| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 8th Meeting: Geneva, Switzerland, 23-27 May, 2003 | Document: JVT-H025 Filename: JVT-H025.doc |
| --- | --- |

| Title: | Comments on JVT-G050 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shinya Kadono and Marin Schlockermann 1006 Kadoma, Kadoma City, Osaka 571-8501, Japan | Tel: Email: | +81-6-6900-9689 kadono.shinya@jp.panasonic.com schlockermann@panasonic.de |
| Source: | Matsushita Electric Industrial Co., Ltd. |  |  |

_____________________________

# Item 1

Subclause: 7.4.1.2.3 Order of NAL units and coded pictures and association to access units

Problem: There is no constrains about sequence parameter set in an access unit. For random accessibility, seq_parameter_set_ID of a sequence parameter set has to be the same as that of slice NAL units. And parameter sets shall precede the primary coded picture.

Solution:

Add such constrains to ease random accessibility.

Proposed text:

- If sequence parameter set NAL unit or picture parameter set NAL unit are present in the access unit, they shall precede the primary coded picture.

- If sequence parameter set NAL unit is present in the access unit, seq_parameter_set_ID of the sequence parameter set shall be the same as that of the primary coded picture.

- If picture parameter set NAL unit is present in the access unit, pic_parameter_set_ID of the picture parameter set shall be the same as that of the primary coded picture.

JVT: Not agreed.  An encoder choice. Done.

# Item2

Subclause: 3.56 Definition of IDR and D.2.7 Recovery point SEI message semantics

Problem: JVT standard allows to code very flexibly. For example, as shown in the following figure, P3a, P4a, P5a cannot be decoded correctly when a decoder begins to decode from IDRb. It hurts of random accessibility using IDR access unit. However it is not forbidden by the current document.

Solution:

Cleary prohibit these picture order between the sequences.

Proposed text:

Output time of any coded picture in the sequence shall be precede the output time of the subsequent sequence.

JVT: Agreed to clarify. Done.

# Item 3

Subclause: D.2.8 Decoded reference picture marking repetition SEI message semantics

Problem: original_frame_num indicate the frame_num of the picture where the repeated decoded reference picture marking syntax structure originally occurred. However, the picture originally occurred is unclear if there are more than one frame_num in the stream. Further more, it is meaningless to send this SEI if the coded picture originally occurred preceeds the last IDR access unit.

Solution:

Original_frame_num shall indicate the last coded picture from the last coded IDR access unit.

Proposed text:

original_frame_num shall be equal to the frame_num of the picture where the repeated decoded reference picture marking syntax structure originally occurred. The picture indicated by original_frame_num is the last coded picture from IDR access unit if there are more than one coded pictures in the sequence.

JVT: Agree this needs to be clear. Should say it's the most recent in decoding order with the specified value of frame_num.  Revisit wording. Language used for "most recent" is "the previous". Done.

# Item 4

Subclause: C.4.5 Current decoded picture marking and storage

Problem: This section describes conformance behaviour of DPB storage management. The word “output” does not mean output to a display device as its timing is irrelevant to output time stamp. So, describing about picture cropping operation in this section is not appropriate.

Solution:

Remove the sentence about picture cropping operation from this sub section.

Proposed text:

- C.4.5.3 Storage and marking of a non-reference decoded picture into the DPB

–	If the current decoded picture is a non-reference picture that is not the second field of a complementary non-reference field pair, the following operations are performed:

–	When there is no empty frame buffer (i.e., DPB fullness is equal to DPB size), the following applies

–	If the current picture does not have the lowest value of PicOrderCnt( ) among all pictures in the DPB, a frame buffer is emptied by the "bumping" process described below.

–	Otherwise (the current picture has the lowest value of PicOrderCnt( ) among all pictures in the DPB), the current picture is output

–	When the current decoded picture has not been output, it is stored in an empty frame buffer and the DPB fullness is incremented by one.

–	Otherwise (the current decoded picture is a second field (in decoding order) of a complementary non-reference field pair), it is stored in the same frame buffer as the first decoded field.

- C.4.5.4 "Bumping" process

The "bumping" process operates when an empty frame buffer is needed for a decoded (non IDR) picture, as in the following steps:

- The field marked as "needed for output" that has a lowest value of PicOrderCnt( ) of all fields or frames in the DPB marked as "needed for output", is output, and the field is marked as "not needed for output".

- The frame buffer that included the field or frame output in step 1 is checked, and if one of the following conditions is satisfied, the frame buffer is emptied, DPB fullness is decremented and the bumping operation is terminated. Otherwise, steps 1 and 2 are repeated until termination.

–	The frame buffer includes a non-reference non-paired field

–	The frame buffer includes a non-reference frame with both fields marked as "not needed for output"

–	The frame buffer includes a complementary non-reference field pair with both fields marked as "not needed for output".

–	The frame buffer includes a non-paired reference field marked as "unused for reference" and "not needed for output".

–	The frame buffer includes a reference frame with both fields marked as "unused for reference" and "not needed for output".

–	The frame buffer includes a complementary reference field pair with both fields marked as "unused for reference" and "not needed for output".

JVT: Cropping is intended to normatively take place for all output.  Not agreed to change. Done.