| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 8th Meeting: Geneva, Switzerland, 23-27 May, 2003 | Document: JVT-H031r2 Filename: JVT-H031r2.doc |
| --- | --- |

| Title: | Transform, Scaling & Color Space Impact of Professional Extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Henrique Malvar and Gary Sullivan One Microsoft Way Redmond, WA 98052 USA | Tel: Email: | +1 425 882 8080 malvar@microsoft.com garys@microsoft.com |
| Source: | Microsoft Corp. |  |  |

_____________________________

# 0.0	Introduction

In addition to the basic concepts previously presented in JVT-C137 and JVT-B039 for 4:2:2, 4:4:4, and increased sample bit depth, we provide some additional remarks herein regarding the following:

- The impact of adjusting for 10 and 12 bit sample bit depth without requiring a change of the transform.

- A discussion of color spaces with respect to the usefulness of compressing video in the RGB domain, and an introduction to the YCoCg color transformation.

Equation number references are relative to JVT-G050.

# 1.0	Support for 10 & 12 bit Samples in Residual 4x4

For the luma or chroma residuals inverse 4x4 transform, any valid wij value in the coefficient matrix in Eqn. (8-268) has a dynamic range of ~15.54 bits. So it fits just right in a 16-bit code word.

Thus, one way to ensure that intermediate values fit in 16-bit arithmetic for increased bit-depth input is to reduce the final scaling factor after inverse quantization and transform. That leads us to:

Proposed modification: in JVT-G050, Eqn. (8-278) should be changed from

to

where NBP is the number of bits per pixel in the input data, i.e. NBP = 8, 10, or 12. That way, the dynamic range of the wij values remains at ~15.54 bits and the dynamic range of the xij values remains at 15 bits. Thus, all the row and column inverse transform operations, as well as Eqn. (8-278) can be computed in 16-bit arithmetic.

We recall that the wij values are produced by Eqn. (8-267):

Thus, as the dynamic range of input pixel data increases from NBP = 8 to NBP = 10 or 12, the dynamic range of the cij values should remain the same. That requires a corresponding modification in the quantization equations at the encoder, but no other decoder changes are necessary.

This modification is in the spirit of the JVT-B039 design.  Note that what is achieved here is a change of bit depth, but not a change of the quantization fidelity range relative to the location of the MSB and therefore not a change of the achievable range of PSNR fidelity.  The current range of quantization step sizes in the H.264/AVC design contains some degree of "overkill" for 8-bit samples (no other standard supports quantization this fine, for example).  So for 10-bit samples, the range of quantization step sizes is probably still quite sufficient.

For 12-bit samples, a greater range of step sizes may be desirable.  There are two straightforward ways we see to achieve that.

- One is to extend the QP range down by 6 (one factor of two in step size) and, for the lowest 6 values, divide the values in the V table of Equation 8-255 by 2 rather than right-shifting by three positions  in Equation 8-258.

- The other is to allow an increased in decoding word length beyond 16 bits.  If this is done, we should scale up the V table of Equation 8-255 and the shifting factors to maximize use of the full larger range for best accuracy.  As before when we used more than 15.5 bits of the available 16-bit range, we should use the maximal amount of the new increased range (e.g., 24 bits)

# 2.0	Support for 4:2:2 and 4:4:4 in Chroma DC

In these cases, the chroma DC blocks will change in dimension from 2x2 to 2x4 and 4x4. In that case, we need to change the 2x2 transforms to 2x4 and 4x4 transforms. Since we already know from luma processing that the DC transforms can be Hadamard transforms (because there’s no significant coding gain advantage of using DCT-like transform), we propose that the chroma DC transform should be:

- 4:2:0 chroma: length-2 Hadamard transform for both rows and columns (that’s what’s currently in the standard).

- 4:2:2 chroma: length-2 Hadamard transform for columns and the length-4 Hadamard transform for rows. The length-4 Hadamard transform should be the same as that used for luma, as specified in Eqn. (8-256) of document JVT-G050.

- 4:4:4 chroma: length-4 Hadamard transform for both rows and columns.

The change of the chroma DC transforms from length-2 to length-4 Hadamard doesn’t lead to an increase in dynamic range, so the scaling equations (8-260) and (8-261) will not need to be changed.

# 3.0	YCoCg and the Value of RGB Coding in 4:4:4 Operation

Support for coding in the RGB color space has been discussed.  However, the RGB color space is neither a perceptually-optimized color space, nor does it take advantage of the statistical properties of the source video content.

In fact, our experiments have shown that when the color transformation matrix is considered as a decorrelation transform (like a DCT, for example), the RGB space is not the best choice.

Even if the goal is to minimize MSE in the RGB domain, rather than minimizing perceptual loss, transformation away from the RGB color space provides a decorrelation benefit that increases coding efficiency. Coding in the appropriate color space can increase the PSNR in the RGB domain by more than 3 dB.

For the encoding of RGB data, we propose the use of the YCoCg (luminance + offset orange + offset green) color space, instead of the YCbCr color space found in the current H.264/AVC design.  YCoCg has several beneficial properties.

Direct YCoCg color space transform:

Inverse YCoCg color space transform:

Thus, the encoder needs only additions and shifts to convert to YCoCg, and the decoder needs just 4 additions per pixel to convert back to RGB, using:

G = Y + Cg;  tmp = Y – Cg;  R = tmp + Co;  B = tmp – Co.

## 3.1 Coding Gain

Besides being the simplest possible color space transform to implement, in terms of decoder operations, the YCoCg space is justified by its better compression performance.

Assuming that the original data was in RGB domain, let us consider the PSNR for the reconstructed RGB values. As it is well known in coding theory, we can get an increase is PSNR by transforming each RGB triplet to an appropriate domain, and coding in that domain. The coding gain is the high-rate increase in PSNR by encoding in that domain. It is well known that the optimal transform, in terms of maximizing the coding gain, is the Karhunen-Loève transform (KLT), whose basis vectors are the eigenvectors of the autocorrelation matrix for the RGB components.

Let us consider a reasonably diverse data set of images, e.g. the widely used “Kodak set” of 24 images of size 512x768, captured with a high-quality 3-CCD camera (no Bayer mosaic artifacts). The normalized RGB autocorrelation matrix for that set is

Given that autocorrelation matrix, it is easy to compute the coding gain. In Table 1 we compare the coding gain of YCoCg to that of the several YCbCr transforms specified in H.264/AVC Annex E, and we see that the YCoCg leads to a 0.7 dB improvement in coding gain when compared to those transforms.

| Color Transform | Coding gain, dB |
| --- | --- |
| Optimal (KLT) | 4.54 |
| ITU-R BT.709 [1] | 3.44 |
| FCC [4] | 3.53 |
| ITU-R BT.470-2 [5] | 3.54 |
| SMPTE 170M [6] | 3.54 |
| SMPTE 240M [7] | 3.48 |
| Proposed YCoCg | 4.21 |

Table 1: Theoretical coding gain for several color space transforms, for the 24 RGB images in the Kodak test set. KLT is the statistically-optimal Karhunen-Loève transform. The numbers in square brackets are the values of the parameter matrix_coefficients, in Table E.5 of H.264/AVC.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Microsoft Corp. |  |  |
| Mailing address | One Microsoft Way Redmond, WA 98052 USA |  |  |
| Country | USA |  |  |
| Contact person | Gary Sullivan or Dawna Hoerle |  |  |
| Telephone | +1 425 882 8080 |  |  |
| Fax | +1 425 706 7329 |  |  |
| Email | garys@microsoft.com |  |  |
| Place and date of submission | Geneva JVT Meeting 23-27 May 2003 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | H.264/AVC |  |
| Title |  | Transform & Scaling Impact of Professional Extensions |  |
| Contribution number |  | JVT-H031 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
| X | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: