| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 8th Meeting: Geneva, Switzerland, 23-27 May, 2003 | Document: JVT-H032 Filename: JVT-H032.doc |
| --- | --- |

| Title: | Extension for 4:2:2 and 4:4:4 YUV coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Response to CfP, Proposal |  |  |
| Author(s) or Contact(s): | Gisle Bjøntegaard Tandberg Philip Pedersens v. 22 1366 Lysaker, Norway | Tel: Email: | +47 98 28 98 79 gbj@tandberg.no |
| Source: | TANDBERG |  |  |

_____________________________

# Introduction

This document is a response to the CfP issued by document JVT-G048.  It deals with extensions to YUV formats 4:2:2 and 4:4:4.  The aim has been to keep the changes to the present standard to a minimum.  As this standard is developed for YUV coding formats, no attempt has been made to code other formats directly.  However, it is believed that all threecomponent formats can easily be converted to and from YUV.

# Different chroma resolutions

The difference between videoformats 4:2:0,  4:2:2 and 4:4:4 is chroma resolution.  In the H.264 coding standard the frame/field is divided into macroblocks of 16x16 luma samples.  Due to lower resolution for chroma for the 4:2:0 and 4:2:2 formats, the number of chroma pixels in a macroblock is smaller.  Below is an indication of the number of pixels for one chroma component in a macroblock.  A partition into subblocks with 4x4 pixels is also shown for the different formats.

4:2:0                                       4:2:2                                        4:4:4

x   x   x   x   x   x   x   x     x   x   x   x   x   x   x   x     x x x x x x x x x x x x x x x x

x   x   x   x   x   x   x   x     x x x x x x x x x x x x x x x x

x   x   x   x   x   x   x   x     x   x   x   x   x   x   x   x     x x x x x x x x x x x x x x x x

x   x   x   x   x   x   x   x     x x x x x x x x x x x x x x x x

x   x   x   x   x   x   x   x     x   x   x   x   x   x   x   x     x x x x x x x x x x x x x x x x

x   x   x   x   x   x   x   x     x x x x x x x x x x x x x x x x

x   x   x   x   x   x   x   x     x   x   x   x   x   x   x   x     x x x x x x x x x x x x x x x x

x   x   x   x   x   x   x   x     x x x x x x x x x x x x x x x x

x   x   x   x   x   x   x   x     x   x   x   x   x   x   x   x     x x x x x x x x x x x x x x x x

x   x   x   x   x   x   x   x     x x x x x x x x x x x x x x x x

x   x   x   x   x   x   x   x     x   x   x   x   x   x   x   x     x x x x x x x x x x x x x x x x

x   x   x   x   x   x   x   x     x x x x x x x x x x x x x x x x

x   x   x   x   x   x   x   x     x   x   x   x   x   x   x   x     x x x x x x x x x x x x x x x x

x   x   x   x   x   x   x   x     x x x x x x x x x x x x x x x x

x   x   x   x   x   x   x   x     x   x   x   x   x   x   x   x     x x x x x x x x x x x x x x x x

x   x   x   x   x   x   x   x     x x x x x x x x x x x x x x x x

# Modifications due to 4:2:2 and 4:4:4 formats

Below the different modifications are described briefly.

## Prediction

### Intra

Proper action must be taken to produce DC and plane mode.  For 4:4:4 the calculations will be the same as for the appropriate modes for luma 16x16 intra.  For 4:2:2:

DC:	(2(sum of pixels above) + (sum of pixels to the left))//32

Plane:	Take 8x16 block size into account

### Inter

4:4:4	Motion vectors are used directly.  Bilinear interpolation is based on

¼ pel motion resolution

4:2:2	Vertical motion vectors are used directly.  Horizontal vectors are scaled

by 2 as for 4:2:0.  Bilinear interpolation is based on 1/8 pel horizontal

and ¼ pel vertical motion resolution

## Transformation of chroma data

Transformation of an 8x8 block in 4:2:0 is done in two steps:

- 4x4 transforsform of the subblocks shown above

- 2x2 transform of the DC values.

### 4:2:2

A similar approach is proposed:

- 4x4 transform of the subblocks shown above

- 2x4 transform (Hadamard) of the DC values.

Proposed basis vectors:

¼ ¼          ¼ -¼

¼ ¼          ¼ -¼

¼ ¼          ¼ -¼

¼ ¼          ¼ -¼      etc.

Division with 4 is chosen instead of 22.  The reason is the low frequency characteristics of chroma.  Dividing by 4 will result in chroma coefficient values more consistent with those used in 4:2:0

### 4:4:4

A similar approach is again proposed:

- 4x4 transforsform of the subblocks shown above

- 4x4 transform (Hadamard)of the DC values.

Proposed basis vectors:

1/8  1/8  1/8  1/8    1/8  1/8 -1/8 -1/8

1/8  1/8  1/8  1/8    1/8  1/8 -1/8 -1/8

1/8  1/8  1/8  1/8    1/8  1/8 -1/8 -1/8

1/8  1/8  1/8  1/8    1/8  1/8 -1/8 -1/8  etc

Division with 8 is chosen instead of 4 is used for the same reason as mentioned above.

## VLC entropy coding

Only VLC entropy coding is covered here.  A similar extension is assumed to be obtainable with CABAC.

The only changes needed are for coding of chroma DC coefficients.  In addition the number of AC blocks will be different, but coding of each AC 4x4 block is unchanged.

### 4:2:2

- The table total_ceff()/trailing_ones():Num-VLC_Chroma_DC must be changed reflecting that there may be 0 to 8 coefficients.  Proposed code table:

Trailing_ones	0	1	2	3

Total_coeff

0	1	-	-	-

1	0001111	01	-	-

2	0001110	0001101	001	-

3	000000111	0001100	0001011	00001

4	000000110	000000101	0001010	000001

5	0000000111	0000000110	000000100	0001001

6	00000000111	00000000110	0000000101	0001000

7	000000000111	000000000110	00000000101	0000000100

8	0000000000111	000000000101	000000000100	00000000100

- A scanning order for the 2x4 transform must be defined as for instance:

0	2

1	5

3	6

4	7

- The table for total_zeroes_chroma_DC must be changed reflecting that there may be 0 to 7 zeroes.  Proposed code table:

NumCoeff	1	2	3	4	5	6	7

Total zeroes

0	1	000	000	110	00	00	0

1	010	01	001	00	01	01	1

2	011	001	01	01	10	1

3	0010	100	10	10	11

4	0011	101	110	111

5	0001	110	111

6	00001	111

7	00000

### 4:4:4

Similar action is taken here.  However, suitable tables already exist for 4x4 blocks.

- Use total_ceff()/trailing_ones():Num-VLC0

- Use normal Zig-zag scanning

- Use the same table for total_zeroes as for luma

# Simulation results

Preliminary tests have been performed with the described approach.  We produced special test material for this initial test.  It was based on the 4:2:2 sequences:

- Mobile

- Canoe

- Rugby

- Formula1

We used every other field and produced 3 different formats:

- “4:2:0”	Luma - 352x288 chroma 176x144  (standard CIF)

- “4:2:2”	Luma - 352x288 chroma 176x288

- “4:4:4”	Luma - 352x288 chroma 352x288

All the sequences have the same luma content.  All sequences are also progressive.  We wanted to compare the bitusage with the different formats.  I used my own software (not test software) and produced a modified version that could take the 4:2:2 and 4:4:4 formats.  In summary:

- The chroma PSNRs were similar or slightly better for 4:2:2 and 4:4:4 (computations relative to uncoded values for each format)

- Increase in bitrate with 4:2:2 and 4:4:4 is shown in the figure below for the 4 sequences and  Qp = 22,26,30,34

# Conclusion

This proposal demonstrates that an extension to YUV 4:2:2 and 4:4:4 is achievable with a simple modifications to the current 4:2:0 design. It is proposed that the extension to these YUV formats shall be based on the described strategy

Regarding other chroma formats, future investigation should be done to find whether conversion to YUV as an intermediate format is feasible.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Tandberg Telecom AS |  |  |
| Mailing address | Philip Pedersens v. 22 1366 Lysaker |  |  |
| Country | Norway |  |  |
| Contact person | Tom-Ivar Johansen |  |  |
| Telephone | +47 67 125 125 |  |  |
| Fax | +47 67 125 234 |  |  |
| Email | tij@tandberg.no |  |  |
| Place and date of submission | Lysaker, Norway, 22. May 2003 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Extension for 4:2:2 and 4:4:4 YUV coding |  |
| Contribution number |  | JVT-H032 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: