| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 8th Meeting: Geneva, Switzerland, 23-27 May, 2003 | Document: JVT-H037r1 Filename: JVT-H037r1.doc |
| --- | --- |

| Title: | Draft Prof. Ext Amendment |  |  |
| --- | --- | --- | --- |
| Status: | Approved Output Document from JVT |  |  |
| Purpose: | Draft Text |  |  |
| Author(s) or Contact(s): | Tom McMahon Dolby Laboratories, Inc. 3601 W. Alameda Ave. Burbank, CA 91505-5300 Thomas Wiegand Heinrich Hertz Institute (FhG), Einsteinufer 37, D-10587 Berlin, Germany Gary Sullivan Microsoft Corporation One Microsoft Way Redmond, WA 98052 USA | Tel: Email: Tel: Fax: Email: Tel: Fax: Email: | +1 (818) 823-2800 tom@dolby.com +49 - 30 - 31002 617 +49 - 30 - 392 72 00 wiegand@hhi.de +1 (425) 703-5308 +1 (425) 706-7329 garysull@microsoft.com |
| Source: | JVT |  |  |

_____________________________

# Annex G
Sample Bit Depth and Chroma Format Extensions

# G.1	Introduction

- This document is intended to serve as the basis for a new Professional Extensions Annex to ITU-T Rec. H.264 & ISO/IEC 14496-10.  This is not a proposal to change the base text of the Recommendation | International Standard.

- The Recommendation | International Standard [1] has been examined for areas where extended sample bit depth or chroma format support may require new or substitute text (documented in the form of an Annex) in order to implement an extended decoder.  The goal has been to keep changes to the present standard to a minimum.

- Please also reference companion output document, “Report – Prof. Ext. Issues of Investigation“ JVT-H038, that lists items and issues still under discussion.

- Subclause G.2 of this document identifies subclause modifications common to both bit depth and chroma format extensions.

- Subclause G.3 of this document identifies changes specific to increased sample bit depth extensions.

- Subclause G.4 identifies changes specific to chroma format extensions.

- Subclause G.5 is the updated Amendment schedule.

# G.2	Changes common to both Sample Bit Depth and Chroma Format

## G.2.1	Subclause: A.2 “Profiles”

- [Ed: This section has been moved to the Discussion document JVT-H0xx.]

## G.2.2	Subclause: 7.3.2.1 “Sequence parameter set RBSP syntax”

Conditionally based on profile_idc equal to Professional profile, add fixed length fields in the sequence parameter set for bit_depth_idc and chroma_format_idc, following level_idc.

Fixed length fields early in the sequence parameter set, prior to any variable length coded fields, allows easy detection of supported formats.  bit_depth_idc and chroma_format_idc  each require 3 bits minimum.  However, allocating 4 bits each, thereby adding one total byte, may provide benefits.

Add “constraint_set3_flag” to Sequence Parameter Set, using a bit from reserved_zero_5bits.

constraint_set3_flag equal to 1 indicates that the bitstream obeys all constraints specified in subclause <insert new Profile definition subclause reference>. constraint_set3_flag equal to 0 indicates that the bitstream may or may not obey all constraints specified in subclause <insert new Profile definition subclause reference>.

[Ed: Need to examine the need and use of these constraint bits for PExt backward compatibility.]

| seq_parameter_set_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| profile_idc | 0 | u(8) |
| constraint_set0_flag | 0 | u(1) |
| constraint_set1_flag | 0 | u(1) |
| constraint_set2_flag | 0 | u(1) |
| if (profile_idc != ***) { |  |  |
| reserved_zero_5bits /* equal to 0 */ | 0 | u(5) |
| } else { |  |  |
| constraint_set3_flag | 0 | u(1) |
| reserved_zero_4bits /* equal to 0 */ | 0 | u(4) |
| } |  |  |
| level_idc | 0 | u(8) |
| seq_parameter_set_id | 0 | ue(v) |
| if( profile_idc == *** ) { |  |  |
| chroma_format_idc | 0 | u(3) |
| bit_depth_idc | 0 | u(3) |
| } |  |  |
| log2_max_frame_num_minus4 | 0 | ue(v) |
| pic_order_cnt_type | 0 | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |  |
| log2_max_pic_order_cnt_lsb_minus4 | 0 | ue(v) |
| else if( pic_order_cnt_type = = 1 ) { |  |  |
| delta_pic_order_always_zero_flag | 0 | u(1) |
| offset_for_non_ref_pic | 0 | se(v) |
| offset_for_top_to_bottom_field | 0 | se(v) |
| num_ref_frames_in_pic_order_cnt_cycle | 0 | ue(v) |
| for( i = 0; i < num_ref_frames_in_pic_order_cnt_cycle; i++ ) |  |  |
| offset_for_ref_frame[ i ] | 0 | se(v) |
| } |  |  |
| num_ref_frames | 0 | ue(v) |
| gaps_in_frame_num_value_allowed_flag | 0 | u(1) |
| pic_width_in_mbs_minus1 | 0 | ue(v) |
| pic_height_in_map_units_minus1 | 0 | ue(v) |
| frame_mbs_only_flag | 0 | u(1) |
| if( !frame_mbs_only_flag ) |  |  |
| mb_adaptive_frame_field_flag | 0 | u(1) |
| direct_8x8_inference_flag | 0 | u(1) |
| frame_cropping_flag | 0 | u(1) |
| if( frame_cropping_flag ) { |  |  |
| frame_crop_left_offset | 0 | ue(v) |
| frame_crop_right_offset | 0 | ue(v) |
| frame_crop_top_offset | 0 | ue(v) |
| frame_crop_bottom_offset | 0 | ue(v) |
| } |  |  |
| vui_parameters_present_flag | 0 | u(1) |
| if( vui_parameters_present_flag ) |  |  |
| vui_parameters( ) | 0 |  |
| rbsp_trailing_bits( ) | 0 |  |
| } |  |  |

## G.2.3	Subclause: 6.2 “Source, decoded, and output picture formats”

Substitute the following for section 6.2:

6.2 Source, decoded, and output picture formats

This subclause specifies the relationship between source and decoded frames and fields that is given via the bitstream.

The video source that is represented by the bitstream is a sequence of either or both frames or fields (called collectively pictures) in decoding order.

The source and decoded pictures (frames or fields) are each comprised of one or more sample arrays:

- Luma only (monochrome)

- Luma and two Chroma (YCbCr) [Ed: with optional Alpha TBD]

- Red, Green and Blue [Ed: with optional Alpha TBD]

- X, Y, Z [Ed: with optional Alpha TBD]

[Ed. Specifics of RGB and XYZ handling are TBD – see JVT-H038.]

The variable of BitDepth specified in Table 6-1 is the number of bits per sample in each of the sample arrays.  If not otherwise specified the value of BitDepth shall be inferred equal to 8.

Table 6-1 – BitDepth values

| bit_depth_idc | BitDepth |
| --- | --- |
| 0 | 8 |
| 1 | 10 |
| 2 | 12 |

[Ed: No mechanism to specify different bit depths for luma and chroma (or alpha).   Draft incomplete in this regard.]

The variable ChromaFormatFactor is specified in Table 6-2, depending on the chroma format sampling structure.  If not otherwise specified the value of ChromaFormatFactor shall be inferred equal to 1.5, indicating 4:2:0 sampling.

Table 6-2 – ChromaFormatFactor values

| chroma_format_idc | Chroma Format | ChromaFormatFactor |
| --- | --- | --- |
| 0 | monochrome | 1 |
| 1 | 4:2:0 | 1.5 |
| 2 | 4:2:2 | 2 |
| 3 | 4:4:4 | 3 |
| 4 | 4:2:2:4 | 3 [Ed: Alpha TBD] |
| 5 | 4:4:4:4 | 4 [Ed: Alpha TBD] |
| 6,7 | Undefined | Undefined |

In monochrome sampling there is only one sample array, which may nominally be considered a luma array.

In 4:2:0 sampling, each of the two chroma arrays has half the height and half the width of the luma array.

In 4:2:2 sampling, each of the two chroma arrays has the same height and half the width of the luma array.  In 4:2:2:4 YCbCrA sampling, each of the two chroma arrays has the same height and half the width of the luma array, and the A array is the same height and width as the luma array.

In 4:4:4 YCbCr sampling, each of the two chroma arrays has the same height and width as the luma array. In 4:4:4 RGB or XYZ sampling, the G or Y arrays may nominally be considered luma arrays and all three arrays have the same height and width. In 4:4:4:4 RGBA or XYZA sampling, the G or Y arrays may nominally be considered the luma array, and all four arrays have the same height and width.

The width and height of the luma, red, green, blue, alpha [Ed: Alpha TBD], X, Y or Z sample arrays are each a multiple of 16.  In bitstreams using 4:2:0 chroma sampling, the width and height of chroma sample arrays are each an integer multiple of 8. In bitstreams using 4:2:2 sampling, the with of the chroma sample arrays is an integer multiple of 8 and the height is an integer multiple of 16.  The height of a luma array that is coded as two separate fields or in macroblock-adaptive frame-field coding (see below) is an integer multiple of 32 samples. In bitstreams using 4:2:0 chroma sampling, the height of each chroma array that is coded as two separate fields or in macroblock-adaptive frame-field coding (see below) is an integer multiple of 16 samples. The width or height of pictures output from the decoding process need not be an integer multiple of 16 and can be specified using a cropping rectangle.

The width of fields coded referring to a specific sequence parameter set is the same as that of frames coded referring to the same sequence parameter set (see below). The height of fields coded referring to a specific sequence parameter set is half that of frames coded referring to the same sequence parameter set (see below).

In bitstreams of chroma_format_idc value equal to one, the nominal vertical and horizontal relative locations of luma and chroma samples in frames are shown in Figure 6-1.  Alternative chroma sample relative locations may be indicated in video usability information (see Annex E).

Figure 6-1 – Nominal vertical and horizontal locations of 4:2:0 luma and chroma samples in a frame

A frame consists of two fields as described below.  A coded picture may represent a coded frame or an individual coded field.  A coded video sequence conforming to this Recommendation | International Standard may contain arbitrary combinations of coded frames and coded fields.  The decoding process is also specified in a manner that allows smaller regions of a coded frame to be coded either as a frame or field region, by use of macroblock-adaptive frame-field coding.

Source and decoded fields are one of two types: top field or bottom field. When two fields are output at the same time, or are combined to be used as a reference frame (see below), the two fields (which shall be of opposite parity) are interleaved. The first (i.e., top), third, fifth, etc. rows of a decoded frame are the top field rows. The second, fourth, sixth, etc. rows of a decoded frame are the bottom field rows. A top field consists of only the top field rows of a decoded frame. When the top field or bottom field of a decoded frame is used as a reference field (see below) only the even rows (for a top field) or the odd rows (for a bottom field) of the decoded frame are used.

In bitstreams of chroma_format_idc idc value equal to one, the nominal vertical and horizontal relative locations of luma and chroma samples in top and bottom fields are shown in Figure 6-2. The nominal vertical sampling relative locations of the chroma samples in a top field are specified as shifted up by one-quarter luma sample height relative to the field-sampling grid. The vertical sampling locations of the chroma samples in a bottom field are specified as shifted down by one-quarter luma sample height relative to the field-sampling grid. Alternative chroma sample relative locations may be indicated in the video usability information (see Annex E).

NOTE – The shifting of the chroma samples is in order for these samples to align vertically to the usual location relative to the full-frame sampling grid as shown in Figure 6-1.

Figure 6-2 – Nominal vertical and horizontal sampling locations of samples top and bottom fields.

In bitstreams of chroma_format_idc value equal to 2 or 4, the luma and chroma samples are co-sited and the nominal locations in a frame and in fields are as shown in figures 6-3 and 6-4 respectively.

Figure 6-3 – Nominal vertical and horizontal locations of 4:2:2 luma and chroma samples in a frame

Figure 6-4 – Nominal vertical and horizontal sampling locations of 4:2:2 samples top and bottom fields.

In bitstreams of chroma_format_idc value equal to 3 or 5, all array samples are co-sited.

[Ed: There may be an implicit assumption above that chroma_format_idc = 3&5 are progressive only.  Also, Do we need to say anything about chroma_format_idc = 0? ]

# G.3	Sample Bit Depth (BD) Extensions

## G.3.1	Subclauses: 5.7 “Mathematical functions”, Equations (8-76, 8-98, 8-187, 8-188, 8-191, 8-192, 8-193, 8-218, 8-219, 8-220, 8-245, 8-247, 8-252, 8-292, 8-299, 8-308, 8-312).

Define new clipping function:

ClipB(x) = Clip3(0, (1<<BitDepth) – 1, x)

Replace Clip1() with ClipB() in equations (8-76, 8-98, 8-187, 8-188, 8-191, 8-192, 8-193, 8-218, 8-219, 8-220, 8-245, 8-247, 8-252, 8-292, 8-299, 8-308, 8-312).

## G.3.2	Subclause: 8.3.1 “Intra_4x4 prediction process for luma samples and 8.3.3 Intra prediction process for chroma samples, equations (8-50, 8-75, 8-85, 8-88, 8-91, and 8-95)”.

Change the right side of the equations from

= 128

to

= 2BitDepth – 1

## G.3.3	Subclause: 8.5.6 QP-Invariant Modifications

Replace equations (8-257), (8-258) and surrounding text with:

- If QPY is greater than or equal to 6*(10-BitDepth), the scaled result shall be derived as

(8-257)

- Otherwise (QPY is less than 6*(10-BitDepth)), the scaled results shall be derived as

(8-258)

## G.3.4	Subclause: 8.5.7 QP-Invariant Modifications

Replace equations (8-260), (8-261) and surrounding text with:

- If QPC is greater than or equal to 6*(9-BitDepth), the scaling result shall be derived as

(8-1)

- Otherwise (QPC is less than 6*(9-BitDepth)), the scaling results shall be derived by

(8-261)

## G.3.5	Subclause: 8.5.8 QP-Invariant Modifications

Replace equation (8-267) with:

## G.3.6	Subclause: Annex E.2.1 “video_full_range_flag”

Change text to read as follows:

video_full_range_flag indicates the black level and range of the luma and chroma signals as derived from E’Y, E’PB, and E’PR or E’R, E’G, and E’B analogue component signals and X = 2(BitDepth-8):

If video_full_range_flag is equal to 0,

Y, R, G, or B = Round( X * 219 * E’YRGB + (X * 16) )

Cb = Round( (X * 224 * E’PB) + (X * 128)  )

Cr = Round( (X * 224 * E’PR) + (X * 128)  )

Otherwise (video_full_range_flag is equal to 1),

Y, R, G, or B = Round(( 2BitDepth -1) * E’YRGB )

Cb = Round( ( 2BitDepth -1)  * E’PB + 2BitDepth-1 )

Cr = Round( (2BitDepth -1)  * E’PR + 2BitDepth-1 )

When the video_full_range_flag syntax element is not present, video_full_range_flag value shall be inferred to be equal to 0.

For BitDepth greater than 10, component values are always full range. (Note potential PCM issue.)

## G.3.7	Subclauses: 7.4.2.2, 7.4.3, 7.4.5 and 8.5.5. Extending QP Range

In subclause 7.4.2.2:

In the description of pic_init_qp_minus26 replace the sentence “The value of pic_init_qp_minus26 shall be in the range of -26 to +25, inclusive” with “The value of pic_init_qp_minus26 shall be in the range of (-26-6*(BitDepth-8)) to +25, inclusive”.

In subclause 7.4.3:

After equation (7-16) replace the phrase “QPY is in the range of 0 to 51, inclusive” with “QPY is in the range of -6*(BitDepth-8) to 51, inclusive”.

In subclause 7.4.5:

Replace equation (7-23) with

QPY = Clip3( -6*(BitDepth-8), 51, QPY,PREV + mb_qp_delta)		(7-2)

In subclause 8.5.5:

Replace the sentence “QP quantisation parameter values QPY, QPC, QSY, and QSC shall be in the range of 0 to 51, inclusive” with “QP quantisation parameter values QPY, QPC, QSY, and QSC shall be in the range of -6*(BitDepth-8) to 51, inclusive”.

## G.3.8	Subclauses: 8.7.2.2 and 8.7.2.3, Loop filter parameters must be scaled to BitDepth

Multiply the values of α and β from Table 8-14 and the value of tC0 from Table 8-15 by 2(BitDepth-8).

## G.3.9	Different Bit depths for Luma & Chroma (e.g. 10 bits luma, 8 bits chroma)

[Ed: approved for initial design but syntax specification is incomplete.]

# G.4	Chroma Format (CF) Extensions

## G.4.1	Subclause: 7.3.2.2 “Picture Parameter Set RBSP Syntax”, other places TBD.

Define three new pic_init_qp_minus26 parameters in addition to the current Luma or Luma equivalent.  Define pic_init_qpc0_minus26 (for second component in the Chroma Format list), pic_init_qpc1_minus26 (for third component), pic_init_qpc2_minus26 (for fourth component).

[Ed: We agreed to change this from pic_inits for each of the additional components to QP offsets for the chroma (or XZ, BR) and optional alpha values.]

Add these optional component QP fields to the Picture Parameter Set:

| if (profile_idc == ??? && chroma_format_idc>2) { |  |  |
| --- | --- | --- |
| pic_init_qpc0_minus26 /* relative to 26 */ | 1 | se(v) |
| pic_init_qpc1_minus26 /* relative to 26 */ | 1 | se(v) |
| pic_init_qpc2_minus26 /* relative to 26 */ | 1 | se(v) |
| } |  |  |
| if(profile_idc == ??? && chroma_format_idc>2) { |  |  |
| pic_init_qpc1_minus26 /* relative to 26 */ | 1 | se(v) |
| } |  |  |

## G.4.2	Prediction and Transform modifications for 4:2:2 and 4:4:4 formats

### G.4.2.1

### Intra 4:4:4

#### G.4.2.1.1	Specification of Intra_Chroma_DC prediction mode

The values of the prediction samples predC[ x, y ] with x = 0..7 and y = 0..7 are derived as follows.

-	If the samples p[ x, –1 ] with x = 0..7 and the samples p[ –1, y ] and y = 0..7 are marked as “available for Intra chroma prediction”,

predC[ x, y ] = (x'=07px', −1+y'=07p−1, y'+8)>>4 ,with x = 0..7 and y = 0..7

Otherwise, if the samples p[ x, –1 ] with x = 0..7 are marked as “available for Intra chroma prediction” and the samples p[ –1, y ] with y = 0..7 are marked as “not available for Intra chroma prediction”,

predC[ x, y ] =(x'=07px', −1+4)>>3, with x = 0..7 and y = 0..7

Otherwise, if the samples p[ x, –1 ] with x = 0..7 are marked as “not available for Intra chroma prediction” and the samples p[ -1, y ] with y = 0..7 are marked as “available for Intra chroma prediction”,

predC[ x, y ] =(y'=07p−1,y'+4)>>3, with x = 0..7 and y = 0..7

Otherwise (the samples p[ x, –1 ] with x = 0..7 and the samples p[ -1, y ] with y = 0..7 are marked as “not available for Intra chroma prediction”),

predC[ x, y ] = 128, with x = 0..7 and y = 0..7

The values of the prediction samples predC[ x, y ] with x = 8..15 and y = 0..7 are derived as follows.

-	If the samples p[ x, -1 ] with x = 8..15 are marked as “available for Intra chroma prediction”,

predC[ x, y ] = (x'=815px', −1+4)>>3, with x = 8..15 and y = 0..7

Otherwise, if the samples p[ -1, y ] with y = 0..7 are marked as “available for Intra chroma prediction”,

predC[ x, y ] = (y'=07p−1,y'+4)>>3, with x = 8..15 and y = 0..7

Otherwise (the samples p[ x, –1 ] with x = 8..15 and the samples p[ -1, y ] with y = 0..7 are marked as “not available for Intra chroma prediction”),

predC[ x, y ] = 128, with x = 8..15 and y = 0..7

The values of the prediction samples predC[ x, y ] with x = 0..7 and y = 8..15 are derived as follows.

If the samples p[ -1, y ] with y = 8..15 are marked as “available for Intra chroma prediction”,

predC[ x, y ] = (y'=815p−1,y'+4)>>3, with x = 0..7 and y = 8..15

-	Otherwise, if the samples p[ x, -1 ] with x = 0..7 are marked as “available for Intra chroma prediction”,

predC[ x, y ] = (x'=07px', −1+4)>>3, with x = 0..7 and y = 8..15

Otherwise (the samples p[ x, –1 ] with x = 0..7 and the samples p[ -1, y ] with y = 8..15 are marked as “not available for Intra chroma prediction”),

predC[ x, y ] = 128, with x = 0..7 and y = 8..15

The values of the prediction samples predC[ x, y ] with x = 8..15 and y = 8..15 are derived as follows.

-	If the samples p[ x, –1 ] with x = 8..15 and the samples p[ –1, y ] and y = 8..15 are marked as “available for Intra chroma prediction”,

predC[ x, y ] = (x'=815px', −1+y'=815p−1, y'+8)>>4, with x = 8..15 and y = 8..15

Otherwise, if the samples p[ x, –1 ] with x = 8..15 are marked as “available for Intra chroma prediction” and the samples p[ –1, y ] with y = 8..15 are marked as “not available for Intra chroma prediction”,

predC[ x, y ] = (x'=815px', −1+4)>>3, with x = 8..15 and y = 8..15

Otherwise, if the samples p[ x, –1 ] with x = 8..15 are marked as “not available for Intra chroma prediction” and the samples p[ -1, y ] with y = 8..15 are marked as “available for Intra chroma prediction”,

predC[ x, y ] = (y'=815p−1,y'+4)>>3, with x = 8..15 and y = 8..15

Otherwise (the samples p[ x, –1 ] with x = 8..15 and the samples p[ -1, y ] with y = 8..15 are marked as “not available for Intra chroma prediction”),

predC[ x, y ] = 128, with x = 8..15 and y = 8..15

#### G.4.2.1.2	Vertical, horizontal and plane prediction

The process for vertical, horizontal and plane modes are equivalent to the definitions for luma 16x16 described in 8.3.2.1, 8.3.2.2, 8.3.2.4

### G.4.2.2	Intra 4:2:2

#### G.4.2.2.1	Specification of Intra_Chroma_DC prediction mode

The values of the prediction samples predC[ x, y ] with x = 0..3 and y = 0..7 are derived as follows.

-	If the samples p[ x, –1 ] with x = 0..3 and the samples p[ –1, y ] and y = 0..7 are marked as “available for Intra chroma prediction”,

predC[ x, y ] = (2x'=03px', −1+y'=07p−1, y'+8)>>4 ,with x = 0..3 and y = 0..7

- Otherwise, if the samples p[ x, –1 ] with x = 0..3 are marked as “available for Intra chroma prediction” and the samples p[ –1, y ] with y = 0..7 are marked as “not available for Intra chroma prediction”,

predC[ x, y ] =(x'=03px', −1+2)>>2, with x = 0..3 and y = 0..7

- Otherwise, if the samples p[ x, –1 ] with x = 0..3 are marked as “not available for Intra chroma prediction” and the samples p[ -1, y ] with y = 0..7 are marked as “available for Intra chroma prediction”,

predC[ x, y ] =(y'=07p−1,y'+4)>>3, with x = 0..3 and y = 0..7

- Otherwise (the samples p[ x, –1 ] with x = 0..3 and the samples p[ -1, y ] with y = 0..7 are marked as “not available for Intra chroma prediction”),

predC[ x, y ] = 128, with x = 0..3 and y = 0..7

The values of the prediction samples predC[ x, y ] with x = 4..7 and y = 0..7 are derived as follows.

-	If the samples p[ x, -1 ] with x = 4..7 are marked as “available for Intra chroma prediction”,

predC[ x, y ] = (x'=47px', −1+2)>>2, with x = 4..7 and y = 0..7

- Otherwise, if the samples p[ -1, y ] with y = 0..7 are marked as “available for Intra chroma prediction”,

predC[ x, y ] = (y'=07p−1,y'+4)>>3, with x = 4..7 and y = 0..7

- Otherwise (the samples p[ x, –1 ] with x = 4..7 and the samples p[ -1, y ] with y = 0..7 are marked as “not available for Intra chroma prediction”),

predC[ x, y ] = 128, with x = 4..7 and y = 0..7

The values of the prediction samples predC[ x, y ] with x = 0..3 and y = 8..15 are derived as follows.

- If the samples p[ -1, y ] with y = 8..15 are marked as “available for Intra chroma prediction”,

predC[ x, y ] = (y'=815p−1,y'+4)>>3, with x = 0..3 and y = 8..15

-	Otherwise, if the samples p[ x, -1 ] with x = 0..3 are marked as “available for Intra chroma prediction”,

predC[ x, y ] = (x'=03px', −1+2)>>2, with x = 0..3 and y = 8..15

- Otherwise (the samples p[ x, –1 ] with x = 0..3 and the samples p[ -1, y ] with y = 8..15 are marked as “not available for Intra chroma prediction”),

predC[ x, y ] = 128, with x = 0..3 and y = 8..15

The values of the prediction samples predC[ x, y ] with x = 4..7 and y = 8..15 are derived as follows.

-	If the samples p[ x, –1 ] with x = 4..7 and the samples p[ –1, y ] and y = 8..15 are marked as “available for Intra chroma prediction”,

predC[ x, y ] = (2x'=47px', −1+y'=815p−1, y'+8)>>4, with x = 4..7 and y = 8..15

- Otherwise, if the samples p[ x, –1 ] with x = 4..7 are marked as “available for Intra chroma prediction” and the samples p[ –1, y ] with y = 8..15 are marked as “not available for Intra chroma prediction”,

predC[ x, y ] = (x'=47px', −1+2)>>2, with x = 4..7 and y = 8..15

- Otherwise, if the samples p[ x, –1 ] with x = 4..7 are marked as “not available for Intra chroma prediction” and the samples p[ -1, y ] with y = 8..15 are marked as “available for Intra chroma prediction”,

predC[ x, y ] = (y'=815p−1,y'+4)>>3, with x = 4..7 and y = 8..15

- Otherwise (the samples p[ x, –1 ] with x = 4..7 and the samples p[ -1, y ] with y = 8..15 are marked as “not available for Intra chroma prediction”),

predC[ x, y ] = 128, with x = 4..7 and y = 8..15

#### G.4.2.2.2	Specification of Intra_Chroma_Horizontal prediction mode

This mode shall be used only when the samples p[ -1, y ] with y = 0..15 are marked as “available for Intra chroma prediction”.

The values of the prediction samples predC[ x, y ] are derived as follows.

predC[ x, y ] = p[ -1, y ], with x = 0..7 and y = 0..15

#### G.4.2.2.3	Specification of Intra_Chroma_Vertical prediction mode

This mode shall be used only when the samples p[ x, -1 ] with x = 0..7 are marked as “available for Intra chroma prediction”.

The values of the prediction samples predC[ x, y ] are derived as follows.

predC[ x, y ] = p[ x, -1 ], with x = 0..7 and y = 0..15

#### G.4.2.2.4	Specification of Intra_Chroma_Plane prediction mode

This mode shall be used only when the samples p[ x, -1 ], with x = 0..7 and p[ -1, y ], with y = -1..15 are marked as “available for Intra chroma prediction”.

The values of the prediction samples predC[ x, y ] are derived as follows.

predC[ x, y ] = Clip1( ( a + b * ( x – 3 ) + c * ( y – 7 ) + 16 ) >> 5 ),  with x = 0..7 and y = 0..15

where:

a = 16 * ( p[ -1, 15 ] + p[ 7, -1 ] )

b = ( 17 * H + 16 ) >> 5

c = ( 5 * V + 32 ) >> 6

and H and V are specified as follows.

H=x'=03(x'+1)∗p4+x',−1−p2−x',−1

V=y'=07(y' +1)∗p−1, 8+y'−p−1,6−y'

### G.4.2.3	Inter Prediction

4:4:4	Same motion vectors are used directly for all components.

Use 6 tap filters for all N components (Alpha also if present).

4:2:2	Vertical motion vectors are used directly.  Horizontal vectors are scaled

by 2 as for 4:2:0.  Use bilinear interpolation based on 1/8 pel horizontal

and ¼ pel vertical motion resolution.

#### G4.2.3.1

#### Scaling and transformation process for chroma DC transform coefficients with 4:2:2

Inputs to this process are transform coefficient level values for chroma DC transform coefficients of one chroma component of the macroblock as a 2x4 array c with elements cij, where i and j form a two-dimensional frequency index.

Outputs of this process are 8 scaled DC values as a 2x4 array dcC with elements dcCij.

The inverse transform for the 2x4 chroma DC transform coefficients is specified by:

f=111111−1−11−1−111−11−1c00c01c10c11c20c20c30c301−11−1

A bitstream conforming to this Recommendation | International Standard shall not contain data that results in any element fij of f that exceeds the range of integer values from –215 to 215–1, inclusive.

After the inverse transform, scaling is performed as follows.

- If QPC is greater than or equal to 12, the scaled result shall be derived as

dcCij=( fij*LevelScale( QPC % 6, 0, 0 ))<<( QPC / 6 −2), with i = 0,3  j = 0,1

- Otherwise (QPC is less than 12), the scaled result shall be derived by

dcCij=( fij*LevelScale( QPC % 6, 0, 0 ))>>( 2-QPC / 6 ), with i = 0,3  j = 0,1

A bitstream conforming to this Recommendation | International Standard shall not contain data that results in any element dcCij of dcC that exceeds the range of integer values from –215 to 215–1, inclusive.

[Note: For 4x2 Hadamard transform the normal scaling would be to divide by 22.  To be more implementation friendly it is assumed that the encoder use a scaling by 2.  This has two consequences concerning quantization:

- The DC quantization can be compensated by a chroma_qp_index_offset of +3

- The dequantization must emulate a scaling of 4 in the inverse transform.  This is reflected by the –2 in the equation above.

The same chroma qp is used for dc and ac coefficients.  By the proposed method ac coefficients will undergo quantization with a qp which is +3 steps higher than for dc.

This discrepancy between quantization of dc and ac can be avoided if we allow different QPC  parameters for dc and ac coefficients.  Compensation with +3 could be included in a QPCdc:

QPCdc = QPC + 3]

#### G.4.2.3.2	Transformation of chroma data for 4:4:4

A similar approach is used:

- 4x4 transforsform of the subblocks shown above

- 4x4 transform (Hadamard)of the DC values.

Basis vectors:

1/8  1/8  1/8  1/8    1/8  1/8 -1/8 -1/8

1/8  1/8  1/8  1/8    1/8  1/8 -1/8 -1/8

1/8  1/8  1/8  1/8    1/8  1/8 -1/8 -1/8

1/8  1/8  1/8  1/8    1/8  1/8 -1/8 -1/8  etc

Division with 8 is chosen instead of 4 is used for the same reason as mentioned above.

## G.4.3	Entropy coding modifications for chroma extensions.

The only changes needed are for coding of chroma DC coefficients.  In addition the number of AC blocks will be different, but coding of each AC 4x4 block is unchanged.

### G.4.3.1	VLC Modification for 4:2:2

The table total_ceff()/trailing_ones():Num-VLC_Chroma_DC must be changed reflecting that there may be 0 to 8 coefficients.  New code table:

Trailing_ones	0	1	2	3

Total_coeff

0	1	-	-	-

1	0001111	01	-	-

2	0001110	0001101	001	-

3	000000111	0001100	0001011	00001

4	000000110	000000101	0001010	000001

5	0000000111	0000000110	000000100	0001001

6	00000000111	00000000110	0000000101	0001000

7	000000000111	000000000110	00000000101	0000000100

8	0000000000111	000000000101	000000000100	00000000100

- A scanning order for the 2x4 transform must be defined and the following is proposed:

0	2

1	5

3	6

4	7

The table for total_zeroes_chroma_DC must be changed reflecting that there may be 0 to 7 zeroes.  New code table:

NumCoeff	1	2	3	4	5	6	7

Total zeroes

0		1	000	000	110	00	00	0

1		010	01	001	00	01	01	1

2		011	001	01	01	10	1

3		0010	100	10	10	11

4		0011	101	110	111

5		0001	110	111

6		00001	111

7		00000

### G.4.3.2	VLC Modification for 4:4:4

Similar action is taken here.  However, suitable tables already exist for 4x4 blocks.

Use total_ceff()/trailing_ones():Num-VLC0

Use normal Zig-zag scanning

Use the same table for total_zeroes as for luma

### G.4.3.3	CABAC Support for 4:2:2 or 4:4:4

Two CABAC related aspects need to be considered when designing an extension of 4:2:0 within H.264/AVC:

#### G.4.3.3.1	If additional bits would be added to the CBP (as suggested in H019), the corresponding binarization and modeling part in CABAC would require a revision. Probably we would not need to specify additional models, but at least a revised context assignment scheme may be required in that case. The latter may even be true, if the CBP would be just re-interpreted. However, in case the interpretation of CodedBlockPatternChroma would be kept the same but with a different (larger) scope, the CABAC coding of CBP could remain unchanged. Of course, in terms of simplicity the last option is the most appealing one.

#### G.4.3.3.2	Since the chroma DC blocks will change in dimension from 2x2 to 2x4 or 4x4, coding of the chroma DC transform coefficients has to be adapted. The flags significant_coeff_flag and last_siginificant_coeff_flag are encoded using a model related to their position in the scanning path. If we want to keep that modeling scheme, 2x4 or 2x8 additional models would be required for each flag. Furthermore, for the coding of the absolute values of chroma DC levels one additional model would be necessary, since the number of decoded levels with abs. value greater than 1 can be larger than 3 in chroma DC blocks with dimension 2x4 or 4x4.

## G.4.4	Chroma-related Deblocking modifications

[Ed: Refine below.]

Deblocking filter process

The deblocking filter process is invoked for the luma and chroma components separately. For each macroblock, vertical edges are filtered first, from left to right, and then horizontal edges are filtered from top to bottom. The luma deblocking filter process is performed on four 16-sample edges, and the deblocking filter process for each chroma components is performed on two 8-sample edges when chroma_format is 1 (4:2:0 format), and on four 16-sample edges when chroma_format is 3 (4:4:4 format), for the horizontal direction as shown on the left side of Error: Reference source not found and for the vertical direction as shown on the right side of Error: Reference source not found [Ed. Note: The Error: Reference source not found should be modified accordingly].

Subclause: 8.7.1 Filtering process for block edges

…

The variable nE is derived as follows.

If chromaEdgeFlag is equal to 0, nE is 16;

If chromaEdgeFlag is equal to 1 and chroma_format is 3, nE is 16;

Otherwise (chromaEdgeFlag is equal to 1 and chroma_format is 1), nE is 8.

## G.5	REFERENCES

- [1] “Final Draft International Standard Joint Video Specification (ITU-T Rec. H.264 | ISO/IEC 14496-10 AVC)” ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6 Document JVT-G050(r1), March/May 2003