| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 8th Meeting: Geneva, Switzerland, 23-27 May, 2003 | Document: JVT-H038r0 Filename: JVT-H038r0.doc |
| --- | --- |

| Title: | Report - Prof. Ext. Issues of Investigation |  |  |
| --- | --- | --- | --- |
| Status: | Non-Final Draft of Approved Output Document from JVT |  |  |
| Purpose: | Prof. Ext. Issues of Investigation |  |  |
| Author(s) or Contact(s): | Tom McMahon Dolby Laboratories, Inc. 3601 W. Alameda Ave. Burbank, CA 91505-5300 Thomas Wiegand Heinrich Hertz Institute (FhG), Einsteinufer 37, D-10587 Berlin, Germany Gary Sullivan Microsoft Corporation One Microsoft Way Redmond, WA 98052 USA | Tel: Email: Tel: Fax: Email: Tel: Fax: Email: | +1 (818) 823-2800 tom@dolby.com +49 - 30 - 31002 617 +49 - 30 - 392 72 00 wiegand@hhi.de +1 (425) 703-5308 +1 (425) 706-7329 garysull@microsoft.com |
| Source: | JVT |  |  |

_____________________________

# Introduction

This document is a compilation of outstanding discussion items from the JVT meetings in Geneva, Switzerland, 23-27 May, 2003 relative to companion output document “Draft Prof. Ext Amendment” JVT-H037.

Section 2 of this document lists general technical issues common to both Sample Bit Depth and Chroma Format

Section 3 of this document pertains to Sample Bit Depth extensions.

Section 4 is specific to Chroma Format extensions.

Section 5 pertains to Amendment process/procedure/schedule issues.

# General technical issues common to both Sample Bit Depth and Chroma Format

## We may end up with more than one new Profile.  For the sake of discussion, listed below is one possible Professional profile example based on the current Main:

Annex A Subclause “A.2.4 Professional profile XYZ”

Bitstreams conforming to the Professional profile shall obey the following constraints:

- Only I, P, and B slice types may be present.

- NAL unit streams shall not contain nal_unit_type values in the range of 2 to 4, inclusive.

- Arbitrary slice order is not allowed.

- Picture parameter sets shall have num_slice_groups_minus1 equal to 0 only.

- Picture parameter sets shall have redundant_pic_cnt_present_flag equal to 0 only.

- The level constraints specified for the Professional profile in subclause A.3.2.4 shall be fulfilled.

Conformance of a bitstream to the Professional profile is specified by profile_idc being equal to ????

Decoders conforming to the Professional profile at a specified level shall be capable of decoding all bitstreams in which profile_idc is equal to ???? or constraint_set3_flag is equal to 1 and in which level_idc represents a level less than or equal to specified level up to and including Level 4.1

Decoders conforming to Professional profile at a specified bit depth shall decode streams of lower BitDepth but are not required to decode streams of higher bit depth.

Decoders conforming to Professional profile supporting any given ChromaFormatFactor (e.g., 4:2:2) shall decode streams of lower chroma format (e.g., 4:2:0) but are not required to decode streams of higher ChromaFormatFactor.

Decoders conforming to Professional profile 4:2:2 or 4:2:2:4 chroma format shall be in YCbCr color space.

Decoders conforming to Professional profile and supporting 4:4:4 chroma format shall support YCbCr, RGB, and XYZ color spaces.

## Profile/Level-Limits

  - Downward-compatibility with Main (conformance points)?

  - Proposal: we need to evaluate compatibility with Base and Extended for 10 bits and 4:x:x as well.

  - Intra-only Profile(s)?

  - 9-element two dimensional matrix for Bit Depth (8/10/12) and Chroma Formats (4:2:0/4:2:2/4:4:4) for conformance points?  A subset of this matrix?  When multiplied times the number of Levels this becomes highly problematic for evaluating possible conformance points.

  - Max bitrate limits?  CPB issues?

  - Max decoded picture buffer size MaxDPB issues?

## Do we need two entropy coders in the Professional profile(s), (CABAC plus CAVLC) or just CABAC or just CAVLC?

  - Bears on Main compatibility discussion.

  - Parallelization of CABAC decoder?  Breaking into slices?

  - A study of hardware implementation complexity (including for CABAC, especially at higher levels) will be forthcoming.

## Do we need “color_space_format” and Color Space table? (Ref: JVT-H018)  Is it implicit in Chroma Format?  Does the decoder need to know RGB versus XYZ?

## Further study of color space proposal in JVT-H031r2.  Test results for Trondheim?

## Table E-1 Aspect Ratios – Verify proposed numbers in JVT-H-019 (values were swapped).  Should these be added now or handled through Extended_SAR?

## Study of film grain is needed.

  - Define testing conditions.

  - Gathering material with suitable film grain characteristics (see process section below).

  - Propose non-normative SEI message syntax (supporting JVT-H022)?

# Extended Sample Bit Depth

## Different sample depths per/component (e.g. 10 bits Luma and Alpha with 8 bits CbCr)?  [Ed: May 27th Plenary – yes this should be in the initial design.]

## Are there requirements for treating Alpha differently as far as the Range Equation?

## Study the need for range equation support with 12 bit content.

# Chroma Format

## JVT-H018 Proposal: In case of 4:4:4 chroma format with RGB components, apply DPCM coding to the residual images of R and B color components using the reconstructed G component residual as a predictor both for the intra and the inter cases. Also: insert chroma_residue_pred_flag in the picture parameter set to signal the DPCM coding of residual images. TBD.

## Loop filter issues

## Motion compensation (bi-linear versus 6 tap filters). [Ed: May 27 Plenary – we will use bi-linear for 4:2:2 and luma method for 4:4:4 in the initial technical design.]

## Should Alpha be supported?  How?

## CBP extensions?  (Adding bits to the coded block pattern flags?)  Luma is currently in the 4 LSBs with Chroma in the next two bits.  May need to extend to 16 bit field.  (4 bits per component for 4:4:4:4.)

## JVT-H019 Proposal: Standardize list order for the various Chroma Format sample arrays.  List the nominal luma-equivalent first and always put Alpha last (if present).  Examples: if chroma_format_idc= 2 or 3 then list member  order is YCbCr, or GBR, or YXZ.  If chroma_format_idc= 4 or 5 then sequence order is YCbCrA, GBRA, YXZA.  TBD.

## Varying QP – offset at the picture level now, however if we have separate QPs per component, should we discuss the ability to change at the macroblock level?

## For Fig. 6.1B, the 4:2:2 sampling position should be defined to be the same as BT.601/SMPTE usage (cosited on the first sample).  [Ed: this may have already been fixed.]

## JVT-C019: Do we really need Table 6-2B?  After further consideration, I don't think that you need Fig. 6.2B since there is no difference between the fields.

## In Table 6.2, do we need to keep a 4:2:2:4 selection?  Depends on Alpha decision.

## Need to check appropriateness of the drafted relation between DC and AC step size for chroma.

# Process & Schedule Issues

## Need 4:4:4 film content  @ 10 (12?) bits and testing conditions/requirements.  Content distribution methodology & medium is potential problem.  Suggest hard drives.

## Need content with an Alpha channel for any 4:x:x:4 testing.

## Evaluation criteria – PSNR? Visual?  Multi-generation testing?

## Equipment required for evaluation (equipment such as monitors, tape machines, projection systems)?

## JM code base schedule.

## Bitstream exchange schedule.