| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) JVT PExt Ad Hoc Group Meeting: 22-24 July 2003, Trondheim | Document: JVT-I014 Filename: JVT-I014.doc |
| --- | --- |

| Title: | YCoCg-R: A Color Space with RGB Reversibility and Low Dynamic Range |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Henrique Malvar and Gary Sullivan One Microsoft Way Redmond, WA 98052 USA | Tel: Email: | +1 425 882 8080 malvar@microsoft.com garys@microsoft.com |
| Source: | Microsoft Corp. |  |  |

_____________________________

# 0.0	Introduction

At the Geneva meeting we presented the YCoCg color space [1], including its simple transformation equations relative to RGB and its improved coding gain relative to both RGB and YCbCr.  We also discussed the reversibility of RGB to YCoCg conversion process in the case that two additional bits of precision are used for YCoCg relative to the precision used for source RGB data.  In this proposal, we additionally present a variation of this color space, which we call YCoCg-R.

YCoCg-R has properties very similar to those of YCoCg, but in addition it enables transformation reversibility with a smaller dynamic range requirement than does YCoCg.  In particular, YCoCg-R produces a Y component with the same dynamic range as the input RGB data, and requires only a one-bit expansion of dynamic range support for the Co and Cg components.

# 1.0	The YCoCg Color Space

As we mentioned in [1], encoding of RGB data directly in RGB space does not lead to the best compression results, because it doesn’t take advantage of the statistical properties of the source video content.

In the same way that the use of transforms improves PSNR in macroblock coding, a transformation applied to RGB can also improve overall PSNR. If we map the original RGB channels into one luma and two chroma channels, the chroma components will require fewer bits, so the overall compression will be better (more than 3dB improvement in PSNR for simple non-predicted intra coding) than encoding directly in the RGB space.

Color spaces such as YCrCb provide good decorrelation, but even better results can be obtained with the YCoCg (luminance + offset orange + offset green).

We recall from [1] that the direct YCoCg color space transform is defined by:

and the inverse YCoCg color space transform is:

Thus, the encoder needs only additions and shifts to convert to YCoCg, and the decoder needs just 4 additions per pixel to convert back to RGB, using:

G = Y + Cg;  tmp = Y – Cg;  R = tmp + Co;  B = tmp – Co.

# 2.0	New Reversible Transform: YCoCg-R

For encoding with the smallest quantization step sizes, or for lossless encoding, it would be useful to employ a reversible transform, that is, one that allows the mapping of a triplet of integer RGB numbers into a triplet of integer YCoCg numbers in such a way that the input RGB integers are exactly recoverable.

We could generate a reversible RGB ↔ YCoCg mapping by multiplying all elements of the direct transform matrix above by four. At the inverse transform, we would divide the final results by four. However, that would generate color components that would need two more bits of precision to represent, and there would also be a loss of coding gain relative to using an equivalent color transform without an increase in bit depth (because the increase in the dynamic range of Y also causes an increase in its entropy).

We can convert the YCoCg transform above to a integer-reversible mapping by applying S-transform concepts [2], [3] (also known as lifting) to factors of the matrices above. For example, given two integer numbers x and y, their integer difference d and truncated average m can be computed by the formulas

where >> is the arithmetic shift operator, which approximates a division by 2. Using this concept, we can convert the RGB ↔ YCoCg mappings to a reversible form by first scaling the Co and Cg components by a factor of two, so the S-transform above can be directly applied.

The final result is:

The corresponding flowgraph is shown in the Figure below:

Figure 1: Flowgraph of the reversible form of the RGB ↔ YCoCb color mapping. Its main advantage is that Y has the same dynamic range as the RGB channels, and Co and Cg require only one additional bit, without loss of coding gain performance.

We can see that the reversible mapping above provides an approximation to the construction

which is identical to the original definition, but with Co and Cg scaled up by a factor of two.

## 2.1 Dynamic Range

It is easy to see from Figure 1 that if each of the RGB channels are integer values with an N-bit range, then the luma channel Y requires N bits, and the chroma channels require N+1 bis. That’s a minimal dynamic range expansion.

# 3.0	Coding Gain

Besides being the simplest possible color space transform to implement, in terms of decoder operations, as we saw above, the YCoCg space is justified by its better compression performance.

Assuming that the original data was in RGB domain, let us consider the PSNR for the reconstructed RGB values. As it is well known in coding theory, we can get an increase is PSNR by transforming each RGB triplet to an appropriate domain, and coding in that domain. The coding gain is the high-rate increase in PSNR by encoding in that domain. It is well known that the optimal transform, in terms of maximizing the coding gain, is the Karhunen-Loève transform (KLT), whose basis vectors are the eigenvectors of the autocorrelation matrix for the RGB components.

Let us consider a reasonably diverse data set of images, e.g. the widely used “Kodak set” of 24 images of size 512x768, captured with a high-quality 3-CCD camera (no Bayer mosaic artifacts). The normalized RGB autocorrelation matrix for that set is

Given that autocorrelation matrix, it is easy to compute the coding gain. In Table 1 we compare the coding gain of YCoCg to that of the several YCbCr transforms specified in H.264/AVC Annex E, and we see that the YCoCg leads to a 0.7 dB improvement in coding gain when compared to those transforms. We have also included the coding gain for the reversible color transform used in JPEG2000.

| Color Transform | Coding gain, dB |
| --- | --- |
| Optimal (KLT) | 4.54 |
| ITU-R BT.709 [1] | 3.44 |
| FCC [4] | 3.53 |
| ITU-R BT.470-2 [5] | 3.54 |
| SMPTE 170M [6] | 3.54 |
| SMPTE 240M [7] | 3.48 |
| JPEG2000 RCT | 3.98 |
| Proposed YCoCg or YCoCg-R | 4.21 |

Table 1: Theoretical coding gain for several color space transforms, for the 24 RGB images in the Kodak test set. KLT is the statistically-optimal Karhunen-Loève transform. The numbers in square brackets are the values of the parameter matrix_coefficients, in Table E.5 of H.264/AVC.

We see from Table 1 that the proposed YCoCg-R color space transformation has three important features:

- Exactly reversible in integer arithmetic.

- Minimal increase in dynamic range; no increase for Y and only 1 bit increase for Co and Cg.

- Higher coding gain than other color spaces, including RCT.

# References

- H. S. Malvar and G. J. Sullivan, “Transform, Scaling & Color Space Impact of Professional Extensions,” ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6 Document JVT-H031, Geneva, May 2003.

- P. Lux, “A novel set of closed orthogonal functions for picture coding,” Arch. Elek. Ubertragung, vol. 31, pp. 267–274, 1977.

- P. Piscaglia and B. Macq, “Multiresolution lossless compression scheme,” Proc. IEEE Int. Conf. Image Processing, Lausanne, Switzerland, Sept. 1996, vol. 1, pp. 69–72.

- The Kodak image set is available at ftp://ftp.ipl.rpi.edu/stills/kodak/color.

- D. S. Taubman and M. W. Marcellin, JPEG2000 Image Compression Fundamentals, Standards, and Practice. Boston, MA: Kluwer, 2000.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Microsoft Corp. |  |  |
| Mailing address | One Microsoft Way Redmond, WA 98052 USA |  |  |
| Country | USA |  |  |
| Contact person | Gary Sullivan or Dawna Hoerle |  |  |
| Telephone | +1 425 882 8080 |  |  |
| Fax | +1 425 706 7329 |  |  |
| Email | garys@microsoft.com |  |  |
| Place and date of submission | Geneva JVT Meeting 23-27 May 2003 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | H.264/AVC |  |
| Title |  | Transform & Scaling Impact of Professional Extensions |  |
| Contribution number |  | JVT-H031 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
| X | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: