| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) AHG Meeting: Trondheim, Norway, 22-24 July, 2003 | Document: JVT-I016 Filename: JVT-I016.doc |
| --- | --- |

| Title: | Extension of Intra Coding for 4:2:2 Video |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Teruhiko Suzuki Gate City Osaki East Tower 20 F 1-11-1 Osaki, Shinagawa-ku Tokyo, 141-0032, Japan | Tel: Email: | +81-3-5435-3669 teruhiko@av.crl.sony.co.jp |
| Source: | Sony Corp. |  |  |

_____________________________

# Introduction

In Geneva, the first working draft for professional extension was produced. However, there are a lot of open issues. In this contribution, the extension of intra coding to support 4:2:2 is studied. In order to improve the coding efficiency for 4:2:2 video, intra prediction for chroma and chroma DC transform should be extended.

# Proposal

## Transform of DC value

In JVT-H037, 2x4 transform for DC value of chroma is describe. In G050r1, 2x2 Hadamard transform is applied to chroma DC matrix. However, energy compaction of 2x2 Hadamard transform is not so high. Larger transform should be applied in case of 4:2:2 and 4:4:4. The simple extension of this transform is to use 4x4 luma transform for vertical basis vector and to use 2x2 chroma DC transform for horizontal basis vector. The forward transform can be described as,

F2x4=Tv(4)×f2x4×Th(2)TT=12111111−1−11−1−111−11−1Th(2)=12111−1

The inverse transform can be described as

F'2x4=Tv(4)×f2x4×Th(2)T122111111−1−11−1−111−11−1f2x4111−1

In order to avoid division by 22, it can be simplified as

F'2x4=Tv(4)×f2x4×Th(2)T122111111−1−11−1−111−11−1f2x4111−114111111−1−11−1−111−11−1f2x4111−1

The scaling factor for chroma DC should be changed from 2 to 4. Therefore, the de-quantization should be changed as follows.

If QPc is greater than or equal to 12, the scaled result shall be derived as

dcCij=(fij×LevelScale(QPC%6,0,0))<<(QPC6−2) (i=0..1, j=0..3)

otherwise (QPc is less than 12), the scaled result shall be derived as

dcCij=(fij×LevelScale(QPC%6,0,0))>>(2−QPC6) (i=0..1, j=0..3)

In case of 4:4:4, the same transform used for luma DC matrix should be used for chroma DC matrix.

F'4x4=T4×f4x4×T4T14111111−1−11−1−111−11−1f4x4111111−1−11−1−111−11−1T4=12111111−1−11−1−111−11−1

The scaling factor is the same as luma. Therefore de-quantization can be described as follows.

If QPc is greater than or equal to 12, the scaled result shall be derived as

dcCij=(fij×LevelScale(QPC%6,0,0))<<(QPC6−2) (i=0..1, j=0..3)

otherwise (QPc is less than 12), the scaled result shall be derived as

dcCij=(fij×LevelScale(QPC%6,0,0))>>(2−QPC6) (i=0..1, j=0..3)

## Chroma Intra Prediction

### DC prediction

In order to apply 2x4 Hadamard transform to DC matrix, the intra prediction should be extended to 8x16. The DC value for lower chroma block is needed before Hadamard transform is applied. In this section, we propose to extend the DC prediction for 4:2:2. In DC prediction, the mean value of the surrounding pixels is used for the prediction. in case of 4:2:2, the number of upper and left neighboring block pixels are 24. It means division by 24 is needed to calculate the DC value. Therefore we propose to modify the DC prediction to avoid the division by 24. There could be several methods.

- Calculate DC value based on the 8x8 block. The DC value is calculated for each 8x8 block using its adjacent pixels.

- Calculate DC value based on the 4x4 block. The current chroma DC prediction uses this method.

- Then, we compare the results with the DC value divided by 24.

In section 3.2.1. the simulation results are presented.

### Plane prediction

In order to introduce 2x4 Hadamard transform to chroma DC matrix, the intra prediction for chroma must be extended to 8x16, likewise. The plane prediction for 8x16 can be defined as follows.

predC[x,y]=Clip1(a+b×(x−3)+c×(y−7)+16)>>5), with x=0..7, y=0..15

a=16×(P(−1,15)+P(7,−1))b=(17×H+16)>>5c=(5×V+32)>>6H=x'=03(x'+1)×(P(4+x',−1)−P(2−x',−1))V=y'=17(y'+1)×(P(−1,8+y')−P(−1,6−y'))

## CAVLC

Since the chroma DC matrix is extended to 2x4, the CAVLC for chroma DC should be extended. The same approach defined in JVT-H037 is used in our experiments.

## CABAC

It may be possible to change the binarization and modeling for CBP as it is pointed out in JVT-H037, however the same definition for 4:2:0 is used for 4:2:2 in our experiments. The same context model for DC coefficients for 4:2:0 is used for 4:2:2.

# Simulation

In order to verify the proposal, the results for intra coding are reported.

## Test Conditions

The following test conditions are used for our experiments. The software to support 4:2:2 is implemented based on JM61e.

Software: JM61e + 4:2:2 extensions

GOP: All intra

Entropy coding: CAVLC and CABAC

RD optimization: On

Picture structure: Field structure is used for interlaced materials and frame structure is used for progressive materials

Bitdepth: 8 bit

Chroma format: 4:2:2

Color space: YUV

Sequence: src2, src5(canoa), src6(F1), src9(rugby), src13, src15(mobile), src19(football), src22(tempete)

Number of frames: 60 frames

The proposed 2x4 DC transform and 8x16 chroma intra prediction is compared with 2x2 DC transform and 8x8 intra prediction.

## Results

### Comparison of DC prediction

Three different DC predictions for 4:2:2 chroma is compared first. Intra chroma prediction mode is fixed to DC mode in this experiment.

| DC prediction | QP | SNR Cb (db) | SNR Cr (db) | Bitrate (kbps) |
| --- | --- | --- | --- | --- |
| 8x16 DC pred with division by 24 | 40 | 29.50 | 28.89 | 9505 |
|  | 36 | 30.90 | 30.39 | 14132 |
|  | 32 | 33.04 | 32.72 | 21533 |
|  | 28 | 35.31 | 35.11 | 30686 |
|  | 24 | 38.51 | 38.40 | 42710 |
|  | 20 | 42.07 | 42.01 | 57308 |
|  | 16 | 45.83 | 45.82 | 73189 |
|  | 12 | 49.37 | 49.36 | 88800 |
| 8x8 DC pred | 40 | 29.51 | 28.89 | 9504 |
|  | 36 | 30.90 | 30.39 | 14130 |
|  | 32 | 33.05 | 32.72 | 21527 |
|  | 28 | 35.31 | 35.11 | 30686 |
|  | 24 | 38.51 | 38.40 | 42699 |
|  | 20 | 42.07 | 42.01 | 57296 |
|  | 16 | 45.83 | 45.82 | 73179 |
|  | 12 | 49.37 | 49.36 | 88785 |
| 4x4 DC pred | 40 | 29.51 | 28.89 | 9573 |
|  | 36 | 30.90 | 30.39 | 14199 |
|  | 32 | 33.05 | 32.72 | 21586 |
|  | 28 | 35.31 | 35.11 | 30748 |
|  | 24 | 38.51 | 38.40 | 42767 |
|  | 20 | 42.07 | 42.01 | 57382 |
|  | 16 | 45.84 | 45.82 | 73297 |
|  | 12 | 49.37 | 49.36 | 88912 |

Table 1: Comparison of intra chroma coding (Src2)

| DC prediction | QP | SNR Cb (db) | SNR Cr (db) | Bitrate (kbps) |
| --- | --- | --- | --- | --- |
| 8x16 DC pred with division by 24 | 40 | 38.12 | 39.09 | 2836 |
|  | 36 | 38.95 | 39.93 | 4271 |
|  | 32 | 40.15 | 41.04 | 6486 |
|  | 28 | 41.38 | 42.17 | 9554 |
|  | 24 | 43,13 | 43.66 | 13866 |
|  | 20 | 45.21 | 45.51 | 20117 |
|  | 16 | 47.68 | 47.77 | 29243 |
|  | 12 | 50.40 | 50.38 | 40912 |
| 8x8 DC pred | 40 | 38.06 | 39.02 | 2835 |
|  | 36 | 38.92 | 39.90 | 4271 |
|  | 32 | 40.14 | 41.04 | 6476 |
|  | 28 | 41.38 | 42.17 | 9540 |
|  | 24 | 43,13 | 43.66 | 13838 |
|  | 20 | 45.21 | 45.51 | 20083 |
|  | 16 | 47.68 | 47.77 | 29196 |
|  | 12 | 50.40 | 50.38 | 40858 |
| 4x4 DC pred | 40 | 37.97 | 38.91 | 2845 |
|  | 36 | 38.85 | 39.81 | 4284 |
|  | 32 | 40.12 | 41.01 | 6496 |
|  | 28 | 41.36 | 42.14 | 9572 |
|  | 24 | 43.12 | 43.64 | 13898 |
|  | 20 | 45.21 | 45.52 | 20160 |
|  | 16 | 47.68 | 47.77 | 29286 |
|  | 12 | 50.40 | 50.38 | 40948 |

Table 2: Comparison of intra chroma coding (Src5)

| DC prediction | QP | SNR Cb (db) | SNR Cr (db) | Bitrate (kbps) |
| --- | --- | --- | --- | --- |
| 8x16 DC pred with division by 24 | 40 | 36.70 | 38.09 | 3080 |
|  | 36 | 37.92 | 39.37 | 4459 |
|  | 32 | 39.69 | 41.01 | 6606 |
|  | 28 | 41.39 | 42.70 | 9473 |
|  | 24 | 43.56 | 44.68 | 13454 |
|  | 20 | 45.88 | 46.68 | 18928 |
|  | 16 | 48.13 | 48.53 | 26643 |
|  | 12 | 50.43 | 50.59 | 37497 |
| 8x8 DC pred | 40 | 36.64 | 38.01 | 3085 |
|  | 36 | 37.88 | 39.33 | 4458 |
|  | 32 | 39.67 | 40.99 | 6604 |
|  | 28 | 41.37 | 42.67 | 9470 |
|  | 24 | 43.57 | 44.65 | 13463 |
|  | 20 | 45.88 | 46.67 | 18924 |
|  | 16 | 48.14 | 48.53 | 26635 |
|  | 12 | 50.43 | 50.59 | 37476 |
| 4x4 DC pred | 40 | 36.56 | 37.91 | 3114 |
|  | 36 | 37.81 | 39.25 | 4485 |
|  | 32 | 39.60 | 40.90 | 6643 |
|  | 28 | 41.32 | 42.61 | 9513 |
|  | 24 | 43.51 | 44.59 | 13526 |
|  | 20 | 45.86 | 46.64 | 19029 |
|  | 16 | 48.12 | 48.51 | 26778 |
|  | 12 | 50.42 | 50.59 | 37654 |

Table 3: Comparison of intra chroma coding (Src6)

| DC prediction | QP | SNR Cb (db) | SNR Cr (db) | Bitrate (kbps) |
| --- | --- | --- | --- | --- |
| 8x16 DC pred with division by 24 | 40 | 36.07 | 37.68 | 3382 |
|  | 36 | 36.94 | 38.59 | 5354 |
|  | 32 | 38.28 | 39.85 | 8532 |
|  | 28 | 39.63 | 41.15 | 12932 |
|  | 24 | 41.58 | 42.81 | 18917 |
|  | 20 | 43.95 | 44.71 | 27195 |
|  | 16 | 46.83 | 47.12 | 38503 |
|  | 12 | 49.89 | 50.06 | 51598 |
| 8x8 DC pred | 40 | 36.05 | 37.63 | 3394 |
|  | 36 | 36.94 | 38.57 | 5361 |
|  | 32 | 38.29 | 39.86 | 8533 |
|  | 28 | 39.63 | 41.15 | 12934 |
|  | 24 | 41.58 | 42.81 | 18912 |
|  | 20 | 43.95 | 44.72 | 27184 |
|  | 16 | 46.82 | 47.12 | 38489 |
|  | 12 | 49.89 | 50.06 | 51575 |
| 4x4 DC pred | 40 | 35.98 | 37.54 | 3421 |
|  | 36 | 36.90 | 38.50 | 5394 |
|  | 32 | 38.27 | 39.82 | 8578 |
|  | 28 | 39.62 | 41.12 | 12999 |
|  | 24 | 41.58 | 42.80 | 19009 |
|  | 20 | 43.96 | 44.72 | 27294 |
|  | 16 | 46.82 | 47.12 | 38617 |
|  | 12 | 49.88 | 50.06 | 51704 |

Table 4: Comparison of intra chroma coding (Src9)

| DC prediction | QP | SNR Cb (db) | SNR Cr (db) | Bitrate (kbps) |
| --- | --- | --- | --- | --- |
| 8x16 DC pred with division by 24 | 40 | 32.93 | 34.69 | 4530 |
|  | 36 | 33.95 | 35.82 | 6595 |
|  | 32 | 35.48 | 37.45 | 9962 |
|  | 28 | 37.04 | 39.06 | 14499 |
|  | 24 | 39.35 | 41.21 | 21321 |
|  | 20 | 42.27 | 43.53 | 31681 |
|  | 16 | 45.87 | 46.27 | 45711 |
|  | 12 | 49.42 | 49.53 | 60984 |
| 8x8 DC pred | 40 | 32.94 | 34.67 | 4536 |
|  | 36 | 33.95 | 35.82 | 6596 |
|  | 32 | 35.48 | 37.45 | 9958 |
|  | 28 | 37.04 | 39.06 | 14493 |
|  | 24 | 39.35 | 41.21 | 21313 |
|  | 20 | 42.27 | 43.53 | 31664 |
|  | 16 | 45.87 | 46.27 | 45692 |
|  | 12 | 49.42 | 49.53 | 60963 |
| 4x4 DC pred | 40 | 32.93 | 34.64 | 4587 |
|  | 36 | 33.96 | 35.80 | 6656 |
|  | 32 | 35.49 | 37.45 | 10023 |
|  | 28 | 37.04 | 39.06 | 14563 |
|  | 24 | 39.35 | 41.21 | 21393 |
|  | 20 | 42.27 | 43.53 | 31756 |
|  | 16 | 45.87 | 46.27 | 45796 |
|  | 12 | 49.42 | 49.53 | 61082 |

Table 5: Comparison of intra chroma coding (Src13)

| DC prediction | QP | SNR Cb (db) | SNR Cr (db) | Bitrate (kbps) |
| --- | --- | --- | --- | --- |
| 8x16 DC pred with division by 24 | 40 | 32.05 | 32.20 | 9864 |
|  | 36 | 33.15 | 33.38 | 14300 |
|  | 32 | 34.93 | 35.22 | 20597 |
|  | 28 | 36.86 | 37.16 | 28186 |
|  | 24 | 39.75 | 39.99 | 37937 |
|  | 20 | 43.06 | 43.26 | 50292 |
|  | 16 | 46.51 | 46.72 | 64556 |
|  | 12 | 49.73 | 49.89 | 79525 |
| 8x8 DC pred | 40 | 32.05 | 32.20 | 9838 |
|  | 36 | 33.15 | 33.37 | 14274 |
|  | 32 | 34.93 | 35.22 | 20567 |
|  | 28 | 36.86 | 37.16 | 28154 |
|  | 24 | 39.75 | 39.99 | 37894 |
|  | 20 | 43.06 | 43.26 | 50240 |
|  | 16 | 46.51 | 46.72 | 64500 |
|  | 12 | 49.73 | 49.89 | 79464 |
| 4x4 DC pred | 40 | 32.04 | 32.18 | 9882 |
|  | 36 | 33.14 | 33.37 | 14325 |
|  | 32 | 34.92 | 35.22 | 20632 |
|  | 28 | 36.86 | 37.15 | 28233 |
|  | 24 | 39.75 | 39.99 | 37980 |
|  | 20 | 43.06 | 43.27 | 50331 |
|  | 16 | 46.51 | 46.72 | 64592 |
|  | 12 | 49.73 | 49.89 | 79568 |

Table 6: Comparison of intra chroma coding (Src15)

| DC prediction | QP | SNR Cb (db) | SNR Cr (db) | Bitrate (kbps) |
| --- | --- | --- | --- | --- |
| 8x16 DC pred with division by 24 | 40 | 35.80 | 36.98 | 2829 |
|  | 36 | 36.79 | 37.78 | 4368 |
|  | 32 | 38.14 | 39.10 | 6976 |
|  | 28 | 39.51 | 40.43 | 10596 |
|  | 24 | 41.26 | 42.22 | 15855 |
|  | 20 | 43.35 | 44.37 | 24219 |
|  | 16 | 46.20 | 46.88 | 36742 |
|  | 12 | 49.54 | 49.77 | 51057 |
| 8x8 DC pred | 40 | 35.79 | 36.95 | 2843 |
|  | 36 | 36.78 | 37.77 | 4381 |
|  | 32 | 38.14 | 39.10 | 6980 |
|  | 28 | 39.51 | 40.43 | 10590 |
|  | 24 | 41.26 | 42.21 | 15847 |
|  | 20 | 43.35 | 44.37 | 24206 |
|  | 16 | 46.20 | 46.88 | 36727 |
|  | 12 | 49.54 | 49.78 | 51033 |
| 4x4 DC pred | 40 | 35.75 | 36.88 | 2877 |
|  | 36 | 36.75 | 37.72 | 4429 |
|  | 32 | 38.13 | 39.08 | 7032 |
|  | 28 | 39.50 | 40.42 | 10642 |
|  | 24 | 41.25 | 42.21 | 15910 |
|  | 20 | 43.35 | 44.37 | 24275 |
|  | 16 | 46.20 | 46.87 | 36802 |
|  | 12 | 49.54 | 49.77 | 51116 |

Table 7: Comparison of intra chroma coding (Src19)

| DC prediction | QP | SNR Cb (db) | SNR Cr (db) | Bitrate (kbps) |
| --- | --- | --- | --- | --- |
| 8x16 DC pred with division by 24 | 40 | 32.69 | 34.43 | 5506 |
|  | 36 | 33.61 | 35.04 | 8424 |
|  | 32 | 35.25 | 36.15 | 13024 |
|  | 28 | 37.04 | 37.56 | 19236 |
|  | 24 | 39.66 | 39.82 | 27984 |
|  | 20 | 42.65 | 42.58 | 40232 |
|  | 16 | 46.03 | 45.95 | 55326 |
|  | 12 | 49.49 | 49.45 | 70401 |
| 8x8 DC pred | 40 | 32.69 | 34.41 | 5489 |
|  | 36 | 33.61 | 35.03 | 8406 |
|  | 32 | 35.25 | 36.15 | 12998 |
|  | 28 | 37.04 | 37.56 | 19204 |
|  | 24 | 39.66 | 39.82 | 27938 |
|  | 20 | 42.65 | 42.58 | 40170 |
|  | 16 | 46.03 | 45.95 | 55254 |
|  | 12 | 49.49 | 49.45 | 70319 |
| 4x4 DC pred | 40 | 32.67 | 34.36 | 5528 |
|  | 36 | 33.60 | 35.00 | 8463 |
|  | 32 | 35.25 | 36.14 | 13083 |
|  | 28 | 37.04 | 37.55 | 19306 |
|  | 24 | 39.66 | 39.82 | 28053 |
|  | 20 | 42.65 | 42.58 | 40281 |
|  | 16 | 46.03 | 45.95 | 55363 |
|  | 12 | 49.48 | 49.45 | 70319 |

Table 8: Comparison of intra chroma coding (Src22)

PSNR for all cases results in almost the same PSNR. It is possible to just compare the generated bits for each case. As the results , 8x8 based DC prediction perform best. Therefore, we propose to use 8x8 DC prediction.

### Comparison of Intra Prediction Block Size

The block size for the intra prediction and the size of DC transform is compared in this experiment. 8x8 block size DC prediction is used in this experiment. The intra prediction mode for chroma is determined by RD optimization.

| DC prediction | QP | SNR Cb (db) | SNR Cr (db) | Bitrate (kbps) |
| --- | --- | --- | --- | --- |
| 8x16 with 2x4 Hadamard | 40 | 29.49 | 28.88 | 9404 |
|  | 36 | 30.88 | 30.39 | 14018 |
|  | 32 | 33.04 | 32.72 | 21389 |
|  | 28 | 35.31 | 35.11 | 30523 |
|  | 24 | 38.51 | 38.40 | 42499 |
|  | 20 | 42.07 | 42.02 | 57066 |
|  | 16 | 45.85 | 45.83 | 72955 |
|  | 12 | 49.39 | 49.38 | 88520 |
| 8x8 with 2x2 Hadamard | 40 | 29.06 | 28.56 | 8897 |
|  | 36 | 30.56 | 30.13 | 13540 |
|  | 32 | 32.82 | 32.52 | 20923 |
|  | 28 | 35.13 | 34.94 | 30050 |
|  | 24 | 38.36 | 38.26 | 41964 |
|  | 20 | 41.93 | 41.88 | 56448 |
|  | 16 | 45.73 | 45.72 | 72297 |
|  | 12 | 49.31 | 49.30 | 87783 |

Table 9: Comparison of intra chroma coding (Src2)

| DC prediction | QP | SNR Cb (db) | SNR Cr (db) | Bitrate (kbps) |
| --- | --- | --- | --- | --- |
| 8x16 with 2x4 Hadamard | 40 | 38.08 | 39.07 | 2751 |
|  | 36 | 38.91 | 39.91 | 4176 |
|  | 32 | 40.10 | 41.06 | 6353 |
|  | 28 | 41.39 | 42.19 | 9384 |
|  | 24 | 43.13 | 43.67 | 13638 |
|  | 20 | 45.22 | 45.52 | 19819 |
|  | 16 | 47.70 | 47.79 | 28909 |
|  | 12 | 50.44 | 50.42 | 40563 |
| 8x8 with 2x2 Hadamard | 40 | 37.21 | 38.09 | 2622 |
|  | 36 | 38.19 | 39.07 | 4015 |
|  | 32 | 39.51 | 40.44 | 6123 |
|  | 28 | 40.89 | 41.65 | 9085 |
|  | 24 | 42.71 | 43.26 | 13220 |
|  | 20 | 44.87 | 45.16 | 19289 |
|  | 16 | 47.41 | 47.46 | 28411 |
|  | 12 | 50.29 | 50.28 | 40175 |

Table 10: Comparison of intra chroma coding (Src5)

| DC prediction | QP | SNR Cb (db) | SNR Cr (db) | Bitrate (kbps) |
| --- | --- | --- | --- | --- |
| 8x16 with 2x4 Hadamard | 40 | 36.64 | 38.08 | 2904 |
|  | 36 | 37.86 | 39.31 | 4241 |
|  | 32 | 39.70 | 41.03 | 6314 |
|  | 28 | 41.37 | 42.71 | 9120 |
|  | 24 | 43.58 | 44.69 | 13010 |
|  | 20 | 45.90 | 46.68 | 18364 |
|  | 16 | 48.15 | 48.55 | 25987 |
|  | 12 | 50.45 | 50.61 | 36814 |
| 8x8 with 2x2 Hadamard | 40 | 35.91 | 37.25 | 2743 |
|  | 36 | 37.21 | 38.53 | 4057 |
|  | 32 | 39.15 | 40.39 | 6086 |
|  | 28 | 40.94 | 42.18 | 8851 |
|  | 24 | 43.29 | 44.38 | 12649 |
|  | 20 | 45.71 | 46.49 | 17944 |
|  | 16 | 47.90 | 48.25 | 25491 |
|  | 12 | 50.27 | 50.42 | 36089 |

Table 11: Comparison of intra chroma coding (Src6)

| DC prediction | QP | SNR Cb (db) | SNR Cr (db) | Bitrate (kbps) |
| --- | --- | --- | --- | --- |
| 8x16 with 2x4 Hadamard | 40 | 36.10 | 37.68 | 3281 |
|  | 36 | 36.92 | 38.59 | 5212 |
|  | 32 | 38.26 | 39.88 | 8331 |
|  | 28 | 39.60 | 41.15 | 12678 |
|  | 24 | 41.56 | 42.80 | 18591 |
|  | 20 | 43.96 | 44.71 | 26838 |
|  | 16 | 46.84 | 47.13 | 38173 |
|  | 12 | 49.92 | 50.09 | 51250 |
| 8x8 with 2x2 Hadamard | 40 | 35.39 | 36.79 | 3088 |
|  | 36 | 36.29 | 37.83 | 4977 |
|  | 32 | 37.72 | 39.27 | 8012 |
|  | 28 | 39.16 | 40.66 | 12290 |
|  | 24 | 41.19 | 42.46 | 18055 |
|  | 20 | 43.69 | 44.42 | 26248 |
|  | 16 | 46.66 | 46.88 | 37630 |
|  | 12 | 49.81 | 49.97 | 50820 |

Table 12: Comparison of intra chroma coding (Src9)

| DC prediction | QP | SNR Cb (db) | SNR Cr (db) | Bitrate (kbps) |
| --- | --- | --- | --- | --- |
| 8x16 with 2x4 Hadamard | 40 | 32.92 | 34.66 | 4406 |
|  | 36 | 33.95 | 35.81 | 6458 |
|  | 32 | 35.48 | 37.46 | 9795 |
|  | 28 | 37.04 | 39.06 | 14301 |
|  | 24 | 39.36 | 41.22 | 21098 |
|  | 20 | 42.28 | 43.53 | 31454 |
|  | 16 | 45.89 | 46.28 | 45491 |
|  | 12 | 49.45 | 49.56 | 60768 |
| 8x8 with 2x2 Hadamard | 40 | 32.21 | 33.84 | 3997 |
|  | 36 | 33.35 | 35.08 | 6003 |
|  | 32 | 35.07 | 36.89 | 9282 |
|  | 28 | 36.76 | 38.63 | 13765 |
|  | 24 | 39.16 | 40.92 | 20483 |
|  | 20 | 42.11 | 43.33 | 30753 |
|  | 16 | 45.78 | 46.09 | 44799 |
|  | 12 | 49.37 | 46.46 | 60097 |

Table 13: Comparison of intra chroma coding (Src13)

| DC prediction | QP | SNR Cb (db) | SNR Cr (db) | Bitrate (kbps) |
| --- | --- | --- | --- | --- |
| 8x16 with 2x4 Hadamard | 40 | 32.03 | 32.19 | 9679 |
|  | 36 | 33.14 | 33.37 | 14087 |
|  | 32 | 34.92 | 35.22 | 20355 |
|  | 28 | 36.86 | 37.15 | 27921 |
|  | 24 | 39.76 | 40.00 | 37639 |
|  | 20 | 43.07 | 43.27 | 49952 |
|  | 16 | 46.53 | 46.73 | 64157 |
|  | 12 | 49.76 | 49.92 | 79095 |
| 8x8 with 2x2 Hadamard | 40 | 31.51 | 31.65 | 9243 |
|  | 36 | 32.68 | 32.92 | 13613 |
|  | 32 | 34.56 | 34.86 | 19832 |
|  | 28 | 36.58 | 36.87 | 27386 |
|  | 24 | 39.55 | 39.78 | 37074 |
|  | 20 | 42.89 | 43.09 | 49360 |
|  | 16 | 46.36 | 46.55 | 63521 |
|  | 12 | 49.65 | 49.81 | 78371 |

Table 14: Comparison of intra chroma coding (Src15)

| DC prediction | QP | SNR Cb (db) | SNR Cr (db) | Bitrate (kbps) |
| --- | --- | --- | --- | --- |
| 8x16 with 2x4 Hadamard | 40 | 35.78 | 36.97 | 2751 |
|  | 36 | 36.78 | 37.78 | 4265 |
|  | 32 | 38.13 | 39.12 | 6817 |
|  | 28 | 39.50 | 40.42 | 10386 |
|  | 24 | 41.26 | 42.22 | 15620 |
|  | 20 | 43.36 | 44.38 | 23974 |
|  | 16 | 46.21 | 46.90 | 36506 |
|  | 12 | 49.58 | 49.81 | 50772 |
| 8x8 with 2x2 Hadamard | 40 | 35.02 | 36.18 | 2501 |
|  | 36 | 36.09 | 37.08 | 3968 |
|  | 32 | 37.60 | 38.53 | 6442 |
|  | 28 | 39.09 | 39.93 | 9947 |
|  | 24 | 40.95 | 41.82 | 15072 |
|  | 20 | 43.12 | 44.09 | 23350 |
|  | 16 | 46.06 | 46.69 | 35976 |
|  | 12 | 49.49 | 49.70 | 50268 |

Table 15: Comparison of intra chroma coding (Src19)

| DC prediction | QP | SNR Cb (db) | SNR Cr (db) | Bitrate (kbps) |
| --- | --- | --- | --- | --- |
| 8x16 with 2x4 Hadamard | 40 | 32.67 | 34.40 | 5330 |
|  | 36 | 33.60 | 35.01 | 8214 |
|  | 32 | 35.23 | 36.13 | 12755 |
|  | 28 | 37.03 | 37.55 | 18924 |
|  | 24 | 39.65 | 39.82 | 27619 |
|  | 20 | 42.65 | 42.58 | 39789 |
|  | 16 | 46.04 | 45.96 | 54817 |
|  | 12 | 49.50 | 49.47 | 69824 |
| 8x8 with 2x2 Hadamard | 40 | 32.13 | 33.95 | 5029 |
|  | 36 | 33.07 | 34.58 | 7832 |
|  | 32 | 34.77 | 35.71 | 12252 |
|  | 28 | 36.69 | 37.16 | 18386 |
|  | 24 | 39.44 | 39.55 | 27099 |
|  | 20 | 42.49 | 42.40 | 39258 |
|  | 16 | 45.91 | 45.83 | 54362 |
|  | 12 | 49.42 | 49.39 | 69375 |

Table 16: Comparison of intra chroma coding (Src22)

BD SNR and BD Bitrate were calculated and were summarized in the following table.

| Sequence | QP range | BD SNR (db) | BD Bitrate (%) |
| --- | --- | --- | --- |
| src2 | 12-24 | -0.0508 | -0.304 |
|  | 28-40 | 0.142 | -2.77 |
| src5 | 12-24 | 0.171 | -2.52 |
|  | 28-40 | 0.556 | -16.5 |
| src6 | 12-24 | 0.0783 | -1.18 |
|  | 28-40 | 0.425 | -9.04 |
| src9 | 12-24 | 0.0870 | -1.026 |
|  | 28-40 | 0.429 | -14.93 |
| src13 | 12-24 | -0.0336 | 0.361 |
|  | 28-40 | 0.276 | -6.79 |
| src15 | 12-24 | 0.0214 | -0.166 |
|  | 28-40 | 0.278 | -5.43 |
| src19 | 12-24 | 0.0645 | -0.896 |
|  | 28-40 | 0.421 | -12.86 |
| src22 | 12-24 | 0.0309 | -0.312 |
|  | 28-40 | 0.337 | -8.44 |

Table 17: BD SNR and BD Bitrate (Proposal/2x2 DC)

At lower bitrate, the advantage of the proposal is quite obvious. It improves up 14 % of the current intra chroma coding method. At higher bitrate, the improvement becomes relatively small. It improves about 1 – 2 % at higher bitrate.

# Conclusions

The simulation results show the advantage of the proposed extension for chroma intra coding for 4:2:2. It should be included in the standard. The extension for 4:4:4 and other bitdepth should be studied. The extension for inter coding should be also verified by the next JVT meeting.

# Annex A.

The definition of DC prediction is specified in this annex.

## 8x8 based DC prediction

The first approach is to calculate DC value based on the 8x8 block. The DC value is calculated for each 8x8 block using its adjacent pixels. In order to avoid the division by 24, the following method is introduced.

When x=0..7, y=0..7, the prediction predc[x, y] can be derived as follows.

If both p[x,-1], and p[-1,y] x=0..7,y=0..7 are not available,

predc[x,y]=x'=07p[x',−1]+y'=07p[−1,y']+8>>4　　　　(x=0..7, y=0..7)

If the left pixels p[x,-1] are not available, and upper pixels p[-1,y] are not available　(x,y=0..7)

predc[x,y]=x'=07p[x',−1]+4>>3　　　　(x, y=0..7)

If the left pixels p[x,-1] are not available, and the upper pixels p[-1,y] are available, (x,y=0..7)

predc[x,y]=y'=07p[−1,y']+4>>3　　　　(x, y=0..7)

If the left pixels p[x,-1] are not available, and the upper pixels p[-1,y] are not available, (x,y=0..7)

predc[x,y]=128　　　　(x, y=0..7)

In the similar way, when x=0..7, y=8..15, the prediction predc[x, y] can be derived as follows.

If the upper pixels p[-1,y] (y=8..15) are available,

predc[x,y]=y'=815p[−1,y']+4>>3　　　　(x=0..7, y=8..15)

If the upper pixels p[-1,y] (y=8..15) are not available, and the left pixels p[x,-1] (x=0..7) are available,

predc[x,y]=x'=07p[x',−1]+4>>3　　　　(x=0..7, y=8..15)

If the left pixels p[x,-1] (x=0..7) are not available, and the upper pixels  p[-1,y] (y=8..15) are not available,

predc=128　　　　(x=0..7, y=8..15)

## 4x4 based DC prediction

The second approach is to calculate DC value based on the 4x4 block. The current chroma DC prediction uses this method. Similar method is applied to 8x16 block. This method can be described as follows.

When x=0..3, y=0..3, the prediction predc[x, y] can be derived using the left adjacent pixels p[x,-1] and the upper adjacent pixels p[-1,y], as follows.

If both p[x,-1], and p[-1,y] x,y=0..3 are available,

predc[x,y]=x'=03p[x',−1]+y'=03p[−1,y']+4>>3　　　　(x, y=0..3)

If the left pixels p[x,-1] are available and the upper pixels p[-1,y] are not available(x,y=0..3)

predc[x,y]=x'=03p[x',−1]+2>>2　　　　(x, y=0..3)

If left pixels p[x,-1] are not available and the upper p[-1,y] are available (x,y=0..3)

predc[x,y]=y'=03p[−1,y']+2>>2　　　　(x, y=0..3)

If the left pixels p[x,-1] are not available, and the upper pixels p[-1,y] are not available (x,y=0..3)

predc[x,y]=128　　　　(x, y=0..3)

By using the similar way, when x=4..7, y=0..3 , the prediction predc[x, y] is derived as follows.

If the left pixels p[x,-1] (x=4..7) are available,

predc[x,y]=x'=47p[x',−1]+2>>2　　　　(x=4..7, y=0..3)

If the left pixels p[x,-1] (x=4..7) are not available, and the upper pixels p[-1,y] (y=0..3) is available

predc[x,y]=y'=03p[−1,y']+2>>2　　　　(x=4..7, y=0..3)

If the left pixels p[x,-1] (x=4..7) are not available and the upper pixels p[-1,y] (y=0..3) are not available ,

predc=128　　　　(x=4..7, y=0..3)

By using the similar way, when x=0..3, y=4..7, the prediction predc[x, y] is derived as follows.

If the upper pixels p[-1,y] (y=4..7) are available,

predc[x,y]=y'=47p[−1,y']+2>>2　　　　(x=0..3, y=4..7)

If the upper pixels p[-1,y] (y=4..7) are not available and the left pixels p[x,-1] (x=0..3) are available,

predc[x,y]=x'=03p[x',−1]+2>>2　　　　(x=0..3, y=4..7)

If the left pixels p[x,-1] (x=0..3) are not available, and the upper pixels p[-1,y] (y=4..7) are not available,

predc=128　　　　(x=0..3, y=4..7)

By using the similar way, when x=4..7, y=4..7, the prediction predc[x, y] can be derived as follows.

If both p[x,-1], and p[-1,y] x,y=4..7 are available,

predc[x,y]=x'=47p[x',−1]+y'=47p[−1,y']+4>>3　　　　(x, y=4..7)

If the left pixels p[x,-1] are available and the upper pixels p[-1,y] are not available (x,y=4..7)

predc[x,y]=x'=47p[x',−1]+2>>2　　　　(x, y=4..7)

If the left pixels p[x,-1] are not available and the upper pixels p[-1,y] are available (x,y=4..7)

predc[x,y]=y'=47p[−1,y']+2>>2　　　　(x, y=4..7)

If the left pixels p[x,-1] are not available and the upper pixels p[-1,y] are not available (x,y=4..7)

predc[x,y]=128　　　　(x, y=4..7)

By using the similar way, when x=0..3, y=8..11, the prediction predc[x, y] can be derived as follows.

If the upper pixels p[-1,y] (y=8..11) are available,

predc[x,y]=y'=811p[−1,y']+2>>2　　　　(x=0..3, y=8..11)

If the upper pixels p[-1,y] (y=8..11) are not available and the left pixels p[x,-1] (x=0..3) are available,

predc[x,y]=x'=03p[x',−1]+2>>2　　　　(x=0..3, y=8..11)

If left pixels p[x,-1] (x=0..3) are not available and the upper pixels p[-1,y] (y=8..11) are not available,

predc=128　　　　(x=0..3, y=8..11)

By using the same way, when x=4..7, y=8..11, the prediction predc[x, y] can be derived as follows.

If both p[x,-1], and p[-1,y] x=4..7,y=8..11 are available,

predc[x,y]=x'=47p[x',−1]+y'=811p[−1,y']+4>>3　　　　(x=4..7, y=8..11)

If left pixels p[x,-1] are available and the upper pixels p[-1,y] are not available, (x=4..7,y=8..11)

predc[x,y]=x'=47p[x',−1]+2>>2　　　　(x=4..7, y=8..11)

If the left pixels p[x,-1] are available and the upper pixels p[-1,y] are available(x=4..7,y=8..11)

predc[x,y]=y'=811p[−1,y']+2>>2　　　　(x=4..7, y=8..11)

If the left pixels p[x,-1] are not available and the upper pixels p[-1,y] are not available, (x=4..7,y=8..11)

predc[x,y]=128　　　　(x=4..7, y=8..11)

By using the similar way, when x=0..3, y=12..15 the prediction predc[x, y] can be derived as follows.

If the upper pixels p[-1,y] (y=12..15) are available,

predc[x,y]=y'=1215p[−1,y']+2>>2　　　　(x=0..3, y=12..15)

If the upper pixels p[-1,y] (y=12..15) are not available and the left pixels p[x,-1] (x=0..3) are available,

predc[x,y]=x'=03p[x',−1]+2>>2　　　　(x=0..3, y=12..15)

If the left pixels p[x,-1] (x=0..3) are not available and the upper pixels p[-1,y] (y=12..15) are not available,

predc=128　　　　(x=0..3, y=12..15)

By using the similar way, when x=4..7, y=12..15, the prediction predc[x, y] can be derived as follows.

If both p[x,-1], and p[-1,y] x=4..7,y=12..15 are available,

predc[x,y]=x'=47p[x',−1]+y'=1215p[−1,y']+4>>3　　　　(x=4..7, y=12..15)

If the left pixels p[x,-1] are available and the upper pixels p[-1,y] are not available, (x=4..7,y=12..15)

predc[x,y]=x'=47p[x',−1]+2>>2　　　　(x=4..7, y=12..15)

If the left pixels p[x,-1] are not available and the upper pixels p[-1,y] are available, (x=4..7,y=12..15)

predc[x,y]=y'=1215p[−1,y']+2>>2　　　　(x=4..7, y=12..15)

If the left pixels p[x,-1] are not available and the upper pixels p[-1,y] are not available, (x=4..7,y=12..15)

predc[x,y]=128　　　　(x=4..7, y=12..15)

## DC prediction with division

Simple way to calculate DC value of 8x16 is to calculate the average of surrounding 24 pixels. We compare the results with the DC value divided by 24.

# Annex B. The Simulation Results

The comparisons between 8x16 intra coding and 8x8 intra coding are summarized in this annex. The graphs show the comparison of Cb.

- Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Sony Corp. |  |  |
| Mailing address | Gate City Osaki East Tower 20F 1-11-1, Osaki, Shinagawa-ku Tokyo, 141-0032 |  |  |
| Country | Japan |  |  |
| Contact person | Teruhiko Suzuki |  |  |
| Telephone | +81-3-5435-3669 |  |  |
| Fax | +81-3-5435-3891 |  |  |
| Email | teruhiko@av.crl.sony.co.jp |  |  |
| Place and date of submission | Trondheim, Norway on July 20th, 2003 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Extension of Intra Coding for 4:2:2 Video |  |
| Contribution number |  | I016 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
| X | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: