| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 9th Meeting: 2-5 September 2003, San Diego | Document: JVT-I018d1 Filename: JVT-I018d1.doc |
| --- | --- |

| Title: | Color format downconversion for test sequence generation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Gary Sullivan One Microsoft Way Redmond, WA 98052 USA | Tel: Email: | +1 425 703 5308 garys@microsoft.com |
| Source: | Microsoft Corp. |  |  |

_____________________________

# Abstract

This document contains a description of a method of downsampling for chroma format conversion that the author believes to be reasonably appropriate for use where needed for downsampling of content to a reduced-resolution chroma sampling grid for encoding (e.g., for the verification test effort for H.264/AVC).  It is based on Kaiser windowed half-band FIR filtering with adjustments for phase offsets.  For interlaced video, it specifies field-based downsampling.

# 1.	Introduction

For 4:4:4 to 4:2:2 and 4:2:2 to 4:2:0 chroma format downconversions (including two flavors of 4:2:0), a filter design is suggested as follows.  This design is motivated by the need to define resampling filters for content used in verification testing of H.264/AVC.

# 2.	Basic Filter Design Structure

First, define a non-scaled filter impulse response by

gp,N,D,w(i)=sincπi+pD∗wND(i+p)

where p is a fractional phase offset in units of input samples in the range 0 to 0.5, wL(·) is some windowing function that is symmetric around 0 and has a zero value outside the range of plus or minus L = ND (and is non-zero within that range, except possibly at the extreme boundaries), D is the nominal downsampling factor that controls the pass band width of the filter, N is a parameter that controls the number of lobes of the sinc function to include within the window (i.e., the main lobe and N-1 side lobes), and

sinc(x)=limt→xsin(t)t.

Then, define

Tp,N,D,w=i= −∞∞gp,N,D,w(i).

The floating-point filter impulse response is then adjusted to provide a DC gain equal to 1 by using the following impulse response.

hp,N,D,w(i)=gp,N,D,w(i)Tp,N,D,w.

Note that for p equal to 0 or p equal to 0.5 the impulse response will be symmetric, with hp,N,D,w(i) equal to hp,N,D,w(-i) for i>0 with p=0 and hp,N,D,w(i) equal to hp,N,D,w(-(i+1)) for i≥0 with p=0.5. This allows the number of multiplies to be cut approximately in half by switching the order of the multiply and add operations for pairs of values of i in these cases.

If the values of hp,N,D,w(i) are scaled up and rounded to integer approximations for ease of computation, care should be taken to ensure that the resulting filter retains a DC gain equal to 1.  One reasonable way to do this, for two's complement integer representations of tap values using b bits each is to use the integerization method specified as follows.  If p is not equal to 0.5, the rounding specified is given by:

hp,N,D,wb(i)=round2b−1∗gp,N,D,w(i)Tpfor i≠0;2b−1−j= −∞−1hp,N,D,wb(j)+j=1∞hp,N,D,wb(j)for i=0;0otherwise.

Otherwise (p is equal to 0.5), symmetry should be maintained as follows:

hp,N,D,wb(i)=round2b−1∗gp,N,D,w(i)Tpfor i≠−1 or 0;2b−2−j=1ceil(ND−1)hp,N,D,wb(j)for i=−1 and 0;0otherwise.

The length of the filter impulse response defined in this formulation is 2*floor(ND), with hp,N,D,w(i) being zero for values of i outside the range from –floor(ND) to floor(ND)-1, except in some special cases when p is equal to 0, as follows.

- When p is equal to 0 and D is an integer and N is an integer, hp,N,D,w(-ND) and hp,N,D,w(ND) will be equal to 0 due to the sinc function term, so in this case the length of the filter impulse response will be 2*ND-1, with hp,N,D,w(i) being zero for values of i outside the range from –(ND-1) to ND-1.

- Similarly, when p is equal to 0 and the product ND is an integer and wND(ND) is equal to zero, hp,N,D,w(-ND) and hp,N,D,w(ND) will be equal to 0 due to the window function term, so in this case the length of the filter impulse response will be 2*ND-1, with hp,N,D,w(i) being zero for values of i outside the range from –(ND-1) to ND-1.

- In the unusual special case when p is equal to zero and wND(ND) is not equal to zero and both N and D are not integers (so the sinc function term in the computation of hp,N,D,w(i) will not be zero at i=ND) but the product ND is an integer, the filter impulse response length will be 2*ND+1, with i ranging from –ND to +ND.

When p is equal to 0 and D is an integer, an additional 2*floor(N-1) impulse response samples will be equal to zero due to zeros of the sinc function, so the total number of non-zero taps within the impulse response will be reduced by that amount in this case.  (There are also likely to be other conditions that produce some additional zero-valued samples within the impulse response in special cases, but this case seems the most typical.)

Typically, the resulting filter will have 3 dB of attenuation at the nominal downsampling critical frequency /D.  If more attenuation is desired at the actual critical frequency, a value of D that is somewhat (e.g., a few percent) larger than the actual downsampling factor can be used for D.

# 3.	The Kaiser Window

The Kaiser window is a well-designed adjustable window.  It is defined as

wL(x)=I0β1−xL2I0βx≤L;0otherwise;

where I0(x) is the modified zero-th order Bessel function of the first kind, and  is a window shape tuning parameter that adjusts the trade-off between stop-band attentuation or pass-band ripple and transition band width.

I0(x) can be computed using the approximation

I0(x)≃1+k=1Kx2kk!2

where K is a sufficiently large number (values of K of about 25 are reported to have been used in the literature for Kaiser-window filter design).

Since the term within the summation is not exactly trivial to compute, note that this term can be computed using logarithms as

x2kk!2=exp2∗k*ln(x2)−j=1kln(j)

Tuning parameter value  = 2.12 reportedly gives approximately 30 dB stop-band attenuation - similar to a von Hann window (also known as a raised-cosine window), but with a narrower transition band width. Tuning parameter  = 4.55 reportedly gives approximately 50 dB stop-band attenuation - close to that of a Hamming window, but with a narrower transition band width.

The transition band width is much smaller for smaller  (for  = 2.12 it is about half as wide as for  = 4.55). The transition band width is also inversely proportional to the filter length.  For a relatively short filter, it is therefore necessary to use a relatively low value of  in order to avoid a wide transition band.

# 4.	Selection of Parameters and Specific Filter Cases

My initial suggestion would be to try D=2, N=4, K=30,  = 2.75.  Given sufficient time, it would be advisable to test other values of N and , and slightly larger values of D.

We can use versions of this with p=0, p=0.25 and p=0.5 for color sampling format conversion.

- p=0.25 is used for field-based downsampling of interlaced 4:2:2 video to 4:2:0.

- p=0.5 is used for vertical downsampling of progressive-scan 4:2:2 video to 4:2:0 and for horizontal downsampling of 4:4:4 to 4:2:2 toward producing the H.261/MPEG-1/H.263 version of 4:2:0.

- p=0 is used for horizontal downsampling of 4:4:4 to 4:2:2 toward producing the MPEG-2/MPEG-4/H.264 version of 4:2:0.