| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 9th Meeting: San Diego, 2-5 September, 2003 | Document: JVT-I022 Filename: JVT-I022.doc |
| --- | --- |

| Title: | Simplified Use of 8x8 Transforms |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Steve Gordon 200 Brickstone, Fourth Floor Andover, MA 01810 USA | Tel: Email: | 978.475.8990 x222 SteveGordon@SandVideo.com |
| Source: | Sand Video, Inc |  |  |

_____________________________

# Introduction

Significant compression performance gains have been reported at Standard Definition (SD) and High Definition (HD) resolutions when larger than 4x4 transforms are used [1]. However, these gains have come at the expense of added complexity to the H.264/MPEG4 Part-10 compression standard. In this document, we propose a simplified way of introducing larger transforms into the JVT specification, while maintaining the rate-distortion performance gain.

# Basic Idea

Based on the observation that at SD resolutions and above, the use of block sizes smaller than 8x8 is limited, we propose adding two new coding tools to the JVT specification: a luma 8x8 transform and a luma 8x8 intra prediction mode. We call our mode “Transform8” to indicate that an 8x8 transform may be in use for block sizes 8x8 and above. The mode is enabled via a new syntax element, transform_8x8_mode_flag, in the Picture Parameter Set RBSP. Relative to Adaptive Block Transforms [2], we propose the following simplifications:

- Do not allow transform sizes to be mixed within a macroblock.

- Limit the larger transform size to 8x8; i.e., do not use 8x4 and 4x8 transforms.

- Limit the new intra modes to 8x8; i.e., do not use 8x4 and 4x8 intra modes.

- Allow the use of Intra16x16 mode when Transform8 mode is enabled.

- Limit the use of the Transform8 mode to when levelx10 is greater than or equal to 40 (i.e, level >= 3). At these levels, direct_8x8_inference_flag is equal to 1 ensuring that direct mode vectors are never applied to smaller than 8x8 blocks.

- Limit the use of the Transform8 mode to when profile is equal to the Professional Extensions.

- Limit the use of the Transform8 mode to when entropy_coding_mode_flag is equal to 1 (i.e., CABAC is enabled).

In addition to the transform_8x8 syntax element, four new syntax elements are required:

- For macroblock types P_8x8 and B_8x8, the syntax element all_8x8 equal to 1 is used to indicate that all four 8x8 subpartitions are using 8x8 block size. In this case, the new 8x8 transform is used. Otherwise (all_8x8 is equal to 0), block sizes smaller than 8x8 may be in use and so the existing 4x4 transform is used.

- For the mb_type equal to 0 in Table 7-8, the syntax element intra_pred_size is used to indicate whether the macroblock type is Intra_4x4 (intra_pred_size equal to 0) or Intra_8x8.

- For macroblock type Intra_8x8, the syntax elements prev_intra8x8_pred_mode_flag and rem_intra8x8_pred_mode are used to determine the specific 8x8 intra mode used.

With the addition of these new syntax elements, the following mapping of macroblock type to luma transform size can be made:

Table 1 - Luma Transform Size

| Macroblock Type | transform_8x8_mode_flag | all_8x8 | Luma Transform Size |
| --- | --- | --- | --- |
| Intra_4x4 | na | na | 4x4 |
| Intra_8x8 | na | na | 8x8 |
| Intra_16x16 | na | na | 4x4 |
| P_16x16 | 0 | na | 4x4 |
|  | 1 | na | 8x8 |
| P_8x16 | 0 | na | 4x4 |
|  | 1 | na | 8x8 |
| P_16x8 | 0 | na | 4x4 |
|  | 1 | na | 8x8 |
| P_8x8 | 0 | na | 4x4 |
|  | 1 | 0 | 4x4 |
|  | 1 | 1 | 8x8 |
| B_Direct | 0 | na | 4x4 |
|  | 1 | na | 8x8 |
| B_16x16 | 0 | na | 4x4 |
|  | 1 | na | 8x8 |
| B_16x8 | 0 | na | 4x4 |
|  | 1 | na | 8x8 |
| B_8x16 | 0 | na | 4x4 |
|  | 1 | na | 8x8 |
| B_8x8 | 0 | na | 4x4 |
|  | 1 | 0 | 4x4 |
|  | 1 | 1 | 8x8 |

# Overview of Specification Changes

## 8x8 Intra Prediction

The proposed luma 8x8 intra prediction tool is a logical extension of the existing luma 4x4 intra prediction tool. Whereas the 4x4 intra prediction uses 13 surrounding luma values, the 8x8 intra prediction uses 25 surrounding luma values. The nine possible prediction modes are identical between the 4x4 and 8x8 intra prediction tools, as is the decoding of the specific prediction mode from the syntax elements in the bitstream.

## 8x8 Transform

The new 8x8 transform is based on a proposal by Frank Bossen[3]. The 8x8 transform is used as per Table 1 above.  The inverse 8x8 transform uses the following basis functions

| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 12/8 | 10/8 | 6/8 | 3/8 | -3/8 | 6/8 | -10/8 | -12/8 |
| 1 | ½ | -1/2 | -1 | -1 | -1/2 | ½ | 1 |
| 10/8 | -3/8 | 12/8 | -6/8 | 6/8 | 12/8 | 3/8 | -10/8 |
| 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 |
| 6/8 | -12/8 | 3/8 | 10/8 | -10/8 | -3/8 | 12/8 | -6/8 |
| ½ | -1 | 1 | -1/2 | -1/2 | 1 | -1 | ½ |
| 3/8 | -6/8 | 10/8 | -12/8 | 12/8 | -10/8 | 6/8 | -3/8 |

which closely approximate the 8x8 IDCT. The inverse 8x8 quantization uses a new scaling matrix V(QP%6,i,j)

| QP%6 | i=0 or 4 && j=0 or 4 | i=1,3,5, or 7 && j=1,3,5, or 7 | i=2 or 6 && j=2 or 6 | (i=0,2,4, or 6 && j=1,3,5, or 7) or (i=1,3,5, or 7 && j=0,2,4,6) | (i=0 or 4 && j=2 or 6 ) or (i=2 or 6 && j=0 or 4) | otherwise |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 10 | 9 | 16 | 9 | 13 | 12 |
| 1 | 11 | 10 | 18 | 10 | 14 | 13 |
| 2 | 12 | 11 | 20 | 12 | 16 | 15 |
| 3 | 14 | 12 | 22 | 13 | 18 | 17 |
| 4 | 16 | 14 | 26 | 15 | 20 | 19 |
| 5 | 18 | 16 | 29 | 17 | 23 | 21 |

Scan Pattern

Two new scan patterns are used for mapping the entropy decoded 8x8 transform coefficients into their raster order. These are shown in Figure 2. The 8x8 zig-zag scan is used for frame macroblocks while the 8x8 field-scan is used

for field macroblocks.

| 0 | 1 | 5 | 6 | 14 | 15 | 27 | 28 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 4 | 7 | 13 | 16 | 26 | 29 | 42 |
| 3 | 8 | 12 | 17 | 25 | 30 | 41 | 43 |
| 9 | 11 | 18 | 24 | 31 | 40 | 44 | 53 |
| 10 | 19 | 23 | 31 | 39 | 45 | 52 | 54 |
| 20 | 22 | 33 | 38 | 46 | 51 | 55 | 60 |
| 21 | 34 | 37 | 47 | 50 | 56 | 59 | 61 |
| 35 | 36 | 48 | 49 | 57 | 58 | 62 | 63 |

Zig-zag scan

| 0 | 3 | 8 | 15 | 23 | 31 | 39 | 47 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 4 | 9 | 16 | 24 | 32 | 40 | 48 |
| 2 | 7 | 14 | 22 | 30 | 38 | 46 | 54 |
| 5 | 10 | 17 | 25 | 33 | 41 | 49 | 55 |
| 6 | 13 | 21 | 29 | 37 | 45 | 53 | 59 |
| 11 | 18 | 26 | 34 | 42 | 50 | 56 | 60 |
| 12 | 20 | 28 | 36 | 44 | 52 | 58 | 62 |
| 19 | 27 | 35 | 43 | 51 | 57 | 61 | 63 |

Field Scan

Figure 2 – 8x8 Scan Patterns

## Deblocking

For macroblocks using the luma 8x8 transform, the deblocking for luma values is only applied at the edges of 8x8 blocks. Note that there are no changes to chroma deblocking.

## CABAC

The four new macroblock layer syntax elements, intra_pred_size, all_8x8, prev_intra4x4_pred_mode_flag, and rem_intra4x4_pred_mode each require the addition of one new CABAC context. In addition, the decoding of luma 8x8 transform coefficients requires the addition of the following new CABAC contexts:

- 4 new contexts for coded_block_flag.

- 63 new contexts for significant_coeff_flag.

- 63 new contexts for last_significant_coeff_flag.

- 10 new contexts for coeff_abs_level_minus1.

Note that no new binarization techniques are required.

- References

[1] Stephen Gordon. ABT for Film Grain Reproduction in High Definition Sequences, JVT-H029r1.

[2] JVT-D157, FCD.

[3] Frank Bossen, ABT cleanup and complexity reduction, JVT-E087.

# Appendix A - Syntax and Semantics Changes

Relative to input.doc, the following edits must be made to clause 7. All edits are in blue.

      - Picture parameter set RBSP syntax

| entropy_coding_mode_flag | 0 | u(1) |
| --- | --- | --- |
| if ( profile_idc == *** && entropy_coding_mode_flag ) { |  |  |
| transform_8x8 | 0 | u(1) |
| } |  |  |

*** is the profile_idc assigned to the Professional Extensions.

7.3.5 Macroblock layer syntax

| mb_type | 2 | ue(v) ¦ ae(v) |
| --- | --- | --- |
| if ( transform_8x8 && mb_type == I_NxN) intra_pred_size | 2 | ae(v) |
| if ( transform_8x8 && MbPartPredMode( mb_type, 0 ) != Intra_4x4 && MbPartPredMode( mb_type, 0 ) != Intra_16x16 && MbPartPredMode( mb_type, 0 ) != Intra_8x8 && NumMbPart( mb_type ) = = 4) all_8x8 | 2 | ae(v) |

| if( MbPartPredMode( mb_type, 0 ) != Intra_4x4 && MbPartPredMode( mb_type, 0 ) != Intra_16x16 && MbPartPredMode( mb_type, 0 ) != Intra_8x8 && NumMbPart( mb_type ) = = 4 ) |  |  |
| --- | --- | --- |
| sub_mb_pred( mb_type ) |  |  |

7.3.5.1	Macroblock prediction syntax

| if( MbPartPredMode( mb_type, 0 ) = = Intra_4x4 ¦ ¦ MbPartPredMode( mb_type, 0 ) = = Intra_8x8 ¦ ¦ MbPartPredMode( mb_type, 0 ) = = Intra_16x16 ) { |  |  |
| --- | --- | --- |
| If( MbPartPredMode( mb_type, 0 ) = = Intra_4x4 ) |  |  |
| for( luma4x4BlkIdx=0; luma4x4BlkIdx<16; luma4x4BlkIdx++ ) { |  |  |
| prev_intra4x4_pred_mode_flag[ luma4x4BlkIdx ] | 2 | u(1) ¦ ae(v) |
| if( !prev_intra4x4_pred_mode_flag[ luma4x4BlkIdx ] ) |  |  |
| rem_intra4x4_pred_mode[ luma4x4BlkIdx ] | 2 | u(3) ¦ ae(v) |
| } |  |  |
| if( MbPartPredMode( mb_type, 0 ) = = Intra_8x8 ) |  |  |
| for( luma8x8BlkIdx=0; luma8x8BlkIdx<4; luma8x8BlkIdx++ ) { |  |  |
| prev_intra8x8_pred_mode_flag[ luma8x8BlkIdx ] | 2 | ae(v) |
| if( !prev_intra8x8_pred_mode_flag[ luma8x8BlkIdx ] ) |  |  |
| rem_intra8x8_pred_mode[ luma8x8BlkIdx ] | 2 | ae(v) |
| } |  |  |

      - Residual data syntax

| for( i8x8 = 0; i8x8 < 4; i8x8++ ) /* each luma 8x8 block */ |  |  |
| --- | --- | --- |
| if ( !transform_8x8 ¦¦ MbPartPredMode( mb_type, 0 ) = = Intra_4x4 ¦¦ MbPartPredMode( mb_type, 0 ) = = Intra_16x16 ¦¦ ( NumMbPart( mb_type ) == 4 && all_8x8 == 0) ) |  |  |
| for( i4x4 = 0; i4x4 < 4; i4x4++ ) /* each 4x4 sub-block of block */ |  |  |
| if( CodedBlockPatternLuma & ( 1 << i8x8 ) ) { |  |  |
| if( MbPartPredMode( mb_type, 0 ) = = Intra_16x16 ) |  |  |
| residual_block( Intra16x16ACLevel[ i8x8 * 4 + i4x4 ], 15 ) | 3 |  |
| Else |  |  |
| residual_block( LumaLevel[ i8x8 * 4 + i4x4 ], 16 ) | 3 ¦ 4 |  |
| } else { |  |  |
| if( MbPartPredMode( mb_type, 0 ) = = Intra_16x16 ) |  |  |
| for( i = 0; i < 15; i++ ) |  |  |
| Intra16x16ACLevel[ i8x8 * 4 + i4x4 ][ i ] = 0 |  |  |
| Else |  |  |
| for( i = 0; i < 16; i++ ) |  |  |
| LumaLevel[ i8x8 * 4 + i4x4 ][ i ] = 0 |  |  |
| } |  |  |
| else /* using 8x8 transform */ |  |  |
| if( CodedBlockPatternLuma & ( 1 << i8x8 ) ) |  |  |
| residual_block( LumaLevel64[ i8x8], 64 ) | 3 ¦ 4 |  |
| else |  |  |
| for( i = 0; i < 64; i++ ) |  |  |
| LumaLevel64[ i8x8 ][ i ] = 0 |  |  |

      - Picture parameter set RBSP semantics

transform_8x8 equal to 1 indicates that the 8x8 transform decoding process may be in use (see subclause 8.4). Otherwise (transform_8x8 equal to 0) indicates that the 8x8 transform decoding process is not in use. If transform_8x8 is not present in the bitstream, it shall be inferred to be 0.

    - Macroblock layer semantics

mb_type

I_4x4: the macroblock is coded as an Intra_4x4 prediction macroblock  I_NxN: the macroblock is coded as an Intra_8x8 prediction macroblock if intra_pred_size equals 1, otherwise the macroblock is coded as an Intra_4x4.

Intra_4x4 specifies the macroblock prediction mode and specifies that the Intra_4x4 prediction process is invoked as specified in subclause Error: Reference source not found.1. Intra_4x4 is an Intra macroblock prediction mode.

Intra_8x8 specifies the macroblock prediction mode and specifies that the Intra_8x8 prediction process is invoked as specified in subclause Error: Reference source not found.2. Intra_4x4 is an Intra macroblock prediction mode.

intra_pred_size equal to 1 indicates that macroblock prediction mode shall be set to Intra_8x8. intra_pred_size equal to 0 indicates that if mb_type equals I_NxN then macroblock prediction mode shall be set to Intra_4x4. If intra_pred_mode is not present in the bitstream, it shall be inferred to be 0.

- Table 7-2 – Macroblock types for I slices

| mb_type | Name of mb_type | Intra_pred_size | MbPartPredMode ( mb_type, 0 ) | Intra16x16PredMode | CodedBlockPatternChroma | CodedBlockPatternLuma |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | I_NxN | 0 | Intra_4x4 | na | na | na |
| 0 | I_NxN | 1 | Intra_8x8 | na | na | na |
| 1 | I_16x16_0_0_0 | na | Intra_16x16 | 0 | 0 | 0 |
| 2 | I_16x16_1_0_0 | na | Intra_16x16 | 1 | 0 | 0 |
| 3 | I_16x16_2_0_0 | na | Intra_16x16 | 2 | 0 | 0 |
| 4 | I_16x16_3_0_0 | na | Intra_16x16 | 3 | 0 | 0 |
| 5 | I_16x16_0_1_0 | na | Intra_16x16 | 0 | 1 | 0 |
| 6 | I_16x16_1_1_0 | na | Intra_16x16 | 1 | 1 | 0 |
| 7 | I_16x16_2_1_0 | na | Intra_16x16 | 2 | 1 | 0 |
| 8 | I_16x16_3_1_0 | na | Intra_16x16 | 3 | 1 | 0 |
| 9 | I_16x16_0_2_0 | na | Intra_16x16 | 0 | 2 | 0 |
| 10 | I_16x16_1_2_0 | na | Intra_16x16 | 1 | 2 | 0 |
| 11 | I_16x16_2_2_0 | na | Intra_16x16 | 2 | 2 | 0 |
| 12 | I_16x16_3_2_0 | na | Intra_16x16 | 3 | 2 | 0 |
| 13 | I_16x16_0_0_1 | na | Intra_16x16 | 0 | 0 | 15 |
| 14 | I_16x16_1_0_1 | na | Intra_16x16 | 1 | 0 | 15 |
| 15 | I_16x16_2_0_1 | na | Intra_16x16 | 2 | 0 | 15 |
| 16 | I_16x16_3_0_1 | na | Intra_16x16 | 3 | 0 | 15 |
| 17 | I_16x16_0_1_1 | na | Intra_16x16 | 0 | 1 | 15 |
| 18 | I_16x16_1_1_1 | na | Intra_16x16 | 1 | 1 | 15 |
| 19 | I_16x16_2_1_1 | na | Intra_16x16 | 2 | 1 | 15 |
| 20 | I_16x16_3_1_1 | na | Intra_16x16 | 3 | 1 | 15 |
| 21 | I_16x16_0_2_1 | na | Intra_16x16 | 0 | 2 | 15 |
| 22 | I_16x16_1_2_1 | na | Intra_16x16 | 1 | 2 | 15 |
| 23 | I_16x16_2_2_1 | na | Intra_16x16 | 2 | 2 | 15 |
| 24 | I_16x16_3_2_1 | na | Intra_16x16 | 3 | 2 | 15 |
| 25 | I_PCM | na | na | na | na | na |

all_8x8 equal to 1 indicates that for macroblock types P_8x8 and B_8x8, all four subpartitions are 8x8 blocks. Otherwise (all_8x8 equal to 0), for macroblock types P_8x8 and B_8x8, all four subpartitions may not be 8x8 blocks. If all_8x8 is not present in the bitstream, it shall be inferred to be zero.

7.4.5.1 	Macroblock prediction semantics

prev_intra8x4_pred_mode_flag[ luma8x8BlkIdx ] and rem_intra8x8_pred_mode[ luma8x8BlkIdx ] specify the Intra_8x8 prediction of the 8x8 luma block with index luma8x8BlkIdx = 0..3.

7.4.5.3	Residual data semantics

Replace

- Otherwise (MbPartPredMode( mb_type, 0 ) is not equal to Intra_16x16), for each of the 16 4x4 luma blocks indexed by i = 0..15,  the 16 transform coefficient levels of the i-th block are parsed into the i-th list LumaLevel[ i ].

With

- If MbPartPredMode( mb_type, 0 ) is equal to Intra_8x8, for each of the 4 8x8 luma blocks indexed by i = 0..3, the 64 transform coefficient levels of the i-th block are parsed into the i-th list LumaLevel64[i].

- If NumMbPart( mb_type ) is equal to 4 and all_8x8 is equal to 1, for each of the 4 8x8 luma blocks indexed by i=0..3, the 64 transform coefficient levels of the i-th block are parsed into the i-th list LumaLevel64[i].

- Otherwise for each of the 16 4x4 luma blocks indexed by i = 0..15, the 16 transform coefficient levels of the i-th block are parsed into the i-th list LumaLevel[ i ].

Appendix B – Detailed Edits

  - Intra prediction process

Inputs to this process are constructed samples prior to the deblocking filter process from neighbouring macroblocks and for Intra_NxN prediction mode, the associated values of IntraNxNPredMode from neighbouring macroblocks (where N is equal to 4 or 8).

Outputs of this process are specified as follows.

- If mb_type is not equal to I_PCM, the Intra prediction samples of components of the macroblock or in case of the Intra_NxN prediction process for luma samples, the outputs are NxN luma sample arrays as part of the 16x16 luma array of prediction samples of the macroblock (where N is equal to 4 or 8).

-	Otherwise (mb_type is equal to I_PCM), constructed macroblock samples prior to the deblocking filter process.

Depending on the value of mb_type the following applies.

- If mb_type is equal to I_PCM, the process specified in subclause 8.3.5 is invoked.

-	Otherwise (mb_type is not equal to I_PCM), the following applies.

-	The decoding processes for Intra prediction modes are described for the luma component as follows.

- 	If the macroblock prediction mode is equal to Intra_4x4, the specification in subclause Error: Reference source not found applies.

-	If the macroblock prediction mode is equal to Intra_8x8, the specification in subclause 8.3.2 applies.

-	Otherwise (the macroblock prediction mode is equal to Intra_16x16), the specification in subclause 8.3.3 applies.

-	The decoding processes for Intra prediction modes for the chroma components are described in subclause 8.3.4.

Samples used in the Intra prediction process shall be sample values prior to alteration by any deblocking filter operations.

### Intra_8x8 prediction process for luma samples

This process is invoked when the macroblock prediction mode is equal to Intra_8x8.

Inputs to this process are constructed luma samples prior to the deblocking filter process from neighbouring macroblocks and the associated values of Intra8x8PredMode from the neighbouring macroblocks or macroblock pairs.

Outputs of this process are 8x8 luma sample arrays as part of the 16x16 luma array of prediction samples of the macroblock predL.

The luma component of a macroblock consists of 4 blocks of 8x8 luma samples. These blocks are inverse scanned using the 8x8 luma block inverse scanning process as specified in subclause 6.4.2.

For all the 8x8 luma blocks of the luma component of a macroblock with luma8x8BlkIdx = 0..3, the variable Intra8x8PredMode[ luma8x8BlkIdx ] is derived as specified in subclause 8.3.2.1.

For the each luma block of 8x8 samples indexed using luma8x8BlkIdx = 0..3,

- The Intra_8x8 sample prediction process in subclause  is invoked with luma8x8BlkIdx and constructed samples prior (in decoding order) to the deblocking filter process from adjacent luma blocks as the input and the output are the Intra_8x8 luma prediction samples pred8x8L[ x, y ] with x, y = 0..7.

- The position of the upper-left sample of a 8x8 luma block with index luma8x8BlkIdx inside the current macroblock is derived by invoking the inverse 8x8 luma block scanning process in subclause Error: Reference source not found2 with luma8x8BlkIdx as the input and the output being assigned to ( xO, yO ) and x, y = 0..7.

predL[ xO + x, yO + y ] = pred8x8L[ x, y ]

- The transform coefficient decoding process and picture construction process prior to deblocking filter process in subclause 8.3 is invoked with predL and luma8x8BlkIdx as the input and the constructed samples for the current 8x8 luma block S’L as the output.

#### 8.3.2.1	Derivation process for the Intra8x8PredMode

Inputs to this process are the index of the 8x8 luma block luma8x8BlkIdx and variable arrays Intra8x8PredMode that are previously (in decoding order) derived for adjacent macroblocks.

Output of this process is the variable Intra8x8PredMode[ luma8x8BlkIdx ].

Table 8-3 specifies the values for Intra8x8PredMode[ luma8x8BlkIdx ] and the associated names.

- Table 8-3 – Specification of Intra8x8PredMode[ luma8x8BlkIdx ] and associated names

| Intra8x8PredMode[ luma8x8BlkIdx ] | Name of Intra8x8PredMode[ luma8x8BlkIdx ] |
| --- | --- |
| 0 | Intra_8x8_Vertical (prediction mode) |
| 1 | Intra_8x8_Horizontal (prediction mode) |
| 2 | Intra_8x8_DC (prediction mode) |
| 3 | Intra_8x8_Diagonal_Down_Left (prediction mode) |
| 4 | Intra_8x8_Diagonal_Down_Right (prediction mode) |
| 5 | Intra_8x8_Vertical_Right (prediction mode) |
| 6 | Intra_8x8_Horizontal_Down (prediction mode) |
| 7 | Intra_8x8_Vertical_Left (prediction mode) |
| 8 | Intra_8x8_Horizontal_Up (prediction mode) |

Let intra8x8PredModeA and intra8x8PredModeB be variables that specify the intra prediction modes of neighbouring 8x8 luma blocks.

Intra8x8PredMode[ luma8x8BlkIdx ] is derived as follows.

- The process specified in subclause Error: Reference source not found.2 is invoked with luma8x8BlkIdx given as input and the output is assigned to mbAddrA, luma8x8BlkIdxA, mbAddrB, and luma8x8BlkIdxB.

- The variable dcOnlyPredictionFlag is derived as follows.

- If one of the following conditions is true, dcOnlyPredictionFlag is set equal to 1

- the macroblock with address mbAddrA is not available

- the macroblock with address mbAddrB is not available

- the macroblock with address mbAddrA is available and coded in Inter prediction mode and constrained_intra_pred_flag is equal to 1

- the macroblock with address mbAddrB is available and coded in Inter prediction mode and constrained_intra_pred_flag is equal to 1

- Otherwise, dcOnlyPredictionFlag is set equal to 0.

- For N being either replaced by A or B, the variables intra8x8PredModeN are derived as follows.

- If dcOnlyPredictionFlag is equal to 1 or the macroblock with address mbAddrN is not coded in Intra_8x8 macroblock prediction mode, intra8x8PredModeN is set equal to 2 (Intra_8x8_DC prediction mode).

- Otherwise (dcOnlyPredictionFlag is equal to 0 and the macroblock with address mbAddrN is coded in Intra_8x8 macroblock prediction mode), intra8x8PredModeN is set equal to Intra8x8PredMode[ luma8x8BlkIdxN ], where Intra8x8PredMode is the variable array assigned to the macroblock mbAddrN.

- Intra8x8PredMode[ luma8x8BlkIdx ] is derived by applying the following procedure.

predIntra8x8PredMode = Min( intra8x8PredModeA, intra8x8PredModeB )
if( prev_intra8x8_pred_mode_flag[ luma8x8BlkIdx ] )
	Intra8x8PredMode[ luma8x8BlkIdx ] = predIntra8x8PredMode
else					
	if( rem_intra8x8_pred_mode[ luma8x8BlkIdx ] < predIntra8x8PredMode )
		Intra8x8PredMode[ luma8x8BlkIdx ] = rem_intra8x8_pred_mode[ luma8x8BlkIdx ]
	else
		Intra8x8PredMode[ luma8x8BlkIdx ] = rem_intra8x8_pred_mode[ luma8x8BlkIdx ] + 1

#### 8.3.2.2	Intra_8x8 sample prediction

This process is invoked for each 8x8 luma block of a macroblock with prediction mode equal to Intra_8x8 followed by the transform decoding process and picture construction process prior to deblocking for each 8x8 luma block.

Inputs to this process are the index of the 8x8 luma block with index luma8x8BlkIdx and constructed samples prior (in decoding order) to the deblocking filter process from adjacent luma blocks.

Output of this process are the prediction samples pred8x8L[ x, y ], with x, y = 0..7 for the 8x8 luma block with index luma8x8BlkIdx.

The position of the upper-left sample of a 8x8 luma block with index luma8x8BlkIdx inside the current macroblock is derived by invoking the inverse 8x8 luma block scanning process in subclause 6.4.2 with luma8x8BlkIdx as the input and the output being assigned to ( xO, yO ).

The 25 neighbouring samples p[ x, y ] that are constructed luma samples prior to the deblocking filter process, with x = -1, y = -1..7 and x = 0..15, y = -1, are derived as follows.

- The luma location ( xN, yN ) is specified by

xN = xO + x

yN = yO + y

- The derivation process for neighbouring locations in subclause Error: Reference source not found is invoked for luma locations with ( xN, yN ) as input and mbAddrN and ( xW, yW ) as output.

- Each sample p[ x, y ] with x = -1, y = -1..7 and x = 0..15, y = -1 is derived as follows.

- If any of the following conditions is true, the sample p[ x, y ] is marked as “not available for Intra_8x8 prediction”

-	mbAddrN is not available,

-	the macroblock mbAddrN is coded in Inter prediction mode and constrained_intra_pred_flag is equal to 1.

- x is greater than 7 and luma8x8BlkIdx is equal to 3

- Otherwise, the sample p[ x, y ] is marked as “available for Intra_8x8 prediction” and the luma sample at luma location ( xW, yW ) inside the macroblock mbAddrN is assigned to p[ x, y ].

When samples p[ x, -1 ], with x = 8..15 are marked as “not available for Intra_8x8 prediction,” and the sample p[ 7, -1 ] is marked as “available for Intra_8x8 prediction,” the sample value of p[ 7, -1 ] is substituted for sample values p[ x, -1 ], with x = 7..15 and samples p[ x, -1 ], with x = 7..15 are marked as “available for Intra_8x8 prediction”.

NOTE – Each block is assumed to be constructed into a frame prior to decoding of the next block.

Depending on Intra8x8PredMode[ luma8x8BlkIdx ], one of the Intra_8x8 prediction modes specified in subclauses 8.3.2.2.1 to 8.3.2.2.9 shall be used.

##### Specification of Intra_8x8_Vertical prediction mode

This Intra_8x8 prediction mode shall be used when Intra8x8PredMode[ luma8x8BlkIdx ] is equal to 0.

This mode shall be used only when the samples p[ x, -1 ] with x = 0..7 are marked as “available for Intra_8x8 prediction”.

The values of the prediction samples pred8x8L[ x, y ], with x, y = 0..7 are derived by

pred8x8L[ x, y ] = p[ x, -1 ], with x, y = 0..7

##### Specification of Intra_8x8_Horizontal prediction mode

This Intra_8x8 prediction mode shall be used when Intra8x8PredMode[ luma8x8BlkIdx ] is equal to 1.

This mode shall be used only when the samples p[ -1, y ], with y = 0..7 are marked as “available for Intra_8x8 prediction”.

The values of the prediction samples pred8x8L[ x, y ], with x, y = 0..7 are derived by

pred8x8L[ x, y ] = p[ -1, y ], with x,y = 0..7

##### Specification of Intra_8x8_DC prediction mode

This Intra_8x8 prediction mode shall be used when Intra8x8PredMode[ luma8x8BlkIdx ] is equal to 2.

The values of the prediction samples pred8x8L[ x, y ], with x, y = 0..7 are derived as follows.

- If all samples p[ x, -1 ], with x = 0..7 and p[ -1, y ], with y = 0..7 are marked as “available for Intra_8x8 prediction”, the values of the prediction samples pred8x8L[ x, y ], with x, y = 0..7 are derived by

pred8x8L[ x, y ] = ( p[ 0, -1 ] + p[ 1, -1 ] + p[ 2, -1 ] + p[ 3, -1 ] + 
                                p[ 4, -1 ] + p[ 5, -1 ] + p[ 6, -1 ] + p[ 7, -1 ] + 
                                p[ -1, 0 ] + p[ -1, 1 ] + p[ -1, 2 ] + p[ -1, 3 ] + 
		            p[ -1, 4 ] + p[ -1, 5 ] + p[ -1, 6 ] + p[ -1, 7 ] + 8 ) >> 4

- Otherwise, if samples p[ x, -1 ], with x = 0..7 are marked as “not available for Intra_8x8 prediction” and p[ -1, y ], with y = 0..7 are marked as “available for Intra_8x8 prediction”, the values of the prediction samples pred8x8L[ x, y ], with x, y = 0..7 are derived by

pred8x8L[ x, y ] = ( p[ -1, 0 ] + p[ -1, 1 ] + p[ -1, 2 ] + p[ -1, 3 ] + 
 			p[ -1, 4 ] + p[ -1, 5 ] + p[ -1, 6 ] + p[ -1, 7 ] + 4 ) >> 3

- Otherwise, if samples p[ -1, y ], with y = 0..7 are marked as “not available for Intra_8x8 prediction” and p[ x, -1 ], with x = 0 .. 7 are marked as “available for Intra_8x8 prediction”, the values of the prediction samples pred8x8L[ x, y ], with x, y = 0 .. 7 are derived by

pred8x8L[ x, y ] = ( p[ 0, -1 ] + p[ 1, -1 ] + p[ 2, -1 ] + p[ 3, -1 ] +
                                p[ 4, -1 ] + p[ 5, -1 ] + p[ 6, -1 ] + p[ 7, -1 ] + 4 ) >> 3

- Otherwise (all samples p[ x, -1 ], with x = 0..7 and p[ -1, y ], with y = 0..7 are marked as “not available for Intra_8x8 prediction”), the values of the prediction samples pred8x8L[ x, y ], with x, y = 0..7 are derived by

pred8x8L[ x, y ] = 128

NOTE – An 8x8 luma block can always be predicted using this mode.

##### Specification of Intra_8x8_Diagonal_Down_Left prediction mode

This Intra_8x8 prediction mode shall be used when Intra8x8PredMode[ luma8x8BlkIdx ] is equal to 3.

This mode shall be used only when the samples p[ x, -1 ] with x = 0..15 are marked as “available for Intra_8x8 prediction”.

The values of the prediction samples pred8x8L[ x, y ], with x, y = 0..7 are derived as follows.

- If x is equal to 7 and y is equal to 7,

pred8x8L[ x, y ] = ( p[ 14, -1 ] + 3 * p[15, -1 ] + 2 ) >> 2

- Otherwise (x is not equal to 7 or y is not equal to 7),

pred8x8L[ x, y ] = ( p[ x + y, -1 ] + 2 * p[ x + y + 1, -1 ] + p[ x + y + 2, -1 ] + 2 ) >> 2

##### Specification of Intra_8x8_Diagonal_Down_Right prediction mode

This Intra_8x8 prediction mode shall be used when Intra8x8PredMode[ luma8x8BlkIdx ] is equal to 4.

This mode shall be used only when the samples p[ x, -1 ] with x = 0..7 and p[ -1, y ] with y = -1..7 are marked as “available for Intra_8x8 prediction”.

The values of the prediction samples pred8x8L[ x, y ], with x, y = 0..7 are derived as follows.

- If x is greater than y,

pred8x8L[ x, y ] = ( p[ x - y - 2, -1] + 2 * p[ x - y - 1, -1 ] + p[ x - y, -1 ] + 2 ) >> 2

- Otherwise if x is less than y,

pred8x8L[ x, y ] = ( p[ -1, y - x - 2 ] + 2 * p[ -1, y - x - 1 ] + p[ -1, y - x ] + 2 ) >> 2

- Otherwise (x is equal to y),

pred8x8L[ x, y ] = ( p[ 0, -1 ] + 2 * p[ -1, -1 ] + p[ -1, 0 ] + 2 ) >> 2

##### Specification of Intra_8x8_Vertical_Right prediction mode

This Intra_8x8 prediction mode shall be used when Intra8x8PredMode[ luma8x8BlkIdx ] is equal to 5.

This mode shall be used only when the samples p[ x, -1 ] with x = 0..7 and p[ -1, y ] with y = -1..7 are marked as “available for Intra_8x8 prediction”.

Let the variable zVR be set equal to 2 * x – y.

The values of the prediction samples pred8x8L[ x, y ], with x, y = 0..7 are derived as follows.

- If zVR is equal to 0, 2, 4,  6, 8, 10, 12, or 14

pred8x8L[ x, y ] = ( p[ x - ( y >> 1 ) - 1, -1 ] + p[ x - ( y >> 1 ), -1 ] + 1 ) >> 1

- Otherwise, if zVR is equal to 1, 3, 5, 7, 9, 11, or 13

pred8x8L[ x, y ] = ( p[ x - ( y >> 1 ) - 2, -1] + 2 * p[ x - ( y >> 1 ) - 1, -1 ] + p[ x - ( y >> 1 ), -1 ] + 2 ) >> 2

- Otherwise, if zVR is equal to -1,

pred8x8L[ x, y ] = ( p[ -1, 0 ] + 2 * p[ -1, -1 ] + p[ 0, -1 ] + 2 ) >> 2

- Otherwise (zVR is equal to -2, -3, -4, -5, -6, or -7),

pred8x8L[ x, y ] = ( p[ -1, y - 2*x - 1 ] + 2 * p[ -1, y - 2*x - 2 ] + p[ -1, y - 2*x - 3 ] + 2 ) >> 2

##### Specification of Intra_8x8_Horizontal_Down prediction mode

This Intra_8x8 prediction mode shall be used when Intra8x8PredMode[ luma8x8BlkIdx ] is equal to 6.

This mode shall be used only when the samples p[ x, -1 ] with x = 0..7 and p[ -1, y ] with y = -1..7 are marked as “available for Intra_8x8 prediction”.

Let the variable zHD be set equal to 2 * y – x.

The values of the prediction samples pred8x8L[ x, y ], with x, y = 0..7 are derived as follows.

- If zHD is equal to 0, 2, 4, 6, 8, 10, 12, or 14

pred8x8L[ x, y ] = ( p[ -1, y -( x >> 1 ) - 1 ] + p[ -1, y - ( x >> 1 ) ] + 1 ) >> 1

- Otherwise, if zHD is equal to 1, 3, 5, 7, 9, 11, or 13

pred8x8L[ x, y ] = ( p[ -1, y - ( x >> 1 ) - 2 ] + 2 * p[ -1, y - ( x >> 1 ) - 1 ] + p[ -1, y - ( x >> 1 ) ] + 2 ) >> 2

- Otherwise, if zHD is equal to -1,

pred8x8L[ x, y ] = ( p[ -1, 0 ] + 2 * p[ -1, -1 ] + p[ 0, -1 ] + 2 ) >> 2

- Otherwise (zHD is equal to –2, -3, -4, -5, -6, -7),

pred8x8L[ x, y ] = ( p[ x - 2*y - 1, -1 ] + 2 * p[ x - 2*y - 2, -1 ] + p[ x - 2*y - 3, -1 ] + 2 ) >> 2

##### Specification of Intra_8x8_Vertical_Left prediction mode

This Intra_8x8 prediction mode shall be used when Intra8x8PredMode[ luma8x8BlkIdx ] is equal to 7.

This mode shall be used only when the samples p[ x, -1 ] with x = 0..15 are marked as “available for Intra_8x8 prediction”.

The values of the prediction samples pred8x8L[ x, y ], with x, y = 0..7 are derived as follows.

- If y is equal to 0, 2, 4 or 6

pred8x8L[ x, y ] = ( p[ x + ( y >> 1 ), -1 ] + p[ x + ( y >> 1 ) + 1, -1 ] + 1) >> 1

- Otherwise (y is equal to 1, 3, 5, 7),

pred8x8L[ x, y ] = ( p[ x + ( y >> 1 ), -1 ] + 2 * p[ x + ( y >> 1 ) + 1, -1 ] + p[ x + ( y >> 1 ) + 2, -1 ] + 2 ) >>2

##### Specification of Intra_8x8_Horizontal_Up prediction mode

This Intra_8x8 prediction mode shall be used when Intra8x8PredMode[ luma8x8BlkIdx ] is equal to 8.

This mode shall be used only when the samples p[ -1, y ] with y = 0..7 are marked as “available for Intra_8x8 prediction”.

Let the variable zHU be set equal to x + 2 * y.

The values of the prediction samples pred8x8L[ x, y ], with x, y = 0..7 are derived as follows:

- If zHU is equal to 0, 2, 4, 6, 8, 10, or 12

pred8x8L[ x, y ] = ( p[ -1, y + ( x >> 1 ) ] + p[ -1, y + ( x >> 1 ) + 1 ] + 1 ) >> 1

- Otherwise, if zHU is equal to 1, 3, 5, 7, 9, or 11

pred8x8L[ x, y ] = ( p[ -1, y + ( x >> 1 ) ] + 2 * p[ -1, y + ( x >> 1 ) + 1 ] + p[ -1, y + ( x >> 1 ) + 2 ] + 2 ) >>2

- Otherwise, if zHU is equal to 13,

pred8x8L[ x, y ] = ( p[ -1, 6 ] + 3 * p[ -1, 7 ] + 2 ) >> 2

- Otherwise (zHU is greater than 13),

pred8x8L[ x, y ] = p[ -1, 7]

## 8.5	Transform coefficient decoding process and picture construction process prior to deblocking filter process

Inputs to this process are Intra16x16DCLevel (if available), Intra16x16ACLevel (if available), LumaLevel (if available), LumaLevel64 (if available), ChromaDCLevel, ChromaACLevel, and available Inter or Intra prediction sample arrays for the current macroblock for the applicable component predL, predCb, or predCr.

NOTE – When decoding a macroblock in Intra_4x4 (or Intra 8x8) prediction mode, the luma component of the macroblock prediction array may not be complete, since for each 4x4 (8x8) luma block, the Intra_4x4 (8x8) prediction process for luma samples as specified in subclause Error: Reference source not found1 (8.3.2) and the process specified in this subclause are iterated.

NOTE – When decoding a macroblock in Intra_4x4 (or Intra 8x8) prediction mode, the luma component of the macroblock constructed sample arrays prior to the deblocking filter process may not be complete, since for each 4x4 (8x8) luma block, the Intra_4x4 (8x8) prediction process for luma samples as specified in subclause Error: Reference source not found (8.3.2) and the process specified in this subclause are iterated.

When the current macroblock is coded as P_Skip or B_Skip, all values of LumaLevel, LumaLevel64, ChromaDCLevel, ChromaACLevel are set equal to 0 for the current macroblock.

From Table 8.x, lumaTransformSize is derived.

Table 8.x – Derivation of lumaTransformSize

| Macroblock Type | transform_8x8_mode_flag | all_8x8 | lumaTransformSize |
| --- | --- | --- | --- |
| Intra_4x4 | Na | na | 4 |
| Intra_8x8 | Na | na | 8 |
| Intra_16x16 | Na | na | 4 |
| P_16x16 | 0 | na | 4 |
|  | 1 | na | 8 |
| P_8x16 | 0 | na | 4 |
|  | 1 | na | 8 |
| P_16x8 | 0 | na | 4 |
|  | 1 | na | 8 |
| P_8x8 | 0 | na | 4 |
|  | 1 | 0 | 4 |
|  | 1 | 1 | 8 |
| B_Direct | 0 | na | 4 |
|  | 1 | na | 8 |
| B_16x16 | 0 | na | 4 |
|  | 1 | na | 8 |
| B_16x8 | 0 | na | 4 |
|  | 1 | na | 8 |
| B_8x16 | 0 | na | 4 |
|  | 1 | na | 8 |
| B_8x8 | 0 | na | 4 |
|  | 1 | 0 | 4 |
|  | 1 | 1 | 8 |

    - Specification of transform decoding process for 4x4 luma residual blocks

When the current macroblock prediction mode is not equal to Intra_16x16 and lumaTransformSize is equal to 4, …

    - Specification of transform decoding process for 8x8 luma residual blocks

When the current macroblock prediction mode is not equal to Intra_16x16 and lumaTransformSize is equal to 8, the variable LumaLevel64 contains the levels for the luma transform coefficients. For an 8x8 luma block indexed by luma8x8BlkIdx = 0..3, the following ordered steps are specified.

- The inverse transform coefficient scanning process as described in subclause 8.5.4? is invoked with LumaLevel64[ luma8x8BlkIdx ] as the input and the two-dimensional array c as the output.

- The scaling and transformation process for residual 8x8 blocks as specified in subclause ?is invoked with c as the input and r as the output.

- The position of the upper-left sample of an 8x8 luma block with index luma8x8BlkIdx inside the macroblock is derived by invoking the inverse 8x8 luma block scanning process in subclause Error: Reference source not found2.2 with luma8x8BlkIdx as the input and the output being assigned to ( xO, yO ).

- The 8x8 array u with elements uij for i, j = 0..7 is derived as

uij = Clip1( predL[ xO + j, yO + i ] + rij ) 		(8-1)

- The picture construction process prior to deblocking filter process in subclause 8.5.10? is invoked with luma8x8BlkIdx, u as the input and S’ as the output.

### Inverse scanning process for 4x4 transform coefficients

### Inverse scanning process for 8x8 Luma transform coefficients

Input to this process is a list of 64 values.

Output of this process is a variable c containing a two-dimensional array of 8x8 values with level assigned to locations in the transform block.

The decoding process maps the sequence of transform coefficient levels to the transform coefficient level positions. For this mapping, the two inverse scanning patterns shown in Error: Reference source not found9 are used.

The inverse zig-zag scan shall be used for frame macroblocks and the inverse field scan shall be used for field macroblocks.

- a) Zig-zag scan

b) Field scan

Figure 8-9 – 8x8 Scan Orders

Table 8-43 provides the mapping from the index idx of input list of 64 elements to indices i and j of the two-dimensional array c.

- Table 8-43 – Specification of mapping of idx to cij for zig-zag and field scan

| idx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| zig-zag | c00 | c01 | c10 | c20 | c11 | c02 | c03 | c12 | c21 | c30 | c40 | c31 | c22 | c13 | c04 | c05 |
| field | c00 | c10 | c20 | c01 | c11 | c30 | c40 | c21 | c02 | c12 | c31 | c50 | c60 | c41 | c22 | c04 |

| idx | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| zig-zag | c14 | c23 | c32 | c41 | c50 | c60 | c51 | c42 | c33 | c24 | c15 | c06 | c07 | c16 | c25 | c34 |
| field | c14 | c32 | c51 | c70 | c61 | c42 | c23 | c04 | c14 | c33 | c52 | c71 | c62 | c43 | c24 | c05 |

| idx | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| zig-zag | c43 | c52 | c61 | c70 | c71 | c62 | c53 | c44 | c35 | c26 | c17 | c27 | c36 | c45 | c54 | c63 |
| field | c15 | c34 | c53 | c72 | c63 | c44 | c25 | c06 | c16 | c35 | c54 | c73 | c64 | c45 | c26 | c07 |

| idx | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| zig-zag | c72 | c73 | c64 | c55 | c46 | c37 | c47 | c56 | c65 | c74 | c75 | c66 | c57 | c67 | c76 | c77 |
| field | c17 | c36 | c55 | c74 | c65 | c46 | c27 | c37 | c56 | c75 | c66 | c47 | c57 | c76 | c67 | c77 |

    - Derivation process for the quantisation parameters and scaling function

The function LevelScale64( m, i, j ) is specified as follows:

LevelScale64(m,i,j) =	Vm0 for (i,j) in {(0,0), (0,4), (4,0), (4,4)},

Vm1 for (i,j) in {(1,1), (1,3), (1,5), (1,7),

(3,1), (3,3), (3,5), (3,7),

(5,1), (5,3), (5,5), (5,7),

(7,1), (7,3), (7,5), (7,7)},

Vm2 for (i,j) in {(2,2), (2,6), (6,2), (6,6)},

Vm3 for (i,j) in {(0,1), (0,3), (0,5), (0,7),

(2,1), (2,3), (2,5), (2,7),

(4,1), (4,3), (4,5), (4,7),

(6,1), (6,3), (6,5), (6,7),

(1,0), (1,2), (1,4), (1,6),

(3,0), (3,2), (3,4), (3,6),

(5,0), (5,2), (5,4), (5,6),

(7,0), (7,2), (7,4), (7,6)},

Vm4 for (i,j) in {(0,2), (0,6), (4,2), (4,6),

(2,0), (2,4), (6,0), (6,4)},

Vm5 otherwise

where the first and second subscripts of v are row and column indices, respectively, of the matrix specified as:

v=1091691312111018101413121120121615141222131817161426152019181629172321

### Scaling and transformation process for residual 8x8 blocks

Input to this process is an 8x8 array c with elements cij which is an array relating to an 8x8 residual block of the luma component.

Outputs of this process are residual sample values as 8x8 array r with elements rij.

- Scaling of 8x8 block transform coefficient levels cij proceeds as follows.

dij = (cij * LevelScale64 (QPY % 6, i, j)) << (QPY / 6)	,   with i, j = 0..7

The bitstream shall not contain data that results in any element dij of d with i, j = 0..7 that exceeds the range of integer values from –215 to 215–1, inclusive.

The transform process shall convert the block of scaled transform coefficients to a block of output samples in a manner mathematically equivalent to the following.

First, each (horizontal) row of scaled transform coefficients is transformed using a one-dimensional inverse transform as follows.

A set of intermediate values eij is computed as follows.

ei0 = di0 + di4,   with   i = 0..7

ei1 = -di3 + di5 – di7 – (di7 >> 1),   with   i = 0..7

ei2 = di0 – di4,   with   i = 0..7

ei3 = di1 + di7 – di3 – (di3 >> 1),   with   i = 0..7

ei4 = ( di2 >> 1 ) – di6,   with   i = 0..7

ei5 = -di1 + di7 + di5 + (di5 >> 1),   with   i = 0..7

ei6 = di2 + ( di6 >> 1 ),   with   i = 0..7

ei7 = di3 + di5 + di1 + (di1 >> 1),   with   i = 0..7

The bitstream shall not contain data that results in any element eij of e with i, j = 0..7 that exceeds the range of integer values from –215 to 215–1, inclusive.

A second set of intermediate results fij is computed from the intermediate values eij as follows.

fi0 = ei0 + ei6,   with   i = 0..7

fi1 = -(ei1 >> 2) + ei7,   with   i = 0..7

fi2 = ei2 + ei4,   with   i = 0..7

fi3 = ei3 + (ei5 >> 2)  with   i = 0..7

fi4 = ei4 – ei2,   with   i = 0..7

fi5 = (ei3 >> 2) - ei5  with   i = 0..7

fi6 = ei0 – ei6,   with   i = 0..7

fi7 = -(ei1 >> 2) + ei7  with   i = 0..7

The bitstream shall not contain data that results in any element fij of f with i, j = 0..7 that exceeds the range of integer values from –215 to 215–1, inclusive.

Then, the transformed result gij is computed from these intermediate values fij as follows.

gi0 = fi0 + fi7,   with   i = 0..7

gi1 = fi2 + fi5,   with   i = 0..7

gi2 = fi3 + fi4,   with   i = 0..7

gi3 = fi1 + fi6,   with   i = 0..7

gi4 = fi6 - fi1,   with   i = 0..7

gi5 = fi4 - fi3,   with   i = 0..7

gi6 = fi2 - fi5,   with   i = 0..7

gi7 = fi0 - fi7,   with   i = 0..7

The bitstream shall not contain data that results in any element gij of f with i, j = 0..7 that exceeds the range of integer values from –215 to 215–1, inclusive.

Then, each (vertical) column of the resulting matrix is transformed using the same one-dimensional inverse transform as follows.

A set of intermediate values hij is computed from the horizontally transformed value gij as follows.

hi0 = gi0 + gi4,   with   i = 0..7

hi1 = -gi3 + gi5 – gi7 – (gi7 >> 1),   with   i = 0..7

hi2 = gi0 – gi4,   with   i = 0..7

hi3 = gi1 + gi7 – gi3 – (gi3 >> 1),   with   i = 0..7

hi4 = ( gi2 >> 1 ) – gi6,   with   i = 0..7

hi5 = -gi1 + gi7 + gi5 + (gi5 >> 1),   with   i = 0..7

hi6 = gi2 + ( gi6 >> 1 ),   with   i = 0..7

hi7 = gi3 + gi5 + gi1 + (gi1 >> 1),   with   i = 0..7

The bitstream shall not contain data that results in any element hij of g with i, j = 0..7 that exceeds the range of integer values from –215 to 215–1, inclusive.

A second set of intermediate results kij is computed from the intermediate values hij as follows.

ki0 = hi0 + hi6,   with   i = 0..7

ki1 = -(hi1 >> 2) + hi7,   with   i = 0..7

ki2 = hi2 + hi4,   with   i = 0..7

ki3 = hi3 + (hi5 >> 2)  with   i = 0..7

ki4 = hi4 – hi2,   with   i = 0..7

ki5 = (hi3 >> 2) - hi5  with   i = 0..7

ki6 = hi0 – hi6,   with   i = 0..7

ki7 = -(hi1 >> 2) + hi7  with   i = 0..7

The bitstream shall not contain data that results in any element kij of h with i, j = 0..7 that exceeds the range of integer values from –215 to 215–33, inclusive.

Then, the transformed result mij is computed from these intermediate values kij as follows.

mi0 = ki0 + ki7,   with   i = 0..7

mi1 = ki2 + ki5,   with   i = 0..7

mi2 = ki3 + ki4,   with   i = 0..7

mi3 = ki1 + ki6,   with   i = 0..7

mi4 = ki6 - ki1,   with   i = 0..7

mi5 = ki4 - ki3,   with   i = 0..7

mi6 = ki2 - ki5,   with   i = 0..7

mi7 = ki0 - ki7,   with   i = 0..7

After performing both the one-dimensional horizontal and the one-dimensional vertical inverse transforms to produce an array of transformed samples, the final constructed residual sample values shall be derived as

rij = (mij + 26) >> 7 with i, j = 0..7

    - Picture construction process for non 8x8 luma residuals prior to deblocking filter process

    - Picture construction process for 8x8 luma residuals prior to deblocking filter process

Inputs to this process are

- luma8x8BlkIdx

- a constructed residual sample 8x8 array u with elements uij which is a luma residual block

- the prediction sample 8x8 array predL

Outputs of this process are constructed luma sample blocks S'L  prior to the deblocking filter process.

The position of the upper-left luma sample of the current macroblock is derived by invoking the inverse macroblock scanning process in subclause Error: Reference source not found with CurrMbAddr as input and the output being assigned to ( xP, yP ).

For each sample uij of the 8x8 luma block, the following applies.

- The position of the upper-left sample of an 8x8 luma block with index luma8x8BlkIdx inside the macroblock is derived by invoking the inverse 8x8 luma block scanning process in subclause Error: Reference source not found2.2 with luma8x8BlkIdx as the input and the output being assigned to ( xO, yO ).

- Depending on the variable MbaffFrameFlag, the following applies.

- If MbaffFrameFlag is equal to 1 and the current macroblock is a field macroblock

S'L[ xP + xO + j, yP + 2 * ( yO + i ) ] = uij   with i, j = 0..7

- Otherwise (MbaffFrameFlag is equal to 0 or the current macroblock is a frame macroblock),

S'L[ xP + xO + j, yP + yO + i ] = uij   with i, j = 0..7

  - Deblocking filter process

A conditional filtering shall be applied to all 4x4 NxN (where N=lumaTransformSize for luma, and N=4 for chroma) block edges of a picture, except edges at the boundary of the picture …

The deblocking filter process is invoked for the luma and chroma components separately. For each macroblock, vertical edges are filtered first, from left to right, and then horizontal edges are filtered from top to bottom. The luma deblocking filter process is performed on two (when N=8) or four (when N=4) 16-sample edges …

- Given the variables lumaTransformSize, fieldModeMbFlag, filterInternalEdgesFlag, filterLeftMbEdgeFlag and filterTopMbEdgeFlag the deblocking filtering is controlled as follows.

- When filterLeftMbEdgeFlag is equal to 1, the filtering of the left vertical luma edge is specified as follows.

- The process specified in subclause Error: Reference source not found is invoked with mbAddr, chromaEdgeFlag = 0, verticalEdgeFlag = 1, fieldModeFilteringFlag = fieldModeMbFlag, and (xEk, yEk) = (0, k) with k = 0..15 as input and S'L as output.

- When filterInternalEdgesFlag is equal to 1, the filtering of the internal vertical luma edges is specified as follows.

- If lumaTransformSize is equal to 4, Tthe process specified in subclause Error: Reference source not found is invoked with mbAddr, chromaEdgeFlag = 0, verticalEdgeFlag = 1, fieldModeFilteringFlag = fieldModeMbFlag, and (xEk, yEk) = (4, k) with k = 0..15 as input and S'L as output.

- The process specified in subclause Error: Reference source not found is invoked with mbAddr, chromaEdgeFlag = 0, verticalEdgeFlag = 1, fieldModeFilteringFlag = fieldModeMbFlag, and (xEk, yEk) = (8, k) with k = 0..15 as input and S'L as output.

- If lumaTransformSize is equal to 4, Tthe process specified in subclause Error: Reference source not found is invoked with mbAddr, chromaEdgeFlag = 0, verticalEdgeFlag = 1, fieldModeFilteringFlag = fieldModeMbFlag, and (xEk, yEk) = (12, k) with k = 0..15 as input and S'L as output.

- When filterTopMbEdgeFlag is equal to 1, the filtering of the top horizontal luma edge is specified as follows.

- If MbaffFrameFlag is equal to 1, (mbAddr % 2) is equal to 0, mbAddr is greater than or equal to 2 * PicWidthInMbs, the macroblock mbAddr is a frame macroblock, and the macroblock (mbAddr - 2 * PicWidthInMbs + 1) is a field macroblock, the following applies.

-	The process specified in subclause Error: Reference source not found is invoked with mbAddr, chromaEdgeFlag = 0, verticalEdgeFlag = 0, fieldModeFilteringFlag = 1, and (xEk, yEk) = (k, 0) with k = 0..15 as input and S'L as output.

-	The process specified in subclause Error: Reference source not found is invoked with mbAddr, chromaEdgeFlag = 0, verticalEdgeFlag = 0, fieldModeFilteringFlag = 1, and (xEk, yEk) = (k, 1) with k = 0..15 as input and S'L as output.

- Otherwise, the process specified in subclause Error: Reference source not found is invoked with mbAddr, chromaEdgeFlag = 0, verticalEdgeFlag = 0, fieldModeFilteringFlag = fieldModeMbFlag, and (xEk, yEk) = (k, 0) with k = 0..15 as input and S'L as output.

- When filterInternalEdgesFlag is equal to 1, the filtering of the internal horizontal luma edges is specified as follows.

- If lumaTransformSize is equal to 4, Tthe process specified in subclause Error: Reference source not found is invoked with mbAddr, chromaEdgeFlag = 0, verticalEdgeFlag = 0, fieldModeFilteringFlag = fieldModeMbFlag, and (xEk, yEk) = (k, 4) with k = 0..15 as input and S'L as output.

- The process specified in subclause Error: Reference source not found is invoked with mbAddr, chromaEdgeFlag = 0, verticalEdgeFlag = 0, fieldModeFilteringFlag = fieldModeMbFlag, and (xEk, yEk) = (k, 8) with k = 0..15 as input and S'L as output.

- If lumaTransformSize is equal to 4, TtThe process specified in subclause Error: Reference source not found is invoked with mbAddr, chromaEdgeFlag = 0, verticalEdgeFlag = 0, fieldModeFilteringFlag = fieldModeMbFlag, and (xEk, yEk) = (k, 12) with k = 0..15 as input and S'L as output.

CABAC parsing process for slice data

- Table 9-5 – Association of ctxIdx and syntax elements for each slice type in the initialisation process

| macroblock_layer( ) | mb_type | Error: Reference source not found, Error: Reference source not found, Error: Reference source not found. | 0-10 | 3-10 | 14-20 | 27-35 |
| --- | --- | --- | --- | --- | --- | --- |
|  | intra_pred_size | Table 9-17 | n/a | 399 | 399 | 399 |
|  | coded_block_pattern (luma) | Error: Reference source not found | 73-76 | 73-76 | 73-76 | 73-76 |
|  | coded_block_pattern (chroma) | Error: Reference source not found | 77-84 | 77-84 | 77-84 | 77-84 |
|  | mb_qp_delta | Table 9-6 | 60-63 | 60-63 | 60-63 | 60-63 |
| mb_pred( ) | prev_intra4x4_pred_mode_flag | Table 9-6 | 68 | 68 | 68 | 68 |
|  | rem_intra4x4_pred_mode | Table 9-6 | 69 | 69 | 69 | 69 |
|  | prev_intra8x8_pred_mode_flag | Table 9-17 | 400 | 400 | 400 | 400 |
|  | rem_intra8x8_pred_mode | Table 9-17 | 401 | 401 | 401 | 401 |
|  | intra_chroma_pred_mode | Table 9-6 | 64-67 | 64-67 | 64-67 | 64-67 |

| residual_block_cabac( ) | coded_block_flag | Table 9-18, Table 9-24. | 85-104, 402-405 | 85-104, 402-405 | 85-104, 402-405 | 85-104, 402-405 |
| --- | --- | --- | --- | --- | --- | --- |
|  | significant_coeff_flag[ ] | Error: Reference source not found, Error: Reference source not found. | 105-165, 277-337 | 105-165, 277-337 | 105-165, 277-337 | 105-165, 277-337 |
|  | last_significant_coeff_flag[ ] | Error: Reference source not found, Error: Reference source not found. | 166-226, 338-398 | 166-226, 338-398 | 166-226, 338-398 | 166-226, 338-398 |
|  | coeff_abs_level_minus1[ ] | Error: Reference source not found | 227-275 | 227-275 | 227-275 | 227-275 |

- Table 9-6 – Values of variables m and n for ctxIdx from 60 to 69, and 399 to 401

| Initialisation variables | ctxIdx |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 60 | 61 | 62 | 63 | 64 | 65 | 66 | 67 | 68 | 69 | 399 | 400 | 401 |
| M | 0 | 0 | 0 | 0 | -9 | 4 | 0 | -7 | 13 | 3 | 0 | 13 | 3 |
| N | 41 | 63 | 63 | 63 | 83 | 86 | 97 | 72 | 41 | 62 | 63 | 41 | 62 |

9.3.2 Binarization Process

The possible values of the context index ctxIdx are in the range 0 to 398401

Table 9-7 – Syntax elements and associated types of binarization, maxBinIdxCtx, and ctxIdxOffset

| intra_pred_size | FL, cMax=1 | 0 | 399 |
| --- | --- | --- | --- |
| prev_intra8x8_pred_mode_flag | FL, cMax=7 | 0 | 400 |
| rem_intra8x8_pred_mode | FL, cMax=1 | 0 | 401 |

Table 9-8 – Binarization for macroblock types in I slices

Change

| 0 (I_4x4NxN) | 0 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Table 9-9 – Assignment of ctxIdxInc to binIdx for all ctxIdxOffset values except those related to the syntax elements coded_block_flag, significant_coeff_flag, last_significant_coeff_flag, and coeff_abs_level_minus1

| 399 | 0 | na | na | na | na | na | na |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 400 | 0 | na | na | na | na | na | na |
| 401 | 0 | 0 | 0 | na | na | na | na |

- Table 9-10 – Assignment of ctxIdxBlockCatOffset to ctxBlockCat for syntax elements coded_block_flag, significant_coeff_flag, last_significant_coeff_flag, and coeff_abs_level_minus1

| Syntax element | ctxBlockCat (as specified in Table 9-11) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| coded_block_flag | 0 | 4 | 8 | 12 | 12 | 16 |
| significant_coeff_flag | 0 | 15 | 29 | 44 | 44 | 47 |
| last_significant_coeff_flag | 0 | 15 | 29 | 44 | 44 | 47 |
| coeff_abs_level_minus1 | 0 | 10 | 20 | 30 | 30 | 39 |

          - Derivation process of ctxIdxInc for the syntax element mb_type

- ctxIdxOffset is equal to 3 and mb_type for the macroblock mbAddrN is equal to I_4x4NxN

###### Derivation process of ctxIdxInc for the syntax element coded_block_flag

- Otherwise, if ctxBlockCat is equal to 3, luma8x8BlkIdx

- Otherwise, if ctxBlockCat is equal to 34, the chroma component index iCbCr

- Otherwise (ctxBlockCat is equal to 45), chroma4x4BlkIdx and the chroma component index compIdx

- Otherwise, if ctxBlockCat is equal to 3, the following applies.

- The derivation process for neighbouring 8x8 luma blocks specified in subclause Error: Reference source not found2 is invoked with luma8x8BlkIdx as input and the output is assigned to mbAddrN, luma8x8BlkIdxN (with N being either A or B).

- The variable transBlockN is derived as follows.

- If mbAddrN is available, the macroblock mbAddrN is not skipped, mb_type for the macroblock mbAddrN is not equal to I_PCM, and (( CodedBlockPatternLuma >> luma8x8BlkIdxN ) & 1 ) is not equal to 0 for the macroblock mbAddrN, the 8x8 luma block with luma8x8BlkIdxN of macroblock mbAddrN is assigned to transBlockN.

- Otherwise, transBlockN is marked as not available.

- Otherwise, if ctxBlockCat is equal to 34, the following applies.

- Otherwise (ctxBlockCat is equal to 45), the following applies.

        - Assignment process of ctxIdxInc using prior decoded bin values

- Table 9-11 – Specification of ctxBlockCat for the different blocks

| Block description | maxNumCoeff | ctxBlockCat |
| --- | --- | --- |
| block of luma DC transform coefficient levels (for macroblock coded in Intra_16x16 prediction modei.e., list Intra16x16DCLevel as described in 7.4.5.3) | 16 | 0 |
| block of luma AC transform coefficient levels (for macroblock coded in Intra_16x16 prediction mode i.e., list Intra16x16ACLevel[i] as described in 7.4.5.3) | 15 | 1 |
| block of 16 luma transform coefficient levels (for macroblock not coded in Intra_16x16 prediction mode i.e., list LumaLevel[i] as described in 7.4.5.3) | 16 | 2 |
| block of 64 luma transform coefficient levels (i.e., list LumaLevel64[i] as described in 7.4.5.3) | 64 | 3 |
| block of chroma DC transform coefficient levels | 4 | 34 |
| block of chroma AC transform coefficient levels | 15 | 45 |

Change

The scanning position for frame coded blocks relates to the zig-zag scan; the scanning position for field coded blocks relates to the field scan relates to the scan orders as specified in subclause 8.5.4.

#### JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Sand Video |  |  |
| Mailing address | 200 Brickstone, Fourth Floor Andover, MA, 01810 |  |  |
| Country | ##### USA |  |  |
| Contact person | Steve Gordon |  |  |
| Telephone | 978.475.8990 x222 |  |  |
| Fax | 978.475.8995 |  |  |
| Email | SteveGordon@SandVideo.com |  |  |
| Place and date of submission | August 27, 2003 for JVT San Diego (Sept 2-7) |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Simplified Use of 8x8 Transforms |  |
| Contribution number |  | I022 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
| X | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: