| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 9th Meeting: San Diego, CA, USA, 2-5 September, 2003 | Document: JVT-I024 Filename: JVT-I024r1.doc |
| --- | --- |

| Title: | Comment on Lossless Video Coding for JVT Professional Extension |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Shijun Sun 5750 NW Pacific Rim Blvd Camas, WA 98607, USA | Tel: Email: | 360-817-7531 ssun@sharplabs.com |
| Source: | Sharp Labs of America |  |  |

_____________________________

# Introduction

This document is to support the lossless coding proposal in document JVT-I012 [1]. The basic idea of this approach is to skip transform and quantization stage to circumvent possible losses, and apply the entropy coding mechanisms directly to the prediction error signal. We have made similar lossless coding proposal in document JVT-C023 [2].  Mathias Wien of RWTH Aachen has successfully verified the previous proposal in document JVT-D054 [3].

A mathematically lossless coding option is critical to many applications, such as film archiving, medical imaging, scientific research, and national defense, etc..  The required modification to the standard for supporting the lossless coding option is fairly simple.

Woo-Shik Kim of Samsung provided an implementation of the lossless RGB coding proposal in JVT-I012 [1] based on JM61e. Sharp Labs of America has tested the software and verified that there is no any mismatch for the tested sequence segments.

## Experiments

Test conditions:

- Sequences: Crew, Harbour (1280x720@60p, 8-bit and 10-bit RGB, first 90 frames only)

- Search range: 32

- Entropy Coding: CABAC

- RD-optimization: ON

- Number of reference frames: 3

- GOP structure: N 30, M 3 (IBBP…)

Lossless coding results:

| Sequences | Crew_8 | Crew_10 | Harbour_8 | Harbour_10 |
| --- | --- | --- | --- | --- |
| Bitrate (Mbps) | 560 | 899 | 601 | 921 |

## Conclusion

We suggest that JVT adopt the lossless RGB coding feature into the Professional Extension.

References

- [1] W. Kim, D. Cho and H. M. Kim, “Proposal for the unsolved issues in Professional Extensions,” JVT-I012, September 2003.

- [2] S. Sun, “Lossless Coding and QP Range Selection,” JVT-C023, May 2002.

- [3] M. Wien, “Verification of Sharps Lossless Coding Proposal,” JVT-D054, July 2002.