| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) - 7th Meeting: San Diego, California, USA - 2-5 September, 2003 | Document: input.doc Filename: input.doc.doc |
| --- | --- |

| Title: | 16-Bit DC coefficient reconstruction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Louis Kerofsky Sharp Labs of America 5750 NW Pacific Rim Blvd. Camas WA 98607 | Tel: Email: | 360-817-7644 lkerofsky@sharplabs.com |
| Source: | Sharp Labs of America |  |  |

_____________________________

# Introduction

Item 22 of the errata in [1] describes situations where reconstructing the DC coefficient from the quantized level appears to require intermediates exceeding 16-bits. A straightforward implementation may exceed 16-bits calculating the DC luma when QP is less than 12 or the DC chroma when QP is less than 6.    We present methods for calculating the DC coefficient with intermediates not exceeding 16-bits.

- Current Text

The relevant text is given in sections 8.5.6 and 8.5.7 primarily by equations 8-256 and 8-259.  These equations are repeated below.

dcYij=( fij∗LevelScale( QPY % 6, 0, 0 )+21−QPY/6)>>( 2 −QPY / 6 ), with i,j=0..3	(8-1)

dcCij=( fij* LevelScale(QPC % 6, 0, 0 ))>>1, with i,j=0, 1	(8-2)

The text limits the values of dcYij and dcCij but the intermediates appear to require more bits due to the right shifts present in these equations.

# 16-bit Calculation

To compute the DC coefficients when QPY<12 or QPC<6with 16-bit intermediates we represent the value f as a multiple of a power of 2 plus a remainder.

For DC luma , the remainder is chosen such that it has the same sign as the level.

Given a level f, define quotient q by:

q=f/(2−QPY/6) if f>0

q=f/(2−QPY/6) if f<0

Define remainder r by

r=f−q<<(2−QPY/6)

The values of f, q and r satisfy:

f=q<<(2−QPY/6)+rsign(r)=sign(f)

Letting L=LevelScale(QPY%6,0,0), equation 8-256 can be rewritten as:

dcY=( f*L+21−QPY/6)>>( 2 −QPY / 6 )dcY=((q<<(2-QPY/6)+r)*L+21−QPY/6)>>( 2 −QPY / 6 )dcY=q*L+( r*L+21−QPY/6)>>( 2 −QPY / 6 )

The expressions in square brackets have the same sign, or are zero, and sum to give a result which has 16-bit dynamic range therefore the first expression has 16-bit dynamic range.  The value L is at most 18 and the value r has absolute value less than 4 therefore all intermediates of the second expression have 16-bit dynamic range (much less).  Using the quotient and remainder decomposition of f, this last expression for dcY can be computed with intermediates having 16-bit dynamic range.

For DC chroma, there is no rounding offset prior to the left shift making analysis simpler.

Given level f, define quotient q and remainder r by:

q=f>>1r=f−q<<1

The values of f, q and r satisfy:

f=q<<1+r

Letting L=LevelScale(QPC%6,0,0), equation 8-259 can be rewritten as:

dcC=( f* L)>>1dcC=(((q<<1)+r)* L)>>1dcC=q*L+(r*L)>>1

- As with dcY, this last expression allows dcC to be computed using 16-bit intermediates.

References

- [1] “Errata List with Revision-Marked Corrections for H.264/AVC,” ITU-T SG16 Q.6 Document JVT-I010d1, September 2003

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Sharp Labs of America |  |  |
| Mailing address | 5750 NW Pacific Rim Blvd. CAMAS WA 98607 |  |  |
| Country | USA |  |  |
| Contact person | Louis Kerofsky |  |  |
| Telephone | 360-817-7644 |  |  |
| Fax | 360-817-8436 |  |  |
| Email | lkerofsky@sharplabs.com |  |  |
| Place and date of submission | Geneva, October 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | 16-bit dequantization |  |
| Contribution number |  | JVT-J000 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: