| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) JVT Meeting: San Diego, CA, USA, 2-5 September, 2003 | Document: JVT-I029 Filename: JVT-I029.doc |
| --- | --- |

| Title: | Additional Experiments on Intra DC Prediction for 4:2:2 Video |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Teruhiko Suzuki Gate City Osaki East Tower 20 F 1-11-1 Osaki, Shinagawa-ku Tokyo, 141-0032, Japan Kyohei Koyabu and Katsumi Tahara 4-14-1 Asahicho, Atsugishi, Kanagawa, 243-0014, Japan | Tel: Email: | +81-3-5435-3669 teruhiko@av.crl.sony.co.jp |
| Source: | Sony Corp. |  |  |

_____________________________

# Introduction

In Trondheim, the first results on Intra coding of 4:2:2 were reported. The document is submitted to this meeting as JVT-I016. In the meeting, other two method of DC prediction were suggested. In this contribution, suggested two methods are compared with our proposal in JVT-I016.

# Proposal

## Chroma Intra Prediction

### DC prediction

In addition to the method in I016, two methods are suggested in Trondheim. One method calculates DC value based on 8x16 block and the other calculates DC based on 4x8. The sum of the upper pixels is multiplied by two to avoid division operation. Those two methods are compared with 8x8 based DC calculation, which was the best method reported in I016.

# Simulation

In order to verify the proposal, the results for intra coding are reported.

## Test Conditions

The following test conditions are used for our experiments. The software to support 4:2:2 is implemented based on JM61e.

Software: JM61e + 4:2:2 extensions

GOP: All intra

Entropy coding: CABAC

RD optimization: On

Picture structure: Field structure is used for interlaced materials and frame structure is used for progressive materials

Bitdepth: 8 bit

Chroma format: 4:2:2

Color space: YUV

Sequence: src5(canoa), src9(rugby), src15(mobile), src22(tempete)

Number of frames: 60 frames

The following Chroma DC predictions are compared.

  - 8x8 DC prediction

  - 8x16 DC prediction

  - 4x8 DC prediction

## Results

### Comparison of DC prediction

Three different DC prediction for 4:2:2 chroma is compared first.

The results are as follows.

| DC prediction | QP | SNR Cb (db) | SNR Cr (db) | Bitrate (kbps) |
| --- | --- | --- | --- | --- |
| 8x16 DC pred with division by 24 | 40 | 38.12 | 39.09 | 2836 |
|  | 36 | 38.95 | 39.93 | 4271 |
|  | 32 | 40.15 | 41.04 | 6486 |
|  | 28 | 41.38 | 42.17 | 9554 |
|  | 24 | 43,13 | 43.66 | 13866 |
|  | 20 | 45.21 | 45.51 | 20117 |
|  | 16 | 47.68 | 47.77 | 29243 |
|  | 12 | 50.40 | 50.38 | 40912 |
| 8x8 DC pred | 40 | 38.06 | 39.02 | 2835 |
|  | 36 | 38.92 | 39.90 | 4271 |
|  | 32 | 40.14 | 41.04 | 6476 |
|  | 28 | 41.38 | 42.17 | 9540 |
|  | 24 | 43.13 | 43.66 | 13838 |
|  | 20 | 45.21 | 45.51 | 20083 |
|  | 16 | 47.68 | 47.77 | 29196 |
|  | 12 | 50.40 | 50.38 | 40858 |
| 8x16 DC pred with division by 32 | 40 | 38.1 | 39.08 | 2856 |
|  | 36 | 38.94 | 39.93 | 4292 |
|  | 32 | 40.13 | 41.03 | 6505 |
|  | 28 | 41.37 | 42.17 | 9580 |
|  | 24 | 43.13 | 43.66 | 13892 |
|  | 20 | 45.21 | 45.51 | 20146 |
|  | 16 | 47.68 | 47.77 | 29272 |
|  | 12 | 50.4 | 50.38 | 40943 |
| 4x8 DC pred. with division 16 | 40 | 38 | 38.96 | 2865 |
|  | 36 | 38.85 | 39.81 | 4307 |
|  | 32 | 40.1 | 40.98 | 6532 |
|  | 28 | 41.34 | 42.13 | 9622 |
|  | 24 | 43.11 | 43.64 | 13958 |
|  | 20 | 45.21 | 45.52 | 20232 |
|  | 16 | 47.68 | 47.77 | 29372 |
|  | 12 | 50.4 | 50.38 | 40943 |

Table 1: Comparison of intra chroma coding (Src5)

| DC prediction | QP | SNR Cb (db) | SNR Cr (db) | Bitrate (kbps) |
| --- | --- | --- | --- | --- |
| 8x16 DC pred with division by 24 | 40 | 36.07 | 37.68 | 3382 |
|  | 36 | 36.94 | 38.59 | 5354 |
|  | 32 | 38.28 | 39.85 | 8532 |
|  | 28 | 39.63 | 41.15 | 12932 |
|  | 24 | 41.58 | 42.81 | 18917 |
|  | 20 | 43.95 | 44.71 | 27195 |
|  | 16 | 46.83 | 47.12 | 38503 |
|  | 12 | 49.89 | 50.06 | 51598 |
| 8x8 DC pred | 40 | 36.05 | 37.63 | 3394 |
|  | 36 | 36.94 | 38.57 | 5361 |
|  | 32 | 38.29 | 39.86 | 8533 |
|  | 28 | 39.63 | 41.15 | 12934 |
|  | 24 | 41.58 | 42.81 | 18912 |
|  | 20 | 43.95 | 44.72 | 27184 |
|  | 16 | 46.82 | 47.12 | 38489 |
|  | 12 | 49.89 | 50.06 | 51575 |
| 8x16 DC pred with division by 32 | 40 | 36.07 | 37.67 | 3392 |
|  | 36 | 36.95 | 38.59 | 5364 |
|  | 32 | 38.29 | 39.85 | 8541 |
|  | 28 | 39.63 | 41.14 | 12946 |
|  | 24 | 41.58 | 42.81 | 18930 |
|  | 20 | 43.95 | 44.71 | 27209 |
|  | 16 | 46.83 | 47.12 | 38525 |
|  | 12 | 49.89 | 50.06 | 51621 |
| 4x8 DC pred. with division 16 | 40 | 36.01 | 37.58 | 3410 |
|  | 36 | 36.91 | 38.51 | 5391 |
|  | 32 | 38.26 | 39.81 | 8584 |
|  | 28 | 39.62 | 41.12 | 13007 |
|  | 24 | 41.58 | 42.8 | 19014 |
|  | 20 | 43.95 | 44.72 | 27306 |
|  | 16 | 46.82 | 47.12 | 38627 |
|  | 12 | 49.89 | 50.06 | 51728 |

Table 2: Comparison of intra chroma coding (Src9)

| DC prediction | QP | SNR Cb (db) | SNR Cr (db) | Bitrate (kbps) |
| --- | --- | --- | --- | --- |
| 8x16 DC pred with division by 24 | 40 | 32.05 | 32.20 | 9864 |
|  | 36 | 33.15 | 33.38 | 14300 |
|  | 32 | 34.93 | 35.22 | 20597 |
|  | 28 | 36.86 | 37.16 | 28186 |
|  | 24 | 39.75 | 39.99 | 37937 |
|  | 20 | 43.06 | 43.26 | 50292 |
|  | 16 | 46.51 | 46.72 | 64556 |
|  | 12 | 49.73 | 49.89 | 79525 |
| 8x8 DC pred | 40 | 32.05 | 32.20 | 9838 |
|  | 36 | 33.15 | 33.37 | 14274 |
|  | 32 | 34.93 | 35.22 | 20567 |
|  | 28 | 36.86 | 37.16 | 28154 |
|  | 24 | 39.75 | 39.99 | 37894 |
|  | 20 | 43.06 | 43.26 | 50240 |
|  | 16 | 46.51 | 46.72 | 64500 |
|  | 12 | 49.73 | 49.89 | 79464 |
| 8x16 DC pred with division by 32 | 40 | 32.06 | 32.2 | 9880 |
|  | 36 | 33.15 | 33.38 | 14314 |
|  | 32 | 34.93 | 35.22 | 20615 |
|  | 28 | 36.86 | 37.16 | 28210 |
|  | 24 | 39.75 | 39.99 | 37958 |
|  | 20 | 43.06 | 43.26 | 50319 |
|  | 16 | 46.51 | 46.71 | 64584 |
|  | 12 | 49.73 | 49.89 | 79554 |
| 4x8 DC pred. with division 16 | 40 | 32.05 | 32.19 | 9932 |
|  | 36 | 33.15 | 33.37 | 14371 |
|  | 32 | 34.93 | 35.22 | 20682 |
|  | 28 | 36.86 | 37.16 | 28283 |
|  | 24 | 39.75 | 39.99 | 38044 |
|  | 20 | 43.06 | 43.26 | 50409 |
|  | 16 | 46.51 | 46.71 | 64689 |
|  | 12 | 49.73 | 49.89 | 79677 |

Table 3: Comparison of intra chroma coding (Src15)

| DC prediction | QP | SNR Cb (db) | SNR Cr (db) | Bitrate (kbps) |
| --- | --- | --- | --- | --- |
| 8x16 DC pred with division by 24 | 40 | 32.69 | 34.43 | 5506 |
|  | 36 | 33.61 | 35.04 | 8424 |
|  | 32 | 35.25 | 36.15 | 13024 |
|  | 28 | 37.04 | 37.56 | 19236 |
|  | 24 | 39.66 | 39.82 | 27984 |
|  | 20 | 42.65 | 42.58 | 40232 |
|  | 16 | 46.03 | 45.95 | 55326 |
|  | 12 | 49.49 | 49.45 | 70401 |
| 8x8 DC pred | 40 | 32.69 | 34.41 | 5489 |
|  | 36 | 33.61 | 35.03 | 8406 |
|  | 32 | 35.25 | 36.15 | 12998 |
|  | 28 | 37.04 | 37.56 | 19204 |
|  | 24 | 39.66 | 39.82 | 27938 |
|  | 20 | 42.65 | 42.58 | 40170 |
|  | 16 | 46.03 | 45.95 | 55254 |
|  | 12 | 49.49 | 49.45 | 70319 |
| 8x16 DC pred with division by 32 | 40 | 32.69 | 34.42 | 5520 |
|  | 36 | 33.61 | 35.04 | 8440 |
|  | 32 | 35.25 | 36.15 | 13040 |
|  | 28 | 37.04 | 37.56 | 19256 |
|  | 24 | 39.66 | 39.82 | 28009 |
|  | 20 | 42.65 | 42.58 | 40254 |
|  | 16 | 46.03 | 45.95 | 55353 |
|  | 12 | 49.49 | 49.45 | 70433 |
| 4x8 DC pred. with division 16 | 40 | 32.68 | 34.38 | 5579 |
|  | 36 | 33.61 | 35.01 | 8515 |
|  | 32 | 35.24 | 36.14 | 13130 |
|  | 28 | 37.04 | 37.56 | 19360 |
|  | 24 | 39.66 | 39.82 | 28117 |
|  | 20 | 42.65 | 42.58 | 40364 |
|  | 16 | 46.03 | 45.95 | 55478 |
|  | 12 | 49.48 | 49.45 | 70576 |

Table 4: Comparison of intra chroma coding (Src22)

BD SNR and BD Bitrate were calculated and were summarized in the following table.

| Sequence | QP range | BD SNR (db) | BD Bitrate (%) |
| --- | --- | --- | --- |
| src5 | 12-24 | -0.019 | 0.28 |
|  | 28-40 | -0.0051 | 0.21 |
| src9 | 12-24 | -0.044 | 0.045 |
|  | 28-40 | 0.046 | -0.18 |
| src15 | 12-24 | -0.019 | 0.14 |
|  | 28-40 | -0.010 | 0.218 |
| src22 | 12-24 | -0.021 | 0.20 |
|  | 28-40 | -0.012 | 0.36 |

Table 5: BD SNR and BD Bitrate (8x16DCdiv32/8x8 DC)

| Sequence | QP range | BD SNR (db) | BD Bitrate (%) |
| --- | --- | --- | --- |
| src5 | 12-24 | -0.044 | 0.67 |
|  | 28-40 | -0.078 | 2.76 |
| src9 | 12-24 | -0.033 | 0.399 |
|  | 28-40 | -0.044 | 1.66 |
| src15 | 12-24 | -0.043 | 0.32 |
|  | 28-40 | -0.028 | 0.62 |
| src22 | 12-24 | -0.050 | 0.46 |
|  | 28-40 | -0.043 | 1.28 |

Table 6: BD SNR and BD Bitrate (4x8DCdiv16/8x8 DC)

# Conclusions

The simulation results show 8x8 based DC calculation performs best in three methods. As reported in I016, the advantage of the proposed extension for chroma intra coding for 4:2:2 is shown. It should be included in the standard. The extension for inter coding is proposed in I030.

Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Sony Corp. |  |  |
| Mailing address | Gate City Osaki East Tower 20F 1-11-1, Osaki, Shinagawa-ku Tokyo, 141-0032 |  |  |
| Country | Japan |  |  |
| Contact person | Teruhiko Suzuki |  |  |
| Telephone | +81-3-5435-3669 |  |  |
| Fax | +81-3-5435-3891 |  |  |
| Email | teruhiko@av.crl.sony.co.jp |  |  |
| Place and date of submission | San Deigo, CA, USA, September 2, 2003 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Additional Experiments on Intra DC Prediction for 4:2:2 Video |  |
| Contribution number |  | I029 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
| X | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: