| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) JVT Meeting: San Diego, CA, USA, 2-5 September, 2003 | Document: JVT-I030 Filename: JVT-I030.doc |
| --- | --- |

| Title: | Extension of Inter Coding for 4:2:2 Video |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kyohei Koyabu, Atsuo Yada and Katsumi Tahara 4-14-1 Asahicho, Atsugishi, Kanagawa, 243-0014, Japan Teruhiko Suzuki Gate City Osaki East Tower 20 F 1-11-1 Osaki, Shinagawa-ku Tokyo, 141-0032, Japan | Tel: Email: | +81-3-5435-3669 teruhiko@av.crl.sony.co.jp |
| Source: | Sony Corp. |  |  |

_____________________________

# Introduction

In the last meeting, the results on the extension of intra coding to support 4:2:2 were reported. However, the results of inter frame coding was not reported yet. This contribution describes the extension for inter coding to support 4:2:2 and reports the simulation results.

# Proposal

## Modes and reference frame

In the case of 4:2:0 video, mode and reference frame of each 4x4 luma sub-block are associated with that of the corresponding 2x2 chroma sub-block as shown in Figure 1.

16

16

8

8

Figure 1: Association of mode and reference frame for chroma sub-block (4:2:0)

In the case of 4:2:2 video, this association rule should be changed since the vertical resolution of chroma is doubled. The mode and reference frame of 4x4 luma block are associated with that of the corresponding 2x4 chroma block as shown in Figure 2.

16

16

16

8

Figure 2: Association of mode and reference frame for chroma sub-block (4:2:2)

## Chroma inter prediction

The same approach of chroma inter prediction defined in JVT-H037 is used in our experiments. In the document, the chroma inter prediction is defined as follows.

A

C

B

D

Figure 3: Chroma inter prediction

The prediction value v is derived as follows.

v=(8−dx)×(4−dy)×A+dx×(4−dy)×B+(8−dx)×dy×C+dx×dy×D+168×4

# Simulation

In order to verify the proposal, the results for inter coding are reported in this section.

## Test Conditions

The following test conditions are used for our experiments. The software to support 4:2:2 is implemented based on JM61e.

Software: JM61e + 4:2:2 extensions

GOP: n 15 m 3

Entropy coding: CABAC

RD optimization: On

Picture structure: Field structure is used for interlaced materials and frame structure is used for progressive materials

Bitdepth: 8 bit

Chroma format: 4:2:2

Color space: YUV

Sequence: src2, src5(canoa), src6(F1), src9(rugby), src13, src15(mobile), src19(football), src22(tempete)

Number of frames: 58 frames

## Results

The result that encode by field coding, and the result that encode by macroblock adaptive frame/field coding are put on below.

| QP | SNR Y (db) | SNR Cb (db) | SNR Cr (db) | Bitrate (kbps) |
| --- | --- | --- | --- | --- |
| 12 | 46.41 | 46.43 | 46.41 | 51402 |
| 16 | 42.7 | 42.85 | 42.79 | 35844 |
| 20 | 39.2 | 39.54 | 39.42 | 22569 |
| 24 | 35.73 | 36.11 | 35.81 | 11765 |
| 28 | 32.6 | 33.23 | 32.76 | 5549 |
| 32 | 29.92 | 31.38 | 30.84 | 2817 |
| 36 | 27.36 | 29.45 | 28.66 | 1303 |
| 40 | 24.88 | 27.94 | 26.92 | 637 |

Table 1: field coding (Src2)

| QP | SNR Y (db) | SNR Cb (db) | SNR Cr (db) | Bitrate (kbps) |
| --- | --- | --- | --- | --- |
| 12 | 46.56 | 46.61 | 46.58 | 47378 |
| 16 | 42.76 | 43.07 | 42.99 | 31672 |
| 20 | 39.34 | 39.95 | 39.8 | 19127 |
| 24 | 36.08 | 36.83 | 36.52 | 9938 |
| 28 | 33.16 | 34.15 | 33.64 | 4874 |
| 32 | 30.61 | 32.34 | 31.77 | 2623 |
| 36 | 28.15 | 30.49 | 29.79 | 1390 |
| 40 | 25.3 | 28.47 | 27.5 | 709 |

Table 2: macroblock adaptive frame/field coding (Src2)

| QP | SNR Y (db) | SNR Cb (db) | SNR Cr (db) | Bitrate (kbps) |
| --- | --- | --- | --- | --- |
| 12 | 47.31 | 48.18 | 48.25 | 27696 |
| 16 | 44.1 | 45.73 | 45.96 | 17761 |
| 20 | 41.2 | 43.72 | 44.14 | 11289 |
| 24 | 38.24 | 42.02 | 42.64 | 7058 |
| 28 | 35.36 | 40.61 | 41.35 | 4407 |
| 32 | 32.42 | 39.41 | 40.1 | 2655 |
| 36 | 29.39 | 38.25 | 38.7 | 1479 |
| 40 | 26.59 | 37.13 | 37.23 | 762 |

Table 3: field coding (Src5)

| QP | SNR Y (db) | SNR Cb (db) | SNR Cr (db) | Bitrate (kbps) |
| --- | --- | --- | --- | --- |
| 12 | 47.34 | 48.19 | 48.26 | 27090 |
| 16 | 44.11 | 45.72 | 45.92 | 16951 |
| 20 | 41.21 | 43.73 | 44.14 | 10662 |
| 24 | 38.26 | 42.03 | 42.65 | 6616 |
| 28 | 35.37 | 40.65 | 41.39 | 4102 |
| 32 | 32.42 | 39.46 | 40.15 | 2460 |
| 36 | 29.44 | 38.31 | 38.76 | 1344 |
| 40 | 26.81 | 37.24 | 37.37 | 706 |

Table 4: macroblock adaptive frame/field coding (Src5)

| QP | SNR Y (db) | SNR Cb (db) | SNR Cr (db) | Bitrate (kbps) |
| --- | --- | --- | --- | --- |
| 12 | 47.48 | 48.52 | 48.83 | 21966 |
| 16 | 44.4 | 46.48 | 47.06 | 13506 |
| 20 | 41.66 | 44.55 | 45.33 | 8177 |
| 24 | 38.98 | 42.67 | 43.59 | 4750 |
| 28 | 36.27 | 40.9 | 41.8 | 2702 |
| 32 | 33.57 | 39.35 | 40.23 | 1488 |
| 36 | 30.96 | 37.83 | 38.61 | 772 |
| 40 | 28.57 | 36.66 | 37.2 | 442 |

Table 5: field coding (Src6)

| QP | SNR Y (db) | SNR Cb (db) | SNR Cr (db) | Bitrate (kbps) |
| --- | --- | --- | --- | --- |
| 12 | 47.5 | 48.57 | 48.88 | 21639 |
| 16 | 44.38 | 46.48 | 47.04 | 13250 |
| 20 | 41.65 | 44.53 | 45.32 | 7997 |
| 24 | 38.95 | 42.64 | 43.58 | 4642 |
| 28 | 36.26 | 40.9 | 41.82 | 2625 |
| 32 | 33.56 | 39.37 | 40.25 | 1440 |
| 36 | 30.97 | 37.85 | 38.63 | 738 |
| 40 | 28.6 | 36.66 | 37.2 | 419 |

Table 6: macroblock adaptive frame/field coding (Src6)

| QP | SNR Y (db) | SNR Cb (db) | SNR Cr (db) | Bitrate (kbps) |
| --- | --- | --- | --- | --- |
| 12 | 47.07 | 47.41 | 47.68 | 34450 |
| 16 | 43.66 | 44.57 | 45.21 | 22662 |
| 20 | 40.51 | 42.25 | 43.42 | 14515 |
| 24 | 37.35 | 40.28 | 41.81 | 8906 |
| 28 | 34.36 | 38.77 | 40.43 | 5426 |
| 32 | 31.41 | 37.58 | 39.16 | 3210 |
| 36 | 28.63 | 36.45 | 37.86 | 1762 |
| 40 | 26.15 | 35.57 | 36.68 | 961 |

Table 7: field coding (Src9)

| QP | SNR Y (db) | SNR Cb (db) | SNR Cr (db) | Bitrate (kbps) |
| --- | --- | --- | --- | --- |
| 12 | 47.12 | 47.46 | 47.72 | 34248 |
| 16 | 43.66 | 44.58 | 45.21 | 22331 |
| 20 | 40.51 | 42.27 | 43.43 | 14218 |
| 24 | 37.36 | 40.28 | 41.8 | 8699 |
| 28 | 34.36 | 38.78 | 40.41 | 5280 |
| 32 | 31.41 | 37.59 | 39.14 | 3096 |
| 36 | 28.6 | 36.47 | 37.83 | 1671 |
| 40 | 26.15 | 35.6 | 36.68 | 894 |

Table 8: macroblock adaptive frame/field coding (Src9)

| QP | SNR Y (db) | SNR Cb (db) | SNR Cr (db) | Bitrate (kbps) |
| --- | --- | --- | --- | --- |
| 12 | 47 | 46.75 | 47.14 | 33604 |
| 16 | 43.52 | 43.15 | 44.33 | 20872 |
| 20 | 40.6 | 40.21 | 42.11 | 12066 |
| 24 | 37.86 | 37.84 | 39.98 | 6749 |
| 28 | 35.11 | 35.94 | 38 | 3827 |
| 32 | 32.24 | 34.42 | 36.3 | 2111 |
| 36 | 29.52 | 32.89 | 34.56 | 1097 |
| 40 | 27.01 | 31.81 | 33.24 | 603 |

Table 9: field coding (Src13)

| QP | SNR Y (db) | SNR Cb (db) | SNR Cr (db) | Bitrate (kbps) |
| --- | --- | --- | --- | --- |
| 12 | 47.21 | 46.86 | 47.32 | 32378 |
| 16 | 43.7 | 43.29 | 44.63 | 19714 |
| 20 | 40.88 | 40.49 | 42.54 | 11393 |
| 24 | 38.27 | 38.28 | 40.54 | 6540 |
| 28 | 35.56 | 36.37 | 38.51 | 3794 |
| 32 | 32.58 | 34.71 | 36.65 | 2100 |
| 36 | 29.87 | 33.15 | 34.89 | 1127 |
| 40 | 27.12 | 31.86 | 33.31 | 596 |

Table 10: macroblock adaptive frame/field coding (Src13)

| QP | SNR Y (db) | SNR Cb (db) | SNR Cr (db) | Bitrate (kbps) |
| --- | --- | --- | --- | --- |
| 12 | 46.36 | 47.07 | 47.35 | 45237 |
| 16 | 42.67 | 43.86 | 44.22 | 31524 |
| 20 | 39.25 | 40.8 | 41.21 | 20522 |
| 24 | 35.86 | 37.85 | 38.37 | 12046 |
| 28 | 32.56 | 35.51 | 36.06 | 6618 |
| 32 | 29.36 | 33.93 | 34.39 | 3393 |
| 36 | 26.52 | 32.52 | 32.86 | 1684 |
| 40 | 23.89 | 31.54 | 31.73 | 891 |

Table 11: field coding (Src15)

| QP | SNR Y (db) | SNR Cb (db) | SNR Cr (db) | Bitrate (kbps) |
| --- | --- | --- | --- | --- |
| 12 | 46.42 | 47.28 | 47.6 | 41944 |
| 16 | 42.72 | 44.18 | 44.59 | 28660 |
| 20 | 39.36 | 41.22 | 41.69 | 18370 |
| 24 | 36.03 | 38.34 | 38.94 | 10638 |
| 28 | 32.86 | 36.08 | 36.7 | 5845 |
| 32 | 29.77 | 34.46 | 35.03 | 3039 |
| 36 | 27 | 33.04 | 33.5 | 1549 |
| 40 | 24.4 | 32.06 | 32.39 | 860 |

Table 12: macroblock adaptive frame/field coding (Src15)

| QP | SNR Y (db) | SNR Cb (db) | SNR Cr (db) | Bitrate (kbps) |
| --- | --- | --- | --- | --- |
| 12 | 46.99 | 46.94 | 47.74 | 31456 |
| 16 | 43.44 | 44.1 | 45.55 | 18304 |
| 20 | 40.75 | 42.31 | 43.76 | 10266 |
| 24 | 38.4 | 40.82 | 42.19 | 5970 |
| 28 | 35.94 | 39.51 | 40.85 | 3605 |
| 32 | 33.41 | 38.14 | 39.65 | 2125 |
| 36 | 31.05 | 36.66 | 38.43 | 1204 |
| 40 | 28.9 | 34.99 | 37.33 | 698 |

Table 13: field coding (Src19)

| QP | SNR Y (db) | SNR Cb (db) | SNR Cr (db) | Bitrate (kbps) |
| --- | --- | --- | --- | --- |
| 12 | 47.04 | 47 | 47.79 | 31377 |
| 16 | 43.46 | 44.1 | 45.55 | 18048 |
| 20 | 40.76 | 42.29 | 43.74 | 10039 |
| 24 | 38.35 | 40.77 | 42.14 | 5793 |
| 28 | 35.9 | 39.46 | 40.82 | 3471 |
| 32 | 33.37 | 38.08 | 39.62 | 2030 |
| 36 | 30.98 | 36.58 | 38.4 | 1130 |
| 40 | 28.85 | 34.98 | 37.33 | 647 |

Table 14: macroblock adaptive frame/field coding (Src19)

| QP | SNR Y (db) | SNR Cb (db) | SNR Cr (db) | Bitrate (kbps) |
| --- | --- | --- | --- | --- |
| 12 | 46.52 | 46.8 | 46.71 | 41667 |
| 16 | 42.78 | 43.53 | 43.41 | 26630 |
| 20 | 39.51 | 40.72 | 40.76 | 15129 |
| 24 | 36.57 | 38.08 | 38.47 | 7877 |
| 28 | 33.67 | 35.92 | 36.74 | 4019 |
| 32 | 30.77 | 34.41 | 35.66 | 1994 |
| 36 | 28.13 | 33.13 | 34.77 | 976 |
| 40 | 25.66 | 32.22 | 34.18 | 506 |

Table 15: field coding (Src22)

| QP | SNR Y (db) | SNR Cb (db) | SNR Cr (db) | Bitrate (kbps) |
| --- | --- | --- | --- | --- |
| 12 | 46.68 | 46.98 | 46.87 | 38971 |
| 16 | 42.82 | 43.75 | 43.59 | 23756 |
| 20 | 39.6 | 41.1 | 41.14 | 12910 |
| 24 | 36.79 | 38.63 | 39.01 | 6580 |
| 28 | 34.03 | 36.61 | 37.33 | 3313 |
| 32 | 31.37 | 35.16 | 36.24 | 1685 |
| 36 | 28.85 | 33.82 | 35.27 | 870 |
| 40 | 26.42 | 32.88 | 34.63 | 487 |

Table 16: macroblock adaptive frame/field coding (Src22)

Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Sony Corp. |  |  |
| Mailing address | Gate City Osaki East Tower 20F 1-11-1, Osaki, Shinagawa-ku Tokyo, 141-0032 |  |  |
| Country | Japan |  |  |
| Contact person | Teruhiko Suzuki |  |  |
| Telephone | +81-3-5435-3669 |  |  |
| Fax | +81-3-5435-3891 |  |  |
| Email | teruhiko@av.crl.sony.co.jp |  |  |
| Place and date of submission | San Deigo, CA, USA, September 2, 2003 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Extension of Inter Coding for 4:2:2 Video |  |
| Contribution number |  | I030 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
| X | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: