| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) JVT Meeting: Waikoloa, Hawaii, USA, 7-12 December, 2003 | Document: JVT-J015 Filename: JVT-J015.doc |
| --- | --- |

| Title: | Profile and level definition for Professional extension |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Teruhiko Suzuki Gate City Osaki East Tower 20 F 1-11-1 Osaki, Shinagawa-ku Tokyo, 141-0032, Japan Kyohei Koyabu and Atsuo Yada 4-14-1 Asahicho, Atsugishi, Kanagawa, 243-0014, Japan | Tel: EMAIL: | +81-3-5435-3669 teruhiko@av.crl.sony.co.jp +81-46-230-6126 koyabu@vns.cpg.sony.co.jp yada@vns.cpg.sony.co.jp |
| Source: | Sony Corp. |  |  |

_____________________________

# Introduction

In the current draft of professional extension doesn’t include profile and level definition. In this contribution, some initial idea to define profile and level for professional extension. The following issues should be considered when we define profile and level.

- Implementation complexity both for hardware and software

- The combination of the tools that is required from industry

- Separate conformance point for bitdepth and chroma format

- Define levels based on the original sample rate (not data rate)

- Keep the same definition of Level.

# Extension of the existing profile

Storage and authoring applications and Camcoder (VCR) are very important application of JVT professional extension. For those applications, the hardware implementation complexity must be feasible, Since the requirements of those applications are similar to that of main profile of JVT ver.1., we propose to extend main profile for professional extension. If error resilience is needed in some applications, the different profile should be defined extended by baseline or extended profile.

# Bitdepth and Chroma format

The architecture is quite different if bitdepth of video sample is completely different, especially in case of hardware. For example, memory management of 8 bit video and 10 bit video are completely different. In order to reduce the hardware complexity, the different conformance point should be defined for each bitdepth.

We propose the following profiles for PExt.

| profile_idc | Profile Name |
| --- | --- |
| 78 | PExt_main_8bit (only 8 bit video) |
| 79 | PExt_main_10bit (only 10 bit video) |
| 80 | PExt_main_12bit (only 12 bit video) |

Table 3-1: Profile definition for PExt

On the other hand, the number of conformance point should be limited, We propose to define each level based on the original sample rate, including the chroma sample. Within this constrain, it is allowed to use 4:4:4 for smaller picture size and 4:2:2 for larger size. Since we propose to add new column for original sample rate including chroma. (The current definition of original sample rate is only for luma.) The constraint on the number of MB per second is defined for each level. We propose to change this to the constraint on the number of block per second, in order to take the chroma size into account. The following definition could be the used for PExt.

| Level number | Max block processing rate MaxBPS (B/s) | Max frame size MaxFS (MBs) ?? (To be changed) | Max decoded picture buffer size MaxDPB (1024 bytes) | Max video bit rate MaxBR (1000 bits/s or 1200 bits/s) | Max CPB size MaxCPB (1000 bits or 1200 bits) | Vertical MV component range MaxVmvR (luma frame samples) | Min compression ratio MinCR | Max number of motion vectors per two consecutive MBs MaxMvsPer2Mb |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 17 820 | 99 | 148.5 | 64 | 175 | [-64,+63.75] | 2 | - |
| 1.1 | 360 000 | 396 | 337.5 | 192 | 500 | [-128,+127.75] | 2 | - |
| 1.2 | 720 000 | 396 | 891.0 | 384 | 1 000 | [-128,+127.75] | 2 | - |
| 1.3 | 154 560 | 396 | 891.0 | 768 | 2 000 | [-128,+127.75] | 2 | - |
| 2 | 154 560 | 396 | 891.0 | 2 000 | 2 000 | [-128,+127.75] | 2 | - |
| 2.1 | 237 600 | 792 | 1 782.0 | 4 000 | 4 000 | [-256,+255.75] | 2 | - |
| 2.2 | 243 000 | 1 620 | 3 037.5 | 4 000 | 4 000 | [-256,+255.75] | 2 | - |
| 3 | 486 000 | 1 620 | 3 037.5 | 10 000 | 10 000 | [-256,+255.75] | 2 | 32 |
| 3.1 | 1 296 000 | 3 600 | 6 750.0 | 14 000 | 14 000 | [-512,+511.75] | 4 | 16 |
| 3.2 | 2 592 000 | 5 120 | 7 680.0 | 20 000 | 20 000 | [-512,+511.75] | 4 | 16 |
| 4 | 2 949 120 | 8 192 | 12 288.0 | 20 000 | 25 000 | [-512,+511.75] | 4 | 16 |
| 4.1 | 2 949 120 | 8 192 | 12 288.0 | 50 000 | 62 500 | [-512,+511.75] | 2 | 16 |
| 4.2 | 5 898 240 | 8 192 | 12 288.0 | 50 000 | 62 500 | [-512,+511.75] | 2 | 16 |
| 5 | 7 077 888 | 22 080 | 41 310.0 | 135 000 | 135 000 | [-512,+511.75] | 2 | 16 |
| 5.1 | 11 796 480 | 36 864 | 69 120.0 | 240 000 | 240 000 | [-512,+511.75] | 2 | 16 |

Table 3-2: Level definition for PExt

We also propose to keep the current constraints on each level. For example, max bitrate should be defined by the max rate of the network bandwidth or the capacity of media. Therefore, those numbers should be the same as it is defined for version 1.

# Conclusions

In this contribution, the initial idea on the profile and level definition is proposed. The key ideas are

- Implementation complexity both for hardware and software

- The combination of the tools that is required from industry

- Separate conformance point for bitdepth and chroma format

- Define levels based on the original sample rate (not data rate)

- Keep the same definition of Level.

In order to reduce the number of conformance point, we propose to define levels based on the maximum number of block per second as defined in Table 3-2. However,if people think the number of conformance point is not the issue, it is to define independent conformance point for both chroma format and bitdepth to reduce the implementation complexity.

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Sony Corp. |  |  |
| Mailing address | Gate City Osaki East Tower 20F, 1-11-1, Osaki Shinagawa-ku, Tokyo, 141-0032, JAPAN |  |  |
| Country | Japan |  |  |
| Contact person | Teruhiko Suzuki |  |  |
| Telephone | +81-3-5435-3669 |  |  |
| Fax | +81-3-5435-3891 |  |  |
| Email | Teruhiko@av.crl.sony.co.jp |  |  |
| Place and date of submission |  |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Profile and level definition for Professional extension |  |
| Contribution number |  | JVT-J015 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
| X | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: