| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 10th Meeting: Waikoloa, Hawaii, USA, 8-12 December, 2003 | Document: JVT-J016 Filename: JVT-J016.doc |
| --- | --- |

| Title: | Proposal for alpha channel coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Woo-Shik Kim, Dae-Sung Cho, Hyun Mun Kim Multimedia Lab. Samsung AIT P.O.Box 111, Suwon 440-600 Korea | Tel: Email: | 82-31-280-9204 hyunmun27.kim@samsung.com |
| Source: | Samsung AIT |  |  |

- Introduction

In this proposal, we introduce a coding method for alpha channel that was addressed in Section 4.5 of the issues of the investigation document [1] from the previous meeting in San Diego.

- Proposed Methods

In this section, we summarize one of the possible coding methods for alpha channel. Figure 1 shows the block diagram of the proposed method.

α image

(MB)

Entropy

coding

CBP

Generation

0: transp.

1: opaque

2: mixed

bitstream

Intra/inter

Prediction

(XOR)

Figure 1: Proposed block diagram of alpha macroblock coding

Figure 2: Example of an alpha channel image.

0

0

2

2

0

0

0

0

2

2

2

0

0

0

2

1

2

0

0

2

2

1

2

2

0

2

1

1

1

2

Figure 3: Proposed CBP coding for alpha channel.

First we divide the given macroblock into 4 8x8 blocks. For each block we decide whether it is transparent (0), opaque (1) or mixed (2). Based on this classification, we have 34=81 cases for each macroblock. This CBP information is coded as the same method of Luma CBP coding in [2].

For the mixed 8x8 block we need to code the pixel values. For this case we can use the intra and inter prediction. But, instead of doing the normal prediction (subtraction), we use exclusive OR (XOR) operation to make the residue image binary. Intra prediction mode and alpha channel motion vector are coded independently from the Luma and Chroma ones. MC interpolation is not applied.

Next we need to code the residue values. We use the same coefficient coding method based on 4x4 block as specified in H.264 [4]. The difference is we do not need to code sign information and the level consists of only 0 and 1.

We also use the “mb_skip_flag” to signal the skipped macroblock in inter prediction mode. If that flag is set to ‘1’, the CBP coding, motion vector coding and transform coefficient coding is skipped.

Loop-filter is not appllied.

- Conclusions

We proposed solutions for the possible coding method of binary alpha channel. This is just the draft proposal and many improvements can be made using the existing coding tools in H.264 along with the special characteristics of alpha channel. For example we can skip the intra and inter prediction and directly code the 4x4 block coefficients. In this case all alpha channel becomes the intra picture and we can increase the random access using alpha information only. To see the effect of coding efficiency we have to estimate the side information overhead such as mode information and motion vector information. The gray scale alpha channel was not addressed in this document and further study is required.

References

- [1] T. McMahon, T. Wiegand and G. Sullivan, “Report - Prof. Ext. Issues of Investigation,” ISO/IEC JTC1/SC29/WG11 and ITU-T Q6/SG16, Document JVT-I048, September 2003

[2] “Final Draft International Standard Joint Video Specification (ITU-T Rec. H.264 | ISO/IEC 14496-10 AVC)” ISO/IEC JTC1/SC29/WG11 and ITU-T Q6/SG16, Document JVT-G050r1, May 2003

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Samsung Elctronics. Co. Ltd. |  |  |
| Mailing address | POBox 111 Suwon Samsung AIT Multimedia |  |  |
| Country | Korea |  |  |
| Contact person | Hyun Mun Kim |  |  |
| Telephone | 82-31-280-9204 |  |  |
| Fax | 82-31-280-9207 |  |  |
| Email | hyunmun27.kim@samsung.com |  |  |
| Place and date of submission | Hawaii, USA, 8-12 December, 2003 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Proposal for alpha channel coding |  |
| Contribution number |  | JVT-J016 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
| X | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| # X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: