| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 10th Meeting: Waikoloa, Hawaii, USA, 8-12 December, 2003 | Document: JVT-J017 Filename: JVT-J017.doc |
| --- | --- |

| Title: | Inter-plane Prediction for RGB Coding II |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Woo-Shik Kim, Dae-Sung Cho, Hyun Mun Kim Multimedia Lab. Samsung AIT P.O.Box 111, Suwon 440-600 Korea | Tel: Email: | 82-31-280-9204 hyunmun27.kim@samsung.com |
| Source: | Samsung AIT |  |  |

_____________________________

- Introduction

In the previous JVT meetings we presented the effectiveness of INTER-PLANE PREDICTION (IPP) coding for RGB coding [1-3]. It avoids the color distortion resulting from the color space conversion as well as coding error propagation. By applying IPP we achieved significant coding gain than coding RGB directly [1,2].

In this proposal, we tested our IPP method over various image set including 10 bit film material and Viper material. We achieved better coding efficiency by controlling the IPP option depending on image characteristics since there are some cases where the correlation between color components is decreased after inter/intra prediction. In such cases the color space conversion based codec works very poorly too. By controlling the IPP option in the sequence parameter set we obtained the best coding results among the other proposed methods [4-5].

IPP is also applied to the XYZ data treating Y component as pivot component and showed its effectiveness.

Section 2 briefly describes the proposed algorithm. The test results are summarized in Section 3. We conclude in Section 4.

- Inter-Plane Prediction Coding

We propose to use the inter-plane prediction as shown in Figure 1. The function F(⋅) in Figure 1 is an identity function. This scheme can be easily incorporated into the existing JVT specification [6] since it needs one simple predictor block using the reconstructed G plane residue for each R and B residue. It will be turned on as proposed in JVT-J038 [8]. The G-plane predicted R residue and B residue follow the same coding procedure of the existing JVT specification as the G residue does. In the decoder, we just add the reconstructed G residue to decode R and B residue value if IPP was used.

We can easily decide to use IPP using the input data information such as film material or use of pre-processing. So we can avoid the frequent switching of IPP option.

G plane

F(•)

-

+

G plane

Predicted

R-Residue

Reconstructed

G plane

Spatial/temporal

prediction

G plane

Residue

R plane

Reconstructed

R plane

Spatial/temporal

prediction

B plane

Reconstructed

B plane

Spatial/temporal

prediction

Reconstructed

G plane Residue

-

+

G plane

Predicted

B-Residue

To encoder

To encoder

To encoder

Reconstructed

G plane Residue

F(•)

Figure 1: Block diagram of the proposed Inter-Plane Prediction

- Experimental Results

We compared our IPP coding with YCoCg-R and YFbFr proposed in [4,5] using the common test conditions and test image sets. The test conditions are summarized as follows:

- Search range: 32

- Entropy coding: CABAC

- RD-optimized mode selection: On

- Number of reference frames: 3

- GOP structure: IBBPBBPBB…

- Number of slice groups: 1

- Direct mode: Temporal

- Weighted prediction: No

- Stored B-picture: No

As observed and discussed in [7], encoding the film grain costs. The film grain pushes the bit rate to the extremely high range in low Qp values. Due to the randomness of the film grain, the coding gain obtained using color space conversion has been diluted. It also propagates coding error when color space is converted back to RGB domain, so the coding efficiency of the color space converted methods become ineffective.

For viper material, the image sequences are not pre-processed. We found out that after intra/inter prediction, the inter-color residue correlation is very small due to the small residue amplitude and noise component that is coming from shot noise. So we turned off the IPP option and achieved better coding efficiency.

On the other hand, the pre-processed HD material shows very high correlation even after intra/inter prediction so that IPP becomes very effective as shown in [1-2].

The color space converted coding methods turned out to be ineffective for the following reasons:

- Even though the correlation between color components is high, the color space converted data are bad fit for the JVT codec for film material or unprocessed material such as Viper data.

- The use of reversible transform still propagates the coding error unless it is lossless coding. In that case the reverse transform is more important since the error is proportional to the square of the conversion coefficients.

Based on this observation and experimental results, the best method to code RGB material is to code the given image in the given space. Using IPP at the discretion of encoder, we achieve the best coding efficiency for RGB material in most cases no matter where the source is coming from. The complete test results are available in the accompanying excel file (JVT-J017.xls).

- Conclusions

The use of IPP coding on residue images in RGB space achieves great coding efficiency for various kinds of RGB data including film material and professional filmstream material. We can easily select this option depending on image characteristics at encoder’s discretion. By putting the Interplane_prediction_flagon sequence parameter set, we minimize the switching of IPP option. It helps achieve the best coding efficiency for any type of input data. RGB coding efficiency using color space conversion cannot achieve the coding efficiency as expressed in terms of theoretical gain [5,6] since coding involves more than color transform. Due to the coding error propagation or the statistical properties of the color space converted data that do not conform to the assumed intra/inter redundancy, it cannot achieve the efficiency as claimed. The IPP was also tested in XYZ data set and showed its efficiency. It makes the proposed JVT codec a complete package for various needs of video compression regardless of color space.

References

- [1] W.-S. Kim, D. Cho and H. M. Kim, “Color Format Extension,” ISO/IEC JTC1/SC29/WG11 and ITU-T Q6/SG16, Document JVT-H018, May 2003.

- [2] W.-S. Kim, D. Cho and H. M. Kim, “Proposal for the unsolved issues in professional extensions,” ISO/IEC JTC1/SC29/WG11 and ITU-T Q6/SG16, Document JVT-I012, July 2003.

- [3] W.-S. Kim, D. Cho and H. M. Kim, “Inter-plane Prediction for RGB Coding,” ISO/IEC JTC1/SC29/WG11 and ITU-T Q6/SG16, Document JVT-I023, September 2003.

- [4] H. Malvar and G. Sullivan, “YCoCg-R: A Color Space with RGB Reversibility and Low Dynamic Range,” ISO/IEC JTC1/SC29/WG11 and ITU-T Q6/SG16, Document JVT-I014, September 2003.

[5] P. Topiwala and C. Tu, “New Invertible Integer Color Transforms Based on Lifting Steps and Coding of 4:4:4 Video,” ISO/IEC JTC1/SC29/WG11 and ITU-T Q6/SG16, Document JVT-I015, September 2003.

[6] “Final Draft International Standard of Joint Video Specification (ITU-T Rec. H.264 | ISO/IEC 14496-10 AVC),” ISO/IEC JTC1/SC29/WG11 and ITU-T Q6/SG16, Document JVT-G050, March 2003.

- [7] C. Gomila, “SEI message for film grain encoding: syntax and results,” ISO/IEC JTC1/SC29/WG11 and ITU-T Q6/SG16, Document JVT-I013r1, September 2003.

- [8] W.-S. Kim and H. M. Kim, “Residue transform”, ISO/IEC JTC1/SC29/WG11 and ITU-T Q6/SG16, Document JVT-J038, December 2003.

- Appendix: Text for the Draft Prof. Ext Amendment (JVT-I048)

- Note that this modification points out the necessary changes for inter-plane prediction based on JVT-I048 text. The complete text change that covers the 4:4:4 format is addressed in JVT-H018.

- Changes to clause 7 "Syntax and semantics"

  - Changes to subclause 7.3.2.1 "Sequence parameter set RBSP syntax"

| seq_parameter_set_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| … |  |  |
| interplane_prediction_flag |  |  |
| … | 1 | u(1) |
| } |  |  |

  - Changes to subclause 7.4.2.1 "Sequence parameter set RBSP semantics"

- interplane_prediction_flag equal to 0 indicates that the inter-plane prediction is not applied to I, P, B, SI, and SP slices. interplane_prediction_flag to equal to 1 indicates that the inter-plane prediction is applied to I, P, B, SI and SP slices (see subclause XXX).

- Changes to clause 8 "Decoding process"

  - Changes to subclause 8.5.3 “Specification of transform decoding process for chroma samples” Equation 8-250

- For each 4x4 chroma block indexed by chroma4x4BlkIdx = 0..3 of the component indexed by iCbCr, the following ordered steps are specified.

  - The scaling and transformation process for residual 4x4 blocks as specified in subclause Error: Reference source not found is invoked with c as the input and p as the output.

- If interplane_prediction_flagis equal to 1, r is derived as

r = p+ rluma 			 (8-x)

where rluma is the ouput of the step 2.c in subcluase 8.5.2. Otherwise, r = p.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Samsung Elctronics. Co. Ltd. |  |  |
| Mailing address | POBox 111 Suwon Samsung AIT Multimedia |  |  |
| Country | Korea |  |  |
| Contact person | Hyun Mun Kim |  |  |
| Telephone | 82-31-280-9204 |  |  |
| Fax | 82-31-280-9207 |  |  |
| Email | hyunmun27.kim@samsung.com |  |  |
| Place and date of submission | Hawaii, USA, 8-12 December, 2003 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Inter-plane Prediction for RGB Coding II |  |
| Contribution number |  | JVT-J017 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
| X | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| # X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: