| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 10th Meeting: Waikoloa, Hawaii, USA, 8-12 Dec, 2003 | Document: JVT-J030 Filename: JVT-J030.doc |
| --- | --- |

| Title: | Simplified Use of 8x8 Transforms - Results |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Steve Gordon Sand Video 200 Brickstone, Fourth Floor Andover, MA 01810 USA Detlev Marpe Heinrich Hertz Institute (FhG), Einsteinufer 37, D-10587 Berlin, Germany Thomas Wiegand Heinrich Hertz Institute (FhG), Einsteinufer 37, D-10587 Berlin, Germany Thomas Wiegand | Tel: Email: Tel: Email: Tel: Email: | 978.475.8990 x222 SteveGordon@SandVideo.com +49 30 31002 619 marpe@hhi.de +49 30 31002 617 wiegand@hhi.de |
| Source: | Sand Video, Inc & Fraunhofer HHI |  |  |

_____________________________

# Introduction

The emergence of consumer HD applications, including HD-DVD and broadcast, cable, and satellite HD-TV, creates an excellent opportunity for H.264/MPEG-4 AVC adoption, replacing the eight year old technology of MPEG-2. However, these high definition applications, as the name implies, require image quality standards well above what’s typical in lower bit-rate applications. In particular, video codecs are evaluated on their preservation of fine detail and film grain, in addition to PSNR.

This submission shows that when using an 8x8 transform in addition to the 4x4 transform in H.264/AVC a roughly 10 % BD bit-rate reduction can be achieved across a wide range of content and coding parameters. Moreover, the improved subjective quality (which is not only because of the bit-rate reduction) makes the presented 8x8 transform feature a critical coding tool for reaching these image quality requirements at acceptable bit rates.

# Background

Significant compression performance gains were reported at Standard Definition (SD) and High Definition (HD) resolutions when larger than 4x4 transforms were used [1]. However, these gains came at the expense of much added complexity to the H.264/MPEG4 Part-10 compression standard. In [2], we proposed a simple way of incorporating an 8x8 transform into the compression standard. Document [3] is an update to this proposal, while [4] outlines the details of implementing the proposal in the JM7.3 code base. This document summarizes the coding gain results of using the modified JM code. Detailed results can be found in the companion spreadsheet JVT-I030.xls.

- Test Conditions

The following test conditions were used throughout this study:

Software: JM7.3a (modified)

Entropy coding: CABAC

Search Range: +/-48

RDOpt: On

Hardamard: On

QP Settings: 20, 24, 28, 32 (QPI=QPP=QP, QPB=QP+2)

Frame Coding

Loop Filter Off (LoopFilterParameters=LoopFilterDisable=1)

GOP Structure: IPPP… or IBBP… as indicated

Frames Coded: 1 second for IPPP, 1 second less 2 frames for IBBP

I Frame Period: ½ second

Reference Frames: L0 Active=1 or 4 as indicated. L1 Active=1 (B frames only)

HD Test Sequences:

- Five film scans (1920x1080 @ 24P) of major release Hollywood movies (designated Movies 1-5).

- One film scan (1920x1080 @ 24P) from Crawford Communications, Atlanta, GA (designated Crawford)

- Riverbed Black – 1920x1080 @ 30P video. MPEG test sequence.

- City – 1280x720 @ 60P video. MPEG test sequence

- Crew – 1280x720 @ 60P video. MPEG test sequence

- Harbour – 1280x720 @ 60P video. MPEG test sequence

- Raven – 1280x720 @ 60P video. MPEG test sequence

- RD Results

# Summary

The eleven sequences were run in three different modes: IPPP with one reference frame, IPPP with four reference frames, and IBBP with one forward and one backward reference frame. The bit-rate reductions were measured using the Bjontegaard [5] technique (referred to as BD bit-rate reductions). The overall average BD bit-rate reduction when using 8x8 transforms in addition to 4x4 transforms is 10.20 %, with the gains being about 2 % higher on average for the film material over the video material.

At the Geneva JVT meeting in May 2003, concerns were raised that the gains associated with Adaptive Block Transforms (ABT) were noticeably diminished when using multiple reference frames and B slices. In private testing with the JM4.0d code, the latter claim was verified. This was particularly disappointing as IBBP is the standard GOP structure used in HD-DVD and HD broadcast environments.

The current implementation with 8x8 transforms based on the JM7.3a code base does not exhibit these problems. The average BD bit-rate reduction when using four reference frames is 9.55 %, compared to a 10.13 % BD bit-rate reduction when using one reference frame. When using B frames, the average BD bit-rate reduction actually increases to 10.94 %.

# IPPP – 1 Reference Frame

The BD bit-rate reductions using one reference frame are tabulated in Table 1. The average BD bit-rate reduction is 10.13%.

Table 1 – BD Bit-rate Reduction using 8x8 Transforms, IPPP, One Reference Frame

| Sequence | % BD bit-rate reduction |
| --- | --- |
| Movie 1 | 9.99 |
| Movie 2 | 15.41 |
| Movie 3 | 11.95 |
| Movie 4 | 9.91 |
| Movie 5 | 12.31 |
| Crawford | 8.35 |
| Riverbed | 13.84 |
| City | 5.33 |
| Crew | 7.39 |
| Harbour | 7.83 |
| Raven | 9.08 |
| Film Average | 11.32 |
| Video Average | 8.69 |
| Overall Average | 10.13 |

# IPPP – 4 Reference Frames

The BD bit-rate reductions using four reference frames are tabulated in Table 2. The average BD bit-rate reduction performance gain is 9.55 % (0.58 % reduced from the one reference frame case).

Table 2 – BD Bit-rate Reduction using 8x8 Transforms, IPPP, Four Reference Frames

| Sequence | % BD bit-rate reduction |
| --- | --- |
| Movie 1 | 9.80 |
| Movie 2 | 14.64 |
| Movie 3 | 11.72 |
| Movie 4 | 9.21 |
| Movie 5 | 11.5 |
| Crawford | 7.32 |
| Riverbed | 13.72 |
| City | 4.77 |
| Crew | 6.27 |
| Harbour | 7.28 |
| Raven | 8.78 |
| Film Average | 10.70 |
| Video Average | 8.16 |
| Overall Average | 9.55 |

# IBBP – 1 Forward and 1 Backward Reference Frame

The BD bit-rate reductions using four reference frames are tabulated in Table 3. The average BD bit-rate reduction is 10.94 %, (0.81 % better than the IPPP, one reference frame case).

Table 3 – BD Bit-rate Reduction using 8x8 Transforms, IBBP, One Forward Reference Frame, One Backward Reference Frame

| Sequence | % BD bit-rate reduction |
| --- | --- |
| Movie 1 | 11.22 |
| Movie 2 | 12.31 |
| Movie 3 | 12.22 |
| Movie 4 | 11.17 |
| Movie 5 | 13.31 |
| Crawford | 11.14 |
| Riverbed | 15.65 |
| City | 4.42 |
| Crew | 9.21 |
| Harbour | 8.99 |
| Raven | 10.66 |
| Film Average | 11.89 |
| Video Average | 9.79 |
| Overall Average | 10.94 |

- References

[1] Gordon, S. ABT for Film Grain Reproduction in High Definition Sequences, JVT-H029.

[2] Gordon, S. Simplified Use of 8x8 Transforms, JVT-I022.

[3] Gordon, S. et. al. Simplified Use of 8x8 Transforms – An Update, JVT-J029.

[4] Gordon, S. et. al. Simplified Use of 8x8 Transforms – JM Software Implementation Notes, JVT-J031.

[5] Bjontegaard, G. Calculation of average PSNR differences between RD-curves, VCEG-M33.

#### JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Sand Video |  |  |
| Mailing address | 200 Brickstone, Fourth Floor Andover, MA, 01810 |  |  |
| Country | ##### USA |  |  |
| Contact person | Steve Gordon |  |  |
| Telephone | 978.475.8990 x222 |  |  |
| Fax | 978.475.8995 |  |  |
| Email | SteveGordon@SandVideo.com |  |  |
| Place and date of submission |  |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Simplified Use of 8x8 Transforms |  |
| Contribution number |  | JVT-J029, J030 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
| X | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks:

Due to licensing arrangements with the Hollywood Studios, Sand Video cannot disclose the names of the movies nor show the results of the processing to non-DVD Forum members.