| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 10th Meeting: Waikoloa, Hawaii, USA, 8-12 December, 2003 | Document: JVT-J032 Filename: JVT-J032.doc |
| --- | --- |

| Title: | Proposal of a new level for super high-resolution video to MPEG-4 AVC/H.264 Profiles |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kazumasa Yamazawa, Hideaki Kimata, Ryuji Kitaura, Hitoshi Habe 3-11 Kanda Surugadai, Chiyoda-ku, Tokyo 101-0062, Japan | Tel: Email: | +81-3-3518-6435 yamazawa@is.aist-nara.ac.jp |
| Source: | JEITA 3DMM Committee |  |  |

# Introduction

A super high-resolution image brings immersive experience for almost all visual application scenarios. Currently the supposed maximum resolution is 4000x4000, which was concluded in 3DAV activities for omni-directional video applications [1][2].  This number is not only for omni-directional video but any future video application requires a high resolution.  Moreover the transmission environments for such a super high-resolution video, whose bitrate becomes over 20 Mbps, is now ready with the spread of wide bandwidth such as FTTH (Fiber To The Home).  Since such applications are in our ideas, we request to support of a super high-resolution image for MPEG-4 AVC/H.264 specifications.

# Proposed properties at a new level

Since the current specification of any MPEG-4 AVC/H.264 Profiles and Levels do not support a 4000x4000 size of image, we request to define Level 5.2 of MPEG-4 AVC/H.264 to support such an image size. Compared to the already existing Level 5.1, the proposed Level 5.2 is specifically targeting 4096x4096@15.0 Hz. In the proposed Level 5.2, only Max frame size is expanded to 65536 [MBs] (65536[MBs] was derived from 4096/16 x 4096/16), though Level 5.1 provides 36864 [MBs] as Max frame size (See “Annex A Table A-1 - with a new Level 5.2 introduced allowing 4Kx4K@15.0Hz”). Also Level 5.2 allows such a frame rate as 15.0fps for 4096x4096 (See “Annex A Table A-5 - with a new Level 5.2 introduced allowing 4Kx4K@15.0Hz”).

- Annex A Table A-1 - with a new Level 5.2 introduced allowing 4Kx4K@15.0Hz

| Level number | Max macroblock processing rate MaxMBPS (MB/s) | Max frame size MaxFS (MBs) | Max decoded picture buffer size MaxDPB (1024 bytes) | Max video bit rate MaxBR (1000 bits/s or 1200 bits/s) | Max CPB size MaxCPB (1000 bits or 1200 bits) | Vertical MV component range MaxVmvR (luma frame samples) | Min compression ratio MinCR | Max number of motion vectors per two consecutive MBs MaxMvsPer2Mb |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 485 | 99 | 148.5 | 64 | 175 | [-64,+63.75] | 2 | - |
| 1.1 | 3 000 | 396 | 337.5 | 192 | 500 | [-128,+127.75] | 2 | - |
| 1.2 | 6 000 | 396 | 891.0 | 384 | 1 000 | [-128,+127.75] | 2 | - |
| 1.3 | 11 880 | 396 | 891.0 | 768 | 2 000 | [-128,+127.75] | 2 | - |
| 2 | 11 880 | 396 | 891.0 | 2 000 | 2 000 | [-128,+127.75] | 2 | - |
| 2.1 | 19 800 | 792 | 1 782.0 | 4 000 | 4 000 | [-256,+255.75] | 2 | - |
| 2.2 | 20 250 | 1 620 | 3 037.5 | 4 000 | 4 000 | [-256,+255.75] | 2 | - |
| 3 | 40 500 | 1 620 | 3 037.5 | 10 000 | 10 000 | [-256,+255.75] | 2 | 32 |
| 3.1 | 108 000 | 3 600 | 6 750.0 | 14 000 | 14 000 | [-512,+511.75] | 4 | 16 |
| 3.2 | 216 000 | 5 120 | 7 680.0 | 20 000 | 20 000 | [-512,+511.75] | 4 | 16 |
| 4 | 245 760 | 8 192 | 12 288.0 | 20 000 | 25 000 | [-512,+511.75] | 4 | 16 |
| 4.1 | 245 760 | 8 192 | 12 288.0 | 50 000 | 62 500 | [-512,+511.75] | 2 | 16 |
| 4.2 | 491 520 | 8 192 | 12 288.0 | 50 000 | 62 500 | [-512,+511.75] | 2 | 16 |
| 5 | 589 824 | 22 080 | 41 310.0 | 135 000 | 135 000 | [-512,+511.75] | 2 | 16 |
| 5.1 | 983 040 | 36 864 | 69 120.0 | 240 000 | 240 000 | [-512,+511.75] | 2 | 16 |
| 5.2 | 983 040 | 65 536 | 69 120.0 | 240 000 | 240 000 | [-512,+511.75] | 2 | 16 |

- Annex A Table A-5 (concluded) – with a new Level 5.2 introduced allowing 4Kx4K@15.0Hz

| Level number: |  |  |  |  | 5 | 5.1 | 5.2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Max frame size (macroblocks): |  |  |  |  | 21 696 | 36 864 | 65 536 |
| Max macroblocks/second: |  |  |  |  | 589 824 | 983 040 | 983 040 |
| Max frame size (samples): |  |  |  |  | 5 554 176 | 9 437 184 | 16 777 216 |
| Max samples/second: |  |  |  |  | 150 994 944 | 251 658 240 | 251 658 240 |
| Format | Luma Width | Luma Height | MBs Total | Luma Samples |  |  |  |
| SQCIF | 128 | 96 | 48 | 12 288 | 172.0 | 172.0 | 172.0 |
| QCIF | 176 | 144 | 99 | 25 344 | 172.0 | 172.0 | 172.0 |
| QVGA | 320 | 240 | 300 | 76 800 | 172.0 | 172.0 | 172.0 |
| 525 SIF | 352 | 240 | 330 | 84 480 | 172.0 | 172.0 | 172.0 |
| CIF | 352 | 288 | 396 | 101 376 | 172.0 | 172.0 | 172.0 |
| 525 HHR | 352 | 480 | 660 | 168 960 | 172.0 | 172.0 | 172.0 |
| 625 HHR | 352 | 576 | 792 | 202 752 | 172.0 | 172.0 | 172.0 |
| VGA | 640 | 480 | 1 200 | 307 200 | 172.0 | 172.0 | 172.0 |
| 525 4SIF | 704 | 480 | 1 320 | 337 920 | 172.0 | 172.0 | 172.0 |
| 525 SD | 720 | 480 | 1 350 | 345 600 | 172.0 | 172.0 | 172.0 |
| 4CIF | 704 | 576 | 1 584 | 405 504 | 172.0 | 172.0 | 172.0 |
| 625 SD | 720 | 576 | 1 620 | 414 720 | 172.0 | 172.0 | 172.0 |
| SVGA | 800 | 600 | 1 900 | 486 400 | 172.0 | 172.0 | 172.0 |
| XGA | 1024 | 768 | 3 072 | 786 432 | 172.0 | 172.0 | 172.0 |
| 720p HD | 1280 | 720 | 3 600 | 921 600 | 163.8 | 172.0 | 172.0 |
| 4VGA | 1280 | 960 | 4 800 | 1 228 800 | 122.9 | 172.0 | 172.0 |
| SXGA | 1280 | 1024 | 5 120 | 1 310 720 | 115.2 | 172.0 | 172.0 |
| 525 16SIF | 1408 | 960 | 5 280 | 1 351 680 | 111.7 | 172.0 | 172.0 |
| 16CIF | 1408 | 1152 | 6 336 | 1 622 016 | 93.1 | 155.2 | 155.2 |
| 4SVGA | 1600 | 1200 | 7 500 | 1 920 000 | 78.6 | 131.1 | 131.1 |
| 1080 HD | 1920 | 1088 | 8 160 | 2 088 960 | 72.3 | 120.5 | 120.5 |
| 2Kx1K | 2048 | 1024 | 8 192 | 2 097 152 | 72.0 | 120.0 | 120.0 |
| 4XGA | 2048 | 1536 | 12 288 | 3 145 728 | 48.0 | 80.0 | 80.0 |
| 16VGA | 2560 | 1920 | 19 200 | 4 915 200 | 30.7 | 51.2 | 51.2 |
| 3616x1536 (2.35:1) | 3616 | 1536 | 21 696 | 5 554 176 | 27.2 | 45.3 | 45.3 |
| 3672x1536 (2.39:1) | 3680 | 1536 | 22 080 | 5 652 480 | 26.7 | 44.5 | 44.5 |
| 4Kx2K | 4096 | 2048 | 32 768 | 8 388 608 | - | 30.0 | 30.0 |
| 4096x2304 (16:9) | 4096 | 2304 | 36 864 | 9 437 184 | - | 26.7 | 26.7 |
| 4Kx4K | 4096 | 4096 | 65 536 | 16 777 216 | - | - | 15.0 |

# References

[1] ISO/IEC JTC1/SC29/WG11 N5877, “Applications and Requirements for 3DAV,” October 2003, Trondheim.

[2] ISO/IEC JTC1/SC29/WG11 N5878, “Report on 3DAV Exploration,” October 2003, Trondheim

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | JEITA 3DMM Committee |  |  |
| Mailing address | 3-11 Kanda Surugadai, Chiyoda-ku, Tokyo 101-0062, Japan |  |  |
| Country | Japan |  |  |
| Contact person | Kazumasa Yamazawa |  |  |
| Telephone | +81-3-3518-6435 |  |  |
| Fax | +81-3-3295-8726 |  |  |
| Email | yamazawa@is.aist-nara.ac.jp |  |  |
| Place and date of submission | Tokyo, Japan – Dec, 2003 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT-J032 |  |
| Title |  |  |  |
| Contribution number |  |  |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
| Ｘ | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Ｘ | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: