| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 10th JVT Meeting: Waikoloa, Hawaii, USA, 8-12 Dec., 2003 | Document: JVT-J034 Filename: input.doc |
| --- | --- |

| Title: | YCoCg and YCoCg-R Color Transform Support |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Henrique S. Malvar and Gary J. Sullivan One Microsoft Way Redmond, WA 98052 USA | Tel: Fax: Email: | +1 425 706-2978 +1 425 703-5308 +1 425 706-7329 malvar@microsoft.com garysull@microsoft.com |
| Source: | Microsoft Corp. |  |  |

_____________________________

Abstract – This document contains remarks regarding proposed additional color spaces for the H.264/MPEG-4 AVC specification's "professional extensions".  We believe that there is merit in the addition of VUI indications for reversible integer color transforms -- including in particular YCoCg and its reduced-dynamic-range variant YCoCg-R, and very possibly others as well.  We believe the threshold for adoption of a color transform VUI indication should not be set very high, as these indicators, like SEI, have no normative effect.  However, if there is a need to select between different such color transform designs, we suggest using simplicity as the primary deciding factor, as we do not expect significant differences in coding efficiency between reasonably well-designed such color transforms in this context.

Slightly-expanded remarks:

- We continue to find merit in proposing support of the YCoCg-R color space introduced in [JVT-I014 (r3)] as a simple and effective color transform that offers reduced bit-depth expansion within the constraints of integer invertibility.

- We also continue to find merit in proposing support of the YCoCg color space introduced in [JVT-H031 (r2)] with additional remarks in [JVT-I014 (r3)] as a simple and effective color transform providing a form of integer invertibility with equal bit depth per component.

- However, we continue to be unable to supply coded video test results due to the lack of availability of JVT reference software supporting N-bit 4:4:4 video.  (Some software became available on the day before the contribution deadline for this meeting, so hopefully it will be possible to provide test results in the near future.)

- We suggest that the JVT not be overly "stingy" with the assignment of indicators for color spaces.  The color space indication is only the provision of an Annex E VUI indication that does not affect normative decoding behavior in any way.   Similar to SEI, the VUI is only there for those who want to use it.  We also note that "unspecified" is a value of color-space indication already supported in the VUI of the current specification, and therefore it is already possible to use such additional color spaces (e.g., with an external indication of the color space in use, or an indication found in an externally-specified but in-band user data SEI message) – it is just not documented how to express it.  Providing such documentation of reserved VUI values will not significantly "bloat" the specification (probably only a half-page or so of text is needed for each additional color transform), and support of additional color space indications places no normative burden on either encoders or decoders.  As with SEI, no complexity burden is imposed on those who do not want to use this supplemental data.  We therefore suggest that if several new color transform indicators are under consideration in the JVT when a decision needs to be made, a good option for the JVT is to simply adopt all of them that are on the table that appear to have some merit.  For example, there are 250 available reserved entries in Table E-5, so there are at least 250 additional color spaces that could be supported in the specification without difficulty (not that we advocate that many additions).

- However, we expect the coding efficiency performance of the use of various "sensible" color transforms on real video data for this codec design will be very similar.  Therefore, although we do not feel that there is a need to be "stingy" with assignment of color space indicators (see item 4 above), we believe that if there is a desire to choose between different simple integer color transforms, the primary appropriate distinction (and therefore the primary appropriate decision factor) will be with regard to the simplicity (or lack thereof) of the intended color conversion processing.  Although that complexity of the conversion process will only be incurred in systems that choose to perform that process (since the VUI requires no normative support), it may be a reasonably good indicator of whether someone would be likely to make that choice or not.

# References

[JVT-I014 (r3)]    H. S. Malvar and G. J. Sullivan, "YCoCg-R: A Color Space with RGB Reversibility and Low Dynamic Range", Document JVT-I014 (r3) of Joint Video Team (JVT) of ISO/IEC JTC1/SC29/WG11 (MPEG) and ITU-T SG16 Q.6 (VCEG), Trondheim, July 2003 (August revision).

[JVT-H031 (r2)]    H. S. Malvar and G. J. Sullivan, “Transform, Scaling & Color Space Impact of Professional Extensions,” Document JVT-H031 (r2) of Joint Video Team (JVT) of ISO/IEC JTC1/SC29/WG11 (MPEG) and ITU-T SG16 Q.6 (VCEG), Geneva, May 2003.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Microsoft Corp. |  |  |
| Mailing address | One Microsoft Way Redmond, WA 98052 USA |  |  |
| Country | USA |  |  |
| Contact person | Gary J. Sullivan |  |  |
| Telephone | +1 425 703-5308 |  |  |
| Fax | +1 425 706-7329 |  |  |
| Email | garysull@microsoft.com |  |  |
| Place and date of submission | 10th JVT Meeting: Waikoloa, Hawaii, USA, 8-12 Dec., 2003 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | ITU-T Rec. H.264 ¦ ISO/IEC 14496-10 Professional Extensions |  |
| Title |  | Advanced Video Coding for Generic Audiovisual Services: Professional Extensions Amendment |  |
| Contribution number |  | JVT-J034 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
| X | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: