| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 10th Meeting: Waikoloa, Hawaii, USA, 8-12 December, 2003 | Document: JVT-J038 Filename: JVT-J038.doc |
| --- | --- |

| Title: | Residue Transform |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Woo-Shik Kim, Hyun Mun Kim Multimedia Lab. Samsung AIT P.O.Box 111, Suwon 440-600 Korea | Tel: Email: | 82-31-280-9204 hyunmun27.kim@samsung.com |
| Source: | Samsung AIT |  |  |

_____________________________

- Introduction

In this proposal, we propose new coding method that exploits the redundancy in RGB space after intra/inter prediction. This method was motivated by the observation that there still exists redundancy among data even after intra/inter prediction. As addressed in JVT-J017, for some images, the statistical properties of the color space converted data do not conform to the assumed intra/inter redundancy of current JVT specification [1]. In that case it is more efficient to apply intra/inter directly to RGB data first. Then we can find suitable transform that matches well to the statistical properties of the residue.

We can control this option using a table that specifies the possible transform. It can be inter-plane prediction, or no transform at all.

Section 2 briefly describes the proposed algorithm. The test results are summarized in Section 3. We conclude in Section 4.

- Residue Transform Coding

The proposed residue transform is to use the residue redundancy more effectively. Figure 1 shows the block diagram of the proposed algorithm. This scheme can be easily incorporated into the existing JVT specification [1]. The rationale is that the statistical properties of the residue vary depending on input materials as observed in [2]. So we propose to use a table that specifies the possible transform in the sequence parameter set as defined below.

Transform/

Quant.

RGB

Intra/inter

prediction

Entropy

Coding

Residue

Transform

Figure 1: Block diagram of the proposed residue transform

Table 1: Residue Transform value

| Residue transform indicator | Transform |
| --- | --- |
| 0 | IPP |
| 1 | No transform |
| 2 | YCoCg |

In Table 1, there are four options to adapt to the different characteristics of the input materials. We can select this option depending on residue characteristics at encoder’s discretion. Option 0 is inter-plane prediction (IPP) that is described in [2, 3]. Option 1 is to code the material directly without any color space conversion. Option 2 is to apply YCoCg as defined in [4].

Table 1 shall be coded using Exp-Golomb code.

- Residue Sampling

We can further achieve coding efficiency using residue transform followed by residue subsampling. Figure 2 shows the block diagram of the proposed algorithm. The idea is after performing residue transform we apply subsampling to the transformed coefficients. It can be 4:2:2, 4:2:0 or any other user defined subsampling depending on the residue transform applied.

Transform/

Quant.

RGB

Intra/inter

prediction

Entropy

Coding

Residue

Transform

Residue

Sampling

Figure 2: Block diagram of the proposed residue sampling

- Experimental Results

We compared our residue transform method using the above options with YCoCg-R proposed in [4]. The test conditions are summarized as follows:

- Search range: 32

- Entropy coding: CABAC

- RD-optimized mode selection: On

- GOP structure: I frame only (Intra4x4 mode only)

- Number of slice groups: 1

We tested this residue transform options over various image set. The complete test results are available in the accompanying excel file (JVT-J038.xls). The simulation results manifest that it is desirable to apply inter/intra prediction followed by color transform. If we apply color transform first before coding, the coding efficiency deteriorates. It becomes severe especially on film material or unprocessed images. Due to the film grain or shot noise, the color space converted data are bad fit for the JVT inter/intra prediction. It tells that the coding gain of using intra/inter redundancy is bigger than that of using inter-color redundancy.

Based on this observation and experimental results, the best method to code RGB material is to code the given image in the given space. Especially for high bit rate it is the best way to code RGB material without color space conversion. By properly using the color transform option, we can always achieve good coding efficiency over wide range of bit rate.

- Conclusions

The use of residue transform in RGB space achieves great coding efficiency for various kinds of RGB data including film material and professional filmstream material. The advantage is clearly shown when we compared the residue transform using YCoCg with YCoCg-R. It tells that it is desirable to apply inter/intra prediction followed by color transform. We can select residue transform option depending on image characteristics at encoder’s discretion. By putting this selection table on sequence parameter set, we minimize the switching of different transform. It helps achieve the best coding efficiency for any type of input data.

References

[1] “Final Draft International Standard of Joint Video Specification (ITU-T Rec. H.264 | ISO/IEC 14496-10 AVC),” ISO/IEC JTC1/SC29/WG11 and ITU-T Q6/SG16,Q.6 Document JVT-G050, March 2003.

[2] W.-S. Kim, D. Cho and H. M. Kim, “Inter-plane Prediction for RGB Coding II,” ISO/IEC JTC1/SC29/WG11 and ITU-T Q6/SG16,Document JVT-J017, December 2003.

- [3] W.-S. Kim, D. Cho and H. M. Kim, “Inter-plane Prediction for RGB Coding,” ISO/IEC JTC1/SC29/WG11 and ITU-T Q6/SG16,Document JVT-I023, September 2003.

- [4] H. Malvar and G. Sullivan, “YCoCg-R: A Color Space with RGB Reversibility and Low Dynamic Range,” ISO/IEC JTC1/SC29/WG11 and ITU-T Q6/SG16,Document JVT-I014, September 2003.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Samsung Elctronics. Co. Ltd. |  |  |
| Mailing address | POBox 111 Suwon Samsung AIT Multimedia |  |  |
| Country | Korea |  |  |
| Contact person | Hyun Mun Kim |  |  |
| Telephone | 82-31-280-9204 |  |  |
| Fax | 82-31-280-9207 |  |  |
| Email | hyunmun27.kim@samsung.com |  |  |
| Place and date of submission | Hawaii, USA, 8-12 December, 2003 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Residue Transform |  |
| Contribution number |  | JVT-J038 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
| X | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| # X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: