| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) JVT Meeting: Waikoloa, HI, USA, 7-12 December, 2003 | Document: JVT-J045r1 Filename: JVT-J045r1.doc |
| --- | --- |

| Title: | Editor's Proposed Draft AVC¦H.264 Conformance Spec. |  |  |
| --- | --- | --- | --- |
| Status: | Output Document from JVT |  |  |
| Purpose: | Draft Conformance Specification |  |  |
| Author(s) or Contact(s): | Teruhiko Suzuki 6-7-35, Kitashinagawa, Shinagawa-ku Tokyo, 141-0001, Japan | Tel: Email: | +81-3-5795-7966 teruhiko@av.crl.sony.co.jp |
| Source: | Sony Corp. |  |  |

_____________________________

- ISO/IEC JTC 1/SC 29 N

Date:   2003-12-XX

- ISO/IEC 14496-4:2003/FPDAM 6

- ISO/IEC TC JTC 1/SC 29/WG 11

Secretariat:

Information technology — Coding of audio-visual objects -- — Part 4: Conformance Testing - AMENDMENT 6: Advanced Video Coding Conformance

Élément introductif — Élément central — Partie 2: Titre de la partie

- Warning

This document is not an ISO International Standard. It is distributed for review and comment. It is subject to change without notice and may not be referred to as an International Standard.

Recipients of this document are invited to submit, with their comments, notification of any relevant patent rights of which they are aware and to provide supporting documentation.

- Copyright notice

This ISO document is a working draft or committee draft and is copyright-protected by ISO. While the reproduction of working drafts or committee drafts in any form for use by participants in the ISO standards development process is permitted without prior permission from ISO, neither this document nor any extract from it may be reproduced, stored or transmitted in any form for any other purpose without prior written permission from ISO.

Requests for permission to reproduce this document for the purpose of selling it should be addressed as shown below or to ISO's member body in the country of the requester:

[Indicate the full address, telephone number, fax number, telex number, and electronic mail address, as appropriate, of the Copyright Manger of the ISO member body responsible for the secretariat of the TC or SC within the framework of which the working document has been prepared.]

Reproduction for sales purposes may be subject to royalty payments or a licensing agreement.

Violators may be prosecuted.

- Introduction

In this document, the additional text to ITU-T Rec. H.264.1 | ISO/IEC 14496-4 is specified for the conformance test of ITU-T Rec. H.264 | ISO/IEC 14496-10 video decoders. This specification is specifically applied to ITU-T Rec. H.264 | ISO/IEC 14496-10, Advanced Video Coding.

The following subclauses specify the normative tests for verifying conformance of ITU-T Rec. H.264 | 14496-10 video bitstreams and video decoders. Those normative tests make use of test data (bitstream test suites) provided as an electronic annex to this document, and of the reference software decoder specified in ITU-T Rec. H.264.2 | ISO/IEC 14496-5 with source code available in electronic format.

Information technology — Coding of audio-visual objects -- — Part 4: Conformance Testing -- AMENDMENT 6: Advanced Video Coding Conformance

- Add the following clauses after the SNHC chapter (as ISO/IEC 14496-4 Amendment), or as Rec. H.264.1 (as ITU-T Recommendation):

# Conformance for ITU-T Rec. H.264 | ISO/IEC 14496-10

## Introduction

This text provides normative conformance tests of ITU-T Rec. H.264 | ISO/IEC 14496-10. In this clause, except where stated otherwise, the following terms are used for practical purposes. The term 'bitstream' refers to an ITU-T Rec. H.264 | ISO/IEC 14496-10 video bitstream. A bitstream may contain IDR, I, P, B, SI, and SP slices. The term 'decoder' refers to an ITU-T Rec. H.264 | ISO/IEC 14496-10 video decoder, i.e., an embodiment of the decoding process specified by ITU-T Rec. H.264 | ISO/IEC 14496-10. The decoder does not include the display process, which is outside the scope of this standard.

The bitstream conformance and the decoder conformance are specified in the Annex C.3 and C.4 of ITU-T Rec. H.264 | ISO/IEC 14496-10. The tested output from a decoder are the output samples including both luma and chroma.

The term ‘reference software decoder’ means the software decoders contained in ITU-T Rec. H.264.2 | ISO/IEC 14496-5 AMD6. It is possible to use this software to test and verify that some of the requirements specified in ITU-T Rec. H.264 | ISO/IEC 14496-10 are met by the bitstream. If any statement stated in this text accidentally contradicts a statement or requirement defined in ITU-T Rec. H.264 | ISO/IEC 14496-10, the text of ITU-T Rec. H.264 | ISO/IEC 14496-10 prevails.

The following subclauses specify the normative tests for verifying conformance of video bitstreams decoders. Those normative tests make use of test data (bitstream test suites) provided as an electronic annex to this document, and of a reference software decoder specified in ITU-T Rec. H.264.2 | ISO/IEC 14496-5 AMD6 with source code available in electronic format.

## Procedure to test decoder

A bitstream that claims conformance with this standard shall pass the following normative test:

The bitstream shall be decoded by processing it with the reference software decoder specified in ITU-T Rec. H.264.2 | ISO/IEC 14496-5 AMD6.  When processed by the reference software decoder, the bitstream shall not cause any error or non-conformance messages to be reported by the reference software decoder. This test should not be applied to bitstreams that are known to contain errors introduced by transmission, as such errors are highly likely to result in bitstream that lacks conformance to ITU-T Rec. H.264 | ISO/IEC 14496-10.

Successfully passing the reference software decoder test provides only a strong presumption that the bitstream under test is conforming, i.e. that it does indeed meet all the requirements specified in ITU-T Rec. H.264 | ISO/IEC 14496-10 that are tested by the reference software decoder.

Additional tests may be necessary to more thoroughly check that the bitstream properly meets all the requirements specified in ITU-T Rec. H.264 | ISO/IEC 14496-10. These complementary tests may be performed using other video bitstream verifiers that perform more complete tests than those implemented by the reference software decoder.

ITU-T Rec. H.264 | ISO/IEC 14496-10 contains several informative recommendations that are not an integral part of that Recommendation | International Standard. When testing a bitstream for conformance, it may also be useful to test whether or not the bitstream follows those recommendations.

To check correctness of a bitstream, it is necessary to parse the entire bitstream and to extract all the syntactic elements and other values derived from those syntactic elements and used by the decoding process specified in ITU-T Rec. H.264 | ISO/IEC 14496-10.

A verifier does not necessarily perform all stages of the decoding process described in ITU-T Rec. H.264 | ISO/IEC 14496-10 in order to verify bitstream correctness. Many tests are performed on syntax elements in a state prior to their use in some processing stages.

### Requirement on output of the decoding process and timing

The output of the decoding process is specified by clause 8 and Annex C of ITU-T Rec. H.264 | ISO/IEC 14496-10. It is a requirement that all samples of all the decoded pictures specified for output in clause 8 and Annex C of ITU-T Rec. H.264 | ISO/IEC 14496-10 be output by a conforming decoder. It is a requirement that a conforming decoder outputs the reconstructed samples at the rates and times specified in clause 8 and Annex C of ITU-T Rec. H.264 | ISO/IEC 14496-10.

The display process, which ordinarily follows the output of the decoding process, is outside the scope of this Recommendation | International Standard.

### Recommendations (informative)

In addition to the requirements, it is desirable that conforming decoders implement various informative recommendations defined in ITU-T Rec. H.264 | ISO/IEC 14496-10 that are not an integral part of that Recommendation | International Standard. This subclause lists some of these recommendations.

It is recommended that a conforming decoder be able to resume the decoding process as soon as possible after the loss or corruption of part of a bitstream. In most cases it is possible to resume decoding at the next start code or slice header. It is recommended that a conforming decoder be able to perform concealment for the macroblocks or video packets for which all the coded data has not been received.

## Procedure to test decoder conformance

In this subclause, except where stated otherwise, the term "bitstream" refers to a conforming ITU-T Rec. H.264 | ISO/IEC 14496-10 video bitstream (as defined in this document), that has values of profile_idc, level_idc, and constraint_setX_flag values (where X is a number in the range of 0 to 2, inclusive) corresponding to a set of specified constraints on a bitstream for which a decoder conforming to a specified profile and level is required in Annex A of ITU-T Rec. H.264 | ISO/IEC 14496-10 to properly perform the decoding process.

### Contents of bitstream file

The following information is included in a single zipped file for each stream.

- ITU-T Rec. H.264 | ISO/IEC 14496-10 video bitstream

- Reconstructed pictures or hashes of decoded pictures

- Short description of the bitstream

- Trace file (The bitstream in ASCII format)

### Static tests

Static tests of a video decoder requires testing of the reconstructed samples. This subclause will explain how this test can be accomplished when the reconstructed samples at the output of the decoding process are available. It may not be possible to perform this type of test with a production decoder. In that case this test should be performed by the manufacturer during the design and development phase. Static tests are used for testing the  decoding process. The test will check that the values of the samples reconstructed by the decoder under test shall be identical to the values of the samples attached to the bitstream file, or shall be identical to the values of the samples reconstructed by the reference decoder in cases where the values of the samples are not attached to the bitstream file.  When a hash of the values of the samples of the decoded pictures is attached to the bitstream file, a corresponding hash operation performed on the values of the samples of the decoded pictures produced by the decoder under test shall produce the same results.

### Dynamic tests

Dynamic tests are applied to check that all the reconstructed samples are output to the display process and that the timing of the output of the decoder's reconstructed samples conforms to the specification of clause 8 and Annex C of ITU-T Rec. H.264 | ISO/IEC 14496-10, and to verify that the HRD models (as defined by Annex C of ITU-T Rec. H.264 | ISO/IEC 14496-10, CPB and DPB specification) are not violated when the bits are delivered at the proper rate.

## Specification of the test bitstreams

[Ed.Note: The bitstreams in this subclause are temporally defined. The specification of each bitstream is the same as the bitstream verified through bitstream exchange activity. The bitstreams verified through bitstream exchange activity are available at the following ftp site.

ftp://ftp.imtc-files.org/jvt-experts/draft_conformance

]

### Test Bitstreams – General

Test bitstream #AVCNL-1, #AVCNL-2

Specification:  All slices are coded as I slices. Each picture contains only one slice. disable_deblocking_filter_idc is equal to 1, specifying disabling of the deblocking filter process. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 0. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of I slices.

Purpose:  Check that decoder can properly decode I slices.

Test bitstream #AVCNL-3, #AVCNL-4

Specification:  All slices are coded as I or P slices. Each picture contains only one slice. disable_deblocking_filter_idc is equal to 1, specifying disabling of the deblocking filter process. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 0. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of P slices.

Purpose:  Check that decoder can properly decode P slices.

Test bitstream #AVCBA-1

Specification:  All slices are coded as I slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 0. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of I slices with the deblocking filter process enabled.

Purpose:  Check that the decoder can properly decode I slices with the deblocking filter process enabled.

Test bitstream #AVCBA-2

Specification:  All slices are coded as I slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 2. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of I slices with the deblocking filter process enabled.

Purpose:  Check that the decoder can properly decode I slices with the deblocking filter process enabled.

Test bitstream #AVCBA-3

Specification:  All slices are coded as I or P slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 0. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of P slices with the deblocking filter process enabled.

Purpose:  Check that the decoder can properly decode P slice with the deblocking filter process enabled.

Test bitstream #AVCBA-4

Specification:  All slices are coded as I or P slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 2. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of P slices with the deblocking filter process enabled.

Purpose:  Check that the decoder can properly decode P slices with the deblocking filter process enabled.

Test bitstream #AVCBA-5, #AVCBA-6

Specification:  All slices are coded as I or P slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 0. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of P slices with the deblocking filter process enabled.

Purpose:  Check that the decoder can properly decode P slices with the deblocking filter process enabled.

Test bitstream #AVCMQ-1

Specification:  All slices are coded as I slices. Each picture contains only one slice. disable_deblocking_filter_idc is equal to 1, specifying disabling of the deblocking filter process. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 1. mb_qp_delta is equal to a non-zero value to change the quantizer scale at each MB. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of I slices with mb_qp_delta not equal to 0.

Purpose:  Check that decoder can properly decode I slices with mb_qp_delta not equal to 0.

Test bitstream #AVCMQ-2

Specification:  All slices are coded as I or P slices. Each picture contains only one slice. disable_deblocking_filter_idc is equal to 1, specifying disabling of the deblocking filter process. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 1. mb_qp_delta is equal to a non-zero value to change the quantizer scale at each MB. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of P slices with mb_qp_delta not equal to 0.

Purpose:  Check that decoder can properly decode P slices with mb_qp_delta not equal to 0.

Test bitstream #AVCMQ-3

Specification:  All slices are coded as I slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 1. Spatial direct prediction is used for direct prediction. mb_qp_delta is equal to a non-zero value to change the quantizer scale at each MB. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of I slices with mb_qp_delta not equal to 0.

Purpose:  Check that decoder can properly decode I slices with mb_qp_delta not equal to 0.

Test bitstream #AVCMQ-4

Specification:  All slices are coded as I or P slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 1. Spatial direct prediction is used for direct prediction. mb_qp_delta is equal to a non-zero value to change the quantizer scale at some MBs. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of P slices with mb_qp_delta not equal to 0.

Purpose:  Check that decoder can properly decode P slices with mb_qp_delta not equal to 0.

Test bitstream #AVCSL-1

Specification:  All slices are coded as I or P slices. Each picture contains more than one slice. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 2. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of I and P slices.

Purpose:  Check that decoder can properly decode pictures with multiple slices.

Test bitstream #AVCSL-2

Specification:  All slices are coded as I or P slices. Each picture contains more than one slice. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 0. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of I and P slices.

Purpose:  Check that decoder can properly decode pictures with multiple slices.

Test bitstream #AVCSQ-1

Specification:  All slices are coded as I slices. Each picture contains 20 slices. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 0. Spatial direct prediction is used for direct prediction. slice_qp_delta is equal to a non-zero value to change the quantizer scale at each slice. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of I slices with non-zero values of slice_qp_delta.

Purpose:  Check that decoder can properly decode I slices with non-zero values of slice_qp_delta.

Test bitstream #AVCFM-1

Specification:  All slices are coded as I or P slices. The number of slices and slice groups is greater than 1 in each picture. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 0. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10. Multiple parameter sets are included in the bitstream.

Functional stage: Slice groups.

Purpose:  Check that decoder handles multiple slice groups and parameter sets.

Test bitstream #AVCCI-1

Specification:  All slices are coded as I or P slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 0. constrained_intra_pred_flag is equal to 1. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Constrained intra prediction.

Purpose:  Check that decoder handles constrained intra prediction.

Test bitstream #AVCCI-2

Specification:  All slices are coded as I or P slices. Each picture contains more than one slice. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 0. constrained_intra_pred_flag is equal to 1. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Constrained intra prediction

Purpose:  Check that decoder handles constrained intra prediction.

Test bitstream #AVCFC-1

Specification:  All slices are coded as I or P slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 0. Decoded pictures are cropped with frame_croping_flag equal to 1. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of I and P slices with frame cropping.

Purpose:  Check that decoder can properly decode I and P slices with frame cropping.

Test bitstream #AVAUD-1

Specification:  All slices are coded as I slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 0. Access unit delimiter NAL units are included in the stream. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of I slices with Access unit delimiter NAL units.

Purpose:  Check that decoder can properly decode I slices with Access unit delimiter NAL units.

Test bitstream #AVCMIDR-1

Specification:  All slices are coded as I or P slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 0. IDR is inserted in every two frame. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of I slices and more than one IDR.

Purpose:  Check that decoder can properly decode I slices with more than IDR in bitstream.

Test bitstream #AVCNRF-1

Specification:  All slices are coded as I or P slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 0. Two non-reference pictures are present. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of I and P slices with non-reference pictures.

Purpose:  Check that decoder can properly decode I and P slices with non-reference pictures .

Test bitstream #AVCCQP-1

Specification:  All slices are coded as I or P slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 2. chroma_qp_index_offset varies at each picture. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of I and P slices with non-reference pictures and non-zero chroma_qp_index_offset.

Purpose:  Check that decoder can properly decode I and P slices with non-reference pictures and non-zero chroma_qp_index_offset.

Test bitstream #AVCBS-1

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 0. Temporal direct prediction is used for direct prediction. direct_8x8_inference_flag is equal to 1.  All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of B slices with temporal direct prediction.

Purpose:  Check that decoder can properly decode B slices with temporal direct prediction.

Test bitstream #AVCBS-2

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 0. Spatial direct prediction is used for direct prediction. direct_8x8_inference_flag is equal to 1.  All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of B slices with spatial direct prediction.

Purpose:  Check that decoder can properly decode B slices with spatial direct prediction.

### Test Bitstreams – I_PCM

Test bitstream #AVCPCM-1, AVCPCM-2

Specification:  All slices are coded as I slices. Each picture contains only one slice. disable_deblocking_filter_idc is equal to 1, specifying disabling of the deblocking filter process. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 0. mb_type is equal to I_PCM for some macroblocks. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of macroblocks with mb_type equal to I_PCM

Purpose:  Check that decoder can properly decode macroblocks with mb_type equal to I_PCM.

### Test Bitstreams – Memory management control operation

Test bitstream #AVCMR-1

Specification:  All slices are coded as I or P slices. Each picture contains more than one slice. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 1. Reference picture list reordering and memory management control operations are used. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Reference picture list reordering and memory management control operations.

Purpose:  Check that decoder handles reference picture list reordering and memory management control operations.

Test bitstream #AVCMR-2

Specification:  All slices are coded as I or P slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 2. Reference picture list reordering and memory management control operations are used. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Reference picture list reordering and memory management control operations.

Purpose:  Check that decoder handles reference picture list reordering and memory management control operations.

Test bitstream #AVCMR-3

Specification:  All slices are coded as I or P slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 2. gaps_in_frame_num_value_allowed_flag is equal to 1. Reference picture list reordering and various memory management control operation is used. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Reference picture list reordering and memory management control operations.

Purpose:  Check that decoder handles gaps in frame_num, reference picture list reordering and memory management control operations.

Test bitstream #AVCMR-4

Specification:  All slices are coded as I or P slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 0. gaps_in_frame_num_value_allowed_flag is equal to 1. Reference picture list reordering and various memory management control operations are used. The decoding order is different from the output order. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Reference picture list reordering, memory management control operations and non-increasing PicOrderCnt values.

Purpose:  Check that decoder handles reference picture list reordering and memory management control operations. Test output order conformance for non-increasing PicOrderCnt values.

Test bitstream #AVCMR-5

Specification:  All slices are coded as I or P slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 1. gaps_in_frame_num_value_allowed_flag is equal to 1. Reference picture list reordering and various memory management control operation is used. The decoding order is different from the output order. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Reference picture list reordering, memory management control operation and non-increasing PicOrderCnt values.

Purpose:  Check that decoder handles gaps_in_frame_num_value_allowed_flag equal to 1, reference picture list reordering and memory management control operation. Test output order conformance for non-increasing PicOrderCnt values.

### Test Bitstreams – Weighted sample prediction process

Test bitstream #AVCWP-1

Specification:  All slices are coded as I or P slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 2. weighted_pred_flag is equal to 1. Plural reference indices are assigned to each reference picture. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Weighted sample prediction process for P slices with plural reference indices.

Purpose:  Check that decoder handles weighted sample prediction for P slices with plural reference indexes.

Test bitstream #AVCWP-2

Specification:  All slices are coded as I or P slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 2. weighted_pred_flag is equal to 1. All NAL units are encapsulated into the byte stream

Functional stage:  Weighted sample prediction process for P slices.

Purpose:  Check that decode handles weighted sample prediction for P slices.

Test bitstream #AVCWP-3

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 0. Temporal direct prediction is used for direct prediction. weighted_bipred_idc is equal to 2. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Weighted sample prediction process for B slices with temporal direct prediction.

Purpose:  Check that decoder handles weighted sample prediction for B slices with temporal direct prediction.

### Test Bitstreams – Slice of coded field

Test bitstream #AVCFI-1

Specification:  All slices are coded as I or P slices. Each picture contains only one slice. Each slice is a coded field. disable_deblocking_filter_idc is equal to 1, specifying disabling of the deblocking filter process. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 0. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Slices of coded fields.

Purpose:  Check that decoder handles I and P slices of coded fields.

Test bitstream #AVCFI-2

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. disable_deblocking_filter_idc is equal to 1, specifying disabling of the deblocking filter process. Each slice is a coded field. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 0. Spatial direct prediction is used for direct prediction. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Slices of coded fields with spatial direct prediction.

Purpose:  Check that decoder handles B slices of coded fields with spatial direct prediction.

Test bitstream #AVCFI-3

Specification:  All slices are coded as I or P slices. Each picture contains only one slice. Each slice is a coded field. disable_deblocking_filter_idc is equal to 1, specifying disabling of the deblocking filter process. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 1. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Slices of coded fields

Purpose:  Check that decoder handles I and P slices of coded fields.

Test bitstream #AVCFI-4

Specification:  All slices are coded as I or P slices. Each picture contains only one slice. Each slice is a coded field. disable_deblocking_filter_idc is equal to 1, specifying disabling of the deblocking filter process. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 1. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Slices of coded fields.

Purpose:  Check that decoder handles I and P slices of coded fields.

Test bitstream #AVCFI-5

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. disable_deblocking_filter_idc is equal to 1, specifying disabling of the deblocking filter process. Each slice is a coded field. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 1. Spatial direct prediction is used for direct prediction. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Slices of coded fields.

Purpose:  Check that decoder handles B slices of coded fields.

Test bitstream #AVCFI-6

Specification:  All slices are coded as I or P slices. Each picture contains more than one slice. Each slice is a coded field. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 0. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Slices of coded fields.

Purpose:  Check that decoder handles I and P slices of coded fields.

Test bitstream #AVCFI-7

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. Each slice is a coded field. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 0. Temporal direct prediction is used for direct prediction. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Slices of coded fields with temporal direct prediction.

Purpose:  Check that decoder handles B slices of coded fields with temporal direct prediction.

Test bitstream #AVCFI-8

Specification:  All slices are coded as I slices. Only one slices is contained in each picture. Each slice is a coded field. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 0. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Slices of coded fields.

Purpose:  Check that decoder handles I slices of coded fields.

### Test Bitstreams – Frame/field coding

Test bitstream #AVCPA-1

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. disable_deblocking_filter_idc is equal to 1, specifying disabling of the deblocking filter process. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 1. Spatial direct prediction is used for direct prediction. Each slice is either a coded frame or a coded field. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Slices of coded frames/fields.

Purpose:  Check that decoder can properly decode slices of coded frames and  fields.

Test bitstream #AVCPA-2

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 0. Spatial direct prediction is used for direct prediction. Each slice is either a coded frame or a coded field. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Slices of coded frames/fields.

Purpose:  Check that decoder can properly decode slices of coded frames and fields.

### Test bitstream - Macroblock adaptive frame/field coding

Test bitstream #AVCMA-1

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. disable_deblocking_filter_idc is equal to 1, specifying disabling of the deblocking filter process. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 0. Spatial direct prediction is used for direct prediction. mb_adaptive_frame_field_coding is equal to 1. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Macroblock adaptive frame field decoding.

Purpose:  Check that decoder can properly decode slices with mb_adaptive_frame_field_flag=1.

Test bitstream #AVCMA-2

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. disable_deblocking_filter_idc is equal to 1, specifying disabling of the deblocking filter process. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 0. Temporal direct prediction is used for direct prediction. mb_adaptive_frame_field_coding is equal to 1. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Macroblock adaptive frame field decoding.

Purpose:  Check that decoder can properly decode slices with mb_adaptive_frame_field_flag=1.

Test bitstream #AVCMA-3

Specification:  All slices are coded as I slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 0. mb_adaptive_frame_field_coding is equal to 1. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Macroblock adaptive frame field decoding.

Purpose:  Check that decoder can properly decode slices with mb_adaptive_frame_field_flag=1.

Test bitstream #AVCMA-4

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 0. Spatial direct prediction is used for direct prediction. mb_adaptive_frame_field_coding is equal to 1. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Macroblock adaptive frame field decoding.

Purpose:  Check that decoder can properly decode slices with mb_adaptive_frame_field_flag=1.

### Test Bitstreams – S picture

Test bitstream #AVCSP-1

Specification:  All slices are coded as I, P and SP slice. Each picture contains more than one slice. disable_deblocking_filter_idc is equal to 1, specifying disabling of the deblocking filter process. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 1. memory_management_operation is seto to 5 on SP slice. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of SP slice.

Purpose:  Check that decoder can properly decode SP slice.

### Test Bitstreams – CABAC: Basic features

Test bitstream #AVCCANL-1

Specification:  All slices are coded as I slices. Each picture contains only one slice. disable_deblocking_filter_idc is equal to 1, specifying disabling of the deblocking filter process. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 2. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of I slice with CABAC parsing.

Purpose:  Check that decoder can properly decode I slices with CABAC parsing.

Test bitstream #AVCCANL-2

Specification:  All slices are coded as I slices. Each picture contains only one slice. disable_deblocking_filter_idc is equal to 1, specifying disabling of the deblocking filter process. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of I slice with CABAC parsing.

Purpose:  Check that decoder can properly decode I slices with CABAC parsing.

Test bitstream #AVCCANL-3

Specification:  All slices are coded as I or P slices. Each picture contains only one slice. disable_deblocking_filter_idc is equal to 1, specifying disabling of the deblocking filter process. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of P slices with CABAC parsing.

Purpose:  Check that decoder can properly decode P slices with CABAC parsing.

Test bitstream #AVCCANL-4

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. disable_deblocking_filter_idc is equal to 1, specifying disabling of the deblocking filter process. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. Spatial direct prediction is used for direct prediction. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of B slices with CABAC parsing.

Purpose:  Check that decoder can properly decode B slices with CABAC parsing.

Test bitstream #AVCCABA-1

Specification:  All slices are coded as I slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of I slice with the deblocking filter process enabled and CABAC.

Purpose:  Check that decoder can properly decode I slices with CABAC parsing.

Test bitstream #AVCCABA-2

Specification:  All slices are coded as I or P slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of P slices with CABAC parsing.

Purpose:  Check that decoder can properly decode P slices with CABAC parsing.

Test bitstream #AVCCABA-3

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. Temporal direct prediction is used for direct prediction. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of B slices with CABAC parsing.

Purpose:  Check that decoder can properly decode B slices with CABAC parsing.

Test bitstream #AVCCABA-4

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 2. Temporal direct prediction is used for direct prediction. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of B slices with CABAC parsing.

Purpose:  Check that decoder can properly decode B slices with CABAC parsing.

### Test Bitstreams – CABAC: Initialization

Test bitstream #AVCCAIN-1

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. Temporal direct prediction is used for direct prediction. direct_8x8_inference_flag is equal to 0. cabac_init_idc is equal to 0, 1, or 2 at slice header. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Initialization of CABAC.

Purpose:  Check that decoder can initialize CABAC with cabac_init_idc=0, 1, or 2.

### Test Bitstreams – CABAC: MB QP Delta

Test bitstream #AVCCAQP-1

Specification:  All slices are coded as I slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 1. mb_qp_delta is equal to non-zero value to change the quantizer scale at each MB. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of I slices with mb_qp_delta not equal to 0.

Purpose:  Check that decoder can properly decode I slices with mb_qp_delta not equal to 0.

Test bitstream #AVCCAQP-2

Specification:  All slices are coded as I, P or B slices. Each picture contains more than one slice. Each slice has different size. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. Temporal direct prediction is used for direct prediction. mb_qp_delta is equal to non-zero value to change the quantizer scale at each MB. disable_deblocking_filter_idc is equal to 2. chroma_qp_index_offset is equal to non-zero value. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of I, P, and B slices with mb_qp_delta not equal to 0.

Purpose:  Check that decoder can properly decode I slices with mb_qp_delta not equal to 0, disable_deblocking_filter_idc equal to 2, and non-zero chroma_qp_index_offset.

### Test Bitstreams – CABAC: Slice

Test bitstream #AVCCASL-1

Specification:  All slices are coded as I, P or B slices. Each picture contains more than one slice. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. Temporal direct prediction is used for direct prediction. direct_8x8_inference_flag is equal to 0. Each picture contains more than one slice. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of different slice types in a picture with CABAC parsing.

Purpose:  Check that decoder can properly decode different slice types in a picture with CABAC parsing.

Test bitstream #AVCCASL-2

Specification:  All slices are coded as I, P or B slices. Each picture contains more than one slice. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. Temporal direct prediction is used for direct prediction. direct_8x8_inference_flag is equal to 0. Slices with different slice types are included in a picture. Stored B slices are included in the stream. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of different slice types in a picture with CABAC parsing.

Purpose:  Check that decoder can properly decode different slice types in a picture with CABAC parsing.

### Test Bitstreams – CABAC: I_PCM

Test bitstream #AVCCAPCM-1

Specification:  All slices are coded as I slices. Each picture contains only one slice. disable_deblocking_filter_idc is equal to 1, specifying disabling of the deblocking filter process. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. mb_type is equal to I_PCM at some Macroblocks. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of Macroblock with mb_type equal to I_PCM.

Purpose:  Check that decoder can properly decode Macroblock with mb_type equal to I_PCM.

Test bitstream #AVCCAPCM-2

Specification:  All slices are coded as I slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. mb_type is equal to I_PCM at some Macroblocks. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of Macroblock with mb_type equal to I_PCM.

Purpose:  Check that decoder can properly decode Macroblock with mb_type equal to I_PCM.

Test bitstream #AVCCAPCM-3

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. Temporal direct prediction is used for direct prediction. direct_8x8_inference_flag is equal to 1. mb_type is equal to I_PCM at some Macroblocks. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of Macroblock with mb_type equal to I_PCM.

Purpose:  Check that decoder can properly decode macroblocks with mb_type equal to I_PCM.

### Test Bitstreams – CABAC: Weighted sample prediction process

Test bitstream #AVCCAWP-1

Specification:  All slices are coded as I or P slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 2. weighted_pred_flag is equal to 1. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Weighted sample prediction process for P slice

Purpose:  Check that decode handles weighted sample prediction for P slice.

Test bitstream #AVCCAWP-2

Specification:  All slices are coded as I or P slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 2. weighted_pred_flag is equal to 1. Plural reference indices are assigned to each reference picture. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Weighted sample prediction process for P slices with plural reference indices.

Purpose:  Check that decoder handles weighted sample prediction for P slices with plural reference indexes.

### Test Bitstreams – CABAC: Field coding

Test bitstream #AVCCAFI-1

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. Temporal direct prediction is used for direct prediction. direct_8x8_inference_flag is equal to 1. Each slice is a coded field. Stored B slices are included in the stream. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of coded fields.

Purpose:  Check that decoder can properly decode slice of coded field including stored B slice.

### Test Bitstreams – CABAC: Frame/field decoding

Test bitstream #AVCCAPA-1

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. disable_deblocking_filter_idc is equal to 1, specifying disabling of the deblocking filter process. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 1. Spatial direct prediction is used for direct prediction. direct_8x8_inference_flag is equal to 1. Each slice is either a coded frame or a coded field. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Picture adaptive frame/field decoding.

Purpose:  Check that decoder can properly decode slices of coded frames and fields with direct_8x8_inference_flag=1.

Test bitstream #AVCCAPA-2

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. Temporal direct prediction is used for direct prediction. direct_8x8_inference_flag is equal to 1. Each slice is either a coded frame or a coded field. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Picture adaptive frame/field decoding.

Purpose:  Check that decoder can properly decode slices of coded frames and fields with direct_8x8_inference_flag=1.

### Test bitstream - Macroblock adaptive frame/field decoding

Test bitstream #AVCCAMA-1

Specification:  All slices are coded as I or P slices. Each picture contains only one slice. disable_deblocking_filter_idc is equal to 1, specifying disabling of the deblocking filter process. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. mb_adaptive_frame_field_coding is equal to 1. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Macroblock adaptive frame field decoding.

Purpose:  Check that decoder can properly decode slices with mb_adaptive_frame_field_flag=1.

Test bitstream #AVCCAMA-2

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. disable_deblocking_filter_idc is equal to 1, specifying disabling of the deblocking filter process. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. Spatial direct prediction is used for direct prediction. direct_8x8_inference_flag is equal to 1. mb_adaptive_frame_field_coding is equal to 1. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Macroblock adaptive frame field decoding.

Purpose:  Check that decoder can properly decode slices with mb_adaptive_frame_field_flag=1.

Test bitstream #AVCCAMA-3

Specification:  All slices are coded as I or P slices. Each picture contains only one slice. disable_deblocking_filter_idc is equal to 1, specifying disabling of the deblocking filter process. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. num_ref_frames is equal to 1. mb_adaptive_frame_field_coding is equal to 1. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Macroblock adaptive frame field decoding.

Purpose:  Check that decoder can properly decode slices with mb_adaptive_frame_field_flag=1.

Test bitstream #AVCCAMA-4

Specification:  All slices are coded as I or P slices. Each picture contains only one slice. disable_deblocking_filter_idc is equal to 1, specifying disabling of the deblocking filter process. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. mb_adaptive_frame_field_coding is equal to 1. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Macroblock adaptive frame field decoding.

Purpose:  Check that decoder can properly decode slices with mb_adaptive_frame_field_flag=1.

Test bitstream #AVCCAMA-5

Specification:  All slices are coded as I slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. mb_adaptive_frame_field_coding is equal to 1. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Macroblock adaptive frame field decoding.

Purpose:  Check that decoder can properly decode slices with mb_adaptive_frame_field_flag=1.

Test bitstream #AVCCAMA-6

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. Spatial direct prediction is used for direct prediction. direct_8x8_inference_flag is equal to 1. mb_adaptive_frame_field_coding is equal to 1. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Macroblock adaptive frame field decoding.

Purpose:  Check that decoder can properly decode slices with mb_adaptive_frame_field_flag=1.

Test bitstream #AVCCAMA-7

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. disable_deblocking_filter_idc is equal to 1, specifying disabling of the deblocking filter process. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. Temporal direct prediction is used for direct prediction. direct_8x8_inference_flag is equal to 1. mb_adaptive_frame_field_coding is equal to 1. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Macroblock adaptive frame field decoding.

Purpose:  Check that decoder can properly decode slices with mb_adaptive_frame_field_flag=1.

Test bitstream #AVCCAMA-8

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. Temporal direct prediction is used for direct prediction. direct_8x8_inference_flag is equal to 1. mb_adaptive_frame_field_coding is equal to 1. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Macroblock adaptive frame field decoding.

Purpose:  Check that decoder can properly decode slices with mb_adaptive_frame_field_flag=1.

Test bitstream #AVCCAPAMA-1

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. Both coded frames and coded fields are included in the stream. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. Temporal direct prediction is used for direct prediction. direct_8x8_inference_flag is equal to 1. mb_adaptive_frame_field_coding is equal to 1. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Macroblock adaptive frame field decoding and slices of a coded field.

Purpose:  Check that decoder can properly decode both slice of a coded frame with mb_adaptive_frame_field_flag=1 and slices of a coded field.

Test bitstream #AVCCAPAMA-2

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. Temporal direct prediction is used for direct prediction. direct_8x8_inference_flag is equal to 1. The first field of the first frame only contains I slice and the second field only contains P slice. mb_adaptive_frame_field_coding is equal to 1 in the rest of the frames. The indicated display of this stream is bottom field first. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Macroblock adaptive frame field decoding and slices of a coded field.

Purpose:  Check that decoder can properly decode both slice of a coded frame with mb_adaptive_frame_field_flag=1 and slices of a coded field.

Test bitstream #AVCCAPAMA-3

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. disable_deblocking_filter_idc is equal to 1, specifying disabling of the deblocking filter process. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. Temporal direct prediction is used for direct prediction. direct_8x8_inference_flag is equal to 1. The first field of the first frame only contains I slice and the second field only contains P slice. mb_adaptive_frame_field_coding is equal to 1 in the rest of the frames. The indicated display of this stream is top field first. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Macroblock adaptive frame field decoding and slices of a coded field.

Purpose:  Check that decoder can properly decode both slices of a coded frame with mb_adaptive_frame_field_flag=1 and slices of a coded field.

Test bitstream #AVCCAPAMA-4

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. Temporal direct prediction is used for direct prediction. direct_8x8_inference_flag is equal to 1. The first field of the first frame only contains I slice and the second field only contains P slice. mb_adaptive_frame_field_coding is equal to 1 in the rest of the frames. The indicated display of this stream is top field first. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Macroblock adaptive frame field decoding and slices of a coded field.

Purpose:  Check that decoder can properly decode both slices of a coded frame with mb_adaptive_frame_field_flag=1 and slices of a coded field.

Test bitstream #AVCCAMV-1

Specification:  The stream conforms to MP@L3.0. Frame size is 720x480. All slices are coded as I, P or B slices. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. Temporal direct prediction is used for direct prediction. direct_8x8_inference_flag is equal to 1. In P slices, each macroblock is coded as sixteen 4x4 blocks. Each block has one motion vector in 1/4 sample position. In B slices, each macroblock is coded as eight 8x4 blocks. Each block has two motion vectors, one for list0 the other for list1. Both vectors are in 1/4 sample position. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Prediction bandwidth.

Purpose: Check that the decoder handles the worse case of prediction bandwidth. Prediction bandwidth is at maximum due to largest number of motion vectors (in 1/4 sample position) per macroblock pair (32 as defined in standard). Non-integer position motion vectors require using 6-tap filter always.

Test bitstream #AVCCVCANLMA-1

Specification:  All slices are coded as I or P slices. Each picture contains only one slice. disable_deblocking_filter_idc is equal to 1, specifying disabling of the deblocking filter process. Both entropy_coding_mode_flag equal to 0, specifying the CAVLC parsing proces, and entropy_coding_mode_flag equal to 1, specifying the CABAC parsing process are present within the bitstream. pic_order_cnt_type is equal to 0. mb_adaptive_frame_field_coding is equal to 1. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Macroblock adaptive frame field decoding using both CAVLC and CABAC.

Purpose:  Check that decoder can properly decode slices with mb_adaptive_frame_field_flag=1 Check that the decoder can properly decode both CABAC and CAVLC.

### Normative Test Suites for 14496-10

Legend:

X – Bitstream is for static and dynamic test

Table 1

| Categories | Bitstream | Donated by | Bitstreams Name | Baseline |  |  |  |  | Extended |  |  |  |  |  | Main |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | L1 | L2 | L3 | L4 | L5 |  | L1 | L2 | L3 | L4 | L5 |  | L1 | L2 | L3 | L4 | L5 |
| General | AVCNL-1 | Sony | NL1_Sony_C | X | X | X | X | X |  | X | X | X | X | X |  | X | X | X | X | X |
|  | AVCNL-2 | SVA | SVA_NL1_A |  | X | X | X | X |  |  | X | X | X | X |  |  | X | X | X | X |
|  | AVCNL-3 | Sony | NL2_Sony_E | X | X | X | X | X |  | X | X | X | X | X |  | X | X | X | X | X |
|  | AVCNL-4 | SVA | SVA_NL2_A |  | X | X | X | X |  |  | X | X | X | X |  |  | X | X | X | X |
|  | AVCBA-1 | Sony | BA1_Sony_C | X | X | X | X | X |  | X | X | X | X | X |  | X | X | X | X | X |
|  | AVCBA-2 | SVA | SVA_BA1_A |  | X | X | X | X |  |  | X | X | X | X |  |  | X | X | X | X |
|  | AVCBA-3 | Sony | BA2_Sony_C | X | X | X | X | X |  | X | X | X | X | X |  | X | X | X | X | X |
|  | AVCBA-4 | SVA | SVA_BA2_A |  | X | X | X | X |  | X | X | X | X | X |  |  | X | X | X | X |
|  | AVCBA-5 | MCubeworks | BA_MW_B | X | X | X | X | X |  | X | X | X | X | X |  | X | X | X | X | X |
|  | AVCBA-6 | MCubeworks | BANM_MW_B | X | X | X | X | X |  | X | X | X | X | X |  | X | X | X | X | X |
|  | AVCBA-7 | SVA | BA3_SVA_B |  |  |  |  |  |  |  | X | X | X | X |  |  | X | X | X | X |
|  | AVCMQ-1 | JVC | NLMQ1_JVC_B |  | X | X | X | X |  |  |  |  |  |  |  |  | X | X | X | X |
|  | AVCMQ-2 | JVC | NLMQ2_JVC_B |  | X | X | X | X |  |  |  |  |  |  |  |  | X | X | X | X |
|  | AVCMQ-3 | JVC | BAMQ1_JVC_A |  | X | X | X | X |  |  |  |  |  |  |  |  | X | X | X | X |
|  | AVCMQ-4 | JVC | BAMQ2_JVC_B |  | X | X | X | X |  |  |  |  |  |  |  |  | X | X | X | X |
|  | AVCSL-1 | SVA | SVA_Base_A |  | X | X | X | X |  |  |  |  |  |  |  |  |  |  |  |  |
|  | AVCSL-2 | SVA | SVA_FM1_A |  | X | X | X | X |  |  |  |  |  |  |  |  |  |  |  |  |
|  | AVCSQ-1 | Sony | BASQP1_Sony_B |  | X | X | X | X |  |  | X | X | X | X |  |  | X | X | X | X |
|  | AVCFM-1 | BT | FM1_BT_B | X | X | X | X | X |  | X | X | X | X | X |  |  |  |  |  |  |
|  | AVCCI-1 | MCubeworks | CI_MW_B | X | X | X | X | X |  | X | X | X | X | X |  | X | X | X | X | X |
|  | AVCCI-2 | SVA | SVA_CL1_A |  | X | X | X | X |  |  |  |  |  |  |  |  |  |  |  |  |
|  | AVCFC-1 | Sony | CVFC1_Sony_A | X | X | X | X | X |  | X | X | X | X | X |  | X | X | X | X | X |
|  | AVCAUD-1 | Mcubeworks | AUD_MW_B | X | X | X | X | X |  | X | X | X | X | X |  | X | X | X | X | X |
|  | AVCMIDR-1 | Mcubeworks | MIDR_MW_B | X | X | X | X | X |  | X | X | X | X | X |  | X | X | X | X | X |
|  | AVCNRF-1 | Mcubeworks | NRF_MW_C | X | X | X | X | X |  | X | X | X | X | X |  | X | X | X | X | X |
|  | AVCCQP-1 | UBVideo | CQP1_UBVideo_A | X | X | X | X | X |  | X | X | X | X | X |  | X | X | X | X | X |
|  | AVCBS-1 | Sony | CVBS3_Sony_B |  |  |  |  |  |  | X | X | X | X | X |  | X | X | X | X | X |
|  | AVCBS-2 | SVA | BA3_SVA_B |  |  |  |  |  |  |  | X | X | X | X |  |  | X | X | X | X |
| I_PCM | AVCPCM-1 | SVA | CVPCMNL1_SVA_B |  |  |  | X | X |  |  |  |  | X | X |  |  |  |  | X | X |
|  | AVCPCM-2 | SVA | CVPCMNL2_SVA_B |  |  |  | X | X |  |  |  |  | X | X |  |  |  |  | X | X |
| MMCO | AVCMR-1 | BT | MR1_BT_A | X | X | X | X | X |  | X | X | X | X | X |  | X | X | X | X | X |
|  | AVCMR-2 | Tandberg | MR2_Tandberg_D |  |  | X | X | X |  |  |  | X | X | X |  |  |  |  |  |  |
|  | AVCMR-3 | Tandberg | MR3_Tandberg_A |  |  | X | X | X |  |  |  | X | X | X |  |  |  |  |  |  |
|  | AVCMR-4 | Tandberg | MR4_Tandberg_B |  |  | X | X | X |  |  |  | X | X | X |  |  |  |  |  |  |
|  | AVCMR-5 | Tandberg | MR5_Tandberg_B |  |  | X | X | X |  |  |  | X | X | X |  |  |  |  |  |  |
| WP | AVCWP-1 | Toshiba | CVWP5_TOSHIBA_A |  |  |  |  |  |  | X | X | X | X | X |  | X | X | X | X | X |
|  | AVCWP-2 | Toshiba | CVWP1_TOSHIBA_D |  |  |  |  |  |  | X | X | X | X | X |  | X | X | X | X | X |
|  | AVCWP-3 | Toshiba | CVWP2_TOSHIBA_D |  |  |  |  |  |  | X | X | X | X | X |  | X | X | X | X | X |
| Field coding | AVCFI-1 | Sony | CVNLFI1_Sony_A |  |  |  |  |  |  |  |  | X | X | X |  |  |  | X | X | X |
|  | AVCFI-2 | Sony | CVNLFI2_Sony_E |  |  |  |  |  |  |  |  | X | X | X |  |  |  | X | X | X |
|  | AVCFI-3 | Sharp | Sharp_MP_Field1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X | X | X |
|  | AVCFI-4 | Sharp | Sharp_MP_Field2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X | X | X |
|  | AVCFI-5 | Sharp | Sharp_MP_Field3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X | X | X |
|  | AVCFI-6 | Sony | CVFI1_Sony_B |  |  |  |  |  |  |  |  | X | X | X |  |  |  | X | X | X |
|  | AVCFI-7 | Sony | CVFI2_Sony_E |  |  |  |  |  |  |  |  | X | X | X |  |  |  | X | X | X |
|  | AVCFI-8 | Sony | FI1_Sony_C |  |  |  |  |  |  |  |  | X | X | X |  |  |  | X | X | X |
| Frame/field coding | AVCPA-1 | Sharp | Sharp_MP_PAFF_1r1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X | X | X |
|  | AVCPA-1 | Toshiba | CVPA1_TOSHIBA_A |  |  |  |  |  |  |  | X | X | X | X |  |  | X | X | X | X |
| MBAFF | AVCMA-1 | Toshiba | CVMANL1_TOSHIBA_A |  |  |  |  |  |  |  |  |  |  |  |  |  | X | X | X | X |
|  | AVCMA-2 | Toshiba | CVMANL2_TOSHIBA_A |  |  |  |  |  |  |  |  |  |  |  |  |  | X | X | X | X |
|  | AVCMA-3 | Sony | CVMA1_Sony_C |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X | X | X |
|  | AVCMA-4 | Toshiba | CVMA1_TOSHIBA_A |  |  |  |  |  |  |  |  |  |  |  |  |  | X | X | X | X |
| S picture | AVCSP-1 | BT | SP1_BT_A |  |  |  |  |  |  | X | X | X | X | X |  |  |  |  |  |  |
| CABAC | AVCCANL-1 | Toshiba | CANL1_TOSHIBA_F |  |  |  |  |  |  |  |  |  |  |  |  | X | X | X | X | X |
|  | AVCCANL-2 | Sony | CANL1_Sony_D |  |  |  |  |  |  |  |  |  |  |  |  |  | X | X | X | X |
|  | AVCCANL-3 | Sony | CANL2_Sony_D |  |  |  |  |  |  |  |  |  |  |  |  |  | X | X | X | X |
|  | AVCCANL-4 | Sony | CANL3_Sony_B |  |  |  |  |  |  |  |  |  |  |  |  | X | X | X | X | X |
|  | AVCCABA-1 | Sony | CABA1_Sony_C |  |  |  |  |  |  |  |  |  |  |  |  |  | X | X | X | X |
|  | AVCCABA-2 | Sony | CABA2_Sony_D |  |  |  |  |  |  |  |  |  |  |  |  |  | X | X | X | X |
|  | AVCCABA-3 | Sony | CABA3_Sony_B |  |  |  |  |  |  |  |  |  |  |  |  | X | X | X | X | X |
|  | AVCCABA-4 | Toshiba | CABA3_TOSHIBA_D |  |  |  |  |  |  |  |  |  |  |  |  | X | X | X | X | X |
| CABAC: Initialization | AVCCAIN-1 | Sony | CABACI3_Sony_A |  |  |  |  |  |  |  |  |  |  |  |  |  | X | X | X | X |
| CABAC: MB QP Delta | AVCCAQP-1 | Sony | CAQP1_Sony_A |  |  |  |  |  |  |  |  |  |  |  |  | X | X | X | X | X |
|  | AVCCAQP-2 | Sony | CACQP3_Sony_C |  |  |  |  |  |  |  |  |  |  |  |  |  | X | X | X | X |
| CABAC: Slice | AVCCASL-1 | Sony | CABAST3_Sony_D |  |  |  |  |  |  |  |  |  |  |  |  |  | X | X | X | X |
|  | AVCCASL-2 | Sony | CABASTBR3_Sony_A |  |  |  |  |  |  |  |  |  |  |  |  |  | X | X | X | X |
| CABAC: I_PCM | AVCCAPCM-1 | Sand Video | CAPCMNL1_Sand_D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X | X |
|  | AVCCAPCM-2 | Sand Video | CAPCM1_Sand_D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X | X |
|  | AVCCAPCM-3 | Sony | CAPM3_Sony_C |  |  |  |  |  |  |  |  |  |  |  |  |  | X | X | X | X |
| CABAC: WP | AVCCAWP-1 | Toshiba | CAWP1_TOSHIBA_D |  |  |  |  |  |  |  |  |  |  |  |  | X | X | X | X | X |
|  | AVCCAWP-2 | Toshiba | CAWP5_TOSHIBA_A |  |  |  |  |  |  |  |  |  |  |  |  | X | X | X | X | X |
| CABAC: Field coding | AVCCAFI-1 | Sand Video | CABREF3_Sand_B |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X | X |
| CABAC: Frame/Field Coding | AVCCAPA-1 | Sharp | Sharp_MP_PAFF_2_r1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X | X | X |
|  | AVCCAPA-2 | Toshiba | CAPA1_TOSHIBA_A |  |  |  |  |  |  |  |  |  |  |  |  |  | X | X | X | X |
| CABAC: MBAFF | AVCCAMA-1 | Toshiba | CAMANL1_TOSHIBA_A |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X | X | X |
|  | AVCCAMA-2 | Toshiba | CAMANL2_TOSHIBA_A |  |  |  |  |  |  |  |  |  |  |  |  |  | X | X | X | X |
|  | AVCCAMA-3 | Sony | CANLMA2_Sony_A |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X | X | X |
|  | AVCCAMA-4 | Sony | CANLMA3_Sony_A |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X | X | X |
|  | AVCCAMA-5 | Sony | CAMA1_Sony_B |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X | X | X |
|  | AVCCAMA-6 | Toshiba | CAMA1_TOSHIBA_A |  |  |  |  |  |  |  |  |  |  |  |  |  | X | X | X | X |
|  | AVCCAMA-7 | Sand Video | CAMANL3_Sand_C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X | X |
|  | AVCCAMA-8 | Sand Video | CAMA3_Sand_C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X | X |
|  | AVCCAPAMA-1 | Sand Video | CAPAMA3_Sand_D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X | X |
|  | AVCCAPAMA-2 | VideoTele.com | CAMA1_VTC_C |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X | X | X |
|  | AVCCAPAMA-3 | VideoTele.com | CAMA2_VTC_B |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X | X | X |
|  | AVCCAPAMA-4 | VideoTele.com | CAMA3_VTC_B |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X | X | X |
| CABAC: Prediction Bandwidth | AVCCAMV-1 | Broadcom | MV1_BRCM_B |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X | X | X |
| CABAC/CAVLC | AVCCVCANLMA-1 | A | CVCANLMA2_Sony_A |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X | X | X |