| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 11th Meeting: Munich, Germany, 15-19 March, 2004 | Document: JVT-K018 Filename: JVT-K018.doc |
| --- | --- |

| Title: | Adaptive Residue Transform and Sampling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Woo-Shik Kim, Dmitry Birinov, Hyun Mun Kim Multimedia Lab. Samsung AIT P.O.Box 111, Suwon 440-600 Korea | Tel: Email: | 82-31-280-9204 hyunmun27.kim@samsung.com |
| Source: | Samsung AIT |  |  |

_____________________________

- Introduction

The JVT Professional Extension work has been developed for the high quality video coding applications such as professional production, post production, HD/DVDs, surveillance, broadcasting distribution, contribution, digital cinema and earth resources. To cope with this purpose, the call for proposal was issued to cover the increased sample bit-depth, YUV 4:2:2 chroma format, and 4:4:4 format for YUV, RGB and XYZ data [1].

During the course of MPEG-4 AVC/H.264 Professional Extension standardization, we showed the effectiveness of the INTER-PLANE PREDICTION (IPP) coding technology [2-4] for 4:4:4 RGB coding. It showed the better coding efficiency than that of coding in other color spaces at relatively high bit rates. It also showed that the high color fidelity is maintained, which would be very important in the professional application area.

In this proposal, we propose a new coding method called “Adaptive Residue Transform and Sampling”. It exploits the redundancy among the residue data of each RGB component after intra/inter prediction. This method was motivated by the observation that there still exists redundancy among the residue data of each color component even after intra/inter prediction. As addressed in JVT-J017, for some images, the statistical properties of the color space converted data do not conform to the assumed intra/inter redundancy of current JVT specification [5]. In that case it is more efficient to apply intra/inter prediction directly to RGB data first. Then we can find suitable transform that matches well to the statistical properties of the residue. We can further compress the data by applying sub-sampling to the transformed residue data.

Section 2 briefly describes the proposed algorithm. The test results are summarized in Section 3. We conclude in Section 4.

- Residue Transform Coding

In the conventional coding methods, RGB data is converted into the YCbCr space. During the color transform, the correlation among color components is removed, which results in high coding efficiency. Recently, there has been researches to further decorrelate the redundancy among color components using Karhunen-Loève transform (KLT) whose basis vectors are the eigenvectors of the autocorrelation matrix of the RGB components [6-8]. These new transforms not only show the good coding efficiency, but also result in a complexity reduction using integer transform derived from the lifting scheme.

Though these methods remove the redundancy among color components effectively, the color distortion cannot be avoided, since the chrominance components are distorted more than the luminance component. The color fidelity can be achieved when all components are equally well processed.

But, if the RGB data is coded directly to keep the color fidelity, the coding efficiency is degraded much. To solve this problem, we propose a residue transform coding. It removes the remaining redundancy among color components after intra/inter prediction.

The correlation among color components still exists even after the intra/inter prediction. We can achieve high coding efficiency by decorrelating the residue data of each color component after intra/inter prediction. Because most information is included in the predictor and less information is remained in the residue data, the color fidelity can be kept by decorrelating the residue data using the residue transform.

To decorrelate the residue data, the YCoCg transform [7] has been selected after extensive simulations using several color transforms recently developed. The transform is presented below where R, G, and B are the residue data and Y, Co, and Cg are the residue transformed data, respectively.

Forward YCoCg color space transform:

ΔYΔCoΔCg=1/4121/41/20−1/2−1/41/2−1/4ΔRΔGΔB

Inverse YCoCg color space transform:

ΔRΔGΔB=11−11011−1−1ΔYΔCoΔCg.

Figure 1: Block diagram of the proposed residue transform

Figure 1 shows the block diagram of the proposed algorithm. This scheme can be easily incorporated into the existing JVT specification [5], since only one additional block has been added.

To elevate the coding efficiency, we introduced the chroma intra 4x4 mode in case of luma intra 4x4 mode selection. The mode and the prediction method shall be the same between the luma and the chroma components, so that the co-located RGB residue would have more correlation. In this case, the chroma DC transform shall be skipped, and the DC value shall be coded with the AC values in the same 4x4 block.

The residue transform can be performed adaptively according to the various characteristics of the input data and the requirement of the application. In very high bit rate for near-lossless and lossless coding, it is more appropriate to code the RGB data directly without residue transform. We propose to add a flag, “residue_transform_flag” to the sequence parameter set to signal the residue transform feature. If residue_transform_flag equals to ‘1’, the residue transform shall be performed, otherwise, the input data shall be coded without residue transform.

Since the proposed method is robust to the color distortion, it can be applied effectively to other input data such as XYZ.

- Residue Sampling

The 4:2:0 and 4:2:2 formats are the mostly well-known sample formats. In these formats, only chrominance components are sub-sampled in the ratio of 1/4 and 1/2, respectively, compared to the luminance sampling rate, because the luminance components has more information than the chrominance components.

In the professional applications, it would be better not to use the chrominance sub-sampling, which would results in the color distortion. However, the chrominance sampling would be required for the low bit rate applications.

To adapt to the variety of applications, we propose to use the adaptive sampling method. If the RGB data is sub-sampled directly, the distortion would be very large. We propose to apply the sub-sampling to the residue data. Once the residue transform is performed, there is luminance-like component, Y, which have more information than other components. As same as the 4:2:0 format, Co and Cg components are sub-sampled vertically and horizontally. Figure 2 show the block diagram of the proposed residue transform and sampling procedure.

Figure 2: Block diagram of the proposed residue sampling

Figure 3: Up-sampling procedure

In case of intra 4x4 mode, the residue sampling is not performed.

On the decoder side, the reconstructed data from the inverse integer transform is up-sampled to the residue transformed data. The bi-linear interpolation is used for the up-sampling, except for the samples at the left and the upper boundary position of the macroblock. These boundary pixels are up-sampled by copying the nearest neighbor pixel in the current macroblock. Figure 3 shows this process.

We propose to apply this residue sampling adaptively in macroblock basis. We propose to add two flags, “residue_sampling_flag” to the sequence parameter set, and “mb_residue_sampling_flag” to the macroblock layer after macroblock type. If residue_sampling_flag equals to ‘1’ and macroblock type is not intra 4x4, mb_residue_sampling_flag shall be coded and decoded. Otherwise, this flag shall not be coded and decoded. The sampling is performed depending on encoder’s discretion by setting these flags. When decoding, if mb_residue_sampling_flag is present and equals to ‘1’, the reconstructed chrominance data from the inverse integer transform shall be up-sampled as shown in Figure 3.

- Experimental Results

We compared our residue transform method using the above options with YCoCg-R proposed in [7]. And the coding efficiency of the residue sampling method is compared with that of YCbCr 4:2:0 coding. The test conditions are summarized as follows:

- Search range: 32

- Number of reference frame: 3

- Entropy coding: CABAC

- RD-optimized mode selection: On

- GOP structure: IBBP…

- Number of slice groups: 1

We tested this residue transform and sampling options over various image set. The complete test results are available in the accompanying excel file (JVT-K018.xls). The simulation results manifest that it is desirable to apply inter/intra prediction followed by color transform. If we apply color transform first before coding, the coding efficiency deteriorates. It becomes severe especially on film material or unprocessed images. Due to the film grain or shot noise, the color space converted data are bad fit for the JVT inter/intra prediction. It tells that the coding gain of using intra/inter redundancy is bigger than that of using inter-color redundancy.

Based on this observation and experimental results, the best method to code RGB material is to code the given image in the given space. Especially for high bit rate it is the best way to code RGB material without color space conversion. By properly using the residue transform and sampling options, we can always achieve good coding efficiency over wide range of bit rate.

- Conclusions

The use of residue transform and sampling in RGB space achieves great coding efficiency for various kinds of RGB data including film material and professional filmstream material. The advantage is clearly shown in the simulation results. It tells that it is desirable to apply inter/intra prediction followed by color transform. We can select residue transform and sampling options depending on image characteristics and requirements of the given application at encoder’s discretion. It helps achieve the best coding efficiency for any type of input data.

References

[1] G. Sullivan, A. Luthra, and T. Wiegand, “Call for proposals for extended sample bit depth and chroma format support in the advanced video coding standard (ITU-T H.264 & ISO/IEC 14496-10),” ISO/IEC JTC1/SC29/WG11 and ITU-T Q6/SG16, Document JVT-G048, Mar. 2003.

[2] W.-S. Kim, D. Cho and H. M. Kim, “Inter-plane prediction for RGB coding II,” ISO/IEC JTC1/SC29/WG11 and ITU-T Q6/SG16, Document JVT-J017, Dec. 2003.

- [3] W.-S. Kim, D. Cho and H. M. Kim, “Inter-plane prediction for RGB coding,” ISO/IEC JTC1/SC29/WG11 and ITU-T Q6/SG16, Document JVT-I023, Sep. 2003.

- [4] W.-S. Kim, D. Cho and H. M. Kim, “Color format extension,” ISO/IEC JTC1/SC29/WG11 and ITU-T Q6/SG16, Document JVT-H018, May 2003.

[5] “Final Draft International Standard of Joint Video Specification (ITU-T Rec. H.264 | ISO/IEC 14496-10 AVC),” ISO/IEC JTC1/SC29/WG11 and ITU-T Q6/SG16, Document JVT-G050, Mar. 2003.

- [6] H. Malvar and G. Sullivan, “YCoCg-R: a color space with RGB reversibility and low dynamic range,” ISO/IEC JTC1/SC29/WG11 and ITU-T Q6/SG16, Document JVT-I014, Sep. 2003.

- [7] H. Malvar and G. Sullivan, “Transform, scaling & color space impact of professional extensions,” ISO/IEC JTC1/SC29/WG11 and ITU-T Q6/SG16, Document JVT-H031, May 2003.

- [8] P. Topiwala and C. Tu, “New invertible integer color transforms based on lifting steps and coding of 4:4:4 video,” ISO/IEC JTC1/SC29/WG11 and ITU-T Q6/SG16, Document JVT-I015, Sep. 2003.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Samsung Elctronics. Co. Ltd. |  |  |
| Mailing address | POBox 111 Suwon Samsung AIT Multimedia |  |  |
| Country | Korea |  |  |
| Contact person | Hyun Mun Kim |  |  |
| Telephone | 82-31-280-9204 |  |  |
| Fax | 82-31-280-9207 |  |  |
| Email | hyunmun27.kim@samsung.com |  |  |
| Place and date of submission | Munich, Germany, 15-19 March, 2004 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Adaptive Residue Transform and sampling |  |
| Contribution number |  | JVT-K018 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
| X | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| # X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: