| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 11th Meeting: Munich, Germany, 15-19 March, 2004 | Document: JVT-K019 Filename: JVT-K019.doc |
| --- | --- |

| Title: | Alpha channel coding using the JVT coding tools |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Woo-Shik Kim, Jin-Hyung Park, Dae-Sung Cho, Hyun Mun Kim Multimedia Lab. Samsung AIT P.O.Box 111, Suwon 440-600 Korea | Tel: Email: | 82-31-280-9204 hyunmun27.kim@samsung.com |
| Source: | Samsung AIT |  |  |

- Introduction

In the previous meeting in Hawaii, there was an SMPTE liaison document [1], which described the alpha channel applications. And in that document, SMPTE expressed that it supports the inclusion of alpha channel coding in the JVT tool kit.

It would be very nice to have the alpha channel coding feature in the JVT if it can be achieved using the current JVT coding tools [2].

In this document, we investigated how to support the alpha channel coding in the JVT coding structure, and showed the experimental results in comparison with the results of MPEG-4 part 2 shape coding tool.

- Proposed Methods

  - Coding structure

The alpha channel data can be combined with any 4:4:4/4:2:2/4:2:0 luma and chroma data, which form a 4:4:4:4/4:2:2:4 macroblock (MB). This format information can be indicated in the sequence parameter set as chroma_format_idc as discussed in the previous meetings [3].

If chroma_format_idc indicates that the bitstream contains the alpha channel data, the alpha channel data is to be decoded in MB unit along with the luma and chroma data.

The mb_skip_flag shall be set considering the luma and chroma and the alpha channel data.

This means that, if the current MB is skipped MB, not only the luma and chroma data but also the alpha channel data shall be skipped.

We propose to add a transp_not_code_flag in the slice header. When this flag is set to ‘1’, if the current MB has only transparent alpha data, the luma and chroma data is not coded in the bitstream. For this purpose, the decoding order of the alpha channel data shall precede that of the luma and chroma data.

If the MB is skipped MB, and transp_not_code_flag is set to ‘1’, and the current MB has only transparent alpha data, there are no luma and chroma data and they shall not be copied from the previous decoded frames.

In the MB layer, the alpha channel data shall have independent MB type, intra prediction mode, and motion vectors from those of the luma and chroma.

  - Grey scale alpha channel coding

If chroma_format_idc indicates that the bitstream contains the alpha channel data, the binary_alpha_flag shall be followed in the sequence parameter set.

If the binary_alpha_flag is ‘1’, the alpha channel data consists of only two level data, transparent and opaque. The coding scheme for the binary alpha data is described in the next section. Otherwise, the alpha channel data is the grey alpha, and shall be coded in the same way of the monochrome data coding.

  - Binary alpha channel coding

A binary alpha channel image consists of two values – transparent and opaque. The values for the transparent and the opaque are coded in the sequence parameter set and they are coded as ‘0’ and ‘1’ only.

The current JVT coding structure consists of the intra/inter prediction, the transform and quantization, the entropy coding, and the loop-filter. To support alpha channel coding in the current JVT coding structure, we used the same intra/inter prediction except that the subtraction operation is replaced with the XOR operation to make the residue binary.

The intra prediction modes and the motion vectors are coded independently from the luma and chroma ones. The half-pel and quarter-pel motion interpolation is not applied considering the binary image characteristics.

The transform and quantization module is skipped to code the alpha channel in high accuracy. The loop-filter is not applied, either.

After intra/inter prediction, the CBP values are formed. If whole 8x8 block is transparent, this block is marked as ‘0’, otherwise marked as ‘1’. The CBP is formed by combining these four numbers. The range of CBP is from 0 to 15.

Each 4x4 blocks in the 8x8 blocks of CBP value ‘1’ are zigzag scanned and entropy coded in the same way of transform coefficient coding method in JVT [2]. Since the residue is binary, the sign value shall not be coded.

Figure 1: Proposed block diagram of binary alpha macroblock coding

- Experimental Results

We compared the proposed binary alpha channel coding method with MPEG-4 part 2 shape coding tool. We up-sampled the CIF test images to 4CIF, since the professional application would require higher image resolution. The test conditions are summarized as follows:

- Search range: 16

- Number of reference frame: 3

- Entropy coding: CABAC

- RD-optimized mode selection: On

- GOP structure: IPPP…

- Number of slice groups: 1

- Conclusions

We proposed an alpha channel coding method using the current JVT coding structure. We introduced a method to utilize the correlation between the alpha channel data and the video data. The experimental results of the binary alpha coding show a comparable coding efficiency to that of the MPEG-4 part 2 shape coding tool.

References

[1] SMPTE, “Incoming LS: Alpha channel and ProfExt,” ISO/IEC JTC1/SC29/WG11 and ITU-T Q6/SG16, Document JVT-J008, Dec. 2003.

[2] “Final Draft International Standard Joint Video Specification (ITU-T Rec. H.264 | ISO/IEC 14496-10 AVC),” ISO/IEC JTC1/SC29/WG11 and ITU-T Q6/SG16, Document JVT-G050, May 2003.

[3] T. McMahon, G. Sullivan, and T. Wiegand, “Draft Prof. Ext Amendment,” ISO/IEC JTC1/SC29/WG11 and ITU-T Q6/SG16, Document JVT-H037, May 2003.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Samsung Elctronics. Co. Ltd. |  |  |
| Mailing address | POBox 111 Suwon Samsung AIT Multimedia |  |  |
| Country | Korea |  |  |
| Contact person | Hyun Mun Kim |  |  |
| Telephone | 82-31-280-9204 |  |  |
| Fax | 82-31-280-9207 |  |  |
| Email | hyunmun27.kim@samsung.com |  |  |
| Place and date of submission |  |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  |  |  |
| Contribution number |  | JVT-K0xx |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
| X | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| # X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: