| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 11th Meeting: Munich, DE, 15-19 March, 2004 | Document: JVT-K021 Filename: input.doc |
| --- | --- |

| Title: | Fast Mode Decision for B slice |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jeyun Lee School of ECE, SungKyunKwan Univ. 300 Chunchun-dong, Suwon, Korea Inchoon Choi School of ECE, SungKyunKwan Univ. 300 Chunchun-dong, Suwon, Korea Woongil Choi School of ECE, SungKyunKwan Univ. 300 Chunchun-dong, Suwon, Korea Byeungwoo Jeon School of ECE, SungKyunKwan Univ. 300 Chunchun-dong, Suwon, Korea | Tel: Email: | +82-31-290-7186 jeyun22@ece.skku.ac.kr +82-31-290-7186 sonne@ece.skku.ac.kr +82-31-290-7186 creata@ece.skku.ac.kr +82-31-290-7144 bjeon@yurim.skku.ac.kr |
| Source: | Sung Kyun Kwan University |  |  |

_____________________________

## Abstract

- The Waikoloa JVT meeting accepted a fast mode decision method [1] in the test model. This method which is based on early detection of P-skip mode and selective intra mode decision is developed mainly for baseline profile applications. Since the complexity of mode decision in the main profile is much higher than the baseline profile due to B slice, in this proposal, we further extend the idea in [1] to early detection of B- skip. The simulation results show that the proposed method reduces encoding time by 66% on average without noticeable performance degradation.

## Fast mode decision for B slice

- Table 1 shows the ratio of each macroblock type selected in B slice in various video sequences at Qp=28. As can be seen in the table, in real sequences, B-skip mode is selected in considerable times: the average ratio of SKIP mode is about 75% (maximum 98% in container and minimum 54% in foreman). Consequently, if SKIP mode for B slice can be detected at early stage (that is, without checking other modes), huge amount of computation can be saved.

- Table 1. The ratio of each macroblock type at Qp=28

| MB mode | Container | Foreman | Mobile | News | Paris | Silent | Tempete | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SKIP mode | 98.133 | 53.949 | 54.534 | 86.139 | 86.016 | 84.855 | 62.877 | 75.215 |
| Other inter modes | 1.847 | 45.648 | 45.447 | 13.861 | 13.962 | 15.016 | 36.514 | 24.614 |
| Intra mode | 0.020 | 0.403 | 0.019 | 0.000 | 0.022 | 0.129 | 0.609 | 0.172 |

- The definition of SKIP mode in B slice is different from that in the P slice. To be decided as the SKIP in B slice, both condition of direct mode and CBP(coded block pattern) = 0 should be satisfied. Therefore, motion vector under direct mode should be calculated first, and the rate distortion cost of direct mode should be also computed. The fast mode decision method proposed in this proposal is an extension of the previous method [1] for B-slices. Therefore, the selective intra mode decision in [1] is also applied after the inter mode decision to further reduce the complexity. Fig 1 shows the flowchart of proposed method.

- Fig 1. The flowchart of proposed method

## Experimental results

Table 2 shows simulation conditions. JM6.1d encoder is used as reference software, and the options of RD optimization and Hadamard transform are turned on.

- Table 2. Test condition

| Codec |  | JM 6.1d |  |
| --- | --- | --- | --- |
| MV search range |  | 16 |  |
| QP |  | 28, 34, 36, 40 |  |
| Coding option |  | Rate distortion optimization, Hadamard transform, #REF=3, CABAC |  |
| Test sequence | CONTAINER | QCIF (300 frames) | IBBP coding |
|  | FOREMAN | QCIF (300 frames) |  |
|  | NEWS | QCIF (300 frames) |  |
|  | MOBILE | CIF (300 frames) |  |
|  | SILENT | QCIF (300 frames) | IBP coding |
|  | PARIS | CIF (300 frames) |  |
|  | TEMPETE | CIF (260 frames) |  |

For degradation measure of encoding, two measurements of BDBR and BDPSNR [3] are calculated according to Qp values in Table 2. Table 3 shows experimental results. TS(Time Saving) indicates the percentage of processing time difference as defined below:

- Table 3. Experimental results

| Sequences | Early detecting P skip |  |  | Early detecting B skip |  |  | Selective intra coding |  |  | Combined all |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BDBR [%] | BDPSNR [dB] | TS [%] | BDBR [%] | BDPSNR [dB] | TS [%] | BDBR [%] | BDPSNR [dB] | TS [%] | BDBR [%] | BDPSNR [dB] | TS [%] |
| Container | -0.71 | 0.05 | 4.63 | -0.36 | 0.02 | 69.19 | 0.73 | -0.06 | 21.16 | 0.35 | -0.04 | 80.47 |
| Foreman | -0.12 | 0.01 | -1.56 | 0.91 | -0.05 | 50.56 | 0.84 | -0.04 | 18.82 | 4.49 | -0.23 | 61.78 |
| News | 0.47 | -0.03 | 5.22 | -0.52 | 0.03 | 63.17 | 0.53 | -0.03 | 20.08 | 2.67 | -0.15 | 76.28 |
| Mobile | -0.18 | 0.01 | -1.26 | 2.36 | -0.09 | 41.50 | -0.07 | 0.00 | 20.85 | 2.48 | -0.10 | 57.44 |
| Silent | -0.17 | 0.01 | 8.94 | -0.40 | 0.02 | 48.64 | 0.52 | -0.02 | 20.47 | 0.80 | -0.04 | 66.75 |
| Paris | 0.18 | -0.01 | 9.64 | -0.54 | 0.02 | 45.53 | 0.51 | -0.02 | 23.97 | 0.73 | -0.03 | 65.63 |
| Tempete | 0.06 | 0.00 | -0.18 | 0.52 | -0.02 | 38.99 | 0.56 | -0.02 | 19.04 | 2.84 | -0.10 | 55.30 |
| Average | -0.07 | 0.01 | 3.63 | 0.28 | -0.01 | 51.08 | 0.52 | -0.03 | 20.63 | 2.05 | -0.10 | 66.24 |

Experimental results show that the proposed method reduces encoding time by about 66% on average. However, BDBR and BDPSNR degradation is only about 2% and 0.1dB, respectively. Especially, in the case of Container sequence, the encoding time is reduced by 80%. That is, H.264 encoder using the proposed technique can encode 5 times faster than conventional encoding method. Therefore, the proposed technique significantly reduces computational complexity without any noticeable coding loss.

- Conclusions

- A macroblock of B slice in H.264 can be coded with various coding modes according to the prediction direction: Forward, Backward, and Bi-direction. Therefore, the mode decision process using a rate distortion cost needs huge computational complexity. As an extension of the previous proposal [1], this document proposed ‘early SKIP mode decision’ in B slice. Since both baseline and main profile allow intra mode in P or B slice, ‘selective intra mode decision’ of [1] can be also applied without any modification. The proposed algorithm provides considerable reduction in encoding time only with small coding loss. Furthermore, the proposed method can be also implemented easily in current reference software without considerable modification.

## References

[1] B. Jeon and J. Lee, “Fast mode decision for H.264,” ITU-T Q.6/16, Doc. #JVT-J033, 2003.

[2] Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG, “Draft ITU-T Recommendation and Final Draft International Standard of Joint Video Specification (ITU-T Rec. H.264 | ISO/IEC 14496-10 AVC),” Doc. G050r1, Mar. 2003.

[3] G. Bjontegaard, “Calculation of Average PSNR Differences between RD-curves,” Doc. VCEG-M33, Apr. 2001.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Sung Kyun Kwan University |  |  |
| Mailing address | 300 Chunchundong, Jangan-gu, Suwon |  |  |
| Country | Korea |  |  |
| Contact person | Byeungwoo Jeon |  |  |
| Telephone |  |  |  |
| Fax |  |  |  |
| Email | bjeon@yurim.skku.ac.kr |  |  |
| Place and date of submission |  |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  |  |  |
| Title |  |  |  |
| Contribution number |  |  |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
| O | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: