| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 11th Meeting: Munich, DE, 15-19 March, 2004 | Document: JVT-K031 Filename: input.doc |
| --- | --- |

| Title: | Dynamic Range of Outcome of Simplified 8x8 Transform and Quantization/De-quantization |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tao Chen Panasonic Hollywood Laboratory 100 Universal City Plaza Universal City, CA 91608 Jiuhuai Lu Panasonic Hollywood Laboratory 100 Universal City Plaza Universal City, CA 91608 Yoshiichiro Kashiwagi Panasonic Hollywood Laboratory 100 Universal City Plaza Universal City, CA 91608 Shinya Kadono Matsushita Electric Industrial Co., Ltd. 1006, Kadoma Kadoma City Osaka, 571-85001, Japan | Tel: Email: | +1 818-777-6175 chent@research.panasonic.com jiuhuai@research.panasonic.com kashiwagiy@research.panasonic.com kadono.shinya@jp.panasonic.com |
| Source: | Panasonic/MEI |  |  |

_____________________________

# Introduction

In [1], the simplified 8x8 transform previously introduced in [2] has been employed for luminance coding, while the chrominance coding still uses 4x4 transform. In previous meetings, 16-bit IDCT definition and associated quantization have been adopted, and it has been verified for 4x4 transform and its associated quantization [3]. This document discusses the dynamic range for simplified 8x8 transform. The main focus is on the decoding process since at the decoder the IDCT and de-quantization is normative, while results at the encoder side will also be presented.

- 8x8 transform dynamic range

When using the simplified 8x8 transform, we have the following forward and inverse transform and quantization/de-quantization processes (specifics on transform and quantization/scaling are referred to [1,2]).

A=T⋅XB=Y⋅TTC=q⋅BD=C>>(16+n)E=(D⋅r)<<nF=E⋅TG=TT⋅FH=G>>8

Since the transform does not have a single norm and different rows of the transform matrix [1, 2] introduce different increases in dynamic range, we need to consider all the different cases as in designing quantization/de-quantization tables [1]. The analysis follows a similar way that was used to show the dynamic range of ABT transforms [4]. Corresponding to the six cases (i.e., six columns of matrix M in [1]) based on the coefficient positions (i, j), the dynamic range of the outcome of these equations is given in Tables 1-6. Here, it assumes that the input signal X has a dynamic range of 9 bits including sign. Note that variable C is an intermediate result and will not be stored in memory.

Table 1 Dynamic range for case 1: i=[0,4] & j=[0,4]

| Output | dynamic of operation | resulting range [bits] |
| --- | --- | --- |
| X | - | 9 |
| A = T X | 9 + 6 – 3 | 12 (12) |
| B = A  TT | 12 + 6 – 3 | 15 (15) |
| C = q B | 15 + 13.7 | 28.7 (29) |
| D = C>>(16+n) | 28.7 – 16 | 12.7 (13) |
| E = (D  r) << n | 12.7 + 4.3 | 17 (17) |
| F = E  T | 17 + 3 – 3 | 17 (17) |
| G = TT  F | 17 + 3 – 3 | 17 (17) |
| H = G >> 8 | 17 – 8 | 9 (9) |

Table 2 Dynamic range for case 2: i=[1,3,5,7] & j=[1,3,5,7]

| Output | dynamic of operation | resulting range [bits] |
| --- | --- | --- |
| X | - | 9 |
| A = T X | 9 + 6 – 3 | 12 (12) |
| B = A  TT | 12 + 6 – 3 | 15 (15) |
| C = q B | 15 + 13.5 | 28.5 (29) |
| D = C>>(16+n) | 28.5 – 16 | 12.5 (13) |
| E = (D  r) << n | 12.5 + 4.1 | 16.6 (17) |
| F = E  T | 16.6 + 3.2 – 3 | 16.8 (17) |
| G = TT  F | 16.8 + 3.2 – 3 | 17 (17) |
| H = G >> 8 | 17 – 8 | 9 (9) |

Table 3 Dynamic range for case 3: i=[2,6] & j=[2,6]

| Output | Dynamic of operation | resulting range [bits] |
| --- | --- | --- |
| X | - | 9 |
| A = T X | 9 + 5.6 – 3 | 11.6 (12) |
| B = A  TT | 11.6 + 5.6 – 3 | 14.2 (15) |
| C = q B | 14.2 + 14.4 | 28.6 (29) |
| D = C>>(16+n) | 28.6 – 16 | 12.6 (13) |
| E = (D  r) << n | 12.6 + 5 | 17.6 (18) |
| F = E  T | 17.6 + 2.7 – 3 | 17.3 (18) |
| G = TT  F | 17.3 + 2.7 – 3 | 17 (17) |
| H = G >> 8 | 17 – 8 | 9 (9) |

Table 4 Dynamic range for case 4: (i=[0,4] & j=[1,3,5,7]) || (i=[1,3,5,7] & j=[0,4])

| Output | Dynamic of operation | resulting range [bits] |
| --- | --- | --- |
| X | - | 9 |
| A = T X | 9 + 6 – 3 | 12 (12) |
| B = A  TT | 12 + 6 – 3 | 15 (15) |
| C = q B | 15 + 13.6 | 28.6 (29) |
| D = C>>(16+n) | 28.6 – 16 | 12.6 (13) |
| E = (D  r) << n | 12.6 + 4.2 | 16.8 (17) |
| F = E  T | 16.8 + 3 – 3 | 16.8 (17) |
| G = TT  F | 16.8 + 3.2 – 3 | 17 (17) |
| H = G >> 8 | 17 – 8 | 9 (9) |

Table 5 Dynamic range for case 5: (i=[0,4] & j=[2,6]) || (i=[2,6] & j=[0,4])

| Output | Dynamic of operation | resulting range [bits] |
| --- | --- | --- |
| X | - | 9 |
| A = T X | 9 + 6 – 3 | 12 (12) |
| B = A  TT | 12 + 5.6 – 3 | 14.6 (15) |
| C = q B | 14.6 + 14.0 | 28.6 (29) |
| D = C>>(16+n) | 28.6 – 16 | 12.6 (13) |
| E = (D  r) << n | 12.6 + 4.7 | 17.3 (18) |
| F = E  T | 17.3 + 3 – 3 | 17.3 (18) |
| G = TT  F | 17.3 + 2.7 – 3 | 17 (17) |
| H = G >> 8 | 17 – 8 | 9 (9) |

Table 6 Dynamic range for case 6: (i=[2,6] & j=[1,3,5,7]) || (i=[1,3,5,7] & j=[2,6])

| Output | dynamic of operation | resulting range [bits] |
| --- | --- | --- |
| X | - | 9 |
| A = T X | 9 + 6 – 3 | 12 (12) |
| B = A  TT | 12 + 5.6 – 3 | 14.6 (15) |
| C = q B | 14.6 + 13.9 | 28.5 (29) |
| D = C>>(16+n) | 28.5 – 16 | 12.5 (13) |
| E = (D  r) << n | 12.5 + 4.6 | 17.1 (18) |
| F = E  T | 17.1 + 3.2 – 3 | 17.3 (18) |
| G = TT  F | 17.3 + 2.7 – 3 | 17 (17) |
| H = G >> 8 | 17 – 8 | 9 (9) |

- Discussion

Based on the above analysis, the maximum dynamic range is 18 bits at the decoder side. In order to ensure 16-bit memory access, the associated de-quantization shall have an additional 2-bit right-shit prior to inverse transform and this can be compensated by reducing the final shift of G from 8 bits to 6 bits. This will introduce loss of precision in integer calculation; however, variable E shall have enough precision even after a right-shift of 2 bits. Otherwise, saturation process (as previously in MPEG-2 and MPEG-4 Part 2) needs to be in place to avoid the problem at decoder side. On the other hand, in Professional Extension Profile, the 16-bit restriction might be relaxed especially for input that is more that 8 bits.

References:

[1] Steve Gordon, et al., “Simplified use of 8x8 transforms – proposal”, JVT-J029.doc, Dec. 8-12, 2003.

[2] Frank Bossen, “ABT cleanup and complexity reduction”, JVT-E087.doc, October 9-17 2002.

[3] Louis Kerofsky, “Notes on JVT IDCT”, JVT-C24.doc, May 6-10, 2002.

[4] Mathias Wien, “Clean-up and improved design consistency for ABT”, JVT-E025r1.doc, October 9-17, 2002.

(Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name |  |  |  |
| Mailing address |  |  |  |
| Country |  |  |  |
| Contact person | Tao Chen |  |  |
| Telephone | +1 818-777-6175 |  |  |
| Fax |  |  |  |
| Email | chent@research.panasonic.com |  |  |
| Place and date of submission |  |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  |  |  |
| Title |  |  |  |
| Contribution number |  |  |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
| x | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: