| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 12th Meeting: Redmond, USA, 19-23 July, 2004 | Document: JVT-L017 Filename: JVT-L017.doc |
| --- | --- |

| Title: | ## Lossless coding for Professional Extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yung-Lyul Lee, Ki-Hun Han Department of Internet Engineering, Sejong University, Seoul, Korea. | Tel: Email: | +82-3408-3753 yllee@sejong.ac.kr |
| Source: | Sejong University |  |  |

1. Introduction

In this document, we propose a lossless coding for Intra picture by changing the semantics (meaning) of the existing 4x4 Intra predictions in Luma signal and the existing 8x8 Intra predictions in Chroma signals. The block diagram for the lossless coding is shown in Fig. 1 and the Intra4x4 prediction modes are shown in Fig. 2. The DC prediction is used as it is in H.264 standard. The Semantics of the 4x4Intra prediction modes in Luma signal are changed except the DC prediction in this lossless coding proposal. Also the semantics of horizontal and vertical predictions in Chroma signals are changed. The results were cross-verified in Appendix by Sharp Lab. using YUV 4:2:0 data format and will be cross-verified by Samsung AIT using RGB 4:4:4 data format. The proposed method can be called pixel-wise prediction.

Fig. 1. Block diagram of the proposed lossless coding for intra frame

Fig. 2. Intra4x4 prediction mode for 9 directions

- 2. Proposed Lossless Coding for Intra picture

The proposed method improves the lossless coding efficiency in intra picture by using the nearest neighbor pixels for each direction to predict the current pixel, while the existing Luma and Chroma prediction modes syntax is not changed. The main idea of the proposed method is that the correlations of the nearest pixels inside the current block are usually higher than the pixels on the neighboring block boundary. For the proposed method, the semantics of intra 4x4 prediction modes are changed except the DC prediction.

1.  Luma Prediction Semantics Change

#### 1.1. Mode 0: Intra_4x4_Vertical Prediction

Fig. 3. Intra_4x4_Vertical prediction mode

The pixel a is predicted from the pixel A.

The pixel e is predicted from the pixel a.

The pixel i is predicted from the pixel e.

The pixel m is predicted from the pixel i.

The pixel b is predicted from the pixel B.

The pixel f is predicted from the pixel b.

The pixel j is predicted from the pixel f.

The pixel n is predicted from the pixel j.

The pixel c is predicted from the pixel C.

The pixel g is predicted from the pixel c.

The pixel k is predicted from the pixel g.

The pixel o is predicted from the pixel k.

The pixel d is predicted from the pixel D.

The pixel h is predicted from the pixel d.

The pixel l is predicted from the pixel h.

The pixel p is predicted from the pixel l.

The decoding order for the pixels in the Intra_4x4_Vertical prediction mode is the same as the encoding order by the reverse operation.

#### 1.2. Mode 1: Intra_4x4_Horizontal Prediction

Fig. 4. Intra_4x4_Horizontal prediction mode

The pixel a is predicted from pixel I.

The pixel b is predicted from pixel a.

The pixel c is predicted from pixel b.

The pixel d is predicted from pixel c.

The pixel e is predicted from pixel J.

The pixel f is predicted from pixel e.

The pixel g is predicted from pixel f.

The pixel h is predicted from pixel g.

The pixel i is predicted from pixel K.

The pixel j is predicted from pixel i.

The pixel k is predicted from pixel j.

The pixel l is predicted from pixel k.

The pixel m is predicted from pixel L.

The pixel n is predicted from pixel m.

The pixel o is predicted from pixel n.

The pixel p is predicted from pixel o.

The decoding order for the pixels in the Intra_4x4_Horizontal prediction mode is the same as the encoding order by the reverse operation.

#### 1.3. Mode 3: Intra_4x4_Diagonal_Down_Left Prediction

Fig. 5. Intra_4x4_Diagonal_Down_Left prediction mode

The pixel a is predicted from pixels (A + 2B + C + 2) / 4

The pixel b is predicted from pixels (B + 2C + D + 2) / 4

The pixel c is predicted from pixels (D + 2D + E + 2) / 4

The pixel d is predicted from pixels (D + 2E + F + 2) / 4

The pixel e is predicted from pixels (a + 2b + c + 2) / 4

The pixel f is predicted from pixels (b + 2c + d + 2) / 4

The pixel g is predicted from pixels (c + 2d + d + 2) / 4

The pixel h is predicted from pixel d

The pixel i is predicted from pixels (e + 2f + g + 2) / 4

The pixel j is predicted from pixels (f + 2g + h + 2) / 4

The pixel k is predicted from pixels (g + 2h + h + 2) / 4

The pixel l is predicted from pixel h

The pixel m is predicted from pixels (i + 2j + k + 2) / 4

The pixel n is predicted from pixels (j + 2k + l + 2) / 4

The pixel o is predicted from pixels (k + 2l + l + 2) / 4

The pixel p is predicted from pixel l

The decoding order for the pixels in the Intra_4x4_Diagonal_Down_Left prediction mode is the same as the encoding order by the reverse operation.

1.4.  Mode 4: Intra_4x4_Diagonal_Down_Right Prediction

Fig. 6. Intra_4x4_Diagonal_Down_Right prediction mode

The pixel a is predicted from the pixels (I + 2X + A + 2) / 4.

The pixel b is predicted from the pixels (X + 2A + B + 2) / 4.

The pixel c is predicted from the pixels (A + 2B + C + 2) / 4.

The pixel d is predicted from the pixels (B + 2C + D + 2) / 4.

The pixel e is predicted from the pixels (J + 2I + a + 2) / 4.

The pixel f is predicted from the pixels (I + 2a + b + 2) / 4.

The pixel g is predicted from the pixels (a + 2b + c + 2) / 4.

The pixel h is predicted from the pixels (b + 2c + d + 2) / 4.

The pixel i is predicted from the pixels (K + 2J + e + 2) / 4.

The pixel j is predicted from the pixels (J + 2e + f + 2) / 4.

The pixel k is predicted from the pixels (e + 2f + g + 2) / 4.

The pixel l is predicted from the pixels (f + 2g + h + 2) / 4.

The pixel m is predicted from the pixels (L + 2K + i + 2) / 4.

The pixel n is predicted from the pixels (K + 2i + j + 2) / 4.

The pixel o is predicted from the pixels (i + 2j + k + 2) / 4.

The pixel p is predicted from the pixels (j + 2k + l + 2) / 4.

The decoding order for the pixels in the Intra_4x4_Diagonal_Down_Right prediction mode is the same as the encoding order by the reverse operation.

#### 1.5.  Mode 5: Intra_4x4_Vertical_Right Prediction

Fig. 7. Intra_4x4_Vertical_Right prediction mode

The pixel a is predicted from the pixels (X + A + 1) / 2.

The pixel b is predicted from the pixels (A + B + 1) / 2.

The pixel c is predicted from the pixels (B + C + 1) / 2.

The pixel d is predicted from the pixels (C + D + 1) / 2.

The pixel e is predicted from the pixels (I + a + 1) / 2.

The pixel f is predicted from the pixels (a + b + 1) / 2.

The pixel g is predicted from the pixels (b + c + 1) / 2.

The pixel h is predicted from the pixels (c + d + 1) / 2.

The pixel i is predicted from the pixels (J + e + 1) / 2.

The pixel j is predicted from the pixels (e + f + 1) / 2.

The pixel k is predicted from the pixels (f + g + 1) / 2.

The pixel l is predicted from the pixels (g + h + 1) / 2.

The pixel m is predicted from the pixels (K + i + 1) / 2.

The pixel n is predicted from the pixels (i + j + 1) / 2.

The pixel o is predicted from the pixels (j + k + 1) / 2.

The pixel p is predicted from the pixels (k + l + 1) / 2.

The decoding order for the pixels in the Intra_4x4_Vertical_Right prediction mode is the same as the encoding order by the reverse operation.

#### Mode 6: Intra_4x4_Horizontal_Down Prediction

Fig. 8. Intra_4x4_Horizontal_Down prediction mode

The pixel a is predicted from the pixels (X + I + 1) / 2.

The pixel e is predicted from the pixels (I + J + 1) / 2.

The pixel i is predicted from the pixels (J + K + 1) / 2.

The pixel m is predicted from the pixels (K + L + 1) / 2.

The pixel b is predicted from the pixels (A + a + 1) / 2.

The pixel f is predicted from the pixels (a + e + 1) / 2.

The pixel j is predicted from the pixels (e + i + 1) / 2.

The pixel n is predicted from the pixels (i + m + 1) / 2.

The pixel c is predicted from the pixels (B + b + 1) / 2.

The pixel g is predicted from the pixels (b + f + 1) / 2.

The pixel k is predicted from the pixels (f + j + 1) / 2.

The pixel o is predicted from the pixels (j + n + 1) / 2.

The pixel d is predicted from the pixels (C + c + 1) / 2.

The pixel h is predicted from the pixels (c + g + 1) / 2.

The pixel l is predicted from the pixels (g + k + 1) / 2.

The pixel p is predicted from the pixels (k + o + 1) / 2.

The decoding order for the pixels in the Intra_4x4_Horizontal_Down prediction mode is the same as the encoding order by the reverse operation.

#### 1.7.  Mode 7: Intra_4x4_Vertical_Left Prediction mode

Fig. 9. Intra_4x4_Vertical_Left prediction mode

The pixel a is predicted from the pixels (A + B + 1) / 2.

The pixel b is predicted from the pixels (B + C + 1) / 2.

The pixel c is predicted from the pixels (C + D + 1) / 2.

The pixel d is predicted from the pixels (D + E + 1) / 2.

The pixel e is predicted from the pixels (a + b + 1) / 2.

The pixel f is predicted from the pixels (b + c + 1) / 2.

The pixel g is predicted from the pixels (c + d + 1) / 2.

The pixel h is predicted from the pixel d

The pixel i is predicted from the pixels (e + f + 1) / 2.

The pixel j is predicted from the pixels (f + g + 1) / 2.

The pixel k is predicted from the pixels (g + h + 1) / 2.

The pixel l is predicted from the pixel h.

The pixel m is predicted from the pixels (i + j + 1) / 2.

The pixel n is predicted from the pixels (j + k + 1) / 2.

The pixel o is predicted from the pixels (k + l + 1) / 2.

The pixel p is predicted from the pixel l.

The decoding order for the pixels in the Intra_4x4_Vertical_Left prediction mode is the same as the encoding order by the reverse operation.

#### 1.8.  Mode 8: Intra_4x4_Horizontal_Up prediction

Fig. 10. Intra_4x4_Horizontal_Up prediction mode

The pixel a is predicted from the pixels (I + J + 1) / 2.

The pixel e is predicted from the pixels (J + K + 1) / 2.

The pixel i is predicted from the pixels (K + L + 1) / 2.

The pixel m is predicted from the pixel L.

The pixel b is predicted from the pixels (a + e + 1) / 2.

The pixel f is predicted from the pixels (e + i + 1) / 2.

The pixel j is predicted from the pixels (i + m + 1) / 2.

The pixel n is predicted from the pixel m.

The pixel c is predicted from the pixels (b + f + 1) / 2.

The pixel g is predicted from the pixels (f + j + 1) / 2.

The pixel k is predicted from the pixels (j + n + 1) / 2.

The pixel o is predicted from the pixel n.

The pixel d is predicted from the pixels (c + g + 1) / 2.

The pixel h is predicted from the pixels (g + k + 1) / 2.

The pixel l is predicted from the pixels (k + o + 1) / 2.

The pixel p is predicted from the pixel o.

The decoding order for the pixels in the Intra_4x4_Horizontal_Up prediction mode is the same as the encoding order by the reverse operation.

- Chroma Prediction Semantics Change

The proposed method is applied to chroma prediction to improve the lossless coding efficiency by modification of the vertical and horizontal prediction semantics, while the semantics of the Intra_Chroma_DC and Intra_Chroma_plane predictions are not changed.

#### 2.1. Mode 1: Intra_Chroma_Horizontal Prediction

Fig. 11. Intra_Chroma_Horizontal Prediction Mode

The pixel a1 is predicted from pixel I.              The pixel b1 is predicted from pixel a1.

The pixel c1 is predicted from pixel b1.             The pixel d1 is predicted from pixel c1.

The pixel a2 is predicted from pixel d1.             The pixel b2 is predicted from pixel a2.

The pixel c2 is predicted from pixel b2.             The pixel d2 is predicted from pixel c2.

The pixel e1 is predicted from pixel J.              The pixel f1 is predicted from pixel e1.

The pixel g1 is predicted from pixel f1.              The pixel h1 is predicted from pixel g1.

The pixel e2 is predicted from pixel h1.              The pixel f2 is predicted from pixel e2.

The pixel g2 is predicted from pixel f2.              The pixel h2 is predicted from pixel g2.

The pixel i1 is predicted from pixel K.               The pixel j1 is predicted from pixel i1.

The pixel k1 is predicted from pixel j1.              The pixel l1 is predicted from pixel k1.

The pixel i2 is predicted from pixel l1.               The pixel j2a is predicted from pixel i2.

The pixel k2a is predicted from pixel j2.             The pixel l2 is predicted from pixel k2.

The pixel m1a is predicted from pixel L.             The pixel n1 is predicted from pixel m1.

The pixel o1 is predicted from pixel n1.              The pixel p1 is predicted from pixel o1.

The pixel m2 is predicted from pixel p1.             The pixel n2 is predicted from pixel m2.

The pixel o2 is predicted from pixel n2.              The pixel p2 is predicted from pixel o2.

The pixel a3 is predicted from pixel M.              The pixel b3 is predicted from pixel a3.

The pixel c3 is predicted from pixel b3.              The pixel d3 is predicted from pixel c3.

The pixel a4 is predicted from pixel d3.              The pixel b4 is predicted from pixel a4.

The pixel c4 is predicted from pixel b4.             The pixel d4 is predicted from pixel c4.

The pixel e3 is predicted from pixel N.              The pixel f3 is predicted from pixel e3.

The pixel g3 is predicted from pixel f3.             The pixel h3 is predicted from pixel g3.

The pixel e4 is predicted from pixel h3.             The pixel f4 is predicted from pixel e4.

The pixel g4 is predicted from pixel f4.              The pixel h4 is predicted from pixel g4.

The pixel i3 is predicted from pixel O.              The pixel j3 is predicted from pixel i3.

The pixel k3 is predicted from pixel j3.              The pixel l3 is predicted from pixel k3.

The pixel i4 is predicted from pixel l3.              The pixel j4 is predicted from pixel i4.

The pixel k4 is predicted from pixel j4.              The pixel l4 is predicted from pixel k4.

The pixel m3 is predicted from pixel P.              The pixel n3 is predicted from pixel m3.

The pixel o3 is predicted from pixel n3.              The pixel p3 is predicted from pixel o3.

The pixel m4 is predicted from pixel p3.             The pixel n4 is predicted from pixel m4.

The pixel o4 is predicted from pixel n4.              The pixel p4 is predicted from pixel o4.

The decoding order for the pixels in the Intra_Chroma_horizontal prediction mode is the same as the encoding order by the reverse operation.

  - Mode 2: Intra_Chroma_Vertical Prediction

Fig. 12. Intra_Chroma_Vertical Prediction Mode

The pixel a1 is predicted from pixel A.              The pixel e1 is predicted from pixel a1.

The pixel i1 is predicted from pixel e1.              The pixel m1 is predicted from pixel i1.

The pixel a3 is predicted from pixel m1.              The pixel e3 is predicted from pixel a3.

The pixel i3 is predicted from pixel e3.              The pixel m3 is predicted from pixel i3.

The pixel b1 is predicted from pixel B.              The pixel f1 is predicted from pixel b1.

The pixel j1 is predicted from pixel f1.              The pixel n1 is predicted from pixel j1.

The pixel b3 is predicted from pixel n1.              The pixel f3 is predicted from pixel b3.

The pixel j3 is predicted from pixel f3.              The pixel n3 is predicted from pixel j3.

The pixel c1 is predicted from pixel C.              The pixel g1 is predicted from pixel c1.

The pixel k1 is predicted from pixel g1.              The pixel o1 is predicted from pixel k1.

The pixel c3 is predicted from pixel o1.              The pixel g3 is predicted from pixel c3.

The pixel k3 is predicted from pixel g3.              The pixel o3 is predicted from pixel k3.

The pixel d1 is predicted from pixel D.              The pixel h1 is predicted from pixel d1.

The pixel l1 is predicted from pixel h1.              The pixel p1 is predicted from pixel l1.

The pixel d3 is predicted from pixel p1.              The pixel h3 is predicted from pixel d3.

The pixel l3 is predicted from pixel h3.              The pixel p3 is predicted from pixel l3.

The pixel a2 is predicted from pixel E.              The pixel e2 is predicted from pixel a2.

The pixel i2 is predicted from pixel e2.              The pixel m2 is predicted from pixel i2.

The pixel a4 is predicted from pixel m2.              The pixel e4 is predicted from pixel a4.

The pixel i4 is predicted from pixel e4.              The pixel m4 is predicted from pixel i4.

The pixel b2 is predicted from pixel F.              The pixel f2 is predicted from pixel b2.

The pixel j2 is predicted from pixel f2.              The pixel n2 is predicted from pixel j2.

The pixel b4 is predicted from pixel n2.              The pixel f4 is predicted from pixel b4.

The pixel j4 is predicted from pixel f4.              The pixel n4 is predicted from pixel j4.

The pixel c2 is predicted from pixel G.              The pixel g2 is predicted from pixel c2.

The pixel k2 is predicted from pixel g2.              The pixel o2 is predicted from pixel k2.

The pixel c4 is predicted from pixel o2.              The pixel g4 is predicted from pixel c4.

The pixel k4 is predicted from pixel g4.              The pixel o4 is predicted from pixel k4.

The pixel d2 is predicted from pixel H.              The pixel h2 is predicted from pixel d2.

The pixel l2 is predicted from pixel h2.              The pixel p2 is predicted from pixel l2.

The pixel d4 is predicted from pixel p2.              The pixel h4 is predicted from pixel d4.

The pixel l4 is predicted from pixel h4.              The pixel p4 is predicted from pixel l4.

The decoding order for the pixels in the Intra_Chroma_vertical prediction mode is the same as the encoding order by the reverse operation.

3. Experimental Results

We compared the proposed lossless coding method to JM73 lossless coding [1-2] without changing any syntax. The basic idea of JM73 lossless coding is to skip transform and quantization stage to circumvent possible lossy coding and apply the entropy coding mechanisms directly to the prediction error signal. For the experiments, we didn’t change any syntax. Tables 1-4 show the test conditions and experimental results. In the Tables, Compression Ratio and Relative Bits (%) are defined as follows :

(1)

(2)

Relative Bit Saving (%) mean the bit saving of the proposed method in comparison to the JM73 lossless coding method.

Experimental results show that the proposed method reduces bitrate approximately 12.6349 % when CABAC is used and approximately 13.5087 % when CAVLC is used.

Table 1. Test condition of Intra lossless coding. (Only I frame is used, P and B frame are not used)

|  | News (QCIF) | Container (QCIF) | Foreman (QCIF) | Silent (QCIF) | Paris (CIF) | Mobile (CIF) | Tempete (CIF) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Total frame | 300 (30Hz) | 300 (30Hz) | 300 (30Hz) | 300 (30Hz) | 300 (30Hz) | 300 (30Hz) | 260 (30Hz) |
| Coding Options | Rate Optimization, CABAC or CAVLC, error tool not used, Intra4x4mode prediction in Luma , Intra8x8 mode prediction in Chroma are used. (Intra16x16 prediction modes are disabled) |  |  |  |  |  |  |

Table 2. Results of Intra lossless coding

| Image | Original Size(Bits) | Method | CABAC |  |  | CAVLC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Total Bits | Compr ession Ratio | Relative Bits (%) | Total Bits | Compr ession Ratio | Relative Bits (%) |
| News (300 Frames) | 91238400 | JM73 | 49062832 | 1.8596 | 100 | 52730184 | 1.7303 | 100 |
|  |  | Sejong | 41581928 | 2.1942 | 84.7524 | 44584128 | 2.0464 | 84.5514 |
| Container (300 Frames) | 91238400 | JM73 | 47836576 | 1.9073 | 100 | 51976808 | 1.7554 | 100 |
|  |  | Sejong | 41989888 | 2.1729 | 87.7778 | 45443536 | 2.0077 | 87.4304 |
| Foreman (300 Frames) | 91238400 | JM73 | 50418312 | 1.8096 | 100 | 54997344 | 1.6590 | 100 |
|  |  | Sejong | 44770832 | 2.0379 | 88.7988 | 48453128 | 1.8830 | 88.1009 |
| Silent (300 Frames) | 91238400 | JM73 | 54273064 | 1.6811 | 100 | 59704832 | 1.5282 | 100 |
|  |  | Sejong | 47083432 | 1.9378 | 86.7529 | 50708848 | 1.7993 | 84.9326 |
| Paris (300 Frames) | 364953600 | JM73 | 224766912 | 1.6237 | 100 | 243763312 | 1.4972 | 100 |
|  |  | Sejong | 192908520 | 1.8918 | 85.8260 | 207292712 | 1.7606 | 85.0385 |
| Mobile (300 Frames) | 364953600 | JM73 | 285423632 | 1.2786 | 100 | 310319680 | 1.1761 | 100 |
|  |  | Sejong | 254943008 | 1.4315 | 89.3209 | 273689024 | 1.3335 | 88.1958 |
| Tempete (260 Frames) | 316293120 | JM73 | 205817192 | 1.5368 | 100 | 225291464 | 1.4039 | 100 |
|  |  | Sejong | 181791504 | 1.7399 | 88.3267 | 196429712 | 1.6102 | 87.1891 |
| Average |  | JM73 |  | 1.6710 | 100 |  | 1.5357 | 100 |
|  |  | Sejong |  | 1.9151 | 87.3651 |  | 1.7772 | 86.4913 |

Table 3. Test condition of IPPP… lossless coding (Only first frame is Intra-coded)

|  | News (QCIF) | Container (QCIF) | Foreman (QCIF) | Silent (QCIF) | Paris (CIF) | Mobile (CIF) | Tempete (CIF) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Total frame | 100 (10Hz) | 100 (10Hz) | 100 (10Hz) | 150 (15Hz) | 150 (15Hz) | 300 (30Hz) | 260 (30Hz) |
| Coding Options | Rate Optimization, CABAC or CAVLC, error tool not used, ±32 search range, Number of reference frames are 5. All MB modes are on except Intra16x16 prediction modes. (Intra16x16 prediction modes are disabled) |  |  |  |  |  |  |

Table 4. Results of IPP.... Lossless coding

| Image | Original Size(Bits) | Method | CABAC |  |  | CAVLC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Total Bits | Compr ession Ratio | Relative Bits (%) | Total Bits | Compr ession Ratio | Relative Bits (%) |
| News (100 Frames) | 30412800 | JM73 | 6474616 | 4.6972 | 100 | 7213696 | 4.2160 | 100 |
|  |  | Sejong | 6378904 | 4.7677 | 98.5217 | 7107024 | 4.2793 | 98.5213 |
| Container (100 Frames) | 30412800 | JM73 | 8349720 | 3.6424 | 100 | 9752880 | 3.1183 | 100 |
|  |  | Sejong | 8320368 | 3.6552 | 99.6485 | 9712456 | 3.1313 | 99.5855 |
| Foreman (100 Frames) | 30412800 | JM73 | 11995600 | 2.5353 | 100 | 13411384 | 2.2677 | 100 |
|  |  | Sejong | 11755696 | 2.5871 | 98.0001 | 13139976 | 2.3145 | 97.9763 |
| Silent (150 Frames) | 45619200 | JM73 | 11963424 | 3.8132 | 100 | 13553208 | 3.3659 | 100 |
|  |  | Sejong | 11795784 | 3.8674 | 98.5987 | 13359272 | 3.4148 | 98.5691 |
| Paris (150 Frames) | 182476800 | JM73 | 61209672 | 2.9812 | 100 | 70555232 | 2.5863 | 100 |
|  |  | Sejong | 60817088 | 3.0004 | 99.3586 | 70049536 | 2.6050 | 99.2833 |
| Mobile (300 Frames) | 364953600 | JM73 | 164983536 | 2.2121 | 100 | 181354448 | 2.0124 | 100 |
|  |  | Sejong | 164813080 | 2.2143 | 99.8967 | 181118432 | 2.0150 | 99.8699 |
| Tempete (260 Frames) | 316293120 | JM73 | 136517976 | 2.3169 | 100 | 151425776 | 2.0888 | 100 |
|  |  | Sejong | 135412424 | 2.3358 | 99.1902 | 150119440 | 2.1069 | 99.1373 |
| Average |  | JM73 |  | 3.1712 | 100 |  | 2.8079 | 100 |
|  |  | Sejong |  | 3.2040 | 99.0306 |  | 2.8381 | 98.9918 |

We applied the proposed algorithm to the RGB 4:4:4 data to see how it works for this kind of images. These results are cross-verified in [3]. For this test we compared the several methods

1) direct RGB

2) in-loop YCoCg-R color transform as proposed in [4]

3) (2) + (3)

As simulation results shows, option (4) performs best that combines the proposed method and the in-loop color transform.

Table 5. Results of IPP.... Lossless coding on RGB 4:4:4

|  | 15 frames, IPP (file size) |  |  |
| --- | --- | --- | --- |
|  | bicycle | card toss | tomatoes |
| Sejong+RT (L014) | 436099744 | 590469704 | 464112320 |
| RT | 444597232 | 619744840 | 507721600 |
| Direct YCoCgR | 465630600 | 632895544 | 511028272 |
| Direct RGB | 475055480 | 620682776 | 550056272 |

Table 5. Results of IPP.... Lossless coding on RGB 4:4:4

|  | 15 frames, IPP (compression ratio) |  |  |
| --- | --- | --- | --- |
|  | bicycle | card toss | tomatoes |
| Sejong+RT (L014) | 1.7751535 | 1.5920072 | 2.0254407 |
| RT | 1.7412254 | 1.5168049 | 1.8514714 |
| direct YCoCgR | 1.6625711 | 1.4852878 | 1.8394912 |
| Direct RGB | 1.6295865 | 1.5145128 | 1.7089742 |

4. Conclusion

In this document, we propose a lossless coding for intra frame by changing the semantics of the existing 4x4 intra prediction method in Luma signal and the semantics of existing intra prediction methods in Chroma signals. The combined method with [4] performs constructively without causing any negative effect. So we propose to adopt this lossless coding method and the in-loop YCoCg-R color transform to Profession Extension Profile.

References

[1] S. Sun, “lossless Coding and QP Range Selection”, JVT-C023, May 2002

[2] S. Sun, “New Results of Lossless Coding”, JVT-D028, May 2002

[3] W.-S. Kim, D. Birinov and H. M. Kim, “Residue Color Transform,” ISO/IEC JTC1/SC29/WG11 and ITU-T Q6/SG16, Document JVT-L025, July 2004

[4] S. Sun, “Residual Color Transform Using YCoCg-R,” ISO/IEC JTC1/SC29/WG11 and ITU-T Q6/SG16, Document JVT-L014, July 2004.

5. Appendix

The cross-verification results using YUV4:2:0 format from Sharp Lab. (Sun, Shijun) are shown in Appendix.

Table A.1 Verification of intra lossless coding by Sharp Labs

| Image | Method | CABAC |  | CAVLC |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Total Bits by Sejong Univ. | Total Bits by Sharp Labs | Total Bits by Sejong Univ. | Total Bits by Sharp Labs |
| News (300 Frames) | JM73 | 49062832 | 49062832 | 52730184 | 52730184 |
|  | Sejong | 41581928 | 41581928 | 44584128 | 44584128 |
| Container (300 Frames) | JM73 | 47836576 | 47836576 | 51976808 | 51976808 |
|  | Sejong | 41989888 | 41989888 | 45443536 | 45443536 |
| Foreman* (300 Frames) | JM73 | 50418312 | 51290768 | 54997344 | 55630840 |
|  | Sejong | 44770832 | 44761680 | 48453128 | 47956992 |
| Silent (300 Frames) | JM73 | 54273064 | 54273064 | 59704832 | 59704832 |
|  | Sejong | 47083432 | 47083432 | 50708848 | 50708848 |
| Paris (300 Frames) | JM73 | 224766912 | 224766912 | 243763312 | 243763312 |
|  | Sejong | 192908520 | 192908520 | 207292712 | 207292712 |
| Mobile (300 Frames) | JM73 | 285423632 | 285423632 | 310319680 | 310319680 |
|  | Sejong | 254943008 | 254943008 | 273689024 | 273689024 |
| Tempete (260 Frames) | JM73 | 205817192 | 205817192 | 225291464 | 225291464 |
|  | Sejong | 181791504 | 181791504 | 196429712 | 196429712 |

* The raw data files seem different between the tests.

Table A.2 Verification of IPP.... lossless coding by Sharp Labs

| Image | Method | CABAC |  | CAVLC |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Total Bits by Sejong Univ. | Total Bits by Sharp Labs | Total Bits by Sejong Univ. | Total Bits by Sharp Labs |
| News (100 Frames) | JM73 | 6474616 | 6474616 | 7213696 | 7213696 |
|  | Sejong | 6378904 | 6378904 | 7107024 | 7107024 |
| Container (100 Frames) | JM73 | 8349720 | 8349720 | 9752880 | 9752880 |
|  | Sejong | 8320368 | 8320368 | 9712456 | 9712456 |
| Foreman* (100 Frames) | JM73 | 11995600 | 11288376 | 13411384 | 12620304 |
|  | Sejong | 11755696 | 10959512 | 13139976 | 12241512 |
| Silent (150 Frames) | JM73 | 11963424 | 11963424 | 13553208 | 13553208 |
|  | Sejong | 11795784 | 11795784 | 13359272 | 13359280 |
| Paris (150 Frames) | JM73 | 61209672 | 61209672 | 70555232 | 70555232 |
|  | Sejong | 60817088 | 60817088 | 70049536 | 70049536 |
| Mobile (300 Frames) | JM73 | 164983536 | 164983536 | 181354448 | 181354448 |
|  | Sejong | 164813080 | 164813080 | 181118432 | 181118432 |
| Tempete (260 Frames) | JM73 | 136517976 | 136517976 | 151425776 | 151425776 |
|  | Sejong | 135412424 | 135412424 | 150119440 | 150119440 |

* The raw data files seem different between the tests.

(Append for Proposal Documents)

# JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Sejong University |  |  |
| Mailing address | 98 Kunja-Dong, KwangJin-Gu, Seoul 143-747, Korea. |  |  |
| Country | Korea |  |  |
| Contact person | Yung-Lyul Lee |  |  |
| Telephone | +82-3408-3753 |  |  |
| Fax | +82-3408-3753 |  |  |
| Email | yllee@sejong.ac.kr |  |  |
| Place and date of submission |  |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  |  |  |
| Contribution number |  | JVT-K0xx |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
| X | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| # X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: