| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) Meeting: Redmond, USA, July 19-23, 2004 | Document: JVT-L023 Filename: JVT-L023.doc |
| --- | --- |

| Title: | SEI message to indicate stereo video information in progressive H.264 streams |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Henrik Karppinen, Nokia Visiokatu 1, 33720 Tampere, Finland | Tel: Email: | +358504821069 henrik.karppinen@nokia.com |
| Source: | Nokia |  |  |

_____________________________

Summary

This document proposes two additional fields to the stereo video SEI message. Current stereo video SEI message does not support indication of stereo video carried in a progressive H.264 stream. The first field proposed indicates the presence of stereo video in a progressive stream and the second one indicates the possible sub-sampling format of the video. Presence of the sub-sampling information can halve the pixel-processing rate and bitrate of the coded video in typical stereo video applications.

Syntax

The current syntax of the stereo video SEI message in Version 3 of H.264/AVC, Chapter D.1.23:

Stereo video fields SEI message syntax

| stereo_video_fields( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| top_field_is_left_view_flag | 5 | u(1) |
| top_field_self_contained_flag | 5 | u(1) |
| bot_field_self_contained_flag | 5 | u(1) |
| } |  |  |

The proposed new syntax:

Stereo video fields SEI message syntax

| stereo_video_fields( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| stereo_video_information_flag | 5 | u(1) |
| top_field_self_contained_flag | 5 | u(1) |
| bot_field_self_contained_flag | 5 | u(1) |
| horizontally_sub_sampled_frames_flag | 5 | U(1) |
| } |  |  |

Semantics

Current SEI message field semantics of the Version 3 of H.264/AVC, Chapter D.2.23, is in appendix.

stereo_video_information_flag: Interpretation of this flag depends on the stream mode. This flag defines how the stereo video frames are ordered in progressive stereo video stream. The same flag defines also how the stereo video slices are ordered in the interlaced stereo video stream and hence it replaces the current top_field_is_left_view_flag.

Due to fact that SEI messages are decoded before decoding of the frame or slice data the semantics of the stereo_video_information_flag is decided during the frame or slice decoding process. During the decoding process of the frame or slice data one is able to check frame_mbs_only_flag. If the frame_mbs_only_flag is 1 then the stream is progressive, if the flag is 0 then the stream is interlaced.

Semantics in a progressive video stream

stereo_video_information_flag equal to 1 indicates that the frames with even picture order counts in the coded video sequence represent a view from the left from the perspective of the viewer. The frames with odd picture order counts in the coded video sequence represent a view from the right from the perspective of the viewer.

stereo_video_information_flag equal to 0 indicates that the frames with even picture order counts in the coded video sequence represent a view from the right from the perspective of the viewer and the frames with odd picture order counts in the coded video sequence represent a view from the left from the perspective of the viewer.

Motion vectors of the left frame can only refer to reference pictures of the left view. Motion vectors of the right frame can only refer to reference pictures of the right view. As a result from this devices without 3D display functionality does not have to decode both left and right view frames. In this case the decoder can choose to use only one of the views.

Semantics in an interlaced video stream

Only name of the flag is changed compared to Version 3 of H.264/AVC, Chapter D.2.23.

stereo_video_information_flag equal to 1 indicates that the top fields in the coded video sequence represent a view from the left from the perspective of the viewer and the bottom fields in the coded video sequence represent a view from the right from the perspective of the viewer.  stereo_video_information_flag equal to 0 indicates that the bottom fields in the coded video sequence represent a view from the left from the perspective of the viewer and the top fields in the coded video sequence represent a view from the right from the perspective of the viewer.

In the interlaced video stream the inter prediction is defined using top_field_self_contained_flag and bot_field_self_contained_flag like the Version 3 of H.264/AVC suggest.

Semantics as a pseudo code

if( frame_mbs_only_flag )

{

if( stereo_video_information_flag )

even frames are for the left view

else

even frames are for the right view

}

else

{

if( stereo_video_information_flag )

top fields are for the left view

else

top fields are for the right view

}

horizonally_sub_sampled_frames_flag equal to 1 indicates that the input frame(s) are horizontally sub-sampled by a factor of two. If the flag is 1 and frame size of the left and right view are equal then frames from the both view are sub-sampled. If the flag is 1 and the frame size of the left and right view is not equal, then either left view or right view frame have full-resolution and another view is sub-sampled by two. The 3D video stream with variable frame size can be rendered on 2D display without up-scaling by choosing the full-resolution frames from the stream. If the flag is 0 the frames are not sub-sampled.

Horizontal sub-sampling is useful in the typical case of parallax barrier or similar displays. In this case the left and right frame are first sub-sampled horizontally by a factor of two and after that the half resolution frames are combined and rendered. See Figure 1. Because only half resolution left and right view frames are needed in order to make the combined stereo video frame it does not make sense to transfer full resolution frames. One can halve the used bitrate and pixel-processing rate by using horizontally sub-sampled frames and get the same stereo view.  Principle of operation of parallax barrier display is described in more detailed in appendix.

Figure 1. Full resolution left and right image, halved horizontal resolution frames, and combined frame. The combined frame can be rendered on parallax barrier display.

Appendix

Version 3 of H.264/AVC, Chapter D.2.23, current stereo video fields SEI message semantics

This SEI message provides the decoder with an indication that the entire coded video sequence consists of a sequence of coded fields in which all fields of a particular parity represent a left view and all fields of the opposite parity represent a right view for stereo-view video content.

The stereo video fields SEI message shall not be present unless all pictures in the coded video sequence are fields. The stereo video fields SEI message shall not be present in any access unit of a coded video sequence unless a stereo video fields SEI message is present in the first access unit of the coded video sequence. The stereo video fields SEI message may also be present in other access units of the coded video sequence in addition to being present in the first access unit of the coded video sequence.

Within a coded video sequence, the values of the syntax elements in all stereo video fields SEI messages shall be the same.

When the stereo video fields SEI message is present, the spatial locations of the samples in each individual field should be interpreted for display purposes as representing complete pictures as shown in Figure 6-1 rather than as spatially-distinct fields within a source frame as shown in Figure 6-2.

top_field_is_left_view_flag equal to 1 indicates that the top fields in the coded video sequence represent a view from the left from the perspective of the viewer and the bottom fields in the coded video sequence represent a view from the right from the perspective of the viewer.  top_field_is_left_view_flag equal to 0 indicates that the bottom fields in the coded video sequence represent a view from the left from the perspective of the viewer and the top fields in the coded video sequence represent a view from the right from the perspective of the viewer.

top_field_self_contained_flag equal to 1 indicates that no inter prediction operations within the decoding process for the top fields of the coded video sequence refer to reference pictures that are bottom fields.  top_field_self_contained_flag equal to 0 indicates that some inter prediction operations within the decoding process for the top fields of the coded video sequence may or may not refer to reference pictures that are bottom fields.

bot_field_self_contained_flag equal to 1 indicates that no inter prediction operations within the decoding process for the bottom fields of the coded video sequence refer to reference pictures that are top fields.  bot_field_self_contained_flag equal to 0 indicates that some inter prediction operations within the decoding process for the bottom fields of the coded video sequence may or may not refer to reference pictures that are top fields.

Principle of operation of parallax barrier display

Parallax barrier is a 3D display requiring no special viewing glasses. 3D effect is generated by controlling the path of travel of light from the display so that slightly different images reach the left and right eyes. In other words, the right eye sees only the image intended for it, and the left eye only the image intended for it. An optical parallax barrier is used to control the path of light and it makes it possible to separate the display images so that slightly different images reach the left and right eyes. By displaying the image intended for the left eye and the image for the right eye as a stereographic pair on a screen each eye sees only the image intended for it and the brain combines the images and perceives them as a 3D representation. See Figure 2.

Figure 2. Parallax barrier display.

(Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Nokia Corporation |  |  |
| Mailing address |  |  |  |
| Country |  |  |  |
| Contact person |  |  |  |
| Telephone |  |  |  |
| Fax |  |  |  |
| Email |  |  |  |
| Place and date of submission | Redmond, WA, 19-24 July, 2004 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | ITU-T Rec. H.264 ¦ ISO/IEC 14496-10 |  |
| Title |  | SEI message to indicate stereo video information in progressive H.264 streams |  |
| Contribution number |  | JVT-L023 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
| X | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: