| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 12th Meeting: Redmond, WA, USA, 17-23 July, 2004 | Document: JVT-L025 Filename: JVT-L025.doc |
| --- | --- |

| Title: | Residue Color Transform |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Woo-Shik Kim, Dmitry Birinov, Hyun Mun Kim Multimedia Lab. Samsung AIT P.O.Box 111, Suwon 440-600 Korea | Tel: Email: | 82-31-280-9204 hyunmun27.kim@samsung.com |
| Source: | Samsung AIT |  |  |

_____________________________

- Introduction

In the last JVT meeting we introduced a residue transform [1] for 4:4:4 RGB coding. In this document we adopted YCoCg-R color transform [2] for the residue transform to support the lossless case as proposed in the last meeting [3]. Section 2 briefly describes the proposed algorithm. The test results are summarized in Section 3. We also cross-verified the lossless coding results using residue transform prepared by Sharp Labs [4] in Section 4. We conclude in Section 5.

- Residue Color Transform

Residue transform exploits the redundancy among the residual data of each RGB component after intra/inter prediction. The correlation among the color components still exists even after the intra/inter prediction. To decorrelate this redundancy, we apply the YCoCg-R color transform to the residual data as shown in Figure 1. This scheme can be easily incorporated into the existing JVT specification [5] as explained in the appendix.

This technique achieves better coding efficiency than applying the color transform outside the coding loop as manifested in the simulation results.

The transform is presented in Figure 2 where R, G, and B are the residual data and Y, Co, and Cg are the residue transformed data, respectively.

Figure 1: Block diagram of the proposed residue transform

Figure 2: Block diagram of the proposed YCoCg-R residue transform

To minimize the syntax change we propose to add a flag, “residue_transform_flag” to the sequence parameter set to signal the residue transform feature. If residue_transform_flag equals to ‘1’, the residue transform shall be performed, otherwise, the input data shall be coded without residue transform.

We also replaced the Hadamard transform with a 2I matrix where I is an identity matrix if residue_transform_flag is equal to 1 and current macroblock prediction mode for luma is equal to Intra_4x4 or Intra_8x8. This avoids the causality problem for residue transform when mode for luam and chrom is different. It is because we do not need to process whole 16x16 macroblcok for chroma components for DC transform. We can apply the replaced transform to the 4x4 blocks since the transform is noting but a scalar operation.

We can also maintain the current quantization scheme using this method as explained below.

Since the current Hadamard transform H is defined as

H=111111−1−11−1−111−11−1

andHH=4I. So by replacing the Hadamard transform with a 2I needs no quantization change.

The residue transform can be performed adaptively depending on the characteristics of the input data and the requirements of the application.

- Experimental Results

We compared the coding efficiency of the residue transform with that of coding directly without it. The test conditions are summarized as follows:

- Search range: 32

- Number of reference frame: 3

- Entropy coding: CABAC

- RD-optimized mode selection: On

- GOP structure: IBBP…

- Number of slice groups: 1

.

Table 1: Simulation Results that show the coding gain for Y component in db.

.

| Seq. | bit rate (Mbps) |  |  |
| --- | --- | --- | --- |
|  | 10 | 40 | 100 |
| Analog TV | na | 1 | 1.5 |
| Trees and Bicycle | 0.5 | 0.6 | 0.8 |
| Man in Restaurant | 0.5 | 0.7 | 1 |
| Rolling Tomatoes | 0.5 | 1.2 | 1.2 |
| Card Toss | na | 1 | 1.2 |
| Dinner | na | 1 | 1.2 |

Table 1 summarizes the coding gain using residue transform for Y component in YCoCg color domain. It shows about 0.5~1.5 db depending on the bit rate.

The complete test results are available in the accompanying excel file (JVT-L025.xls).

- Cross-Verification of Lossless Coding

We cross-verified the lossless coding results using residue transform prepared by Sharp Labs [4]. For this purpose, we decoded the bitstream generated by Sharp Labs encoder using our decoder and vice versa. The experimental results summarized in the JVT-L014.xls were identical for the images we have such as film images.

We also cross-verified the lossless coding results proposed by Sejong University [5].

The combined methods with [4] and [5] perform constructively without causing any negative effect. So we propose to adopt this lossless coding method and the in-loop YCoCg-R color transform for FRExt.

- Conclusions

The in-loop residue color transform using YCoCg-R after intra/inter prediction achieves better coding efficiency than applying the color transform outside the coding loop as shown in the simulation results. We can select this residue transform option depending on image characteristics at encoder’s discretion. It helps achieve good coding efficiency for various image set. We request that JVT adopt the residue color transform for FRExt.

References

[1] W.-S. Kim, D. Birinov and H. M. Kim, “Adaptive Residue Transform and Sampling,” ISO/IEC JTC1/SC29/WG11 and ITU-T Q6/SG16, Document JVT-K018, March 2004

- [2] H. Malvar and G. Sullivan, “YCoCg-R: a color space with RGB reversibility and low dynamic range,” ISO/IEC JTC1/SC29/WG11 and ITU-T Q6/SG16, Document JVT-I014, Sep. 2003.

- [3] S. Sun, “New Results of Lossless Video Coding for JVT Professional Extension,” ISO/IEC JTC1/SC29/WG11 and ITU-T Q6/SG16, Document JVT-K018, March 2004.

- [4] S. Sun, “Residual Color Transform Using YCoCg-R,” ISO/IEC JTC1/SC29/WG11 and ITU-T Q6/SG16, Document JVT-L014, July 2004.

- [5] Y.-L. Lee and K.-H. Han, “Lossless coding for Professional Extensions,” ISO/IEC JTC1/SC29/WG11 and ITU-T Q6/SG16, Document JVT-L017, July 2004.

[6] “Final Draft International Standard of Joint Video Specification (ITU-T Rec. H.264 | ISO/IEC 14496-10 AVC),” ISO/IEC JTC1/SC29/WG11 and ITU-T Q6/SG16, Document JVT-G050, Mar. 2003.

Annex: Draft Text

# 1)	Subclause 7.3.2.1

Insert residue_transform_flag as below:

| seq_parameter_set_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| profile_idc | 0 | u(8) |
| … |  |  |
| if( profile_idc = = 80 ¦ ¦ profile_idc = = 81 ¦ ¦ profile_idc = = 82 ¦ ¦ profile_idc = = 83 ) { |  |  |
| chroma_format_idc | 0 | ue(v) |
| bit_depth_luma_minus8 | 0 | ue(v) |
| bit_depth_chroma_minus8 | 0 | ue(v) |
| residue_transform_flag | 0 | u(1) |
| … |  |  |
| } |  |  |
| … |  |  |
| } |  |  |

# 2)	Subclause 7.4.2.1

Insert the following.

residue_transform_flag equal to 0 specifies that the residue transform is not applied to a luma and chroma residual block. residue_transform_flag equal to 1 specifies that the residue transform is applied to a luma and chroma residual block. When residue_transform_flag is not present, it shall be inferred to be equal to 0.

# 3)	Subclause 8.5.7

Replace below part

- Otherwise (chroma_format_idc is equal to 3), the inverse transform for the 4x4 chroma DC transform coefficients is specified as

f=111111−1−11−1−111−11−1c00c01c02c03c10c11c12c13c20c21c22c23c30c31c32c33111111−1−11−1−111−11−1	(8-257b)

with the following

- Otherwise (chroma_format_idc is equal to 3), the following applies.

- If residue_transform_flag is equal to 1 and current macroblock prediction mode is equal to Intra_4x4 or Intra_8x8,

f=2000020000200002c00c01c02c03c10c11c12c13c20c21c22c23c30c31c32c332000020000200002	(8-257b)

- Otherwise, the inverse transform for the 4x4 chroma DC transform coefficients is specified as

f=111111−1−11−1−111−11−1c00c01c02c03c10c11c12c13c20c21c22c23c30c31c32c33111111−1−11−1−111−11−1	(8-257b)

# 4)	Subclause 8.5.8

Insert the following at the end of the subclause.

If the residue_transform_flag is equal to 1, the inverse residue transform as specified in 8.5.8b is invoked.

# 5)	New subclause 8.5.8b “Inverse residue transform process”

Insert the following at the end of the subclause.

When the residue_transform_flag is equal to 1, the variables rY,ij, rCb,ij, and rCr,ij are defined to specify the rij of each color components which are outputs of Eq.8-282.

The following transform is applied:

t = rY,ij – (rCb,ij >> 1 )		(8-282zza)

rG,ij = t + rCb,ij		(8-282zzb)

rB,ij = t – (rCr,ij >> 1 )		(8-282zzc)

rR,ij = rB,ij + rCr,ij		(8-282zzd)

The rij of each color components, rY,ij, rCb,ij, and rCr,ij shall be replaced with rR,ij, rG,ij, and rB,ij, respectively.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Samsung Elctronics. Co. Ltd. |  |  |
| Mailing address | POBox 111 Suwon Samsung AIT Multimedia |  |  |
| Country | Korea |  |  |
| Contact person | Hyun Mun Kim |  |  |
| Telephone | 82-31-280-9204 |  |  |
| Fax | 82-31-280-9207 |  |  |
| Email | hyunmun27.kim@samsung.com |  |  |
| Place and date of submission | Redmond, WA, USA, 17-23 July, 2004 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Residue Color Transform |  |
| Contribution number |  | JVT-L025 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
| X | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| # X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: