| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 13th Meeting: Palma de Mallorca, Spain 18-22 Oct., 2004 | Document: JVT-M013 Filename: input.doc |
| --- | --- |

| Title: | Performance evaluation of fast mode decision algorithms for H.264 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Inchoon Choi School of ECE, SungKyunKwan Univ. 300 Chunchun-dong, Suwon, Korea Jeyun Lee School of ECE, SungKyunKwan Univ. 300 Chunchun-dong, Suwon, Korea Woong Il Choi School of ECE, SungKyunKwan Univ. 300 Chunchun-dong, Suwon, Korea Byeungwoo Jeon School of ECE, SungKyunKwan Univ. 300 Chunchun-dong, Suwon, Korea | Tel: Email: | +82-31-290-7186 sonne@ece.skku.ac.kr +82-31-290-7186 jeyun22@ece.skku.ac.kr +82-31-290-7186 creata@ece.skku.ac.kr +82-31-290-7144 bjeon@yurim.skku.ac.kr |
| Source: | Sung Kyun Kwan University |  |  |

_____________________________

## Abstract

Several fast mode decision algorithms were adopted for the JVT test model such as JVT-G013 [3], JVG-I020 [4], JVT-J033 [1], and JVT-K021 [2]. It turns out that each method significantly reduces computational complexity for RD optimization-based mode decision without outstanding loss. To know the overall performance of the fast algorithms in terms of speedup and PSNR, we integrated to the JVT reference software, all fast mode decision methods that were adopted in the JVT test model and compared their performances. The simulation results show that the fast high-complexity mode provides considerable reduction in computational complexity and processing time.

## Early skip detection and Selective intra mode decision

The proposed algorithms in JVT-J033 and JVT-K021 provided two algorithms for fast high-complexity mode decision. One of them is ‘early skip detection for P and B slice’. It checks whether the skip mode can be chosen for the current macroblock mode or not. To do so, first of all, one needs to estimate the motion vector and reference frame for the given macroblock in order to check the skip condition in P slice. After the motion vector and the reference frame are found, then we calculate the RD cost of 16x16 macroblock and check the P-SKIP condition. The procedure in case of B-slice is similar to that of P-slice. To check whether the current macroblock mode can be decided as SKIP in B-slice, macroblock is encoded as the direct mode in advance. Another class of fast algorithm is ‘Selective Intra mode decision’: it compares degree of spatial correlation with that of temporal correlation for deciding whether proceeding to intra coding is worthwhile. Since proper decision between intra and inter modes need some objective measure of spatial and temporal correlation, we proposed to utilize boundary errors for spatial correlation and number of bits for encoded motion-compensated residual data for the temporal correlation.

## Fast decision for intra prediction mode and inter block mode

Two fast mode decision algorithms for inter and intra prediction were proposed in Pattaya and San Diego meeting [3,4]. JVT-G013 proposed a fast mode decision scheme for intra prediction that employs Sobel edge operator to obtain local edge direction information of blocks and reduces amount of calculations in intra prediction. When Sobel edge operator generates edge map, an encoder can calculate edge direction and amplitude. Subsequently, the encoder calculates edge histogram and uses most probable direction information. The fast INTER mode selection scheme in JVT-I020 also employs motion detection method and Sobel edge operator. Motion detection method checks motion of macroblock and computes frame difference for macroblock. Sobel edge operator checks homogeneity of macroblock. If a block has many edges, the block is segmented into several blocks. The encoder checks motion and homogeneity of a macroblock, and calculate RD cost for probable mode in order to choose best mode.

Fig. 1 Flowchart of macroblock mode decision under high-complexity mode

## Fast high-complexity mode for intra prediction mode and inter block mode

Under the fast high-complexity mode, macroblock mode decision is made in rate-distortion optimized way but having the computation efficiency in mind. As indicated in Figure 1, inter mode prediction is executed first, and if decided necessary, intra mode prediction follows. To choose the best macroblock mode under the high complexity mode, the encoder needs to calculate the RDcost (rate distortion cost) of every possible mode and chooses the mode having the minimum value. To compute RDcost associated with each mode, same operation of forward and inverse integer transform/quantization, and variable-length coding is repetitively performed. Moreover, although the probability of having intra mode is much less than that of inter modes in inter-coded slices, encoder computes the RDcost of all the possible intra modes at each macroblock simply because the RD optimization technique needs to make sure which mode produces the minimum RDcost. Under the fast high-complexity mode, the computational efficiency is enhanced in two ways. First, in inter-coded slices, it investigates the possibility of early decision of SKIP mode as the final coding mode by checking the skip mode condition first since a macroblock satisfying the skip condition has very high probability of ending up with the SKIP mode as the best coding mode [1,2]. Secondly, it selectively investigates intra coding modes in inter-coded slices after deciding the best inter coding mode [3,4]. In this way, the encoder can avoid quite a number of unfruitful calculation of RDcost values without costing coding performance.

Experimental Results

The video sequences and experimental conditions are shown in Table 1. To evaluate the overall performance, all proposed methods are implemented in JM7.5c encoder and tested under the main profile.

Table 1 Simulation condition

|  |  |  | Exp. 1 | Exp. 2 |
| --- | --- | --- | --- | --- |
| Profile |  |  | Main |  |
| Codec |  |  | JM 7.5c |  |
| MV search range |  |  | 32 |  |
| QP |  |  | 28, 34, 36, 40 |  |
| Number of Reference |  |  | 5 |  |
| Common coding option |  |  | Rate distortion optimization, Hadamard transform CABAC |  |
| PC |  |  | Xeon 2.4GHz(dual CPU), 1GB RAM |  |
| Test sequence | AKIYO | QCIF (100 frames) | IPPP coding | IBBP coding |
|  | SILENT | QCIF (100 frames) |  |  |
|  | FOREMAN | QCIF (100 frames) |  |  |
|  | NEWS | QCIF (100 frames) |  |  |
|  | CONTAINER | QCIF (100 frames) |  |  |
|  | MOBILE | QCIF (100 frames) |  |  |
|  | STEFAN | QCIF (100 frames) |  |  |

Table 4 and 7 shows the experimental results of the fast high-complexity mode. TS indicate the average time saving in encoding process, which is given as below.

Where Time[reference] and Time[proposed] means that the entire encoding time of JM 7.5c reference software and proposed method, respectively.

Table 2 Simulation result for methods [1][2] in Exp 1

|  | TS [%] | BDBR [%] | BDPSNR [dB] |
| --- | --- | --- | --- |
| AKIYO | 25.41 | -0.84 | 0.05 |
| SILENT | 23.33 | -0.54 | 0.03 |
| FOREMAN | 20.21 | 0.09 | 0.00 |
| NEWS | 24.92 | 0.24 | -0.01 |
| CONTAINER | 27.73 | -1.17 | 0.06 |
| MOBILE | 18.44 | -0.30 | 0.01 |
| STEFAN | 17.31 | -0.40 | 0.02 |
| Average | 22.48 | -0.42 | 0.02 |

Table 3 Simulation result for methods [3][4] in Exp 1

|  | TS [%] | BDBR [%] | BDPSNR [dB] |
| --- | --- | --- | --- |
| AKIYO | 23.39 | 2.57 | -0.15 |
| SILENT | 18.68 | 2.02 | -0.10 |
| FOREMAN | 22.91 | 3.36 | -0.18 |
| NEWS | 18.99 | 2.32 | -0.13 |
| CONTAINER | 21.57 | 2.68 | -0.14 |
| MOBILE | 17.32 | 0.76 | -0.04 |
| STEFAN | 19.14 | 0.75 | -0.04 |
| Average | 20.29 | 2.07 | -0.11 |

Table 4 Simulation result for fast-high complexity mode in Exp 1

|  | TS [%] | BDBR [%] | BDPSNR [dB] |
| --- | --- | --- | --- |
| AKIYO | 31.61 | 1.58 | -0.09 |
| SILENT | 28.14 | 2.97 | -0.14 |
| FOREMAN | 29.25 | 2.85 | -0.16 |
| NEWS | 29.29 | 2.32 | -0.13 |
| CONTAINER | 22.40 | 1.79 | -0.10 |
| MOBILE | 22.73 | 0.56 | -0.03 |
| STEFAN | 24.97 | 0.78 | -0.04 |
| Average | 26.91 | 1.84 | -0.10 |

Table 5 Simulation result for methods [1][2] in Exp 2

|  | TS [%] | BDBR [%] | BDPSNR [dB] |
| --- | --- | --- | --- |
| AKIYO | 75.27 | -1.82 | 0.14 |
| SILENT | 68.20 | 0.43 | -0.02 |
| FOREMAN | 56.94 | 1.93 | -0.09 |
| NEWS | 72.24 | -0.30 | 0.01 |
| CONTAINER | 76.18 | -0.74 | 0.06 |
| MOBILE | 51.80 | 2.49 | -0.11 |
| STEFAN | 51.27 | 1.29 | -0.06 |
| Average | 64.56 | 0.47 | -0.01 |

Table 6 Simulation result for methods [3][4] in Exp 2

|  | TS [%] | BDBR [%] | BDPSNR [dB] |
| --- | --- | --- | --- |
| AKIYO | 21.12 | 0.61 | -0.05 |
| SILENT | 18.22 | 3.73 | -0.17 |
| FOREMAN | 18.12 | 3.98 | -0.19 |
| NEWS | 17.94 | 2.86 | -0.15 |
| CONTAINER | 17.76 | 1.96 | -0.16 |
| MOBILE | 11.72 | 0.45 | -0.02 |
| STEFAN | 15.18 | 1.04 | -0.05 |
| Average | 17.15 | 2.09 | -0.11 |

Table 7 Simulation result for fast-high complexity mode in Exp 2

|  | TS [%] | BDBR [%] | BDPSNR [dB] |
| --- | --- | --- | --- |
| AKIYO | 77.83 | -0.89 | 0.05 |
| SILENT | 71.34 | 3.50 | -0.16 |
| FOREMAN | 61.79 | 5.43 | -0.26 |
| NEWS | 73.84 | 1.87 | -0.10 |
| CONTAINER | 78.06 | 1.12 | -0.10 |
| MOBILE | 54.22 | 2.98 | -0.13 |
| STEFAN | 55.07 | 3.59 | -0.18 |
| Average | 67.45 | 2.51 | -0.12 |

In Table 4 and 7, we can see that overall time saving of given methods is about 27% for IPPP case and 67% for IBBP case. It means that H.264 main profile encoder running under fast high-complexity mode is at most 3 times faster than the main profile encoder running under the high-complexity mode. In this case, BDBR and BDPSNR loss are 2.51% and -0.12dB, respectively. Besides, we don’t see any visual artifact from fast-high complexity mode case in compare with conventional high complexity mode case. The result shows that the fast high-complexity mode significantly reduces overall encoding time with only negligible degradation.

## Conclusion

H.264/AVC can choose whether an encoder uses rate distortion optimization method or not. Since RD optimization scheme provides significant coding gain, we cannot easily drop it to reduce complexity. To speed-up the decision process under complex RD optimization, several fast mode decision schemes were adopted. In this information contribution, we showed that the overall performance of the fast high complexity mode  in terms of BDBR and entire encoding time. The experimental results show that it considerably reduces encoding time only with negligible coding loss.

## Reference

[1] B,Jeon and J.Lee, “Fast mode decision for H.264,” ITU-T, Doc. #JVT-J033, 2003

[2] J.Lee, I.Choi, W.Choi and B,Jeon, “ Fast Mode Decision for B slice,” ITU-T, Doc. #JVT-K021

[3] F.Pan, X.Lin, R.Susanto, K.P.Lim, Z.G.Li, G.N.Feng, D.J.Wu and S.Wu, “Fast Mode Decision for Intra Prediction,” ITU-T, Doc. #JVT-G013

[4] K.P.Lim, S.Wu, D.J.Wu, S.Rahardja, X.Lin, F.Pan and Z.G.Li, “Fast INTER Mode Selection,” ITU-T, Doc. #JVT-I020

(Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name |  |  |  |
| Mailing address |  |  |  |
| Country |  |  |  |
| Contact person |  |  |  |
| Telephone |  |  |  |
| Fax |  |  |  |
| Email |  |  |  |
| Place and date of submission |  |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  |  |  |
| Title |  |  |  |
| Contribution number |  |  |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: