| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 14th Meeting: Hong Kong, PRC China 17-21 Jan., 2005 | Document: JVT-N013 Filename: input.doc |
| --- | --- |

| Title: | The Fast Mode Decision with Fast Motion Estimation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Inchoon Choi School of ECE, SungKyunKwan Univ. 300 Chunchun-dong, Suwon, Korea Woong Il Choi School of ECE, SungKyunKwan Univ. 300 Chunchun-dong, Suwon, Korea Jeyun Lee School of ECE, SungKyunKwan Univ. 300 Chunchun-dong, Suwon, Korea Byeungwoo Jeon School of ECE, SungKyunKwan Univ. 300 Chunchun-dong, Suwon, Korea | Tel: Email: | +82-31-290-7186 sonne@ece.skku.ac.kr +82-31-290-7186 creata@ece.skku.ac.kr +82-31-290-7186 jeyun22@ece.skku.ac.kr +82-31-290-7144 bjeon@yurim.skku.ac.kr |
| Source: | Sung Kyun Kwan University |  |  |

_____________________________

## Abstract

JVT test model adopted several fast mode decision algorithms such as JVT-G013 [3], JVG-I020 [4], JVT-J033 [1], and JVT-K021 [2]. In Palma meeting [5], their performance was evaluated in terms of speedup and PSNR. In this document, we evaluate performance of fast high-complexity mode decision algorithm with fast motion estimation algorithm and show comparison with the low complexity mode. We integrate to the latest JVT reference software, all fast mode decision methods that were adopted in the JVT test model and compared their performances. The simulation results show that the fast high-complexity mode with fast motion estimation provides better performance than the low complexity mode.

## Fast high-complexity mode

Under the fast high-complexity mode, macroblock mode decision is made in rate-distortion optimized way but having computation efficiency in mind. As indicated in Figure 1, inter mode prediction is executed first, and if decided necessary, intra mode prediction follows. To choose the best macroblock mode under the high complexity mode, encoder needs to calculate the RDcost (rate distortion cost) of every possible mode and chooses the mode having the minimum value. To compute RDcost associated with each mode, same operation of forward and inverse integer transform/quantization, and variable-length coding is repetitively performed. Moreover, although the probability of having intra mode is much less than that of inter modes in inter-coded slices, encoder computes the RDcost of all the possible intra modes at each macroblock simply because the RD optimization technique needs to make sure which mode produces the minimum RDcost. Under the fast high-complexity mode, the computational efficiency is enhanced in two ways. First, in inter-coded slices, it investigates the possibility of early decision of SKIP mode as the final coding mode by checking the skip mode condition first since a macroblock satisfying the skip condition has very high probability of ending up with the SKIP mode as the best coding mode [1,2]. Secondly, it selectively investigates intra coding modes in inter-coded slices after deciding the best inter coding mode [3,4]. In this way, the encoder can avoid quite a number of unfruitful calculation of RDcost values without costing coding performance.

Figure 1. Macroblock Mode Decision under Fast High Complexity mode

## Fast high-complexity mode with Fast Motion Estimation

To further accelerate the processing time of fast high-complexity mode, the fast motion estimation method implemented in JM9.2 was also used in addition to the fast high-complexity mode. The execution time saving of the fast high-complexity mode with fast motion estimation was compared with that of the low complexity mode. Note that in low-complexity mode, the Rate Distortion Optimization (RDO) is turned off, and the cost of each mode is computed using some biases and Sum of Absolute Difference (SAD) of either the prediction errors or the Hadamard transformed coefficients of the difference.

Experimental Results

The video sequences and experimental conditions are shown in Table 1. To evaluate the overall performance (speedup and BDBR/BDPSNR), the fast methods in J-033[1] and K-021[2] are implemented in JM9.2 encoder with the fast motion estimation method implemented in JM9.2 and tested under the main profile.

Table 1 Simulation condition

|  |  |  | Exp. 1 | Exp. 2 |
| --- | --- | --- | --- | --- |
| Profile |  |  | Main |  |
| Codec |  |  | JM 9.5 |  |
| MV search range |  |  | 32 |  |
| QP |  |  | 28, 34, 36, 40 |  |
| Number of Reference |  |  | 5 |  |
| Common coding option |  |  | Rate distortion optimization, Hadamard transform CABAC |  |
| PC |  |  | Xeon 2.4GHz(dual CPU), 1GB RAM |  |
| Test sequence | AKIYO | QCIF (100 frames) | IPPP coding | IBBP coding |
|  | SILENT | QCIF (100 frames) |  |  |
|  | FOREMAN | QCIF (100 frames) |  |  |
|  | NEWS | QCIF (100 frames) |  |  |
|  | CONTAINER | QCIF (100 frames) |  |  |
|  | MOBILE | QCIF (100 frames) |  |  |
|  | STEFAN | QCIF (100 frames) |  |  |

Table 2 and 5 shows the experimental results of the low complexity mode and other Tables show results of the fast-high complexity with or without fast motion estimation. TS indicate the average time saving in encoding process, which is given as below.

Where Time[reference] and Time[proposed] mean respectively the entire encoding times of JM 9.2 reference software and the proposed method (fast high-complexity mode+fast ME).

Table 2 Performance result of low complexity mode w/ FME in Exp 1

|  | TS [%] | BDBR [%] | BDPSNR [dB] |
| --- | --- | --- | --- |
| AKIYO | 88.27 | 0.44 | -0.02 |
| SILENT | 87.96 | 5.78 | -0.28 |
| FOREMAN | 82.94 | 5.76 | -0.31 |
| NEWS | 81.77 | 5.08 | -0.26 |
| CONTAINER | 87.17 | 5.36 | -0.31 |
| MOBILE | 85.96 | 4.37 | -0.21 |
| STEFAN | 81.15 | 7.08 | -0.39 |
| Average | 85.03 | 4.84 | -0.25 |

Table 3 Performance result of fast-high complexity mode w/o FME in Exp 1

|  | TS [%] | BDBR [%] | BDPSNR [dB] |
| --- | --- | --- | --- |
| AKIYO | 25.15 | -1.25 | 0.07 |
| SILENT | 25.84 | 0.74 | -0.04 |
| FOREMAN | 15.01 | 0.54 | -0.03 |
| NEWS | 11.42 | 0.00 | 0.00 |
| CONTAINER | 26.29 | 0.24 | -0.02 |
| MOBILE | 21.23 | 0.62 | -0.03 |
| STEFAN | 10.61 | 1.11 | -0.06 |
| Average | 19.37 | 0.28 | -0.02 |

Table 4 Performance result of fast-high complexity mode w/ FME in Exp 1

|  | TS [%] | BDBR [%] | BDPSNR [dB] |
| --- | --- | --- | --- |
| AKIYO | 92.41 | -1.27 | 0.07 |
| SILENT | 90.86 | 1.56 | -0.08 |
| FOREMAN | 79.44 | 2.23 | -0.12 |
| NEWS | 74.73 | 0.37 | -0.02 |
| CONTAINER | 89.63 | 1.00 | -0.06 |
| MOBILE | 87.21 | 1.07 | -0.05 |
| STEFAN | 74.15 | 0.54 | -0.03 |
| Average | 84.06 | 0.79 | -0.04 |

Table 5 Performance result of of low complexity mode w/ FME in Exp 2

|  | TS [%] | BDBR [%] | BDPSNR [dB] |
| --- | --- | --- | --- |
| AKIYO | 87.98 | 11.09 | -0.80 |
| SILENT | 87.97 | 8.78 | -0.60 |
| FOREMAN | 83.11 | 31.32 | -1.41 |
| NEWS | 82.25 | 32.89 | -1.26 |
| CONTAINER | 87.29 | 26.04 | -1.31 |
| MOBILE | 85.90 | 28.85 | -1.18 |
| STEFAN | 81.50 | 23.85 | -1.12 |
| Average | 85.14 | 23.26 | -1.10 |

Table 6 Performance result of fast-high complexity mode w/o FME in Exp 2

|  | TS [%] | BDBR [%] | BDPSNR [dB] |
| --- | --- | --- | --- |
| AKIYO | 76.80 | -2.58 | 0.20 |
| SILENT | 77.09 | -0.37 | 0.02 |
| FOREMAN | 57.19 | 1.64 | -0.08 |
| NEWS | 54.99 | 2.19 | -0.10 |
| CONTAINER | 72.69 | 1.01 | -0.06 |
| MOBILE | 67.75 | 0.51 | -0.02 |
| STEFAN | 46.50 | 2.17 | -0.11 |
| Average | 64.72 | 0.65 | -0.02 |

Table 7 Performance result of fast-high complexity mode w/ FME in Exp 2

|  | TS [%] | BDBR [%] | BDPSNR [dB] |
| --- | --- | --- | --- |
| AKIYO | 97.25 | -2.32 | 0.18 |
| SILENT | 96.81 | 0.56 | -0.05 |
| FOREMAN | 89.01 | 3.28 | -0.17 |
| NEWS | 87.28 | 2.59 | -0.11 |
| CONTAINER | 95.22 | 1.25 | -0.07 |
| MOBILE | 93.57 | 0.70 | -0.03 |
| STEFAN | 84.16 | 3.34 | -0.17 |
| Average | 91.90 | 1.34 | -0.06 |

In Table 2 and 7, we can see that overall time saving of given methods with FME is about 84% for IPPP case and 92% for IBBP case. In this case, BDBR and BDPSNR loss are 1.34% and -0.06dB, respectively. We can also see that fast high-complexity mode with Fast ME provides much better result than the low complexity mode with FME. The fast high-complexity mode significantly reduces overall encoding time with only negligible degradation. In this experiment, G013[3] and I020[4] algorithms are not tested, because implementation of the algorithms are not yet finished.

## Conclusion

In this contribution, we showed that the overall performance of the fast-high complexity mode with FME is much better than the low complexity mode in terms of BDBR and entire encoding time. The experimental results show that it considerably reduces encoding time only with negligible coding loss and shows better performance than low complexity mode in terms of encoding time and quality.

## Reference

[1] B,Jeon and J.Lee, “Fast mode decision for H.264,” ITU-T, Doc. #JVT-J033, 2003

[2] J.Lee, I.Choi, W.Choi and B,Jeon, “ Fast Mode Decision for B slice,” ITU-T, Doc. #JVT-K021

[3] F.Pan, X.Lin, R.Susanto, K.P.Lim, Z.G.Li, G.N.Feng, D.J.Wu and S.Wu, “Fast Mode Decision for Intra Prediction,” ITU-T, Doc. #JVT-G013

[4] K.P.Lim, S.Wu, D.J.Wu, S.Rahardja, X.Lin, F.Pan and Z.G.Li, “Fast INTER Mode Selection,” ITU-T, Doc. #JVT-I020

[5] I.Choi, J.Lee, W.Choi and B.Jeon, “Performance evaluation of fast mode decision algorithms for H.264,” ITU-T, Doc. #JVT-M01

(Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name |  |  |  |
| Mailing address |  |  |  |
| Country |  |  |  |
| Contact person |  |  |  |
| Telephone |  |  |  |
| Fax |  |  |  |
| Email |  |  |  |
| Place and date of submission |  |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  |  |  |
| Title |  |  |  |
| Contribution number |  |  |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: