| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 14th Meeting: Hong Kong, CN, 17-21 January, 2005 | Document: JVT-N045 Filename: input.doc |
| --- | --- |

| Title: | Draft Text of FRExt Conformance |  |  |
| --- | --- | --- | --- |
| Status: | Output from JVT |  |  |
| Purpose: | Draft |  |  |
| Author(s) or Contact(s): | Teruhiko Suzuki 6-7-35, Kitashinagawa, Shinagawa-ku Tokyo, 141-0001, Japan | Tel: Email: | +81-3-5795-7966 teruhiko@av.crl.sony.co.jp |
| Source: | Sony Corp. |  |  |

_____________________________

TWIN  TEXT  PUBLICATION

AVC FIDELITY RANGE EXTENSIONS CONFORMANCE

ITU-T DRAFT NEW RECOMMENDATION H.264.1

ISO/IEC 14496-4:200X PDAM 9

# Foreword

This Recommendation | International Standard establishes conformance test requirements for conformance to ITU-T Rec. H.264 | ISO/IEC 14496-10.

# Introduction

In this document, the additional text to ITU-T Rec. H.264.1 | ISO/IEC 14496-4 is specified for the conformance test of ITU-T Rec. H.264 | ISO/IEC 14496-10:2004 video decoders. This specification is specifically applied to ITU-T Rec. H.264 | ISO/IEC 14496-10, Advanced Video Coding.

The following subclauses specify the normative tests for verifying conformance of ITU-T Rec. H.264 | 14496-10 video bitstreams and video decoders. Those normative tests make use of test data (bitstream test suites) provided as an electronic annex to this document, and of the reference software decoder specified in ITU-T Rec. H.264.2 | ISO/IEC 14496-5 with source code available in electronic format.

The numbering in this fidelity range extensions conformance amendment text is relative to the text of ITU-T Rec. H.264.1 | ISO/IEC 14496-4. When a numbered item (i.e., a clause, subclause, figure, table, or equation) or associated content is being replaced or modified, the same number is used for the modified numbered item. When a numbered item is inserted between prior numbered items, the number of the corresponding numbered item immediately preceding it is used and the letter 'a' is appended to this number. When, after this one such inserted numbered item, another numbered item is inserted, the letter 'a' is replaced by the letter 'b' to indicate their relative order, and so on, following ordinary English alphabetical order. If text integrating this amendment with ITU-T Rec. H.264.1 | ISO/IEC 14496-4 is produced, the inserted numbered items are to be assigned the corresponding number in their numerical order without any such letters, and any subsequent numbered items are to be assigned later numbers to avoid conflicts. The purpose of the numbering convention in this amendment text is to avoid renumbering of numbered items in ITU-T Rec. H.264 | ISO/IEC 14496-4 while drafting this amendment. Therefore, if the addition of a numbered item does not require renumbering of numbered items in ITU-T Rec. H.264 | ISO/IEC 14496-4, the final number is assigned to the numbered item herein.

INTERNATIONAL  STANDARD

ISO/IEC 14496-4 :200X / Amendment 9 : (2004 E)

ITU-T Rec. H.264.1 Amendment 1(2004 E)

ITU-T  RECOMMENDATION

AVC FIDELITY RANGE EXTENSIONS
CONFORMANCE

# 1)	Subclause 2.3 “Paired Recommendations | International Standards equivalent in technical content”

Objective: Correct normative reference, reflecting Fidelity Range Extensions

- Replace Section 2.3as follows,

“

## 2.3	Paired Recommendations | International Standards equivalent in technical content

–	ITU-T Recommendation H.264 (2004), Advanced video coding for generic audiovisual services.

ISO/IEC 14496-10, Information technology – Coding of audiovisual objects – Part 10: Advanced video coding.

–	ITU-T Recommendation H.264.2 (2004), Advanced video coding reference software.

ISO/IEC 14496-5:2001, Information technology – Coding of audiovisual objects – Part 5: Reference software.

“

# 2)	Subclause 3 “Definitions”

Objective: Correct definition of reference software, reflecting Fidelity Range Extensions

- Replace the definition of reference software as follows,

“

  - reference software decoder: The software decoders contained in ITU-T Rec. H.264.2 | ISO/IEC 14496-5.

“

# 3)	Subclause 6.1 “Introduction”

Objective: Correct the normative reference, reflecting Fidelity Range Extensions

- Replace Section 6.1 as follows,

- “

## 6.1	Introduction

The following subclauses specify the normative tests for verifying conformance of video bitstreams as well as decoders. Those normative tests make use of test data (bitstream test suites) provided as an electronic annex to this document, and the reference software decoder specified in ITU-T Rec. H.264.2 | ISO/IEC 14496-5 with source code included in electronic format.

“

# 4)	Subclause 6.5.7 “Decoder conformance test of a particular profile and level”

Objective: Conformance with existing profiles

- Replace Section 6.5.7,as follows,

- “

### 6.1.1	Decoder conformance test of a particular profile-and-level

In order for a decoder of a particular profile-and-level to claim output order conformance to the standard as described by this Specification, the decoder shall successfully pass the static test defined in subclause Error: Reference source not found with all the bitstreams of the normative test suite specified for testing decoders of this particular profile-and-level.

In order for a decoder of a particular profile and level to claim output timing conformance to the standard as described by this Specification, the decoder shall successfully pass both the static test defined in subclause Error: Reference source not found and the dynamic test defined in subclause Error: Reference source not found with all the bitstreams of the normative test suite specified for testing decoders of this particular profile-and-level. 2 and Table 2 define the normative test suites for each profile-and-level combination. The test suite for a particular profile-and-level combination is the list of bitstreams that are marked with an ‘X’ in the column corresponding to that profile-and-level combination.

‘X’ indicates that the bitstream is designed to test both the dynamic and static conformance of the decoder.

The bitstream specification indicates the test bitstream specification used for each bitstream.

A decoder compliant with High, High 10, High 4:2:2, or High 4:4:4 shall be capable of decoding Main profile bitstreams. In addition to the streams defined in Table 2, a compliant decoder shall decode Main profile streams in Table 1.

“

# 5)	Subclauses 6.6 “Specification of the test bitstreams”

Objective: Add the reference to Table-2

- Replace the following sentence in 6.6 ,

- “

Some characteristics of each bitstream listed in 2 are described in the subclauses of this subclause.  In 2, the value "29.97" shall be interpreted as an approximation of an exact value of 30000 ÷ 1001.

“

with

“

Some characteristics of each bitstream listed in 2 and Error: Reference source not found are described in the subclauses of this subclause.  In 2 and Error: Reference source not found, the value "29.97" shall be interpreted as an approximation of an exact value of 30000 ÷ 1001.

“

# 6)	Subclauses 6.6.3 “Test Bitstreams – Memory management control operation ”

Objective: Specification of new test bitstreams

- Add the following after Section 6.6.3.7,

- “

#### 6.6.3.8	Test bitstream #AVCMR-8, #AVCMR-9

Specification:  All slices are coded as I or P slices. Each picture contains more than one slice. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 1. Reference picture list reordering and memory management control operations are used. direct_8x8_inference_flag is equal to 1. Each slice is a coded field. VUI is included in the bitstream. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Reference picture list reordering and memory management control operations.

Purpose:  Check that decoder handles reference picture list reordering and memory management control operations.

#### 6.6.3.9	Test bitstream #AVCMR-10

Specification:  All slices are coded as I, P or B slices. Each picture contains more than one slice. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 1. Reference picture list reordering and memory management control operations are used. Temporal direct prediction is used for direct prediction. direct_8x8_inference_flag is equal to 1. Each slice is a coded field. VUI is included in the bitstream. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Reference picture list reordering and memory management control operations.

Purpose:  Check that decoder handles reference picture list reordering and memory management control operations.

“

# 7)	Subclauses 6.6.3 “Test Bitstreams – CABAC: Memory management control operation ”

Objective: Specification of new test bitstreams

- Add the following after Section 6.6.15.3 ,

- “

### 6.6.16	Test Bitstreams – CABAC: Memory management control operation

#### 6.6.16.1	Test bitstream #AVCCAMR-1

Specification:  All slices are coded as I, P or B slices. Each picture contains more than one slice. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 1. Reference picture list reordering and memory management control operations are used. Temporal direct prediction is used for direct prediction. direct_8x8_inference_flag is equal to 1. Each slice is a coded frame.  mb_adaptive_frame_field_coding equal to 1. VUI is included in the bitstream. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Reference picture list reordering and memory management control operations.

Purpose:  Check that decoder handles reference picture list reordering and memory management control operations.

“

Numbers of sub-sub-section after 6.6.16 are incremented according to this addition of new sub-sub-section.

# 8)	New Subclauses 6.6.20 “Test Bitstreams – Fidelity Range Extensions: 4:2:0 8 bit”, 6.6.21 “Test Bitstreams – Fidelity Range Extensions: 4:2:0 10 bit”, 6.6.22 “Test Bitstreams – Fidelity Range Extensions: 4:2:2 10 bit”, and 6.6.23 “Test Bitstreams – Fidelity Range Extensions: 4:4:4 12 bit”

Objective: Specification of new test bitstreams

- Add the following after Section 6.6.19.18,

- “

[Ed.Note: The bitstreams in this subclause are temporarily defined. The specification of each bitstream is the same as the bitstream verified through bitstream exchange activity. The bitstreams verified through bitstream exchange activity are available at the following ftp site.

ftp:// standards.polycom.com/jvt-experts/draft_conformance/FRExt

]

### 6.6.20	Test bitstreams – Fidelity Range Extensions: 4:2:0 8 bit

#### 6.6.20.1	Test bitstream #FREH-1, #FREH-28

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 0. Spatial direct prediction is used for direct prediction. direct_8x8_inference_flag is equal to 0. Transform mode is set to 8x8 block size only. seq_scaling_matrix_present_flag and pic_scaling_matrix_flag are set to 1. Scaling lists are included in the sequence parameter set and the picture parameter set. Each slice is a coded frame. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

- Functional stage:  Tests loading of scaling list in the sequence parameter set and the picture parameter set. Tests 8x8 block size transform mode. Tests decoding of level prefix more than 16 bits in CAVLC entropy coding. Tests deblocking for 8x8 transform.

Purpose:  Check that a decoder can properly decode slices of coded frames with 8x8 block size transform for CAVLC and check that scaling list is implemented correctly for frame only coding.

#### 6.6.20.2	Test bitstream #FREH-2, #FREH-29

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. Temporal direct prediction is used for direct prediction. direct_8x8_inference_flag is set equal to 0. Both 4x4 and 8x8 block size transform modes are used. seq_scaling_matrix_present_flag and pic_scaling_matrix_flag are set to 1. Scaling lists are included in the sequence parameter set and the picture parameter set. Each slice is a coded frame. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Tests 4x4 and 8x8 block size transform modes in CABAC entropy coding. Tests loading of scaling list in the sequence parameter set and the picture parameter set. Tests deblocking for 4x4 and 8x8 transform.

Purpose:  Check that a decoder can properly decode slices of coded frames with both 4x4 and 8x8 block size transform modes and check that scaling list is implemented correctly for CABAC entropy coding for frame only coding.

#### 6.6.20.3	Test bitstream #FREH-3, #FREH-30

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. The value of cabac_init_idc is adaptively changed in slice header. Spatial direct prediction is used for direct prediction. direct_8x8_inference_flag is set equal to 1. Both 4x4 and 8x8 block size transform modes are used. seq_scaling_matrix_present_flag is set to 1 and default scaling lists are used. Each slice is a coded frame. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Tests 4x4 and 8x8 block size transform modes in CABAC entropy coding.

Purpose:  Check that a decoder can properly decode slices of coded frames and fields with both 4x4 and 8x8 block size transform modes.

#### 6.6.20.4	Test bitstream #FREH-4, #FREH-31

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. The value of cabac_init_idc is adaptively changed in slice header.  pic_order_cnt_type is equal to 0. Spatial direct prediction is used for direct prediction. direct_8x8_inference_flag is set equal to 1. Both 4x4 and 8x8 block size transform modes are used. seq_scaling_matrix_present_flag is set to 1 and default scaling lists are used. Each slice is either a coded frame or a coded field. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Tests 4x4 and 8x8 block size transform modes in CABAC entropy coding.

Purpose:  Check that a decoder can properly decode slices of coded frames and fields with both 4x4 and 8x8 block size transform modes.

#### 6.6.20.5	Test bitstream #FREH-5, #FREH-32

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. The value of cabac_init_idc is adaptively changed in slice header.  pic_order_cnt_type is equal to 0. Spatial direct prediction is used for direct prediction. direct_8x8_inference_flag is set equal to 1. Both 4x4 and 8x8 block size transform modes are used. seq_scaling_matrix_present_flag is set to 1 and default scaling lists are used. Each slice is a coded frame. mb_adaptive_frame_field_coding equal to 1. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Macroblock adaptive frame field decoding and slices of a coded frame with both 4x4 and 8x8 block size transform modes in CABAC entropy coding.

Purpose:  Check that a decoder can properly decode slices of coded frames with mb_adaptive_frame_field_flag=1 and with both 4x4 and 8x8 block size transform modes.

#### 6.6.20.6	Test bitstream #FREH-6, #FREH-33

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 0. Temporal direct prediction is used for direct prediction. direct_8x8_inference_flag is equal to 1. Transform mode is set to 8x8 block size only. seq_scaling_matrix_present_flag and pic_scaling_matrix_flag are set to 1. Scaling lists are included in the sequence parameter set and the picture parameter set. Each slice is either a coded frame or a coded field. mb_adaptive_frame_field_coding equal to 1 in coded frames. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

- Functional stage:  Tests loading of scaling list in the sequence parameter set and the picture parameter set. Tests 8x8 block size transform mode. Tests decoding of level prefix more than 16 bits in CAVLC entropy coding. Tests deblocking for 8x8 transform.

Purpose:  Check that a decoder can properly decode slices of coded frames with 8x8 block size transform for CAVLC and check that scaling list is implemented correctly for both slices of a coded frame with mb_adaptive_frame_field_flag=1 and slices of a coded field.

#### 6.6.20.7	Test bitstream #FREH-7, #FREH-34

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. Temporal direct prediction is used for direct prediction. direct_8x8_inference_flag is set equal to 1. Both 4x4 and 8x8 block size transform modes are used. seq_scaling_matrix_present_flag and pic_scaling_matrix_flag are set to 1. Scaling lists are included in the sequence parameter set and the picture parameter set. Each slice is either a coded frame or a coded field. mb_adaptive_frame_field_coding equal to 1 in coded frames. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Tests 4x4 and 8x8 block size transform modes in CABAC entropy coding. Tests loading of scaling list in the sequence parameter set and the picture parameter set. Tests deblocking for 4x4 and 8x8 transform.

Purpose:  Check that a decoder can properly decode slices of coded frames with both 4x4 and 8x8 block size transform modes and check that scaling list is implemented correctly for CABAC entropy coding for both slices of a coded frame with mb_adaptive_frame_field_flag=1 and slices of a coded field.

#### 6.6.20.8	Test bitstream #FREH-8

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. disable_deblocking_filter_idc is equal to 1, specifying disabling of the deblocking filter process. pic_order_cnt_type is equal to 0. Temporal direct prediction is used for direct prediction. direct_8x8_inference_flag is set equal to 1. Both 4x4 and 8x8 block size transform modes are used. Both seq_scaling_matrix_present_flag and pic_scaling_matrix is set to 0. Each slice is a coded frame. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Tests 4x4 and 8x8 block size transform modes in CABAC entropy coding.

Purpose:  Check that a decoder can properly decode slices of coded frames with both 4x4 and 8x8 block size transform modes.

#### 6.6.20.9	Test bitstream #FREH-9

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. Temporal direct prediction is used for direct prediction. direct_8x8_inference_flag is set equal to 1. Both 4x4 and 8x8 block size transform modes are used. Both seq_scaling_matrix_present_flag and pic_scaling_matrix is set to 0. Each slice is a coded frame. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Tests 4x4 and 8x8 block size transform modes in CABAC entropy coding.

Purpose:  Check that a decoder can properly decode slices of coded frames with both 4x4 and 8x8 block size transform modes.

#### 6.6.20.10	Test bitstream #FREH-10

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. disable_deblocking_filter_idc is equal to 1, specifying disabling of the deblocking filter process. pic_order_cnt_type is equal to 0. Temporal direct prediction is used for direct prediction. direct_8x8_inference_flag is set equal to 1. Both 4x4 and 8x8 block size transform modes are used. Both seq_scaling_matrix_present_flag and pic_scaling_matrix is set to 0. Each slice is a coded field. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Tests 4x4 and 8x8 block size transform modes in CABAC entropy coding.

Purpose:  Check that a decoder can properly decode slices of coded fields with both 4x4 and 8x8 block size transform modes.

#### 6.6.20.11	Test bitstream #FREH-11

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. Temporal direct prediction is used for direct prediction. direct_8x8_inference_flag is set equal to 1. Both 4x4 and 8x8 block size transform modes are used. Both seq_scaling_matrix_present_flag and pic_scaling_matrix is set to 0. Each slice is a coded field. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Tests 4x4 and 8x8 block size transform modes in CABAC entropy coding.

Purpose:  Check that a decoder can properly decode slices of coded fields with both 4x4 and 8x8 block size transform modes.

#### 6.6.20.12	Test bitstream #FREH-12, #FREH-39

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. Temporal direct prediction is used for direct prediction. direct_8x8_inference_flag is set equal to 0. Both 4x4 and 8x8 block size transform modes are used. seq_scaling_matrix_present_flag is set to 1 and default scaling lists are used. Each slice is a coded frame. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Tests 4x4 and 8x8 block size transform modes in CABAC entropy coding.

Purpose:  Check that a decoder can properly decode slices of coded frames with both 4x4 and 8x8 block size transform modes.

#### 6.6.20.13	Test bitstream #FREH-13, #FREH-14, #FRE-15

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. Temporal direct prediction is used for direct prediction. direct_8x8_inference_flag is set equal to 1. Both 4x4 and 8x8 block size transform modes are used. seq_scaling_matrix_present_flag is set to 1 and default scaling lists are used. Each slice is a coded frame. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Tests 4x4 and 8x8 block size transform modes in CABAC entropy coding.

Purpose:  Check that a decoder can properly decode slices of coded frames with both 4x4 and 8x8 block size transform modes.

#### 6.6.20.14	Test bitstream #FREH-16

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. Temporal direct prediction is used for direct prediction. direct_8x8_inference_flag is equal to 1. Both 4x4 and 8x8 block size transform modes are used. seq_scaling_matrix_present_flag is set to 1. Scaling lists are included in the sequence parameter set. Each slice is a coded frame. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

- Functional stage:  Tests loading of scaling list in the sequence parameter set. Tests 8x8 block size transform mode.

- Purpose:  Check that a decoder can properly decode slices of a coded frame with 8x8 block size transform for CABAC. Check that scaling list is implemented correctly for frame only coding. Check that a decoder can handle temporal direct mode with direct_inference_flag=1 for coded frames with 8x8 block size transform.

#### 6.6.20.15	Test bitstream #FREH-17

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. Temporal direct prediction is used for direct prediction. direct_8x8_inference_flag is equal to 1. Both 4x4 and 8x8 block size transform modes are used. seq_scaling_matrix_present_flag is set to 1. Scaling lists are included in the sequence parameter set. Each slice is either a coded frame or a coded field. mb_adaptive_frame_field_coding equal to 1 in coded frames. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

- Functional stage:  Tests loading of scaling list in the sequence parameter set. Tests 8x8 block size transform mode.

- Purpose:  Check that a decoder can properly decode slices of a coded frame with 8x8 block size transform for CABAC. Check that scaling list is implemented correctly for field coding and MBAFF. Check that a decoder can handle temporal direct mode with direct_inference_flag=1 for coded frames with 8x8 block size transform.

#### 6.6.20.16	Test bitstream #FREH-18

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 0. Temporal direct prediction is used for direct prediction. direct_8x8_inference_flag is set equal to 1. Both 4x4 and 8x8 block size transform modes are used. Both seq_scaling_matrix_present_flag and pic_scaling_matrix is set to 0. Each slice is a coded frame. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Tests 4x4 and 8x8 block size transform modes in CAVLC entropy coding.

Purpose:  Check that a decoder can properly decode slices of coded frames with both 4x4 and 8x8 block size transform modes.

#### 6.6.20.17	Test bitstream #FREH-19

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. disable_deblocking_filter_idc is equal to 1, specifying disabling of the deblocking filter process. pic_order_cnt_type is equal to 0. Temporal direct prediction is used for direct prediction. direct_8x8_inference_flag is set equal to 1. Both 4x4 and 8x8 block size transform modes are used. Both seq_scaling_matrix_present_flag and pic_scaling_matrix is set to 0. Each slice is a coded frame. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Tests 4x4 and 8x8 block size transform modes in CAVLC entropy coding.

Purpose:  Check that a decoder can properly decode slices of coded frames with both 4x4 and 8x8 block size transform modes.

#### 6.6.20.18	Test bitstream #FREH-20

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. disable_deblocking_filter_idc is equal to 1, specifying disabling of the deblocking filter process. pic_order_cnt_type is equal to 0. Temporal direct prediction is used for direct prediction. direct_8x8_inference_flag is set equal to 1. Both 4x4 and 8x8 block size transform modes are used. Both seq_scaling_matrix_present_flag and pic_scaling_matrix is set to 0. Each slice is a coded field. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Tests 4x4 and 8x8 block size transform modes in CAVLC entropy coding.

Purpose:  Check that a decoder can properly decode slices of coded fields with both 4x4 and 8x8 block size transform modes.

#### 6.6.20.19	Test bitstream #FREH-21

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 0. Temporal direct prediction is used for direct prediction. direct_8x8_inference_flag is set equal to 1. Both 4x4 and 8x8 block size transform modes are used. Both seq_scaling_matrix_present_flag and pic_scaling_matrix is set to 0. Each slice is a coded field. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Tests 4x4 and 8x8 block size transform modes in CAVLC entropy coding.

Purpose:  Check that a decoder can properly decode slices of coded fields with both 4x4 and 8x8 block size transform modes.

#### 6.6.20.20	Test bitstream #FREH-22

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 0. Spatial direct prediction is used for direct prediction. direct_8x8_inference_flag is set equal to 1. Both 4x4 and 8x8 block size transform modes are used. seq_scaling_matrix_present_flag is set to 1 and default scaling lists are used. Each slice is a coded field. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Tests 4x4 and 8x8 block size transform modes in CAVLC entropy coding.

Purpose:  Check that a decoder can properly decode slices of coded fields with both 4x4 and 8x8 block size transform modes.

#### 6.6.20.21	Test bitstream #FREH-23

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 0. Spatial direct prediction is used for direct prediction. direct_8x8_inference_flag is set equal to 1. Both 4x4 and 8x8 block size transform modes are used. seq_scaling_matrix_present_flag is set to 1 and default scaling lists are used. Each slice is either a coded frame or a coded field. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Tests 4x4 and 8x8 block size transform modes in CAVLC entropy coding.

Purpose:  Check that a decoder can properly decode slices of coded fields with both 4x4 and 8x8 block size transform modes.

#### 6.6.20.22	Test bitstream #FREH-24

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 0. Spatial direct prediction is used for direct prediction. direct_8x8_inference_flag is set equal to 1. Both 4x4 and 8x8 block size transform modes are used. seq_scaling_matrix_present_flag is set to 1 and default scaling lists are used. mb_adaptive_frame_field_coding equal to 1. Each slice is a coded frame. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Tests 4x4 and 8x8 block size transform modes in CAVLC entropy coding.

Purpose:  Check that a decoder can properly decode slices of coded fields with both 4x4 and 8x8 block size transform modes.

#### 6.6.20.23	Test bitstream #FREH-25

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 0. Temporal direct prediction is used for direct prediction. direct_8x8_inference_flag is set equal to 1. Both 4x4 and 8x8 block size transform modes are used. seq_scaling_matrix_present_flag is set to 1. Scaling lists are included in the sequence parameter set. Each slice is a coded frame. chroma_format_idc is equal to 0, specifying monochrome chroma format. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Tests monochrome chroma format in CAVLC entropy coding.

Purpose:  Check that a decoder can properly decode slices of coded frame for monochrome chroma format.

#### 6.6.20.24	Test bitstream #FREH-26

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. Temporal direct prediction is used for direct prediction. direct_8x8_inference_flag is set equal to 1. Both 4x4 and 8x8 block size transform modes are used. seq_scaling_matrix_present_flag is set to 0. Scaling lists are included in the sequence parameter set. Each slice is a coded frame. chroma_format_idc is equal to 1, specifying monochrome chroma format. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Tests monochrome chroma format in CABAC entropy coding.

Purpose:  Check that a decoder can properly decode slices of coded frame for monochrome chroma format.

#### 6.6.20.25	Test bitstream #FREH-27

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. Temporal direct prediction is used for direct prediction. direct_8x8_inference_flag is set equal to 1. Both 4x4 and 8x8 block size transform modes are used. seq_scaling_matrix_present_flag is set to 1. Scaling lists are included in the sequence parameter set. Each slice is a coded frame. second_chroma_qp_index_offset is equal to 2. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Tests second_chroma_qp_index_offset.

Purpose:  Check that a decoder can properly decode slices of coded frame with second_chroma_qp_index_offset.

#### 6.6.20.26	Test bitstream #FREH-35

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. disable_deblocking_filter_idc is equal to 1, specifying disabling of the deblocking filter process. pic_order_cnt_type is equal to 0. Temporal direct prediction is used for direct prediction. direct_8x8_inference_flag is set equal to 1. Both 4x4 and 8x8 block size transform modes are used. Both seq_scaling_matrix_present_flag and pic_scaling_matrix_flag are set to 1. Scaling lists are included in the sequence parameter set and the picture parameter set. Each slice is a coded frame. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Tests 4x4 and 8x8 block size transform modes in CABAC entropy coding.

Purpose:  Check that a decoder can properly decode slices of coded frames with both 4x4 and 8x8 block size transform modes.

#### 6.6.20.27	Test bitstream #FREH-36

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. Temporal direct prediction is used for direct prediction. direct_8x8_inference_flag is set equal to 1. Both 4x4 and 8x8 block size transform modes are used. Both seq_scaling_matrix_present_flag and pic_scaling_matrix_flag are set to 1. Scaling lists are included in the sequence parameter set and the picture parameter set. Each slice is a coded frame. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Tests 4x4 and 8x8 block size transform modes in CABAC entropy coding.

Purpose:  Check that a decoder can properly decode slices of coded frames with both 4x4 and 8x8 block size transform modes.

#### 6.6.20.28	Test bitstream #FREH-37

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. disable_deblocking_filter_idc is equal to 1, specifying disabling of the deblocking filter process. pic_order_cnt_type is equal to 0. Temporal direct prediction is used for direct prediction. direct_8x8_inference_flag is set equal to 1. Both 4x4 and 8x8 block size transform modes are used. Both seq_scaling_matrix_present_flag and pic_scaling_matrix_flag are set to 1. Scaling lists are included in the sequence parameter set and the picture parameter set. Each slice is a coded field. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Tests 4x4 and 8x8 block size transform modes in CABAC entropy coding.

Purpose:  Check that a decoder can properly decode slices of coded fields with both 4x4 and 8x8 block size transform modes.

#### 6.6.20.29	Test bitstream #FREH-38

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. Temporal direct prediction is used for direct prediction. direct_8x8_inference_flag is set equal to 1. Both 4x4 and 8x8 block size transform modes are used. Both seq_scaling_matrix_present_flag and pic_scaling_matrix_flag are set to 1. Scaling lists are included in the sequence parameter set and the picture parameter set. Each slice is a coded field. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Tests 4x4 and 8x8 block size transform modes in CABAC entropy coding.

Purpose:  Check that a decoder can properly decode slices of coded fields with both 4x4 and 8x8 block size transform modes.

### 6.6.21	Test bitstreams – Fidelity Range Extensions: 4:2:0 10 bit

#### 6.6.21.1	Test bitstream #FREH10-1

Specification:  All slices are coded as I slices.  Each picture contains only one slice. disable_deblocking_filter_idc is equal to 1, specifying disabling of the deblocking filter process. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 0. frame_mbs_only_flag is equal to 1. chroma_format_idc is equal to 1. Both bit_depth_luma_minus8 and bit_depth_chroma_minus8 are set equal to 2. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of I slices for 4:2:0 10-bit.

Purpose:  Check that a decoder can properly decode I slices for 4:2:0 10-bit.

#### 6.6.21.2	Test bitstream #FREH10-2

Specification:  All slices are coded as I, P or B slices.  Each picture contains only one slice. disable_deblocking_filter_idc is equal to 1, specifying disabling of the deblocking filter process. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 0. frame_mbs_only_flag is equal to 1. chroma_format_idc is equal to 1. Both bit_depth_luma_minus8 and bit_depth_chroma_minus8 are set equal to 2. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of I, P, and B slices for 4:2:0 10-bit.

Purpose:  Check that a decoder can properly decode I, P and B slices for 4:2:0 10-bit.

### 6.6.22	Test bitstreams – Fidelity Range Extensions: 4:2:2 10 bit

#### 6.6.22.1	Test bitstream #FREH422-1

Specification:  All slices are coded as I or P slices. Each picture contains only one slice. disable_deblocking_filter_idc is equal to 1, specifying disabling of the deblocking filter process. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 0. chroma_format_idc is equal to 2, specifying 4:2:2 chroma format. Both bit_depth_luma_minus8 and bit_depth_chroma_minus8 are set equal to 0. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of P slices for 4:2:2 8 bit.

Purpose:  Check that a decoder can properly decode P slices for 4:2:2 8 bit.

#### 6.6.22.2	Test bitstream #FREH422-2

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. disable_deblocking_filter_idc is equal to 1, specifying disabling of the deblocking filter process. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 0. Direct prediction is not used in this bitstream. chroma_format_idc is equal to 2, specifying 4:2:2 chroma format. Both bit_depth_luma_minus8 and bit_depth_chroma_minus8 are set equal to 0. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of B slices for 4:2:2 8 bit.

Purpose:  Check that a decoder can properly decode B slices for 4:2:2 8 bit.

#### 6.6.22.3	Test bitstream #FREH422-3

Specification:  All slices are coded as I or P slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 0. chroma_format_idc is equal to 2, specifying 4:2:2 chroma format. Both bit_depth_luma_minus8 and bit_depth_chroma_minus8 are set equal to 0. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of P slices for 4:2:2 8 bit.

Purpose:  Check that a decoder can properly decode P slices with deblocking filter for 4:2:2 8 bit.

#### 6.6.22.4	Test bitstream #FREH422-4

Specification:  All slices are coded as I slices. Each picture contains only one slice. disable_deblocking_filter_idc is equal to 1, specifying disabling of the deblocking filter process. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 0. chroma_format_idc is equal to 2, specifying 4:2:2 chroma format. Both bit_depth_luma_minus8 and bit_depth_chroma_minus8 are set equal to 0. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of I slices for 4:2:2 8 bit.

Purpose:  Check that a decoder can properly decode I slices for 4:2:2 8 bit without deblocking filter.

#### 6.6.22.5	Test bitstream #FREH422-5

Specification:  All slices are coded as I or P slices. Each picture contains only one slice. disable_deblocking_filter_idc is equal to 1, specifying disabling of the deblocking filter process. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 0. chroma_format_idc is equal to 2, specifying 4:2:2 chroma format. Both bit_depth_luma_minus8 and bit_depth_chroma_minus8 are set equal to 0. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of P slices for 4:2:2 8 bit.

Purpose:  Check that a decoder can properly decode P slices for 4:2:2 8 bit without deblocking filter.

#### 6.6.22.6	Test bitstream #FREH422-6

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. disable_deblocking_filter_idc is equal to 1, specifying disabling of the deblocking filter process. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 0. Spatial direct prediction is used for direct prediction. direct_8x8_inference_flag is equal to 0. chroma_format_idc is equal to 2, specifying 4:2:2 chroma format. Both bit_depth_luma_minus8 and bit_depth_chroma_minus8 are set equal to 0. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of B slices for 4:2:2 8 bit.

Purpose:  Check that a decoder can properly decode B slices for 4:2:2 8 bit without deblocking filter.

#### 6.6.22.7	Test bitstream #FREH422-7

Specification:  All slices are coded as I or P slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 0. chroma_format_idc is equal to 2, specifying 4:2:2 chroma format. Both bit_depth_luma_minus8 and bit_depth_chroma_minus8 are set equal to 0. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of P slices for 4:2:2 8 bit.

Purpose:  Check that a decoder can properly decode P slices for 4:2:2 8 bit with deblocking filter.

#### 6.6.22.8	Test bitstream #FREH422-8

Specification:  All slices are coded as I slices. Each picture contains only one slice. disable_deblocking_filter_idc is equal to 1, specifying disabling of the deblocking filter process. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 0. chroma_format_idc is equal to 2, specifying 4:2:2 chroma format. Both bit_depth_luma_minus8 and bit_depth_chroma_minus8 are set equal to 2, specifying 10 bit video. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of I slices for 4:2:2 10 bit.

Purpose:  Check that a decoder can properly decode I slices for 4:2:2 10 bit without deblocking filter.

#### 6.6.22.9	Test bitstream #FREH422-9

Specification:  All slices are coded as I or P slices. Each picture contains only one slice. disable_deblocking_filter_idc is equal to 1, specifying disabling of the deblocking filter process. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 0. chroma_format_idc is equal to 2, specifying 4:2:2 chroma format. Both bit_depth_luma_minus8 and bit_depth_chroma_minus8 are set equal to 2, specifying 10 bit video. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of P slices for 4:2:2 8 bit.

Purpose:  Check that a decoder can properly decode P slices for 4:2:2 10 bit without deblocking filter.

#### 6.6.22.10	Test bitstream #FREH422-10

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. disable_deblocking_filter_idc is equal to 1, specifying disabling of the deblocking filter process. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 0. Spatial direct prediction is used for direct prediction. direct_8x8_inference_flag is equal to 0. chroma_format_idc is equal to 2, specifying 4:2:2 chroma format. Both bit_depth_luma_minus8 and bit_depth_chroma_minus8 are set equal to 2, specifying 10 bit video. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of B slices for 4:2:2 10 bit.

Purpose:  Check that a decoder can properly decode B slices for 4:2:2 10 bit without deblocking filter.

#### 6.6.22.11	Test bitstream #FREH422-11

Specification:  All slices are coded as I or P slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 0. chroma_format_idc is equal to 2, specifying 4:2:2 chroma format. Both bit_depth_luma_minus8 and bit_depth_chroma_minus8 are set equal to 2, specifying 10 bit video. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of P slices for 4:2:2 10 bit.

Purpose:  Check that a decoder can properly decode P slices for 4:2:2 10 bit with deblocking filter.

#### 6.6.22.12	Test bitstream #FREH422-12

Specification:  All slices are coded as I slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. chroma_format_idc is equal to 2, specifying 4:2:2 chroma format. Both bit_depth_luma_minus8 and bit_depth_chroma_minus8 are set equal to 0. Both 4x4 and 8x8 block size transform modes are used. seq_scaling_matrix_present_flag is set to 1 and default scaling lists are used. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of I slices for 4:2:2 8 bit.

Purpose:  Check that a decoder can properly decode I slices for 4:2:2 8 bit without deblocking filter.

#### 6.6.22.13	Test bitstream #FREH422-13

Specification:  All slices are coded as I or P slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. chroma_format_idc is equal to 2, specifying 4:2:2 chroma format. Both bit_depth_luma_minus8 and bit_depth_chroma_minus8 are set equal to 0. Both 4x4 and 8x8 block size transform modes are used. seq_scaling_matrix_present_flag is set to 1 and default scaling lists are used. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of P slices for 4:2:2 8 bit.

Purpose:  Check that a decoder can properly decode P slices for 4:2:2 8 bit without deblocking filter.

#### 6.6.22.14	Test bitstream #FREH422-14

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. Spatial direct prediction is used for direct prediction. direct_8x8_inference_flag is equal to 0. chroma_format_idc is equal to 2, specifying 4:2:2 chroma format. Both bit_depth_luma_minus8 and bit_depth_chroma_minus8 are set equal to 0. Both 4x4 and 8x8 block size transform modes are used. seq_scaling_matrix_present_flag is set to 1 and default scaling lists are used. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of B slices for 4:2:2 8 bit.

Purpose:  Check that a decoder can properly decode B slices for 4:2:2 8 bit without deblocking filter.

#### 6.6.22.15	Test bitstream #FREH422-15

Specification:  All slices are coded as I slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. chroma_format_idc is equal to 2, specifying 4:2:2 chroma format. Both bit_depth_luma_minus8 and bit_depth_chroma_minus8 are set equal to 2, specifying 10 bit video. Both 4x4 and 8x8 block size transform modes are used. seq_scaling_matrix_present_flag is set to 1 and default scaling lists are used. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of I slices for 4:2:2 10 bit.

Purpose:  Check that a decoder can properly decode I slices for 4:2:2 10 bit without deblocking filter.

#### 6.6.22.16	Test bitstream #FREH422-13

Specification:  All slices are coded as I or P slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. chroma_format_idc is equal to 2, specifying 4:2:2 chroma format. Both bit_depth_luma_minus8 and bit_depth_chroma_minus8 are set equal to 2, specifying 10 bit video. Both 4x4 and 8x8 block size transform modes are used. seq_scaling_matrix_present_flag is set to 1 and default scaling lists are used. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of P slices for 4:2:2 10 bit.

Purpose:  Check that a decoder can properly decode P slices for 4:2:2 10 bit without deblocking filter.

#### 6.6.22.17	Test bitstream #FREH422-14

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. Spatial direct prediction is used for direct prediction. direct_8x8_inference_flag is equal to 0. chroma_format_idc is equal to 2, specifying 4:2:2 chroma format. Both bit_depth_luma_minus8 and bit_depth_chroma_minus8 are set equal to 2, specifying 10 bit video. Both 4x4 and 8x8 block size transform modes are used. seq_scaling_matrix_present_flag is set to 1 and default scaling lists are used. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of B slices for 4:2:2 10 bit.

Purpose:  Check that a decoder can properly decode B slices for 4:2:2 10 bit without deblocking filter.

#### 6.6.22.18	Test bitstream #FREH422-18

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. Spatial direct prediction is used for direct prediction. direct_8x8_inference_flag is equal to 0. chroma_format_idc is equal to 2, specifying 4:2:2 chroma format. Both bit_depth_luma_minus8 and bit_depth_chroma_minus8 are set equal to 2, specifying 10 bit video. Both 4x4 and 8x8 block size transform modes are used. seq_scaling_matrix_present_flag is set to 1. Scaling lists are included in the sequence parameter set and the picture parameter set. Each slice is a coded field. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of B slices for 4:2:2 10 bit.

Purpose:  Check that a decoder can properly decode B slices of coded fields for 4:2:2 10 bit.

#### 6.6.22.19	Test bitstream #FREH422-19

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. Spatial direct prediction is used for direct prediction. direct_8x8_inference_flag is equal to 0. chroma_format_idc is equal to 2, specifying 4:2:2 chroma format. Both bit_depth_luma_minus8 and bit_depth_chroma_minus8 are set equal to 2, specifying 10 bit video. Both 4x4 and 8x8 block size transform modes are used. seq_scaling_matrix_present_flag is set to 1. Scaling lists are included in the sequence parameter set and the picture parameter set. Each slice is a coded frame. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of B slices for 4:2:2 10 bit.

Purpose:  Check that a decoder can properly decode B slices of coded frames for 4:2:2 10 bit.

#### 6.6.22.20	Test bitstream #FREH422-20

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. Spatial direct prediction is used for direct prediction. direct_8x8_inference_flag is equal to 0. chroma_format_idc is equal to 2, specifying 4:2:2 chroma format. Both bit_depth_luma_minus8 and bit_depth_chroma_minus8 are set equal to 2, specifying 10 bit video. Both 4x4 and 8x8 block size transform modes are used. seq_scaling_matrix_present_flag is set to 1. Scaling lists are included in the sequence parameter set and the picture parameter set. Each slice is either a coded frame or a coded field. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of B slices for 4:2:2 10 bit.

Purpose:  Check that a decoder can properly decode B slices of coded frames and fields for 4:2:2 10 bit.

#### 6.6.22.21	Test bitstream #FREH422-21

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. Spatial direct prediction is used for direct prediction. direct_8x8_inference_flag is equal to 0. chroma_format_idc is equal to 2, specifying 4:2:2 chroma format. Both bit_depth_luma_minus8 and bit_depth_chroma_minus8 are set equal to 2, specifying 10 bit video. Both 4x4 and 8x8 block size transform modes are used. seq_scaling_matrix_present_flag is set to 1. Scaling lists are included in the sequence parameter set and the picture parameter set. Each slice is a coded frame. mb_adaptive_frame_field_coding is equal to 1. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of B slices for 4:2:2 10 bit.

Purpose:  Check that a decoder can properly decode B slices with mb_adaptive_frame_field_flag=1 for 4:2:2 10 bit.

### 6.6.23	Test bitstreams – Fidelity Range Extensions: 4:4:4 12 bit

#### 6.6.23.1	Test bitstream #FREH444-1

Specification:  All slices are coded as I slices.  Each picture contains only one slice. disable_deblocking_filter_idc is equal to 1, specifying disabling of the deblocking filter process. entropy_coding_mode_flag is equal to 0, specifying the CAVLC parsing process. pic_order_cnt_type is equal to 0. frame_mbs_only_flag is equal to 1. chroma_format_idc is equal to 3. Both bit_depth_luma_minus8 and bit_depth_chroma_minus8 are set equal to 4. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of I slices for 4:4:4 12-bit

Purpose:  Check that the decoder can properly decode I slices for 4:4:4 12-bit.

#### 6.6.23.2	Test bitstream #FREH444-2

Specification:  All slices are coded as IBBP slices.  Each picture contains only one slice. disable_deblocking_filter_idc is equal to 1, specifying disabling of the deblocking filter process. entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. pic_order_cnt_type is equal to 0. frame_mbs_only_flag is equal to 1. chroma_format_idc is equal to 3. residual_colour_transform_flag is equal to 1. Both bit_depth_luma_minus8 and bit_depth_chroma_minus8 are set equal to 4. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of I, P, and B slices for 4:4:4 12-bit

Purpose:  Check that the decoder can properly decode I, P and B slices for 4:4:4 12-bit with residual colour transform.

### 6.6.24	Auxiliary coded picture

#### 6.6.24.1	Test bitstream #FREAUX-1

Specification:  Coded slices of an auxiliary coded picture are included in this bitstream. The rest of the slices are coded as either an I slice or a P slice.  entropy_coding_mode_flag is equal to 1, specifying the CABAC parsing process. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage:  Decoding of coded slices of an auxiliary coded picture

Purpose:  Check that the decoder can properly handle coded slices of an auziliary coded picture

“

# 9)	Subclause 6.7 “Normative Test Suites for ITU-T Rec. H.264 | ISO/IEC 14496-10”

Objective: Change the title of Table-1

Replace

“

Table 1 – Bitstreams included in this Specification

“

with

“

Table 2 – Bitstreams for Baseline, Extended and Main profile

“

# 10)	Subclause 6.7 “Normative Test Suites for ITU-T Rec. H.264 | ISO/IEC 14496-10”

Objective: Include new test bitstreams in Table-1

Add the following rows after the AVCMR-7 in Table-1.

“

|  | AVCMR-8 | British Telecom | MR6_BT_B |  | X | X | 2.1 and higher | 25 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AVCMR-9 | British Telecom | MR7_BT_B |  | X | X | 2.1 and higher | 25 |
|  | AVCMR-10 | British Telecom | MR8_BT_B |  | X | X | 2.1 and higher | 25 |

“

# 11)	Subclause 6.7 “Normative Test Suites for ITU-T Rec. H.264 | ISO/IEC 14496-10”

Objective: Include new test bitstreams in Table-1

Add the following rows after the AVCCAPCM-3 in Table-1.

“

| CABAC: MMCO | AVCCAMR-1 | British Telecom | MR9_BT_B |  |  | X | 2.1 and higher | 25 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

“

# 12)	Subclause 6.7 “Normative Test Suites for ITU-T Rec. H.264 | ISO/IEC 14496-10”

Objective: Include new Table for High, High 10, High 4:2:2, High 4:4:4 profile

Add the following subclause after Table-1.

“

Table 2 – Bitstreams for High, High 10, High 4:2:2, and High 4:4:4 profile

| Categories | Bitstream | Donated by | File Name | High | High 10 | High 4:2:2 | High 4:4:4 | Level | Frame Rate (Frame/Sec) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4:2:0 8 bit | FREH-1 | Panasonic Singapore Lab. | FRExt1_Panasonic_C | X | X | X | X | 2.1 and higher | 29.97 |
|  | FREH-2 | Panasonic Singapore Lab. | FRExt3_Panasonic_D | X | X | X | X | 2.1 and higher | 29.97 |
|  | FREH-3 | HHI | HCAFR1_HHI_C | X | X | X | X | 3.0 and higher | 15 |
|  | FREH-4 | HHI | HCAFF1_HHI_B | X | X | X | X | 3.0 and higher | 15 |
|  | FREH-5 | HHI | HCAMFF1_HHI_B | X | X | X | X | 3.0 and higher | 15 |
|  | FREH-6 | Panasonic Singapore Lab. | FRExt2_Panasonic_B | X | X | X | X | 2.1 and higher | 29.97 |
|  | FREH-7 | Panasonic Singapore Lab. | FRExt4_Panasonic_A | X | X | X | X | 2.1 and higher | 29.97 |
|  | FREH-8 | Broadcom | HPCANL_BRCM_C | X | X | X | X | 4.0 and higher | 29.97 |
|  | FREH-9 | Broadcom | HPCA_BRCM_C | X | X | X | X | 4.0 and higher | 29.97 |
|  | FREH-10 | Broadcom | HPCAFLNL_BRCM_C | X | X | X | X | 4.0 and higher | 29.97 |
|  | FREH-11 | Broadcom | HPCAFL_BRCM_C | X | X | X | X | 4.0 and higher | 29.97 |
|  | FREH-12 | HHI | HCAFR2_HHI_A | X | X | X | X | 2.0 and higher | 15 |
|  | FREH-13 | HHI | HCAFR3_HHI_A | X | X | X | X | 3.0 and higher | 15 |
|  | FREH-14 | HHI | HCAFR4_HHI_A | X | X | X | X | 3.0 and higher | 15 |
|  | FREH-15 | Broadcom | HPCADQ_BRCM_B | X | X | X | X | 4.0 and higher | 29.97 |
|  | FREH-16 | Broadcom | HPCALQ_BRCM_B | X | X | X | X | 4.0 and higher | 29.97 |
|  | FREH-17 | Broadcom | HPCAMAPALQ_BRCM_B | X | X | X | X | 4.0 and higher | 29.97 |
|  | FREH-18 | Broadcom | HPCV_BRCM_A | X | X | X | X | 4.0 and higher | 29.97 |
|  | FREH-19 | Broadcom | HPCVNL_BRCM_A | X | X | X | X | 4.0 and higher | 29.97 |
|  | FREH-20 | Broadcom | HPCVFL_BRCM_A | X | X | X | X | 4.0 and higher | 29.97 |
|  | FREH-21 | Broadcom | HPCVFLNL_BRCM_A | X | X | X | X | 4.0 and higher | 29.97 |
|  | FREH-22 | Sony | HVLCFI0_Sony_A | X | X | X | X | 3.1 and higher | 29.97 |
|  | FREH-23 | Sony | HVLCPFF0_Sony_A | X | X | X | X | 3.1 and higher | 29.97 |
|  | FREH-24 | Sony | HVLCMFF0_Sony_A | X | X | X | X | 3.1 and higher | 29.97 |
|  | FREH-25 | Broadcom | HPCVMOLQ_BRCM_B | X | X | X | X | 4.0 and higher | 29.97 |
|  | FREH-26 | Broadcom | HPCAMOLQ_BRCM_B | X | X | X | X | 4.0 and higher | 29.97 |
|  | FREH-27 | Broadcom | HPCAQ2LQ_BRCM_B | X | X | X | X | 4.0 and higher | 29.97 |
|  | FREH-28 | Broadcom | brcm_freh1_B | X | X | X | X | 3.0 and higher | 29.97 |
|  | FREH-29 | Broadcom | brcm_freh2_B | X | X | X | X | 3.0 and higher | 29.97 |
|  | FREH-30 | Broadcom | brcm_freh3 | X | X | X | X | 3.0 and higher | 29.97 |
|  | FREH-31 | Broadcom | brcm_freh4 | X | X | X | X | 3.0 and higher | 29.97 |
|  | FREH-32 | Broadcom | brcm_freh5 | X | X | X | X | 3.0 and higher | 29.97 |
|  | FREH-33 | Broadcom | brcm_freh6 | X | X | X | X | 3.0 and higher | 29.97 |
|  | FREH-34 | Broadcom | brcm_freh7_B | X | X | X | X | 3.0 and higher | 29.97 |
|  | FREH-35 | Broadcom | brcm_freh8 | X | X | X | X | 3.0 and higher | 29.97 |
|  | FREH-36 | Broadcom | brcm_freh9 | X | X | X | X | 3.0 and higher | 29.97 |
|  | FREH-37 | Broadcom | brcm_freh10 | X | X | X | X | 3.0 and higher | 29.97 |
|  | FREH-38 | Broadcom | brcm_freh11 | X | X | X | X | 3.0 and higher | 29.97 |
|  | FREH-39 | Broadcom | brcm_freh12_B | X | X | X | X | 3.0 and higher | 29.97 |
| 4:2:0 10 bit | FREH10-1 | Dolby | FREH10-1 |  | X | X | X | 4 and higher | 24 |
|  | FREH10-2 | Dolby | FREH10-2 |  | X | X | X | 4 and higher | 24 |
| 4:2:2 10 bit | FREH422-1 | Tandberg | FREXT3_TANDBERG_A |  |  | X | X | 2.1 and higher | 29.97 |
|  | FREH422-2 | Tandberg | FREXT5_TANDBERG_A |  |  | X | X | 2.1 and higher | 29.97 |
|  | FREH422-3 | Tandberg | FREXT6_TANDBERG_A |  |  | X | X | 2.1 and higher | 29.97 |
|  | FREH422-4 | Sony | Hi422FREXT1_Sony_A |  |  | X | X | 3.1 and higher | 29.97 |
|  | FREH422-5 | Sony | Hi422FREXT2_Sony_A |  |  | X | X | 3.1 and higher | 29.97 |
|  | FREH422-6 | Sony | Hi422FREXT3_Sony_A |  |  | X | X | 3.1 and higher | 29.97 |
|  | FREH422-7 | Sony | Hi422FREXT4_Sony_A |  |  | X | X | 3.1 and higher | 29.97 |
|  | FREH422-8 | Sony | Hi422FREXT6_Sony_A |  |  | X | X | 3.1 and higher | 29.97 |
|  | FREH422-9 | Sony | Hi422FREXT7_Sony_A |  |  | X | X | 3.1 and higher | 29.97 |
|  | FREH422-10 | Sony | Hi422FREXT8_Sony_A |  |  | X | X | 3.1 and higher | 29.97 |
|  | FREH422-11 | Sony | Hi422FREXT9_Sony_A |  |  | X | X | 3.1 and higher | 29.97 |
|  | FREH422-12 | Sony | Hi422FREXT10_Sony_A |  |  | X | X | 3.1 and higher | 29.97 |
|  | FREH422-13 | Sony | Hi422FREXT11_Sony_A |  |  | X | X | 3.1 and higher | 29.97 |
|  | FREH422-14 | Sony | Hi422FREXT12_Sony_A |  |  | X | X | 3.1 and higher | 29.97 |
|  | FREH422-15 | Sony | Hi422FREXT13_Sony_A |  |  | X | X | 3.1 and higher | 29.97 |
|  | FREH422-16 | Sony | Hi422FREXT14_Sony_A |  |  | X | X | 3.1 and higher | 29.97 |
|  | FREH422-17 | Sony | Hi422FREXT15_Sony_A |  |  | X | X | 3.1 and higher | 29.97 |
|  | FREH422-18 | Sony | Hi422FREXT16_Sony_A |  |  | X | X | 4 and higher | 29.97 |
|  | FREH422-19 | Sony | Hi422FREXT17_Sony_A |  |  | X | X | 4 and higher | 29.97 |
|  | FREH422-20 | Sony | Hi422FREXT18_Sony_A |  |  | X | X | 4 and higher | 29.97 |
|  | FREH422-21 | Sony | Hi422FREXT19_Sony_A |  |  | X | X | 4 and higher | 29.97 |
| 4:4:4 12 bit | FREH444-1 | Dolby | FREXT9_Dolby_C |  |  |  | X | 4 and higher | 24 |
|  | FREH444-2 | Samsung AIT | FREXT10_Samsung_A |  |  |  | X | 4 and higher | 24 |
| Auxiliary codec picture | FREAUX-1 | Apple | alphaconformanceA | X | X | X | X | 2.1 and higher | 29.97 |

“