| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 15th Meeting: Busan, KR, 16-22 April, 2005 | Document: JVT-O010 Filename: JVT-O010.doc NOTE – That first character after the dash up there in the document number is an "O", not a zero. |
| --- | --- |

| Title: | Adaptive Upsampling for Spatially Scalable Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Andrew Segall Sharp Laboratories of America 5750 NW Pacific Rim Blvd Camas, WA 98607 Shawmin Lei Sharp Laboratories of America 5750 NW Pacific Rim Blvd Camas, WA 98607 | Tel: Email: | +1-360-817-8487 asegall@sharplabs.com +1-360-817-8446 shawmin@sharplabs.com |
| Source: | Sharp Laboratories of America |  |  |

_____________________________

## Abstract

We present an adaptive method for upsampling low-resolution image frames for the purpose of spatially scalable coding.  The low-resolution data is adaptively upsampled on a block-by-block basis, and the filter for each block is selected by a side-channel in the enhancement bit-stream.  Simulations focus on the improvement of IntraBL coded blocks and show that the method improves the end-to-end performance of the current JSVM.

## Adaptive Upsampler

The algorithm proposed in this document estimates a block of high-resolution image pixels from a block of low-resolution image pixels.  The mapping from low-resolution to high-resolution is accomplished with a separable poly-phase filter and is given in one dimension as

HighResx=t=−23FilterS⋅t−x%S⋅LowResxS+t,	(1)

where x%S and x/S are respectively the mod-S and integer division operator and S is the scale factor.  We assume that S=2 for the rest of this proposal.  In this case, (1) is a two-phase filter where each filter contains six-taps.

In the current WD [1], the coefficients for the filter in (1) do not change across the image frame.  However, we adjust the filter coefficients in our proposed method.  Adjustment is performed on a 4x4 block basis (with respect to the high-resolution image), and it is accomplished by selecting an interpolation filter from a pre-defined filter bank for each of the horizontal and vertical directions.  The filterbank contains four members and is defined in Table 1.  The construction of the filterbank augments the current six-tap filter with three alternative (and computationally simpler) interpolation kernels.  The additional filters include two sample-and-hold procedures as well as the bi-linear interpolator.

| Filter | Taps |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 0 | -5 | 0 | 20 | 32 | 20 | 0 | -5 | 0 | 1 | 0 |
| 1 | 0 | 0 | 0 | 0 | 0 | 32 | 32 | 0 | 0 | 0 | 0 | 0 |
| 2 | 0 | 0 | 0 | 0 | 32 | 32 | 0 | 0 | 0 | 0 | 0 | 0 |
| 3 | 0 | 0 | 0 | 0 | 16 | 32 | 16 | 0 | 0 | 0 | 0 | 0 |

Table 1: Four filters of the proposed filter-bank.  The coefficients in the table are represented with five-bits of precision.  Also, the phases of the filter are color coded to enhance display.

Selection of the interpolation filter is performed at the encoder and signaled to the decoder.  Signaling for the horizontal and vertical filters is performed independently, and it is included within the enhancement bit-stream.

Experimental Results

To evaluate the performance of the proposed method, we incorporate the adaptive upsampler into the JSVM1.0 encoder.  The encoder is modified to apply the adaptive upsampler to IntraBL-mode luminance blocks and to signal the interpolation filters in a separate side-channel.  We utilize the filter bank defined in Table 1, and we measure the impact of the adaptive upsampling tool on the compression efficiency of the IntraBL-mode blocks.  We generate a test bitstream using the simulation conditions in Table 2.

Table 2 Simulation condition

| Number of Layers | 2 |
| --- | --- |
| GOP Size | 1 |
| Intra Period 1 | 1 |
| Base Layer |  |
| Resolution | QCIF |
| AVC Compatible | Yes |
| FRExt Mode | No |
| Intra mode Usage | On |
| Enhancement Layer |  |
| Resolution | CIF |
| AVC Compatible | No |
| FRExt Mode | No |
| Intra mode Usage | On |

Choosing the filters for each block is accomplished at the encoder and utilizes the following procedure.  First, the cost of signaling each candidate filter is estimated with an adaptive arithmetic code. (Context adaptive coding is not used here although it is possible to further reduce the side-channel bits.) Next, the block is interpolated with each candidate filter and the resulting residual coded with the specified Qp.  The filter is then chosen that minimizes the number of total bits, where total bits is defined as the sum of residual and side-channel bits.  To be clear, this solves the problem

argminFRF s.t. DF=D,	(2)

where F is the set of candidate filters, DF and RF are respectively the distortion and rate of signaling the filter F and coding the interpolation residual with the specified Qp, and

RF=SideF+ResidualF

where SideF is the cost (in bits) of signaling a specific filter, and ResidualF is the cost (in bits) of transmitting the difference between the upsampled and original high-resolution blocks.

We process the Mobile and Calendar, Bus, Flower and Paris sequences with Qp values in the range [15,35].  Rate-distortion performance is captured for each enhanced intra-frame by extracting the bit information within the JSVM block decision loop.  In the case of the proposed tool, we extract the bits needed to code the residual as well as the side channel information for the IntraBL mode.  We then extract the bits needed to code the IntraBL residual for the unmodified JSVM.  In both cases, we are only considering the cost of coding the luma channel.  Results appear in Figures 1-4, which contain a representative example from each sequence.  In the figures, the performance of the unmodified JSVM (i.e., without the adaptive upsampler) is denoted by the “JSVM (w/ Six-Tap)” data points.

As can be seen from the figures, the adaptive upsampler improves the performance of the IntraBL blocks for each sequence in the test.  For example, the method provides 0.7 dB of improvement in the Mobile and Calendar sequence when the enhancement PSNR is 38 dB.  For the remaining sequences, the technique provides 0.4 dB of improvement when the enhancement PSNR is 38 dB.

Performance of the adaptive upsampling method depends on the quality of the base layer data.  This is also evident in the figures, as the gain introduced by the adaptive upsampling tool depends on the enhancement PSNR.  (The experiments utilize the same Qp for both base and enhancement layers, and so the PSNR of the two layers are similar.)  For example, the adaptive upsampler provides 0.9 dB of improvement for the Mobile and Calendar sequence at the 45 dB enhancement point.  The other sequences show additional gains as well.  The Bus, Flower Garden and Paris sequences show an improvement of 0.6 dB, 0.5 dB and 0.7 dB, respectively, at the 45 dB enhancement point.

## Conclusion

We propose an upsampling method to improve the compression efficiency of spatially scalable coding.  The technique selects filters on a block-by-block basis and utilizes a four-channel filterbank.  Filter selection is performed at the encoder and signaled to the decoder.  For the case of IntraBL coded blocks, results show a coding improvement of 0.7dB for the Mobile and Calendar sequence and 0.4dB for other sequences, when compared to the current JSVM and encoded with a PSNR of 38dB.  Coding improvements of 0.9dB are observed for high-rate conditions. We propose to the JVT Committee to establish a core experiment to further optimize this method and verify its performance.

## Reference

[1] ITU-T and ISO/IEC JTC1, “Scalable Video Coding - Working Draft 1”, JVT-N020, Jan 2005

Figure 1: Rate-distortion performance of a frame from the Mobile & Calendar sequence.  The adaptive upsampler provides .7dB of improvement when the PSNR is 38dB, as compared to the current JSVM.  Note that the level of improvement varies relative to the quality of the decoded frame.  For example, the adaptive upsampler provides .9dB of improvement at the 45dB quality point.

Figure 2: Rate-distortion performance of a frame from the Bus sequence.  The adaptive upsampler provides .4dB of improvement when the PSNR is 38dB, as compared to the current JSVM.  The amount of improvement increases to .6dB when the PSNR is 45dB.

Figure 3: Rate-distortion performance of a frame from the Flower Garden sequence.  The adaptive upsampler provides .4dB of improvement when the PSNR is 38dB, as compared to the current JSVM.  The amount of improvement increases to .5dB when the PSNR is 45dB.

Figure 4: Rate-distortion performance of a frame from the Paris sequence.  The adaptive upsampler provides .4dB of improvement when the PSNR is 38dB, as compared to the current JSVM.  The amount of improvement increases to .7dB when the PSNR is 45dB.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Sharp Labs of America |  |  |
| Mailing address | 5750 NW Pacific Rim Blvd Camas, WA 98607 USA |  |  |
| Country | USA |  |  |
| Contact person | Andrew Segall |  |  |
| Telephone | +1-360-817-8487 |  |  |
| Fax | +1-360-817-8436 |  |  |
| Email | asegall@sharplabs.com |  |  |
| Place and date of submission | Busan, Korea (April 2005) |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Adaptive Upsampling for Spatially Scalable Coding |  |
| Contribution number |  | JVT-O010 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  | US Provisional Patent Filed |  |
| Inventor(s)/Assignee(s) |  | Andrew Segall and Shawmin Lei |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: