| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 72th Meeting: Busan, Korea, 18-22 Apr, 2005 | Document: JVT-O015 Filename: JVT-O015.doc |
| --- | --- |

| Title: | Simplified update step operation for MCTF |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xianglin Wang, Marta Karczewicz, Yiliang Bao, Justin Ridge Nokia Research Center 6000 Connection Drive Irving, TX 75039, USA | Tel: Email: | +1 972 894 5771 xianglin.wang@nokia.com |
| Source: | Nokia Inc. |  |  |

_____________________________

# Introduction

Although the MCTF (i.e. motion compensated temporal filtering) technique is found to be useful in improving coding performance, complexity has always been a major concern in using it.  In a large part, the complexity is related to the update step. Reducing the update step complexity is very much desired and important.

In this proposal, the interpolation process of update step is simplified with an adaptive but very simple filter. In this case, coding performance is virtually unchanged. Meanwhile, update motion vector derivation process can also be simplified by doing it on an 8x8 block basis instead of 4x4 block. The degradation of coding performance in this case is less than 1%.

# MCTF Update Step

In the current SVC reference software (JSVM version 1.0), MCTF is implemented using lifting steps, which are: prediction step and update step. The prediction step is generally the same as that in traditional block-based video coding without MCTF, except that in MCTF the prediction step is based on open loop structure where the original video frames are used as reference frame in motion estimation. Motion vectors used during the update step, so called update motion vectors, are an inverse of motion vectors found during the prediction step. According to these update motion vectors, prediction residue (high pass frame H) obtained from the prediction step is added back to the reference frame to generate a low pass frame L. In this sense, the update step also includes a motion compensation process.

More specifically, the update step is performed by splitting L frame into 4x4 non-overlapping, adjacent blocks. Motion vectors used in the update step for these blocks are derived from motion vectors used in the prediction step. Let us consider one of the update blocks, block A, in frame L, as shown in Fig. 1.  As it can be seen in Fig 1 two blocks (blocks B1 or B2) in the frame H are predicted from this block. The inverse of the motion vector of the block, which has the largest number of pixels m predicted from block A, is selected to predict block A, i.e., if the motion vector of block B1 is (Δx, Δy) the update motion vector that is used for block A is equal to (-Δx, -Δy). The number of pixels m predicted from the update block A is also used to calculate weight w1 assigned to block A.

Generally interpolation is needed in the update step whenever motion vector (-Δx, -Δy) does not have an integer pixel displacement for horizontal or vertical direction. Such extra interpolation process associated with MCTF update step increases the overall system complexity, especially the decoder complexity, considerably. Taking Mobile sequence as an example, we found that around 90% of blocks need to be updated in the update step. Therefore, extra interpolation may be needed for all those blocks. In the current SVC reference software, the AVC standard interpolation method is used for sub-pixel interpolation in both prediction step and update step.

Figure 1

It is found that the update process in MCTF is helpful in improving coding performance in terms of objective quality of the coded video. However, it may also bring unwanted coding artifacts that may degrade the subjective quality. In order to avoid such coding artifacts, update strength is controlled by two weight factors: w1 and w2. Assume En+1 is the prediction residue block used during the update operation, then instead of using En+1 directly, w1* w2* En+1  is actually used during the update. Weight w2 is derived based on the energy level of the residue block in frame H used for update operation [1]. If the energy is too high, it is more likely that the update operation could produce the unwanted visual artifacts. In this case the update strength needs to be lowered. In the current SVC reference software, the average of squared pixel value in a block is used as an indication of the energy level of that block. To be more specific, interpolation is first performed as needed to get the interpolated prediction residue, followed by energy estimation based on the interpolated pixels.

# Proposed Simplifications

## Energy Calculation

As explained above, a block energy estimation process is involved in the update step and the estimated block energy level is used to control update strength of the corresponding block. In the current SVC reference software, this energy estimation is based on interpolated pixels of the block.

Due to the fact that there is a close correlation between the original integer pixel values and their neighboring interpolated sub-pixel values, it can be expected that the energy estimation result based on interpolated pixels should also be close to the result that is based on their neighboring integer pixels. Therefore, we propose doing the block energy estimation simply based on integer pixels.

With this approach, the complexity of calculations on energy estimation remains the same.  However, computation savings can be expected from the interpolation process. When the estimated energy level is high the block will be excluded from update process (i.e. a weight factor w2 = 0), hence interpolation for the current block can be omitted. This is not be possible if energy estimation is done based on interpolated pixels.

## Adaptive Interpolation

In the current SVC reference software, AVC 6 tap interpolation filter is used both during prediction and update steps. We propose to perform interpolation for an update step by switching between a 4-tap filter and bilinear filter depending on the weight factor of an updated block. We use the same 4-tap filter that has been used in a previously submitted JVT document [2]. The weight factor w can take values in the range [0, 1].  If w>0.5, the 4-tap filter is used in interpolation for the current block. Otherwise, bilinear filter is used.

As explained above, a weight factor is used to control update strength. The weight factor is hence an indicator of how reliable the update motion vector is. When choosing an interpolation filter, we use a relatively long filter, e.g. the 4-tap filter, for interpolation of blocks with a larger weight factor because these blocks are more important in maintaining the coding performance. For blocks with a lower weight factor, bilinear filter is sufficient.

In our simulations generally over 30% blocks were interpolated with bilinear method. However, we found almost no coding performance degradation. In addition, there were no obvious differences in terms of video subjective quality.

# PSNR results

Attached spreadsheet JVT-O015.xls contains RD-curves for the test set and QP values of the common conditions (QP = 22, 28, 34 and 40). For each test sequence, 2 sets of results are given. The first set result is obtained by using the proposed energy calculation based on integer pixel positions and adaptive interpolation method. The filter is switched between a 4-tap filter and bilinear filter with a threshold value 0.5 as explained in the previous section.

The set of results include preliminary results obtained by performing update step using 8x8 instead of 4x4 blocks. Motion vectors corresponding to a block size smaller than 8x8 (such as 8x4, 4x8 and 4x4) are excluded from, and therefore not used for, the update step. The main procedure for the update step as described above remains the same, except that everything is performed on an 8x8 block basis. For majority of sequences there is no performance degradation brought by this simplification.

# References

[1] Heiko Schwarz, etc. “Further Progress on Scalable Extension of H.264”, document VCEG-X08, 24rd Meeting, Palma de Mallorca, Spain, October, 2004

[2] Antti Hallapuro, etc. "4-tap filter for bi-predicted macroblocks", document JVT-D029, Joint Video Team (JVT) Meeting, Klagenfurt, Austria, 22-26 July, 2002

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Nokia Inc. |  |  |
| Mailing address | 6000 Connection Drive, Irving, TX 75039 USA |  |  |
| Country | USA |  |  |
| Contact person | Marta Karczewicz |  |  |
| Telephone | +1 972 894 4188 |  |  |
| Fax |  |  |  |
| Email | marta.karczewicz@nokia.com |  |  |
| Place and date of submission | Busan, Korea, 18/4/2005 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Simplified update step operation for MCTF |  |
| Contribution number |  | JVT-O015 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: