| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 15th Meeting: Busan, KR, 16-22 April, 2005 |  |  | Document: JVT-O020 Filename: JVT-O020.doc |  |
| --- | --- | --- | --- | --- |
| Title: | Adding Selective Enhancement Functionality to Scalable Video Coding |  |  |  |
| Status: | Input Document to JVT |  |  |  |
| Purpose: | Proposal |  |  |  |
| Contact(s): | Wen-Hsiao Peng 1001 Ta-Hsueh Rd., HsinChu 30050, Taiwan | Tel: Email: |  | +886-0-955-028-902 pawn@mail.si2lab.org |
| Source: | National Chiao-Tung University |  |  |  |

- Introduction

Regions-of-interest (ROI) is a desirable feature in some applications as described in the requirement document [4] of MPEG-4 Part 10 Amd. 1 Scalable Video Coding (SVC) [1]. For those applications, the clients at the decoder side may require to receive a better decoded quality in the ROIs when the pre-encoded scalable bit-stream is truncated. Currently, the JSVM1.0 [3] of SVC does not support ROI functionality. In this proposal, we develop a graceful and arbitrary shape selective enhancement scheme to provide the ROI functionality in SVC.

The ROI functionality is commonly supported in scalable video/image coding. In MPEG-4 FGS [5], the ROI functionality is enabled by a selective bit-plane shifting scheme. For the SNR scalability, MPEG-4 FGS encode the SNR enhancement-layer using a bit-plane coding scheme. The coding of transform coefficients is started from the most significant bit-plane to the least significant bit-plane. To provide the ROI, the encoder can shift up the coefficients in the ROI so that they can be firstly encoded and transmitted during the bit-plane coding. At the decoder, the ROI will be updated prior to those excluded regions when the SNR enhancement-layer is truncated. Similar bit-plane shifting concept is also widely used in the other scalable video/image codecs for the ROI functionality.

Different from MPEG-4 FGS, in SVC [1] the bit-planes are replaced with FGS layers which are produced by successive quantization of inter-layer prediction residue. Each FGS layer can be deemed as a group of multiple bit-planes. However, these bit-planes are coded by a cyclical block coding [2] instead of traditional bit-plane coding. In SVC [1], the coding of a FGS layer is partitioned into the significant and refinement passes. The significant pass first encodes the insignificant coefficients which show zero values in the subordinate layers. After that, the refinement pass refines the remaining significant coefficients ranging from -1 to +1. Particularly, the significance pass coding is performed in a cyclical manner. On the other hand, the refinement pass coding is conducted subband by subband [1].

Fig. 1 shows the major differences between the cyclical block coding [2] and the traditional bit-plane coding. For simplicity, we assume that the FGS layer 1 in Fig. 1 has only 4 transform blocks and each block has 3 coefficients. In addition, before the coding, we assume all the transform coefficients have insignificant status, i.e., the entire FGS layer 1 will be coded by the significance pass. Fig. 1 (a) illustrates the cyclical block coding [2]. As shown, for each coding cycle, the coding of a block is continued until a non-zero coefficient in zigzag order is coded and recognized. Particularly, every coding cycle in a block will include the coding of an EOB symbol, the significance indication bits and a non-zero quantization level. As compared to traditional bit-plane coding in Fig. 1 (b), the coefficient is not coded bit-plane by bit-plane. Thus, selectively shifting the bit-planes is not an effective way to offer the ROI functionality in SVC.

- Cyclical block coding scheme in JSVM1.0 of SVC.

(b) Bit-plane coding scheme in MPEG-4 FGS

Figure 1: Differences between (a) cyclical block coding in JSVM1.0 of SVC and (b) bit-plane coding in MPEG-4 FGS.

- Selective Enhancement

  - Prioritized Cyclical Block Coding

To provide the ROI functionality based on cyclical block coding [2], we proposed to code each block unequally in a coding cycle. For illustration, Fig. 2 and Fig. 3 show the differences between the cyclical block coding and our prioritized coding scheme. For simplicity, we use the notation of (EOB, Run, Level) symbol to represent the coefficients of a block that are to be coded in a coding cycle of significance pass. In Fig. 2, the cyclical block coding equally encodes each block with one (EOB, Run, Level) symbol (or one refinement symbol) in every cycle. However, as shown in Fig. 3, to offer the ROI functionality, we should encode the blocks in the ROIs with more symbols by enabling the coding prior to the blocks which are outside the ROIs. We can further extend this concept to enable the coding of different ROIs at different cycles to have graceful selective enhancement. For example in Fig. 3, we have selected two ROIs. The coding of the highest priority region ROI 1, which includes Block 0, is enabled prior to the rest of blocks. After two coding cycles, the coding of ROI 2 which is of lower priority is activated subsequently. In particular, the coding of ROI 2 is started before the coding of background region, i.e., Block 2 and Block 3. With such prioritization, the blocks in the ROIs will be coded with more symbols at the end of each cycle. When the enhancement-layer is truncated, the blocks in the ROIs will be firstly decoded and updated. Figure 2: Cyclical block coding.

Figure 3: Prioritized cyclical block coding for the ROI functionality.

For specifying the priority of a ROI, we use the number of shifting cycles which is defined as the number of coding cycles in the ROI before the coding of background region. For example, the number of shifting cycles of ROI 1 in Fig. 3 is 2 which means that the Block 0 has to be coded with 2 cycles before the coding of background region. Higher number of shifting cycles means higher coding priority.

Table 1. Enable_MB_Coding bit assignment

| ROIs | Shift Factors | Cycle 0 | Cycle 1 | Cycle 2 | Cycle 3 |
| --- | --- | --- | --- | --- | --- |
| Block 0 | 3 | 1* | X | X | X |
| Block 1 | 1 | 0 | X | 1 | X |
| Block 2 | 0 | 0 | X | 0 | X |
| Block 3 | 0 | 0 | X | 0 | X |

*1:Enable MB coding; 0:Disable MB coding; X: No indication bit is coded.

  - Syntax of Priority Information

In this proposal, the granularity of ROI can be down to one macroblock (MB). Hence, the priority information, i.e., the number of shifting cycles, is coded at MB level. To minimize the overhead, we propose an efficient priority information coding scheme.

Firstly, we gather the information of number of non-zero shifting factors existing in the current FGS slice. Such information is coded with a 3-bit syntax, Num_of_NonZero_MB_ShiftingFactors, in the beginning of each FGS slice. After that, we encode each non-zero shifting factor with a 5-bit syntax, NonZero_MB_ShiftingFactor. In other words, we allow maximally 8 different shifting factors in a FGS slice and these shifting factors can be any values from 1 to 32. Particularly, the Num_of_NonZero_MB_ShiftingFactors of value 0 indicates that the ROI functionality is disabled. In Fig. 3, we assume that each MB simply contains only one block. Thus, in the example of Fig. 3, we have Num_of_NonZero_MB_ShiftingFactors = 2 and the associated NonZero_MB_ShiftingFactors have values of 1 and 3.

Given these information at the slice level, we can then appropriately add an Enable_MB_Coding bit prior to the coding of each MB to enable or disable the coding. For example, in Fig. 3, an Enable_MB_Coding bit of value 1 is coded prior to the Block 0 to enable the coding in the Cycle 0. On the other hand, Block 1, 2, and 3 have an Enable_MB_Coding bit of value 0 to temporarily disable the coding. For those disabled blocks, another set of Enable_MB_Coding bits will be coded at later cycles to enable the coding. Specifically, the time to refresh the Enable_MB_Coding bits for the disabled blocks can be derived from the slice level syntax, Num_of_NonZero_MB_ShiftingFactors and NonZero_MB_ShiftingFactors. For example in Fig. 3, the Enable_MB_Coding bits of Block 1, 2 and 3 will be refreshed in the Cycle 2 because there are blocks which have shifting factor of value 1 as indicated by the slice level syntax, NonZero_MB_ShiftingFactors. Particularly, after the Cycle 2, all the blocks will be enabled for coding. There is no need to further encode any Enable_MB_Coding bits.

Table 2. Alternative Enable_MB_Coding bit assignment

| ROIs | Shift Factors | Cycle 0 | Cycle 1 | Cycle 2 | Cycle 3 |
| --- | --- | --- | --- | --- | --- |
| Block 0 | 3 | 1* | 1 | X | X |
| Block 1 | 1 | 1 | 0 | X | X |
| Block 2 | 0 | 0 | X | X | X |
| Block 3 | 0 | 0 | X | X | X |

*1:Enable MB coding; 0:Disable MB coding; X: No indication bit is coded.

Figure 4: Prioritized cyclical block coding with the alternative indication bit assignment.

Table 1 summarizes our Enable_MB_Coding bit assignment for the example of Fig. 3. As shown, the higher priority MBs have less number of indication bits than the lower priority ones. In addition, the maximal number of Enable_MB_Coding bits required for a MB is equivalent to Num_of_NonZero_MB_ShiftingFactors - 1. In the case of binary ROI partition, i.e., the entire frame is partition into the foreground and background regions, we simply have one bit for each MB to indicate if a MB is enabled for coding or not.

  - Improved Syntax of Priority Information

Typically, most of MBs are in the background regions and are assigned with lower priority. However, according to Table 1, the lower priority MBs are assigned with more Enable_MB_Coding bits. To reduce the overhead, Table 2 gives an alternative Enable_MB_Coding bit assignment scheme and Fig. 4 shows the corresponding change to the cyclical block coding.

In the Cycle 0, an Enable_MB_Coding bit of value 1 is coded prior to the Block 0 and 1 to enable the coding of MBs that have non-zero shifting factors. On the other hand, an Enable_MB_Coding bit of value 0 is coded prior to the Block 2 and 3 to temporarily disable the coding. Instead of refreshing the Enable_MB_Coding bits of disabled blocks, the alternative assignment scheme in Table 2 refreshes the ones of enabled blocks. Again, from the slice level syntax, Num_of_NonZero_MB_ShiftingFactors and NonZero_MB_ShiftingFactors, we can decide when to refresh the Enable_MB_Coding bits of those enabled blocks. In the example of Fig. 4, the Enable_MB_Coding bits of Block 0 and 1 are refreshed in the Cycle 1 as indicated in Table 2. In addition, after the Cycle 2, all the blocks will be enabled for coding. As shown in Table 2, the alternative Enable_MB_Coding bit assignment scheme assign less number of Enable_MB_Coding bits for the lower priority MBs. The overhead is minimized. Moreover, as compared to Fig. 3, Fig. 4 shows that the alternative bit assignment scheme has exactly the same MB coding result after the Cycle 2.

To minimize the overhead for the ROI functionality, the priority information bits of each MB, i.e., Enable_MB_Coding, is shared among the FGS layers in a slice. In other words, the Enable_MB_Coding bits of each MB are encoded/decoded once in a slice. During the encoding/decoding of a FGS slice, we assign each MB a register to keep track of the coding state in each cycle. The encoding/decoding of a MB will firstly check if the associated register has been set in the subordinate FGS layers. Once the register is set, the MB coding control will refer to the content of the associated register. On the other hand, a priority information bit will be encoded/decoded if the associated register has not been set.

  - Syntax of Significance Pass Coding Cycles

In the cyclical block coding (Fig. 2), the refinement pass is enabled after all the (EOB, Run, Level) symbols are coded. The blocks with less (EOB, Run, Level) symbols are assigned with multiple waiting cycles to delay the coding of refinement symbols. In our prioritized cyclical block coding, the (EOB, Run, Level) symbol and the refinement symbol could be coded in a cycle. Thus, we need additional information to decide when to enable the coding of refinement symbols in each MB.

To maintain the waiting cycles structure so as to have more (EOB, Run, Level) symbols be coded prior to the refinement symbols, a 4-bit syntax, Num_of_Significance_Pass_Cycles, is coded in the beginning of each FGS layer to record the number of coding cycles in significance pass. Given such information, the number of waiting cycles for each MB can be derived. For instance, in Fig. 2, the Num_of_Significance_Pass_Cycles = 3. Thus, the coding of refinement symbols in the Block 0 will be delayed for one cycle after the only two (EOB, Run, Level) symbols are coded. Particularly, we have such syntax for each component (i.e., Luminance, Chrominance DC and Chrominance AC).

- Context Model for Priority Information Coding

In this proposal, we have additionally introduced side information syntax in the FGS slice and the FGS layer. For the side information syntax, we encode each bit with equal probability.

- Num_of_NonZero_MB_ShiftingFactor (3 bits, coded in each FGS slice)

- NonZero_MB_ShiftingFactor (5 bits, coded in each FGS slice)

- Num_of_Significance_Pass_Cycles for luminance(4 bits, coded in each FGS layer)

- Num_of_Significance_Pass_Cycles for chrominance DC(4 bits, coded in each FGS layer)

- Num_of_Significance_Pass_Cycles for chrominance AC (4 bits, coded in each FGS layer)

Besides, at the MB level, we also introduced an additional syntax for the priority information.

- Enable_MB_Coding (1 bit, coded at MB level)

For efficiently encoding the Enable_MB_Coding bit, we have developed 3 extra context models. Fig. 5 shows our context model configuration. For coding the Enable_MB_Coding bit of MB C, we refer to the coding state of the adjacent MB A and B. Specifically, each MB has either an enabled or a disabled coding state. We sum up the binary coding state of MB A and B as context index. Thus, we require additional 3 context models.

Figure 5: Context model for coding the Enable_MB_Coding bit.

| - Binary Mode, Foreground shift factor = 5 | - Binary Mode, Foreground shift factor = 15 | (c) Graceful Mode, Max. foreground shift factor =5 |
| --- | --- | --- |

Figure 6: Selective enhancement modes for ROI functionality.

Table 2. Encoder Parameters

| Sequence | Resolution | Frame Rate | GOP Size | Base-Layer Qp | # of FGS Layer | # of Spatial Layer |
| --- | --- | --- | --- | --- | --- | --- |
| Foreman | CIF | 30 | 1 | 43.1 | 2 | 1 |

- Experiment

For the experiment, we implement our prioritized cyclical block coding based on JSVM1.0 [3]. Specifically, we use the Foreman sequence as an example to demonstrate the functionality of ROI. We compare the regional PSNR and the subjective quality between the schemes with ROI and without ROI.

Table 2 shows our encoder parameters and Fig. 6 depicts the selective enhancement modes used in the experiments. Fig. 6 (a) and (b) show the binary ROI partition with different cycle shifting factors and Fig. 6 (c) gives an example of graceful ROI partition and enhancement. In the graceful mode, each FGS frame includes more than 1 non-zero shifting factor. In the experiments, the ROI partition is derived from the shape information of Foreman sequence. In the binary mode, MBs which have at least one pixel covered by the foreground region are assigned with a uniform shifting factor. But, in the graceful mode, the shifting factor of a MB is determined by the proportion of pixels which are covered by the foreground region. The foreground intensity in Fig. 6 shows the distribution of MB shifting factors.

  - Coefficient Layer Remapping

In our experiment configuration, each FGS slice includes two FGS layers. In the JSVM1.0 [3], the FGS layer 1 has to be coded prior to the FGS layer 2. Thus, the lower priority background region in the FGS layer 1 is coded prior to the higher priority foreground region in the FGS layer 2. To break this constraint, we perform the layer remapping to have the background coefficients originally in the FGS layer 1 be coded in the FGS layer 2. Note that the scalability is maintained since no coefficients are removed from these two layers.

For the layer remapping, the code_block_pattern (cbp) syntax of the blocks in the background region is set to 0 during the encoding of FGS layer 1. Those background coefficients will therefore be quantized and coded in the FGS layer 2. Such layer remapping will shrink the bit rate range of FGS layer 1 and stretch the range of FGS layer 2. In the FGS layer 1, the layer remapping is a technique of ROI functionality. However, in the FGS layer 2, our prioritized cyclical coding is still essential to simultaneously have the scalability and the ROI functionality over the entire bit rate range.

  - PSNR Comparison

Fig. 7 shows the regional PSNR comparison of different ROI coding modes. The legend SEn denotes the performance of our prioritized cyclical block coding. Specifically, the number n represents the foreground shifting factors. In addition, SEn_Remap shows the performance with both the prioritized coding and layer remapping. As shown, enabling the ROI functionality can dramatically increase the PSNR of foreground region. Without the layer remapping, the PSNR of foreground region is the same as the original cyclical block coding at the end of FGS layer 1 (around the bit rate of 700kbits/s). However, with the layer remapping, we can have ROI functionality over the entire bit rate ranges. Particularly, the curve of JSVM1_Remap shows that simply enabling the layer remapping without our prioritized cyclical block coding is not sufficient to offer the ROI functionality over the entire bit rate range.

Fig. 8 further shows the PSNR comparison on the whole frame basis. As shown, without the layer remapping, the PSNR of our prioritized cyclical coding is very close to the one produced by JSVM1.0 [3]. However, significant PSNR drop is observed when the layer remapping is enabled. It can be explained by the fact that more significant coefficients in the background regions are transmitted after the refinement information in the foreground region. Essentially, such coefficient reordering is not rate-distortion optimized.

To assess the coding efficiency, we find that the bit rates of different approaches are very close at the end of FGS layer 2. In other words, enabling our ROI functionality shows almost no penalty on the coding efficiency. In fact, with layer remapping, the total bit rate is slightly reduced because the significance map of the background region is only transmitted once in the FGS layer 2.

  - Subjective Quality Comparison

Fig. 9 ~ 11 shows the subjective quality comparison of different approaches. As shown, enabling our ROI functionality can significantly improve the subjective quality at different bit rates. In addition, with layer remapping, the improvement is more significant and noticeable.

- Conclusions

In this proposal, we use the cyclical block coding in JSVM1.0 to develop a graceful and arbitrary shape region-of-interest (ROI) functionality for SVC. To enable the ROI functionality, we additionally introduce 2 syntax in the FGS slice header, 3 syntax at the FGS layer level, and one syntax at MB level. Experimental results show that the proposed ROI functionality can significantly improve the subjective quality while maintaining the coding efficiency and scalability. Thus, we suggest the group to look into this technology and consider including ROI functionality in the SVC.

References

- [] J. Reichel, H. Schwarz and M. Wien, “Working draft 1.0 of 14496-10:200x/AMD1 scalable video coding”, ISO/IEC JTC1/SC29/WG11, N6901, 2005.

- [] J. Ridge, Y. Bao, M. Karczewicz and X. Wang, "Cyclical block coding for FGS", ISO/IEC JTC1/SC29/WG11, M11509, 2005.

- [] “JSVM 1.0 Software”, ISO/IEC JTC1/SC29/WG11, N6900, 2005.

- [] “Applications and Requirement for Scalable Video Coding”, ISO/IEC JTC1/SC29/WG11, N6880, 2005.

(a) Foreman foreground PSNR

(b)Foreman background PSNR

Figure 7: Regional PSNR comparison.

Figure 8: Whole frame PSNR comparison.

| JSVM1 | SE5_Graceful |
| --- | --- |
| SE15_ Remap | SE5_Graceful_Remap |

Figure 9: Subjective quality comparison at bit rate 300kbits/s.

| JSVM1 | SE5_Graceful |
| --- | --- |
| SE15_ Remap | SE5_Graceful_Remap |

Figure 10: Subjective quality comparison at bit rate 700kbits/s.

| JSVM1 | SE5_Graceful |
| --- | --- |
| SE15_ Remap | SE5_Graceful_Remap |

Figure 11: Subjective quality comparison at bit rate 1100kbits/s.

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | National Chiao-Tung University |  |  |
| Mailing address | 1001 Ta-Hsueh Rd., 30050, HsinChu |  |  |
| Country | TAIWAN |  |  |
| Contact person | Wen-Hsiao Peng |  |  |
| Telephone | +886-0-955-028-902 |  |  |
| Fax | +886-3-5710580 |  |  |
| Email | pawn@mail.si2lab.org |  |  |
| Place and date of submission | Taiwan, 2005/04/13 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Adding Selective Enhancement Functionality to Scalable Video Coding |  |
| Contribution number |  | JVT-O020 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
| ■ | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  | Pending |  |
| Inventor(s)/Assignee(s) |  | Wen-Hsiao Peng, Tihao Chiang, Hsueh-Ming Hang and Chen-Yi Lee |  |
| Relevance to JVT |  | Relevant |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| █ | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: