| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 15th Meeting: Busan, KR, 16-22 April, 2005 | Document: JVT-O021 Filename: JVT-O021.doc |
| --- | --- |

| Title: | The report of CE 3 Improve coding efficiency of entropy coding |  |  |
| --- | --- | --- | --- |
| Status: | Input |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Woong Il Choi School of ICE, Sungkyunkwan Univ. 300 Chunchun-dong, Suwon, Korea Byeungwoo Jeon School of ICE, Sungkyunkwan Univ. 300 Chunchun-dong, Suwon, Korea | Tel: Email: | +82-31-290-7186 creata@ece.skku.ac.kr +82-31-290-7144 bjeon@yurim.skku.ac.kr |
| Source: | Sungkyunkwan Univ. |  |  |

_____________________________

## 1. Introduction

The current JSVM uses the Context Adaptive Binary Arithmetic Coding (CABAC) scheme that is already adopted in MPEG-4 AVC/H.264 standard for entropy coding of each syntax element. Although many coding parts of the current JSVM adopted the same schemes in MPEG-4 AVC/H.264, the statistical characteristics of JSVM syntax elements could be different from H.264 due to newly adopted coding features for scalability such as MCTF and layered structure. Since the entropy coding scheme for JSVM has not been sufficiently studied in terms of coding efficiency, we believe there is some room to further improvement over the conventional entropy coding scheme. To improve the coding efficiency of CABAC in JSVM, we propose a new context modeling based on inter-layer relationship.

## 2. Context modeling for enhancement layer syntax

To improve coding efficiency of the context adaptive arithmetic coding scheme, it is important to design an adequate model that exploits the statistical dependency between current symbol and pre-coded symbol set. When adaptively estimated conditional probability based on context model provides larger probability of MPS (Most Probable Symbol), the coding efficiency can be significantly increased as long as the estimation of conditional probability is valid.

In scalable video coding, each layer contains information of coding element in base layer and the syntax element is predicted from data of base layer macroblock as long as base layer exists. Therefore, we can improve the context modeling of CABAC by exploiting the statistical dependency among layers. The proposed context modeling scheme of CABAC is depicted in Fig. 1. In conventional context modeling scheme, the context template consists of two neighboring syntax elements, A and B, while the context template of the proposed scheme has the additional syntax element C that is located in the corresponding base layer macroblock as shown in Fig. 1.

Fig. 1. The proposed context modeling scheme by using base layer syntax

## 3. Context modeling and binarization for mb_type

In this contribution, we applied the new context modeling only to the syntax element ‘mb_type’. The assignment of context index of each bin of mb_type for each slice type is illustrated in Table 1. The values of variable m and n for additional context are shown in Table 2.

Table 1. Assignment of context index to bin index of mb_type in each slice type

| Syntax element | Bin index |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | >=2 |
| mb_type (I) | 0 | Same with AVC | Same with AVC |
| mb_type (P) | 1,2,3 (model #P0) | Same with AVC | Same with AVC |
| mb_type (B) | 4,5,6,7,8,9 (model #B0) | 10, 11 (model #B1) | 12 |

(Model #P0) mb_type = Intra

| Context | MB(A) is intra | MB(B) is intra |
| --- | --- | --- |
| 1 | Y | Y |
| 2 | Y N | N Y |
| 3 | N | N |

(Model #B0) mb_type = B_Direct_16x16

| Context | MB(A) is skipped* | MB(B) is skipped* | mb_type(C) = 16x16 or 8x8 or MB(C) is skipped* |
| --- | --- | --- | --- |
| 4 | Y | Y | Y |
| 5 | Y | Y | N |
| 6 | Y N | N Y | Y |
| 7 | Y N | N Y | N |
| 8 | N | N | Y |
| 9 | N | N | N |

* If (base_mode_flag = 1 in MB (N) or mb_type (N) = B_Direct_16x16 or MB (N) is not available)

Then MB (N) is skipped

(Model #B1) mb_type = B_8x8

| Context | mb_type(C) = 8x8 |
| --- | --- |
| 10 | Y |
| 11 | N |

Table 2. Values of variables m and n for additional context index for mb_type in enhancement layer

| Initialisation variables | ctxIdx |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| m | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| n | 3 | 76 | 47 | 9 | 22 | 47 | 54 | 73 | 80 | 88 | 76 | 90 | 63 |

We propose additional binarization process for the syntax element, mb_type of enhancement layer macroblock in B slice as shown in Table 3.

Table 3. Binarization for mb_type of B slice in enhancement layer

| Slice type | Value (name) of mb_type | Bin string |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B slice | 0 (B_Direct_16x16) | 0 |  |  |  |  |  |  |  |
|  | 1 (B_L0_16x16) | 1 | 1 | 0 | 0 | 0 |  |  |  |
|  | 2 (B_L1_16x16) | 1 | 1 | 0 | 0 | 1 |  |  |  |
|  | 3 (B_Bi_16x16) | 1 | 1 | 0 | 1 |  |  |  |  |
|  | 4 (B_L0_L0_16x8) | 1 | 1 | 1 | 0 | 0 | 0 | 0 |  |
|  | 5 (B_L0_L0_8x16) | 1 | 1 | 1 | 0 | 0 | 0 | 1 |  |
|  | 6 (B_L1_L1_16x8) | 1 | 1 | 1 | 0 | 0 | 1 | 0 |  |
|  | 7 (B_L1_L1_8x16) | 1 | 1 | 1 | 0 | 0 | 1 | 1 |  |
|  | 8 (B_L0_L1_16x8) | 1 | 1 | 1 | 0 | 1 | 0 | 0 |  |
|  | 9 (B_L0_L1_8x16) | 1 | 1 | 1 | 0 | 1 | 0 | 1 |  |
|  | 10 (B_L1_L0_16x8) | 1 | 1 | 1 | 0 | 1 | 1 | 0 |  |
|  | 11 (B_L1_L0_8x16) | 1 | 1 | 1 | 0 | 1 | 1 | 1 |  |
|  | 12 (B_L0_Bi_16x8) | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 |
|  | 13 (B_L0_Bi_8x16) | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 1 |
|  | 14 (B_L1_Bi_16x8) | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 0 |
|  | 15 (B_L1_Bi_8x16) | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 1 |
|  | 16 (B_Bi_L0_16x8) | 1 | 1 | 1 | 1 | 0 | 1 | 0 | 0 |
|  | 17 (B_Bi_L0_8x16) | 1 | 1 | 1 | 1 | 0 | 1 | 0 | 1 |
|  | 18 (B_Bi_L1_16x8) | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 0 |
|  | 19 (B_Bi_L1_8x16) | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 |
|  | 20 (B_Bi_Bi_16x8) | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 |
|  | 21 (B_Bi_Bi_8x16) | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 1 |
|  | 22 (B_8x8) | 1 | 0 |  |  |  |  |  |  |
|  | 23 to 48 (Intra, prefix only) | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 1 |
| binIdx |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |

## 4. Simulation Result

We implemented the proposed scheme (SKKU) on JSVM 1.0 software and tested it under the Munich test condition. The simulation result is shown in Fig. 2.

Figure 2. R-D performance of the proposed scheme

Although the initial probability of context does not well match to actual statistics of symbol, the inaccurate probability models can gradually approach to actual probability of the symbol due to probability update of arithmetic coding engine. To evaluate the coding gain of the context modeling without other effect, we tested the proposed context modeling with no probability update. Fig 3 shows the R-D performance of the proposed scheme without probability update of arithmetic coding.

Figure 3. R-D performance of the proposed context modeling scheme when probabilities of context does not updated in arithmetic coding engine

## 5. Conclusion

In this document, we proposed a new context modeling using inter-layer relationship and binarization process for the syntax element mb_type of enhancement layer. In the test result, the overall coding gain of the proposed scheme is less than 0.2 dB. Because the bit portion of the syntax element mb_type is less than 10~15% and the initial probability of context can be updated during symbol coding. If there is no update of initial probability for each context, coding gain of the proposed scheme is up to 0.7dB. The proposed scheme requires additional 13 contexts and one table for binarization.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name |  |  |  |
| Mailing address |  |  |  |
| Country |  |  |  |
| Contact person |  |  |  |
| Telephone |  |  |  |
| Fax |  |  |  |
| Email |  |  |  |
| Place and date of submission |  |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  |  |  |
| Title |  |  |  |
| Contribution number |  |  |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: