| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 15th Meeting: Busan, KR, 16-22 April, 2005 | Document: JVT-O024 Filename: JVT-O024.doc |
| --- | --- |

| Title: | Down/up-sample methods for spatial scalability of SVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ilhong Shin and HyunWook Park Dept. EE, KAIST, 373-1 Guseong-dong, Yuseong-gu, Daejeon 305-701, Korea | Tel: Email: | +82-42-869-5466 ssi@athena.kaist.ac.kr hwpark@athena.kaist.ac.kr |
| Source: | KAIST |  |  |

_____________________________

# 1. Introduction

In a scalable video coding, down and up samplings of video sequences are incorporated for spatial scalability. The rate-distortion performance of spatial scalable video coding directly depends on the down and up sampling methods used in the encoder and decoder. A new up-sampling method is proposed for SVC. Also a new down-sampling method that can be efficiently matched with the proposed up-sampling method is developed. The main idea is derived from the DCT domain down/up-sampling scheme, which is efficiently derived from combination of the forward and backward DCT kernels. Current down/up-sampling filters of SVC have no phase-shift after employment of filters. Typical down-sampling from YUV444 to YUV420 suffers half-pel shift. Therefore we propose two down/up sampling methods, which should be used appropriately in the encoder and decoder of SVC. First, we propose down/up-sampling filters derived from the type-2 DCT, which is a typical DCT for image compression. It suffers half-pel shift due to characteristics of the type-2 DCT. Therefore, encoder and decoder should use the same type-2 DCT filters. Second proposal employs the type-1 DCT filters. Due to characteristics of the type-1 DCT, the developed down/up-sampling kernels have no phase shift. Also, the developed down/up-sampling methods can use overlapping pixels near the block boundary to reduce the block artifacts. Since current SVC requires only two-fold down/up-sampling, we developed the two-fold down/up-sampling filters to meet the requirements. The developed down/up-sampling methods provide more accurate results than that of SVM 3.0. The developed type-2 DCT down/up-sampling and its experimental results are given in sections 3 and 4, respectively. Type-1 DCT down/up-sampling and its experimental results are given in sections 5 and 6, respectively. Section 7 shows frequency response curves of the proposed type-1, type-2, and SVM 3.0 up-sampling filters.

# 2.  Down/up-sampling methods of the current SVM 3.0

Fig. 1 shows the pixel coordinates in 1D for down/up-sampling with/without phase shift. In svm 3.0, the down-sampling filter produces the pixel coordinates represented by star in Fig. 1(a), which uses 13-tap linear phase filter. Also the up-sampling filter of the SVM 3.0 produces the pixel coordinates of X in Fig. 1(a). In the practical implementation, the even-index pixels do not change its values in the two-fold interpolation, and only odd-index pixels are interpolated, which uses 6-tap filter. The down-sampling and interpolation filters are given as follows,

eq. (1)

eq. (2)

In SVM 3.0, up-sampling of the decoded images is performed only for I_BL mode [1], whereas up-sampling of the residual signal uses a simple bi-linear interpolation. Hence, the developed up-sampling method can be used for I_BL mode. Although down-sampling method is an informative issue of standardization, the exact match of filters in down/up sampling is necessary.

Typical down-sampling filters suffer half-pel shift like YUV420 down-sampling as shown in Fig. 1(b). Therefore, we propose new down/up-sampling filters having half-pel shift. Also we propose other filters without phase shift. The latter up-sampling filter can be used in conjunction with the down-sampling filter of SVM 3.0, since the SVM 3.0 down-sampling filter has no phase shift. Also the up-sampling filter of SVM 3.0 can be used with the proposed down-sampling filter without phase-shift.

(a)                                                (b)

Fig. 1. Pixel coordinates in 1D.

# 3.  Down/up-sampling methods using type-2 DCT

DCT family has four classes of DCT, such as type-1, 2, 3, and 4. The most common DCT is the type-2, which is used in the image compression due to asymptotic equivalence of the optimal KL transform. The definition of type-2 DCT is given as follows,

eq. (3)

where  is the input signal and N is the input sample size. If the type-2 DCT is used in down-sampling, the down/up-sampled coordinates have a half-pel shifted result. However, up-sampling using type-2 DCT recovers original pixel coordinates as shown in Fig. 2.

Fig. 2. Pixel coordinate of down/up sampling using type-2 DCT.

Figure 3 represents the down-sampling scheme using type-2 DCT. The signal length of type-2 DCT is N. After type-2 DCT of N-sample signal, take N/2 samples of low frequency. Then, the type-2 IDCT is performed on the N/2 samples, resulting in the down-sampled signal. We can make the efficiently combined down-sampling matrix as follows:

eq. (4)

where D2 is the down-sampling matrix using type-2 DCT, which consists of the N/2xN matrix. The 2D down-sampling can be separately performed using columnwise (vertical) down-sampling followed by row (horizontal) down-sampling.

Fig. 3. Down-sampling method using type-2 DCT.

Figure 4 represents the up-sampling scheme using type-2 DCT. The signal length of type-2 DCT is N, and N zeros are appended in the high frequency region after type-2 DCT. Then type-2 IDCT of 2N samples is performed to obtain two-fold up-sampled data. We can also make the combined up-sampling matrix as follows:

eq. (5)

where U2 is the up-sampling matrix using type-2 DCT, which consists of the 2NxN matrix. The 2D up-sampling can be also  separately performed using columnwise (vertical) up-sampling followed by row (horizontal) up-sampling.

Fig. 4. Up-sampling method using type-2 DCT.

Although the down/up sampling kernels are derived from the floating-point DCT, it is possible to implement using 16-bit integer-type operations in the up-sampling and down-sampling. Even though the down-sampling matrix size is different from the up-sampling one, the calculation procedure for down-sampling is identical to that for up-sampling. Therefore, the computational complexities of down/up-sampling are the same. Although an efficient implementation of the proposed method is not described in this documents, it is possible to make fast filtering of the proposed down/up-sampling filter. Table 1 shows comparison of the computational complexity of the SVM 3.0 and the proposed up-sampling method. The number of additions in Table 1 includes the final rounding operation.

Table 1. The computational complexity of the up-sampling method

(N=8 was applied for the proposed method.)

| Per pixel | Multiplication | Addition |
| --- | --- | --- |
| SVM 3.0 | 1.5 | 4.5 |
| Proposed | 2.34 | 8.66 |

Thankful to efficient filtering scheme, the computational complexity of the proposed method is comparable to that of SVM 3.0. Since the residual signal should also be up-sampled with phase shift in the proposed type-2 filtering method and the bi-linear interpolation of SVM 3.0 has no phase shift, the residual upsampling should be performed by the proposed type-2 up-sampling filter instead of the bi-linear interpolation. Or more simple filter with phase shift can be applied for the efficiency of computational complexity. For example, N=2 type-2 DCT up-sampling filter can be used with phase shift, where its computational complexity is very low.

# 4.  Experimental results using type-2 DCT down/up-sampling filters

We implemented the proposed down/up-sampling method in the SVM 3.0 video codec [2]. For an impartial comparison, the proposed down/up-sampling kernels were converted to the 16-bit integer type. Although the PSNR loss is expected in the integer-type conversion, the loss is negligible. The value of 32 is multiplied to the kernel and then rounded to the nearest integer for converting the proposed kernels to integer format. We can further reduce the number of multiplications by using shift operation in the integer implementation.

We used two video sequences City and Harbour which have a spatial resolution of 704576 (4CIF) and 30 Hz frame rate. The base layer were encoded with 352288 (CIF) after down-sampling of SVM 3.0 and the proposed method. The enhancement layer has the 4CIF resolution by using up-sampling of SVM 3.0 and the proposed method for inter-layer prediction [1]. Although the rate control does not exist in SVC, we tried to set nearly the same bitrate in base layer. The GOP was 1, i.e., all frames were coded as intra frames. The proposed method is efficient for high-bitrate coding of base-layer, i.e., less quantization of base-layer gives more accurate up-sampling results. Therefore, we use an extreme experiment condition, where QP of the base-layer is set to zero. Actually, the base-layer images from the proposed method and SVM 3.0 are different due to their different down-sampling methods. To meet the same bitrate condition in the base layer, we set QPs of base-layer for the proposed method and SVM 3.0 as shown in Table 2.

Table 2. QP of the base layer images for the proposed and SVM 3.0 methods

| Sequence | Proposed (QP) | SVM 3.0 (QP) |
| --- | --- | --- |
| City (high) | 2 | 0 |
| City (middle) | 23 | 21 |
| Harbour (high) | 3 | 0 |
| Harbour (middle) | 21.5 | 20 |

The high and middle in Table 2 denote the high and medium image quality of the base layer images. Fig. 5 shows the rate-PSNR curves of the proposed one and svm 3.0 with high bitrate of the base-layer. In Fig.5, D2 and U2 denote the proposed down and up-sampling filters using type-2 DCT, respectively. SVM (down) and SVM (up) mean the applied SVM 3.0 down and up-sampling filters, respectively.

(a) City 30 Hz

(b) Harbour 30 Hz

Fig. 5. Rate-PSNR curves of proposed type-1 DCT and SVM 3.0 down/up-sampling with high bitrates.

As shown in Fig. 5, the proposed method outperforms the SVM 3.0 down/up-sampling. The maximum improvements of PSNR are about 1~3 dB. Figure 6 shows rate-PSNR curves of middle bitrates of the base layer.

(a) City 30 Hz

(b) Harbour 30 Hz

Fig. 6. Rate-PSNR curves of proposed and SVM 3.0 down/up-sampling with middle bitrates.

The proposed method outperforms SVM 3.0 also in middle bitrate. The improvements of PSNR are 1.5 dB (City), and 2.5 dB (Harbour).

# 5.  Down/up-sampling methods using type-1 DCT

The type-1 DCT has no phase-shift in down/up-sampling. The definition of type-1 DCT is given as follows,

eq. (6)

where  is the input signal and N is the input sample size. If the type-1 DCT is used in down-sampling, the down-sampled coordinate has no phase shift. Also the up-sampling using type-1 DCT introduces no phase shift as shown in Fig. 7.

Fig. 7. Pixel coordinate of down/up sampling using type-1 DCT.

Figure 8 represents the down-sampling scheme using type-1 DCT. The signal length of type-1 DCT is N+1. After the type-1 DCT, take only N/2+1 samples of low frequency. Then, the type-1 IDCT is performed on the N/2+1 samples, resulting in the down-sampled signal. We can make the efficiently combined down-sampling matrix as follows:

eq. (7)

where D1 is the down-sampling matrix using type-1 DCT, which consists of the (N/2+1)x(N+1) matrix. The 2D down-sampling can be separately performed using columnwise (vertical) down-sampling followed by row (horizontal) down-sampling.

Fig. 8. Down-sampling method using type-1 DCT.

Figure 9 represents the up-sampling scheme using type-1 DCT. The original type-1 DCT cannot provide the Nyquist interpolation kernel. To make the Nyquist interpolation kernel, the highest frequency coefficient in the transform domain should be halved. The signal length of type-1 DCT is N+1, and N zeros are appended in the high frequency region after type-1 DCT. Then, type-1 IDCT of 2N+1 samples is performed to obtain up-sampled data.

We can make the combined up-sampling matrix as follows:

eq. (8)

where U1 is the down-sampling matrix using type-1 DCT, which consists of the (2N+1)x(N+1) matrix. The 2D up-sampling can be also separately performed using columnwise (vertical) up-sampling followed by row (horizontal) up-sampling.

Fig. 9. Up-sampling method using type-1 DCT.

In our implementation with pixel overlapping, type-1 DCT with N+1=11 and type-1 IDCT with 2N+1=21 samples are used for the two-fold up-sampling. Therefore, 1D input signal is 11 point data and the up-sampled signal becomes 21 point data. The left and right parts (or upper and lower parts) of the current 11x11 block are from the left and right blocks (or upper and lower blocks in vertical direction) of the current 8x8 block in Fig. 10. The additional 3 points of input signal make the up-sampling process to reduce the block artifacts, which is visually annoying. However, the required number of up-sampling points is 16. Therefore, we can use the remaining up-sampled pixels for overlapping of up-sampling block. Figure 11

Figure F shows the proposed overlapping scheme of up-sampling.

The points of 16~20 at the previous block (or current block) and 0~4 at the current block (or next block) overlap with each other. It should be noted that the even points are un-changed due to characteristics of the two-fold Nyquist interpolation kernel. The points of 2~17 are the up-sampled 16 points of the current block. Therefore, two pairs of 17 (current block) and 1(next block), and 19 (previous block) and 3 (current block) are overlapped as shown in Fig.11. The overlapped points will be averaged or filtered to further reduce the blocking artifacts. Also down-sampling filter can use this overlapping scheme. Actually, for the sake of computational efficiency, the overlapping pixels after up-sampling can be simply truncated instead of averaging.

Fig. 10. Overlapping blocks of input signal for up-sampling.

Fig. 11. Reuse of overlapping pixels in up-sampled signal.

Although the down/up sampling kernels are derived from the floating-point DCT, it is possible to implement using 16-bit integer-type operations in the up-samping and down-sampling. The calculation procedure of down and up-samplings using type-1 DCT are different due to some manipulation to make Nyquist interpolation kernel in up-sampling. However, the down and up-samplings can use fast implementation scheme like type-2 DCT kernels. The efficient implementation of the proposed method is not described in the document. Table 3 shows comparison of the computational complexity of SVM 3.0, and the proposed up-sampling methods.

Table 3. The computational complexity of up-sampling method

(N=10 was applied for the proposed method.)

| Per pixel | Multiplication | Addition |
| --- | --- | --- |
| SVM 3.0 | 1.5 | 4.5 |
| Proposed | 2.21 | 8.23 |

The computational complexity of the proposed method is larger than that of SVM 3.0. Since the type-1 DCT method produces no phase shift, both or one of SVC down/up-sampling filters can be replaced with the proposed type-1 DCT filters.

# 6.  Experimental results of type-1 DCT down/up-sampling filters

We implemented the proposed type-1 down/up-sampling methods in SVM 3.0 video codec [2]. The proposed down/up-sampling kernels were converted to the 16-bit integer type. Although the PSNR loss is expected in the integer-type conversion, the loss is negligible.

We used two video sequences of City and Harbour which have a spatial resolution of 704576 (4CIF), 30 Hz frame rate, and 298 frames. The base layer were encoded with 352288 (CIF) after down-sampling of SVM 3.0 and the proposed method. The enhancement layer uses the 4CIF resolution with up-sampling of SVM 3.0 and the proposed method for inter-layer prediction [1]. Although the rate control does not exist in SVM 3.0, we set nearly the same bitrate in the base layer. The GOP size was 1, i.e., all frames were coded as intra frames. The proposed method is efficient for high-bitrate coding of base-layer, i.e., less quantization of base layer images gives more accurate up-sampling results. Therefore, we use an extreme experiment condition, where QP of base-layer is set to zero. Since the type-1 DCT down/up-sampling introduces no phase shift, we perform three experiments. The comparisons of the proposed type-1 DCT up-sampling and the SVM 3.0 up-sampling is performed for the down-sampled image from the SVM 3.0 down-sampling or the proposed type-1 DCT down-sampling. Also the comparison of the proposed type-1 DCT down/up-sampling and the SVM 3.0 down/up-sampling is performed. To meet the same bitrate condition in the base-layer, we set QP of the base-layer for the proposed method and SVM 3.0 as shown in Table 4.

Table 4. QP of thew base layer images for the proposed and SVM 3.0 methods

| Sequence | Proposed (QP) | SVM 3.0 (QP) |
| --- | --- | --- |
| City (high) | 0 | 0 |
| City (middle) | 20 | 20 |
| Harbour (high) | 0 | 0 |
| Harbour (middle) | 20 | 20 |

The high and middle in Table 4 denote the high and medium image quality of the base-layer images. We can set the same QP due to the same down-sampling filter in Table 4. Figure 12 shows the rate-PSNR curves of the proposed up-sampling and the SVM 3.0 up-sampling both with the proposed type-1 DCT down-sampling filter. In Fig.12, D1 and U1 denote the proposed down and up-sampling using type-1 DCT, respectively. SVM (down) and SVM (up) mean the applied SVM 3.0 down and up-sampling filters, respectively.

(a) City 30 Hz (High)

(b) City 30 Hz (Middle)

(c) Harbour 30 Hz (High)

(d) Harbour 30 Hz (Middle)

Fig. 12. Rate-PSNR curves of proposed and SVM 3.0 up-sampling with down-sampling using type-1 DCT.

As shown in Fig. 12, the proposed method is significantly better than SVM 3.0 up-sampling. The maximum improvements of PSNR are about 0.5 dB (a), 0.35 dB (b), 0.5 dB (c), and 0.4 dB (d). In the middle bitrates with base-layer QP of 20, the performance is still better than SVM 3.0. The proposed up-sampling filter using type-1 DCT gives better performance than SVM 3.0 with slightly increased computational complexity. Figure 13 shows the rate-distortion curves of the proposed type-1 DCT up-sampling and the SVM 3.0 up-sampling with the same SVM 3.0 down-sampling filter.

(a) City 30 Hz (High)

(b) City 30 Hz (Middle)

(c) Harbour 30 Hz (High)

(d) Harbour 30 Hz (Middle)

Fig. 13. Rate-PSNR curves of proposed and SVM 3.0 up-sampling with down-sampling using SVC.

As shown in Fig. 13, the proposed up-sampling method using type-1 DCT is slightly better than SVM 3.0 up-sampling. The maximum improvements of PSNR are about 0.25 dB (a), 0.2 dB (b), 0.2 dB (c), and 0.15 dB (d).

(a) City 30 Hz (High)

(b) Harbour 30 Hz (High)

Fig. 14. Rate-PSNR curves of the proposed type-1 DCT down/up-sampling and the SVM 3.0 down/up-sampling.

Figure 14 shows performance of the proposed down/up-sampling filters using type-1 DCT compared with the SVM 3.0 down/up-sampling filters. The QPs of base-layer in the proposed type-1 up-sampling are selected to 2.5 (a) and 3.0 (b).  As shown in Fig. 14, the proposed down/up-sampling method using type-1 DCT outperforms the SVM 3.0 down/up-sampling method. The maximum improvements of PSNR are about 2 dB (a) and 3 dB (b).

# 7.  Frequency response of the proposed up-sampling filters and the SVM up-sampling filters

We proposed type-1 and type-2 DCT down/up-sampling filters. The proposed down/up-sampling methods are perfomed by using block by block processing. Hence, multirate filter-banks system is required to obtain frequency response of down/up-sampling filters. We obtain frequency response of the proposed method by using multirate filter-banks. Fig. 15 shows frequency response of the proposed up-sampling filter using type-2 DCT and the SVM up-sampling filter. In Fig. 15, the proposed type-2 DCT up-sampling (N=8) has narrow transition region around /2. If we apply larger N, the frequency response can be further improved with increased computational complexity. Fig. 16 shows frequency response of the proposed up-sampling filter using type-1 DCT and the SVM up-sampling filter. In Fig.16, the proposed type-1 DCT up-sampling (N=10) method shows very narrow transition band.

Fig. 15. Frequency response of the proposed up-sampling filter using type-2 DCT and the SVM up-sampling filter.

Fig. 16. Frequency response of the proposed up-sampling filter using type-1 DCT and the SVM up-sampling filter.

# 8.  Conclusion

In this contribution, we proposed two new down/up-sampling methods for performance improvement of spatial scalability in the SVC. When the half-pel shift is required in the down/up-sampling, type-2 DCT kernels can be exploited for spatial scalability in the SVC. Type-1 DCT kernels provides no phase shift. The proposed two methods show the improved rate-PSNR performance. The proposed contribution will help SVC spatial scalability be more realistic in many applications of video coding.

# 9.  References

[1] J. Reichel, M. Wien, H. Schwarz, ‘Scalable Video Model 3.0,’ ISO/IEC JTC1/ SC29/ WG11 N6716, October 2004.

[2] ‘SVM 3.0 Software,’ ISO/IEC JTC1/ SC29/ WG11 N6717, October 2004.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Korea Advanced Institute of Science and Technology (KAIST) |  |  |
| Mailing address | KAIST, Dept. EE, 373-1 Guseong-dong, Yuseong-gu, Daejeon 305-701, Korea |  |  |
| Country | South Korea |  |  |
| Contact person | Professor, HyunWook Park |  |  |
| Telephone | +82-42-869-5466 |  |  |
| Fax | +82-42-869-8066 |  |  |
| Email | hwpark@athena.kaist.ac.kr |  |  |
| Place and date of submission | Busan Korea, April 13,2005. |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Down/up-sample methods for spatial scalability of SVC |  |
| Contribution number |  | JVT-O024 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
| X | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  | Planned |  |
| Inventor(s)/Assignee(s) |  | Ilhong Shin, HyunWook Park |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: