| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 15th Meeting: Busan, KR, 16-22 April, 2005 | Document: JVT-O028 Filename: input.doc NOTE – That first character after the dash up there in the document number is an "O", not a zero. |
| --- | --- |

| Title: | Scalable Video Coding – Proposal to reduce memory requirement for MCTF |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | ChongSoon Lim MenHuang Lee ShengMei Shen Satoshi Kondo | Tel: Email: | (+65) 65505487 (cslim@psl.com.sg) (mhlee@psl.com.sg) (shen@psl.com.sg) (kondo.satoshi@jp.panasonic.com ) |
| Source: | Panasonic Singapore Laboratories Pte. Ltd. |  |  |

_____________________________

- Introduction

The MCTF operation in SVC requires a decoder to store decoded residuals of a high pass picture in a frame buffer for the inverse update operation.  These decoded residuals are also used for the reconstruction of the pictures after the inverse prediction operation.

- Problems

To store the decoded residuals (inter macroblocks) of a high-pass picture, a decoder will require a memory size of more than one frame size due to reason that the dynamic range of the residuals are more than 8 bits.  For some memory types, this will require 2 bytes of memory space to store each sample and thus 3 bytes for each pixel (4:2:0).

To simplify the motion interpolation step of the inverse update operation, a decoder will probably store the intra coded macroblocks of the high pass picture in a separate frame memory.  The reason for performing such a step is because intra macroblocks are set to zero values during the inverse update operation. Thus if they are stored in the same location as inter macroblock residuals, special operation are needed to be performed to mask the intra macroblocks to zero values during the motion compensated update process.

Therefore to efficiently store a high pass picture, some decoders are required to allocate a memory space of three pictures in worst case (Two for a residual picture and one for reconstructed intra samples).

- A Simple Solution

We propose a simple solution to reduce this memory requirement for high pass pictures.  The inverse update operation is currently defined as

UHaarsx, 2k=12 h[x+mU0, k+rU0]

U53sx, 2k=14h[x+mU0, k+rU0]+h[x+mU1, k−1−rU1]

where h is motion-compensated residuals from a high pass picture.

We propose to move the ½ operation to an earlier stage of the decoding process to reduce the memory requirement to store the residuals.  Thus h will become motion compensated 8 bits residuals instead.

Therefore a decoded high pass picture will be stored into two picture memory spaces (one residual picture memory and one reconstructed picture memory).

For every decoded “residuals” of a high pass picture, only the first 8 bits of the decoded residual are stored in a residual picture memory for inverse update operation.  The last bit of the decoded residual is stored in a reconstructed frame memory together with the intra macroblocks of the high pass picture.

For every decoded and reconstructed intra macroblocks of a high pass picture (assuming constrained_intra_pred_flag is set to 1), the reconstructed intra macroblock are stored in the reconstructed frame memory and the same locations in the residual picture memory are set to zero values.

- Unification of Fractional Sample Interpolation Process (informative)

The same process (including the clip1 operations) defined 8.4.2.2 of MPEG AVC can be used for both motion compensated update and prediction process.  Before storing the signed 8 bits residuals in the residual picture memory, the residuals can be converted to unsigned 8 bits residuals by adding a value of 128 to each samples.  By doing such an operation, a common fractional sample interpolation process can be used for both update and prediction operation.  The residual picture will be treated like an unsigned 8 bits reference picture similar to a reconstructed picture.

Thus if the residual picture is an unsigned 8 bits reference picture, the update operation will become:

“

- Each sample S'L[ x, y] with x = xP..xP + 15 and y = yP..yP + 15 is modified by

S'L[ x, y] = Clip1Y( S'L[ x, y] –  ( updMbWL[ x – xP, y – yP ] )>>1 + 128 )

And the prediction operation will become

- Each sample S'L[ x, y] with x = xL..( xL + predWidth – 1 ) and y = yL..( yL + predHeight – 1 ) is modified by

S'L[ x, y] = Clip1Y( S'L[ x, y]  +( R'L[ x, y]<<1) – 256 +  predPartL[ x – xL, y – yL ] )

Where S'L[ x, y] are samples from reconstructed picture and R'L[ x, y] are samples from residual picture.

- Suggested Changes to SVC WD 1.0

- A clipping operation is needed for the decoded residuals.  Currently AVC only defines clipping on the reconstructed samples but not on the decoded residuals.

In 8.5.8 of MPEG-4 AVC

“

The bitstream shall not contain data that results in any element hij of h with i, j = 0..3 that exceeds the range of integer values from –215 to 215–33, inclusive.

After performing both the one-dimensional horizontal and the one-dimensional vertical inverse transforms to produce an array of transformed samples, the final constructed residual sample values shall be derived asb

rij=( hij+ 25)>>6   with   i, j = 0..3

“

This means that the decoded residuals can have legal values that are 10 bits.  Thus additional clipping operation is needed to restrict the residuals to 9 bits in SVC.

r’ij  = Clip3 (-256, 255, rij )  with 	i,j = 0..3

- Changes to last section of S.8.8.1 to remove the right shift.

- The picture updPic, which is stored in the decoded picture buffer, is modified as follows.

- Let S'L[ x, y], S'Cb[ x, y], and S'Cr[ x, y] represent the luma and chroma samples arrays of the picture updPic.

- The inverse macroblock scanning process in subclause S.6.4.1 is invoked with mbAddr as input and the output is assigned to the luma location ( xP, yP ).

- When chroma_format_idc is not equal to 0, a chroma location ( xC, yC ) is derived by

xC = xP / SubWidthC
yC = yP / SubHeightC

- Each sample S'L[ x, y] with x = xP..xP + 15 and y = yP..yP + 15 is modified by

S'L[ x, y] = Clip1Y( S'L[ x, y] – ( updMbWL[ x – xP, y – yP ] >> 1 ) )

- When chroma_format_idc is not equal to 0, each sample S'C[ x, y] (with C being Cb or Cr) with x = xC..xC + MbWidthC – 1 and y = yC..yC + MbHeightC – 1 is modified by

S'C[ x, y] = Clip1Y( S'C[ x, y] – ( updMbWC[ x – xP, y – yP ] >> 1 ) )

- Residual picture definition

residual picture: a picture composed of first 8 bits of the residual and/or decoded samples or data elements.

- Changes to last section of S8.8.2 (Motion-compensated prediction process)

- Each sample S'L[ x, y] with x = xL..( xL + predWidth – 1 ) and y = yL..( yL + predHeight – 1 ) is modified by

S'L[ x, y] = Clip1Y( S'L[ x, y] + ( R'L[ x, y]<<1) + predPartL[ x – xL, y – yL ] )

- When chroma_format_idc is not equal to 0, each sample S'C[ x, y] (with C being Cb or Cr) with x = xC..( xC + predWidthC – 1 ) and y = yC..( yC + predHeightC – 1 ) is modified by

S'C[ x, y] = Clip1Y( S'C[ x, y] +( R'L[ x, y]<<1) + predPartC[ x – xC, y – yC ] )

- References

[1] “Working Draft 1.0 of 14496-10:200x/AMD1 Scalable Video Coding”, MPEG N6901, January 2005, Hong Kong, China.

[2] “Joint Scalable Video Model (JSVM) 1.0 Reference Encoding Algorithm Description”, MPEG N6899, January 2005, Hong Kong, China.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Matsushita Electric Industrial Co., Ltd. (Panasonic) |  |  |
| Mailing address | 1006 Kadoma, Kadoma City, Osaka 571-8501 |  |  |
| Country | Japan |  |  |
| Contact person | Satoshi Kondo |  |  |
| Telephone | +81-6-6900-9689 |  |  |
| Fax | +81-6-6900-9699 |  |  |
| Email | kondo.satoshi@jp.panasonic.com |  |  |
| Place and date of submission | Busan, KR, 16-22 April, 2005 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Scalable Video Coding –Proposal to reduce memory requirement for MCTF |  |
| Contribution number |  | JVT-O028 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
| X | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: