| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 15th Meeting: Busan, Korea 16-22 Apr., 2005 | Document: JVT-O039 Filename: JVT-O039.doc |
| --- | --- |

| Title: | Report on Core experiment 10 : Non Dyadic Spatial Scalability |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | E. Francois, J. Viéron, G. Marquant, N. Burdin, P. Lopez THOMSON R&D France, Video Compression Lab, Corporate Research Rennes, 1, Avenue Belle Fontaine – CS17617, 35 576 Cesson Sevigne Cedex – France | Tel: Email: | +33 2 99 27 32 62 edouard.francois@thomson.net |
| Source: | Thomson |  |  |

_____________________________

# Introduction

This document reports the results of CE10 related to Non Dyadic Spatial Scalability, that is, corresponding to size ratio between high layer and its lower layer non equal to 2. It addresses the  Extended Spatial Scalability functionality of SVC requirements, [N6880].

Coordinator: Thomson

| Proponent | Partner | Contact | Email |
| --- | --- | --- | --- |
| Thomson | France Telecom | E.Francois | edouard.francois@thomson.net |

# CE10 description

As described in [N6898], 3 size ratios between low and high resolution layers are considered :

- low layer picture size / high layer picture size = 3/4,

- low layer picture size / high layer picture size = 2/3,

- low layer picture size / high layer picture size = 3/5.

## Compared solutions

For each of these ratios, 3 different solutions are compared:

- Anchor solution : dyadic scalable coding , such as defined in [JSVM1.0], with upsampling of the low dyadic layer to get the right low-layer dimensions (for instance, if the real ratio is 3/4, a 3/2-upsampling is applied to the low dyadic layer, so the upsampled low-layer dimensions are 1/2*3/2=3/4 of the high-layer dimensions).

decoded

H pict.

upsampled decoded

L pict.

upsampling

dyadic

coding /

decoding

H

pict.

dyadic

L pict.

decoded

L pict.

Figure 1: Extended Spatial Sclaability using Dyadic Spatial Scalability

- Proposal1 : scalable coding with a 2/3 size-ratio between low- and high-layers , and up or downsampling of the low 2/3-layer to get the right low-layer dimensions (for instance, if the real ratio is 3/4, a 9/8  upsampling is applied to the low 2/3-layer, so the upsampled low-layer dimensions are 2/3*9/8=3/4 of the high-layer dimensions). Proposal 1 is fully described in document JVT-MO042.

decoded

H pict.

upsampled decoded

L pict.

resampling

ratio 2/3

coding /

decoding

H

pict.

2/3

L pict.

2/3

decoded

L pict.

Figure 2: Extended Spatial Scalability using Spatial Scalability with ratio H/L of 2/3

- Proposal2 : generic scalable coding ; any value can be considered as high-low size-ratio, and low- and high-layers are directly encoded with their real dimensions. Proposal 2 is fully described in document JVT-MO041.

decoded

H pict.

decoded

L pict.

ratio 2/3

coding /

decoding

H

pict.

L

pict.

Figure 3: Generic Extended Spatial Scalability

## Low layer pictures resampling

Low layer resampling is achieved using the Lanczos filter, defined as follows. For each sample of the high layer, its position in the base layer is first computed as (x,y). The phase is then computed as px=x-floor(x) and py=y-floor(y). Then the sample value is computed using the lanczos3 filter, defined as follows, [KT90]:

L3(k)=sin(π(k+px))π(k+px)sin(π(k+px)/3)π(k+px)/3 ,k<20 , k≥2

# Test conditions definition

The test sequences dimensions proposed in [N6898] are not practically usable because the low-layers dimensions are not multiple of 16. Consequently it has been necessary to define new test sequences dimensions, and related bit-rates conditions.

The new test sequences dimensions have been chosen, for a given ratio between high and low resolution layers, in order to be multiple of 16. This leads to the following dimensions :

| RATIO | HIGH layer dimensions | LOW layer dimensions |
| --- | --- | --- |
| 3 / 4 | 704 * 576 @ 30Hz | 528 * 432 @ 30Hz |
| 2 / 3 | 672 * 576 @ 30Hz | 448 * 384 @ 30Hz |
| 3 / 5 | 640 * 560 @ 30Hz | 384 * 336 @ 30Hz |

As the high- and low-layers dimensions are different from those of the Munich test sequences (MPEG meeting march 04), the related bitrates have to be adjusted. For CITY sequence, the target bitrate at 4CIF@30Hz was 1024 kbps, and for CIF@30Hz, that corresponds to 1/4 of the 4CIF dimensions, the maximum target bitrate was 512 kbps. The bitrate for any other size ratio to 4CIF format can then be computed by interpolating between these two points, as illustrated in Figure 1, in which Munich CITY test points are highlighted in red. The same policy can be applied to CREW, SOCCER and HARBOUR sequences (the interpolation curve is provided in Figure 5).

Figure 4: bitrate – size ratio curve for CITY sequence

Figure 5 : bitrate – size ratio curve for CREW, SOCCER, HARBOUR sequences

Consequently, the new test conditions are the following:

| Ratio Low/High 3/4 | bitrates | Low - 528*432 | High - 704*576 |
| --- | --- | --- | --- |
|  | City | 810 kbps | 1024 kbps |
|  | Crew, Harbour, Soccer | 1190 kbps | 1500 kbps |
| Ratio Low/High 2/3 | bitrates | Low - 448*384 | High - 672*576 |
|  | City | 720 kbps | 1000 kbps |
|  | Crew, Harbour, Soccer | 1050 kbps | 1470 kbps |
| Ratio Low/High 3/5 | bitrates | Low - 384*336 | High - 640*560 |
|  | City | 610 kbps | 980 kbps |
|  | Crew, Harbour, Soccer | 890 kbps | 1430 kbps |

The generation of the low layer sequences has been performed by downsampling the high-layer sequences using the following separable linear polyphase filtering:

| Ratio | Tap n° | Filter taps | Divisor |
| --- | --- | --- | --- |
| 3 / 4 | 1 2 3 | -33,97,384,97,-33 -15,3,333,222,-35,4 4,-35,222,333,3,-15 | 512 512 512 |
| 2 / 3 | 1 2 | 6,-2,-27,77,168,77,-27,-2,6 8,-20,2,138,138,2,-20,8 | 256 256 |
| 3 / 5 | 1 2 3 | -24,-9,88,146,88,-9,-24 -28,17,118,137,53,-26,-15 -15,-26,53,137,118,17,-28 | 256 256 256 |

The new test sequences are available on the ftp site ftp.tnt.uni-hannover.de, in sub-directory 72_Busan\CE10.

# Results

## Ratio Low / High layer of ¾

### Processed simulations

For the dyadic case, an upsampling with a ratio of 3/2 must be applied on the low layer to get the right picture dimensions.

For the ESS_3_2 cases, the low layer must be first cropped to get low layer dimensions multiple of 16. Then the low layer must be upsampled, with a ratio of 9/8 f. The upsampled low layer is smaller than the actual low layer, and thus must be compared to a cropped version of the low layer.

| Simu Type | decoded high layer size | decoded low layer size | decoded Low layer resampling |
| --- | --- | --- | --- |
| Dyadic | 704*576 | 352*288 | 528*432 - 3/2 |
| ESS with Ratio 3/2 | 704*576 | 464*384 (crop) | 522*432 - 9/8 (crop) |
| Generic ESS | 704*576 | 528*432 | no |

Table 1 : processed simulations for ratio 3/4

### Results

| CITY | low layer | 810 | kbps |  |  | high layer | 1024 | kbps |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | real bitrate | psnr Y | psnr U | psnr V | delta bitrate | real bitrate | psnr Y | psnr U | psnr V | delta bitrate |
| dyadic | 804,83 | 29,83 | 43,31 | 45,39 | -0,64% | 1010,48 | 32,24 | 42,76 | 45,34 | -1,32% |
| ratio 3/2 | 822,71 | 34,89 | 43,24 | 45,16 | 1,57% | 1020,97 | 33,69 | 42,91 | 45,24 | -0,30% |
| generic | 827,38 | 35,84 | 43,94 | 45,91 | 2,15% | 1034,01 | 33,96 | 42,92 | 45,21 | 0,98% |
| Generic lower base | 681,44 | 35,236 | 43,411 | 45,442 |  | 1004,5552 | 34,19 | 42,924 | 45,113 | -1,90% |

| CREW | low layer | 1190 | kbps |  |  | high layer | 1500 | kbps |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | real bitrate | psnr Y | psnr U | psnr V | delta bitrate | real bitrate | psnr Y | psnr U | psnr V | delta bitrate |
| dyadic | 1211,20 | 35,23 | 40,78 | 40,03 | 1,78% | 1490,08 | 34,97 | 40,41 | 40,43 | -0,66% |
| ratio 3/2 | 1176,53 | 36,22 | 40,75 | 40,05 | -1,13% | 1511,90 | 35,35 | 40,62 | 40,75 | 0,79% |
| generic | 1201,04 | 36,27 | 41,00 | 40,77 | 0,93% | 1509,83 | 34,99 | 40,42 | 40,45 | 0,66% |
| Generic lower base | 1010,60 | 35,672 | 40,695 | 40,313 |  | 1507,86 | 35,26 | 40,55 | 40,65 | 0,52% |

| HARBOUR | low layer | 1190 | kbps |  |  | high layer | 1500 | kbps |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | real bitrate | psnr Y | psnr U | psnr V | delta bitrate | real bitrate | psnr Y | psnr U | psnr V | delta bitrate |
| dyadic | 1212,24 | 29,75 | 41,28 | 43,74 | 1,87% | 1519,73 | 30,93 | 41,88 | 44,09 | 1,32% |
| ratio 3/2 | 1174,42 | 31,97 | 40,90 | 43,41 | -1,31% | 1500,57 | 31,41 | 41,78 | 43,76 | 0,04% |
| generic | 1183,14 | 31,99 | 41,90 | 43,76 | -0,58% | 1517,93 | 31,22 | 41,76 | 43,75 | 1,20% |
| Generic lower base | 965,04 | 31,34 | 41,53 | 43,40 |  | 1501,66 | 31,54 | 41,93 | 43,87 | 0,11% |

| SOCCER | low layer | 1190 | kbps |  |  | high layer | 1500 | kbps |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | real bitrate | psnr Y | psnr U | psnr V | delta bitrate | real bitrate | psnr Y | psnr U | psnr V | delta bitrate |
| dyadic | 1179,31 | 33,91 | 43,18 | 44,74 | -0,90% | 1499,80 | 34,20 | 42,85 | 44,95 | -0,01% |
| ratio 3/2 | 1181,99 | 35,87 | 43,41 | 44,10 | -0,67% | 1504,34 | 34,84 | 42,95 | 44,95 | 0,29% |
| generic | 1204,31 | 36,46 | 43,66 | 45,52 | 1,20% | 1505,02 | 34,42 | 42,80 | 44,73 | 0,33% |
| Generic lower base | 962,32 | 35,614 | 43,173 | 45,007 |  | 1503,42 | 34,91 | 42,90 | 44,79 | 0,23% |

Table 2 : PSNR results for ratio 4/3

These results show a sensible advantage of the Extended Spatial Scalability solutions against the dyadic one.

Considering the 2 ESS proposals, the following remarks can be made:

- regarding low layer, quality is equivalent for harbour and crew sequences. For city and soccer sequences, an difference of 0.7 to 1 dB can be observed in favour of the generic solution.

- regarding high layer, the 3/2 ratio solution gives superior or comparable results.

Additional simulations have been performed with the generic solution by decreasing the bitrate of the base layer (20% less), in order to give more quality to the high layer. It can be observed that both approaches have similar behaviours, with anyway an overall better quality for the 3/2 ratio solution.

These conclusions are confirmed by sequences viewing.

## Ratio Low / High layer of 2/3

### Processed simulations

For the dyadic case, an upsampling with a ratio of 4/3 must be applied on the low layer to get the right picture dimensions.

| Simu Type | decoded high layer size | decoded low layer size | decoded Low layer resampling |
| --- | --- | --- | --- |
| Dyadic | 672*576 | 336*288 | 448*384 - 4/3 |
| ESS with Ratio 3/2 | 672*576 | 448*384 | No |
| Generic ESS | 672*576 | 448*384 | No |

Table 3 : processed simulations for ratio 2/3

### Results

| CITY | low layer | 720 | kbps |  |  | high layer | 1000 | kbps |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | real bitrate | psnr Y | psnr U | psnr V | delta bitrate | real bitrate | psnr Y | psnr U | psnr V | delta bitrate |
| Dyadic | 705,66 | 29,43 | 42,90 | 43,51 | -1,99% | 981,11 | 32,69 | 45,16 | 46,97 | -1,89% |
| ratio 3/2 | 732,07 | 35,44 | 44,51 | 46,27 | 1,68% | 993,87 | 34,05 | 44,68 | 46,44 | -0,61% |
| generic | 732,07 | 35,44 | 44,51 | 46,27 | 1,68% | 983,97 | 34,02 | 44,68 | 46,45 | -1,60% |

| CREW | low layer | 1050 | kbps |  |  | high layer | 1470 | kbps |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | real bitrate | psnr Y | psnr U | psnr V | delta bitrate | real bitrate | psnr Y | psnr U | psnr V | delta bitrate |
| Dyadic | 1049,62 | 34,77 | 40,03 | 39,28 | -0,04% | 1465,98 | 35,03 | 41,87 | 41,31 | -0,27% |
| ratio 3/2 | 1069,01 | 36,02 | 41,57 | 40,91 | 1,81% | 1458,41 | 35,29 | 42,02 | 41,45 | -0,79% |
| generic | 1069,01 | 36,02 | 41,57 | 40,91 | 1,81% | 1465,22 | 35,33 | 42,03 | 41,47 | -0,32% |

| HARBOUR | low layer | 1050 | kbps |  |  | high layer | 1470 | kbps |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | real bitrate | psnr Y | psnr U | psnr V | delta bitrate | real bitrate | psnr Y | psnr U | psnr V | delta bitrate |
| Dyadic | 1058,18 | 29,28 | 41,41 | 42,21 | 0,78% | 1479,71 | 31,25 | 43,10 | 45,17 | 0,66% |
| ratio 3/2 | 1063,44 | 31,93 | 42,42 | 44,15 | 1,28% | 1481,42 | 31,69 | 42,97 | 44,69 | 0,78% |
| generic | 1063,44 | 31,93 | 42,42 | 44,15 | 1,28% | 1474,86 | 31,70 | 42,97 | 44,69 | 0,33% |

| SOCCER | low layer | 1050 | kbps |  |  | high layer | 1470 | kbps |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | real bitrate | psnr Y | psnr U | psnr V | delta bitrate | real bitrate | psnr Y | psnr U | psnr V | delta bitrate |
| Dyadic | 1044,03 | 33,74 | 41,50 | 42,37 | -0,57% | 1488,46 | 34,67 | 42,85 | 44,96 | 1,26% |
| ratio 3/2 | 1057,82 | 36,24 | 43,31 | 45,22 | 0,74% | 1492,19 | 35,18 | 43,05 | 45,04 | 1,51% |
| generic | 1057,81 | 36,24 | 43,31 | 45,22 | 0,74% | 1452,56 | 35,10 | 43,03 | 45,01 | -1,19% |

Table 4 : PSNR results for ratio 3/2

Figure 6 : PSNR for ratio 2/3 (1:low layer, 2:high layer)

Again, these results show a sensible advantage of the Extended Spatial Scalability solutions against the dyadic one. Moreover both solutions (generic and 3/2 ratio) give equivalent results. These conclusions are confirmed by sequences viewing.

## Ratio Low / High layer of 3/5

### Processed simulations

For the dyadic the ESS_3_2 cases, the low layer must be first cropped to get low layer dimensions multiple of 16. In these two cases, the low layer must be upsampled, with a ratio of 6/5 for the dyadi case, and of 9/10 for the ESS_3_2 case. In both cases, the upsampled low layer is smaller than the actual low layer, and thus must be compared to a cropped version of the low layer.

| Simu Type | decoded high layer size | decoded low layer size | decoded Low layer resampling |
| --- | --- | --- | --- |
| Dyadic with cropping | 640*560 | 320*272 (crop) | 384*326 – 6/5 (crop) |
| ESS with Ratio 3/2 | 640*560 | 416*368 (crop) | 374*330 – 9/10 (crop) |
| Generic ESS | 640*560 | 384*336 | No |

Table 5 : processed simulations for ratio 5/3

### Results

| CITY | low layer | 610 | kbps |  |  | high layer | 980 | kbps |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | real bitrate | psnr Y | psnr U | psnr V | delta bitrate | real bitrate | psnr Y | psnr U | psnr V | delta bitrate |
| dyadic | 601,22 | 29,91 | 42,67 | 44,17 | -1,44% | 997,97 | 33,61 | 42,94 | 45,39 | 1,83% |
| ratio 3/2 | 601,24 | 34,06 | 42,33 | 44,27 | -1,44% | 967,69 | 34,32 | 42,96 | 45,19 | -1,26% |
| generic | 621,78 | 35,30 | 43,21 | 45,11 | 1,93% | 963,08 | 33,99 | 42,89 | 45,18 | -1,73% |
| Generic lower base | 499,31 | 34,56 | 42,64 | 44,56 |  | 970,31 | 34,41 | 43,03 | 45,23 | -0,99% |

| CREW | low layer | 890 | kbps |  |  | high layer | 1430 | kbps |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | real bitrate | psnr Y | psnr U | psnr V | delta bitrate | real bitrate | psnr Y | psnr U | psnr V | delta bitrate |
| dyadic | 887,24 | 33,66 | 39,00 | 37,94 | -0,31% | 1407,53 | 35,07 | 40,32 | 40,40 | -1,57% |
| ratio 3/2 | 906,04 | 34,45 | 39,31 | 38,20 | 1,80% | 1426,67 | 35,36 | 40,48 | 40,67 | -0,23% |
| generic | 898,96 | 35,09 | 39,68 | 39,08 | 1,01% | 1424,63 | 35,11 | 40,37 | 40,51 | -0,38% |
| Generic lower base | 717,61 | 34,25 | 39,14 | 38,36 |  | 1423,66 | 35,32 | 40,37 | 40,54 | -0,44% |

| HARBOUR | low layer | 890 | kbps |  |  | high layer | 1430 | kbps |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | real bitrate | psnr Y | psnr U | psnr V | delta bitrate | real bitrate | psnr Y | psnr U | psnr V | delta bitrate |
| dyadic | 894,55 | 28,31 | 40,56 | 42,14 | 0,51% | 1451,09 | 31,28 | 41,75 | 43,85 | 1,47% |
| ratio 3/2 | 907,37 | 29,83 | 40,31 | 42,60 | 1,95% | 1444,08 | 31,91 | 41,99 | 43,98 | 0,98% |
| generic | 875,03 | 30,81 | 40,83 | 42,58 | -1,68% | 1449,78 | 31,46 | 41,95 | 43,89 | 1,38% |
| Generic lower base | 710,91 | 30,11 | 40,48 | 42,22 |  | 1430,24 | 31,70 | 41,87 | 43,78 | 0,02% |

| SOCCER | low layer | 890 | kbps |  |  | high layer | 1430 | kbps |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | real bitrate | psnr Y | psnr U | psnr V | delta bitrate | real bitrate | psnr Y | psnr U | psnr V | delta bitrate |
| dyadic | 893,80 | 33,17 | 41,25 | 42,67 | 0,43% | 1401,96 | 34,72 | 42,77 | 44,88 | -1,96% |
| ratio 3/2 | 901,77 | 34,57 | 42,16 | 43,18 | 1,32% | 1421,51 | 35,27 | 43,05 | 45,04 | -0,59% |
| generic | 892,08 | 35,58 | 42,43 | 44,42 | 0,23% | 1444,52 | 34,88 | 42,97 | 44,92 | 1,02% |
| Generic lower base | 721,89 | 34,691 | 41,985 | 43,866 |  | 1436,15 | 35,202 | 43,017 | 44,876 | 0,43% |

Table 6 : PSNR results for ratio 4/3

Again, these results show a sensible advantage of the Extended Spatial Scalability solutions against the dyadic one.

Considering the 2 ESS proposals, the following remarks can be made:

- regarding low layer, the generic solution outperforms the 3/2 ratio one by 0.7 to 1.2 dB.

- regarding high layer, the 3/2 ratio solution the generic ratio one by 0.2 to 0.6 dB.

Additional simulations have been performed with the generic solution by decreasing the bitrate of the base layer (20% less), in order to give more quality to the high layer. It can be observed that both approaches have similar behaviours.

These conclusions are confirmed by sequences viewing.

# Conclusions and recommendations

These experiments prove the interest of Extended Spatial Scalability.

Pros and cons of each solution are the following:

- Ratio 3/2 solution

  - Pros

    - Direct and non complex motion and texture inheriting processes (as dyadic solution)

    - Good efficiency

    - Directly usable for SD/HD (720p/SD30Hz) scalability

  - Cons

    - Non genericity

    - Cropping window aligned on MBs

- Generic solution

  - Pros

    - Genericity : any horizontal and vertical ratios, any cropping window position

    - Good efficiency

    - Limited complexity

  - Cons

    - Higher complexity than dyadic or 3/2 ratio solutions

Consequently we recommend to adopt proposal 1 and proposal2 for introduction into the JSVM. Each tool may correspond to different types of applications. For instance, proposal 1 (ratio 3/2) is directly usable for SD/HD (720p/SD30Hz) scalability. Proposal 2 would be for instance well adapted for professional applications (such as flexible content repurposing).

# References

[N6880]	ISO/IEC JTC1/SC29/WG11/N6880, Hong Kong, January 2005, Applications and Requirements for Scalable Video Coding

[N6898]	ISO/IEC JTC1/SC29/WG11/N6898, Hong Kong, January 2005, Description of Core Experiments in SVC

[JSVM1.0]	J.Reichel, H.Schwarz, M.Wien, “Joint Scalable Video Model JSVM 1”, Joint Video Team of ISO/MPEG and ITU-T VCEG, Doc. JVT-N021, Hong-Kong, China, Jan 05.

[JVT-O041]	E. Francois, J. Viéron, G. Marquant, N. Burdin, P. Lopez, “Generic Extended Spatial Scalability”, Joint Video Team of ISO/MPEG and ITU-T VCEG, Doc. JVT-O041, Busan, Korea, April 05.

[JVT-O042]	J. Viéron, E. Francois, G. Marquant, N. Burdin, P. Lopez, “Extended Spatial Scalability with a 3/2 ratio”, Joint Video Team of ISO/MPEG and ITU-T VCEG, Doc. JVT-O042, Busan, Korea, April 05.