| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 13th Meeting: Palma de Mallorca, Spain 18-22 Oct., 2004 | Document: JVT-O040 Filename: input.doc |
| --- | --- |

| Title: | Spatial scalable coding using cropped areas |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | E. Francois, J. Viéron, G. Marquant, N. Burdin, P. Lopez Thomson Multimedia R&D France 1 avenue de belle fontaine 35511 Cesson Sevigne Cedex France | Tel: Email: | +33 2 99 27 32 62 edouard.francois@thomson.net |
| Source: | Thomson |  |  |

_____________________________

1	Overview	2

2	Problem to solve	2

3	Inter-Layer Prediction	3

4	Changes in SVM	4

4.1	Changes in section S.6 ‘Source, coded, decoded and output data formats, scanning processes, and neighbouring relationships’	4

4.2	Changes in section S.7 ‘Syntax and semantics’	6

4.3	Changes in section ‘S.7.4  Semantics’	11

4.4	Changes in section S.8 ‘Decoding Process’	11

5	References	15

(Append for Proposal Documents)	16

JVT Patent Disclosure Form	16

# Overview

This document describes THOMSON proposal for CE9 related to 'Spatial scalable coding using cropped areas'. This tool enables that pictures of a given spatial layer (resolution) are a cropped and downsampled version of the pictures of the higher spatial layer (resolution).

# Problem to solve

We consider two successive spatial layers, a low layer (considered as base layer) and a high layer (considered as enhancement layer), linked by the following geometrical relations (cf Figure 1). Width and height of enhancement layer pictures are defined respectively as wenh and henh. Base layer pictures dimensions are defined as wbase and hbase. Base layer pictures are a downsampled version of sub-pictures of enhancement layer pictures, of dimensions wextract and hextract, positioned at coordinates (xorig , yorig) in the enhancement layer pictures coordinates system. (xorig , yorig) are multiple of 16 and thus aligned on macroblock structure of the high layer. In Figure 1, the enhancement and base layer pictures are divided in macroblocks. In this example, a downsampling ratio of 3/2 from high to low layer is used.

Figure 1: relations between enhancement layer and base layer (downsampling 3/2 in both dimensions)

The problem to solve is the encoding/decoding of the macroblocks of the enhancement layer knowing the decoded base layer. On borders of the enhancement layer picture, macroblocks may either have no base layer corresponding block, or be only partially covered by scaled base layer blocks. Inter layer prediction has to be managed differently for these macroblocks than for the other ones.

In the sequel, according to the semantic defined further, we will name:

- scaled_base_column_in_mbs = xorig / 16

- scaled_base_line_in_mbs = yorig / 16

- scaled_base_width = wextract

- scaled_base_height = hextract

# Inter-Layer Prediction

Inter layer motion and texture prediction, such as described in [JSVM1], is only authorized for high layer macroblocks fully covered by scaled base layer, that is, whose coordinates (MBx,Mby) verify the following conditions (grey area in Figure 2):

MBx >=  scaled_base_column_in_mbs	and

MBx < scaled_base_column_in_mbs + scaled_base_width / 16

And

MBy >= scaled_base_ line_in_mbs	and

MBy < scaled_base_ line_in_mbs + scaled_base_height / 16

Figure 2: Authorized blocks for inter layer texture prediction

Macroblocks following these conditions may use intra prediction, inter-prediction and inter-layer prediction modes. Macroblocks that do not follow these conditions may only use intra prediction and inter-prediction.

# Changes in SVM

## Changes in section S.6 ‘Source, coded, decoded and output data formats, scanning processes, and neighbouring relationships’

Changes in Section ‘S.6.5.2 Derivation process for base macroblocks’

Following lines

The macroblock address mbAddrBase is derived as follows.

- If HalfSpatResBaseFlag is equal to 0,

mbAddrBase = mbAddr

- Otherwise (HalfSpatResBaseFlag is equal to 1),

mbAddrBase = ( PicWidthInMbs / 2 ) * ( mbAddr / ( 2 * PicWidthInMbs ) ) + ( mbAddr % PicWidthInMbs ) / 2

replaced by

The macroblock address mbAddrBase is derived as follows.

- If ExtendedSpatialScalability is equal to 0,

- If HalfSpatResBaseFlag is equal to 0,

mbAddrBase = mbAddr

- Otherwise (HalfSpatResBaseFlag is equal to 1),

mbAddrBase = ( PicWidthInMbs / 2 ) * ( mbAddr / ( 2 * PicWidthInMbs ) ) + ( mbAddr % PicWidthInMbs ) / 2

- Otherwise (ExtendedSpatialScalability is equal to 1),

- If	(mbAddr / PicWidthInMbs < scaled_base_lines_in_mbs)                              or
          (mbAddr / PicWidthInMbs >= scaled_base_lines_in_mbs + scaled_base_height / 16)       or
          (mbAddr % PicWidthInMbs < scaled_base_columns_in_mbs)                             or
          (mbAddr % PicWidthInMbs >= scaled_base_columns_in_mbs + scaled_base_width / 16 ),

there is no base layer macroblock,

mbAddrBase = -1

- Otherwise,

- If HalfSpatResBaseFlag is equal to 0,

mbAddrBase = basePicWidthInMbs  *  (mbAddr / PicWidthInMbs - scaled_base_lines_in_mbs) +  (mbAddr % PicWidthInMbs) - scaled_base_columns_in_mbs)

- Otherwise (HalfSpatResBaseFlag is equal to 1),

mbAddrBase = ( basePicWidthInMbs  / 2 ) *  ( (mbAddr / PicWidthInMbs - scaled_base_lines_in_mbs) / 2 ) + ( (mbAddr % PicWidthInMbs - scaled_base_columns_in_mbs) / 2 )

Changes in Section ‘S.6.5.3 Derivation process for base sub-macroblocks’

Following lines

The macroblock partition index mbPartIdxBase is derived as follows.

- The base luma location ( xB, yB ) is derived as follows

xB = 8 * ( ( mbAddr % PicWidthInMbs ) % 2)

yB = 8 * ( ( mbAddr / PicWidthInMbs ) % 2)

- The macroblock partition index of the macroblock partition in the macroblock mbAddrBase in the picture basePic covering the luma location ( xB, yB ) shall be assigned to mbPartIdxBase.

replaced by

- If (mbAddrBase != -1),

The macroblock partition index mbPartIdxBase is derived as follows.

- The base luma location ( xB, yB ) is derived as follows

xB = 8 * ( ( mbAddr % PicWidthInMbs - scaled_base_columns_in_mbs) % 2)

yB = 8 * ( ( mbAddr / PicWidthInMbs - scaled_base_lines_in_mbs) % 2)

- The macroblock partition index of the macroblock partition in the macroblock mbAddrBase in the picture basePic covering the luma location ( xB, yB ) shall be assigned to mbPartIdxBase.

- Else, there is no base layer macroblock

Changes in Section ‘S.6.5.4 Derivation process for base partitions’

Following lines

The macroblock partition index mbPartIdxBase and the sub-macroblock partition index subMbPartIdxBase are derived as follows.

- The inverse macroblock macroblock partition scanning process in subclause S.6.4.2.1 is invoked with mbPartIdx as the input and ( x, y ) as the output.

- The location of the upper-left luma sample inside a macroblock partition ( xS, yS ) is derived as follows.

- If mb_type is equal to P_8x8, P_8x8ref0, or B_8x8, the inverse sub-macroblock partition scanning process in subclause S.6.4.2.1 is invoked with subMbPartIdx as the input and ( xS, yS ) as the output.

- Otherwise, ( xS, yS ) are set equal to ( 0, 0 ).

- The base luma location ( xB, yB ) is derived as follows.

- If HalfSpatResBaseFlag is equal to 0,

xB = x + xS

yB = y + yS

- Otherwise,

xB = 8 * ( ( mbAddr % PicWidthInMbs ) % 2) + ( x + xS ) / 2

yB = 8 * ( ( mbAddr / PicWidthInMbs ) % 2) + ( y + yS ) / 2

- The macroblock partition index of the macroblock partition in the macroblock mbAddrBase in the picture basePic covering the luma location ( xB, yB ) shall be assigned to mbPartIdxBase.

- The sub-macroblock partition index of the sub-macroblock partition inside the macroblock partition mbPartIdxBase of the macroblock mbAddrBase in the picture basePic covering the luma location ( xB, yB ) shall be assigned to subMbPartIdxBase.

replaced by

- If (mbAddrBase != -1),

The macroblock partition index mbPartIdxBase and the sub-macroblock partition index subMbPartIdxBase are derived as follows.

- The inverse macroblock partition scanning process in subclause S.6.4.2.1 is invoked with mbPartIdx as the input and ( x, y ) as the output.

- The location of the upper-left luma sample inside a macroblock partition ( xS, yS ) is derived as follows.

- If mb_type is equal to P_8x8, P_8x8ref0, or B_8x8, the inverse sub-macroblock partition scanning process in subclause S.6.4.2.2 is invoked with subMbPartIdx as the input and ( xS, yS ) as the output.

- Otherwise, ( xS, yS ) are set equal to ( 0, 0 ).

- The base luma location ( xB, yB ) is derived as follows.

- If HalfSpatResBaseFlag is equal to 0,

xB = x + xS

yB = y + yS

- Otherwise,

xB = 8 * ( ( mbAddr % PicWidthInMbs - scaled_base_columns_in_mbs) % 2) + ( x + xS ) / 2

yB = 8 * ( ( mbAddr / PicWidthInMbs - scaled_base_lines_in_mbs) % 2) + ( y + yS ) / 2

- The macroblock partition index of the macroblock partition in the macroblock mbAddrBase in the picture basePic covering the luma location ( xB, yB ) shall be assigned to mbPartIdxBase.

- The sub-macroblock partition index of the sub-macroblock partition inside the macroblock partition mbPartIdxBase of the macroblock mbAddrBase in the picture basePic covering the luma location ( xB, yB ) shall be assigned to subMbPartIdxBase.

- Else, there is no base layer macroblock

## Changes in section S.7 ‘Syntax and semantics’

Regarding syntax and semantics, the following lines have to be added at the beginning of the section:

For processes requiring the derivation of mbAddrBase as described in section S.6.5.2 ‘Derivation process for base macroblocks’, if the returned mbAddrBase is equal to -1 (meaning that extended spatial scalability is used, and that the base layer macroblock is outside the cropping window), the process that exploits the computed mbAddrBase is not applied.

Changes in Section ‘S.7.2	Specification of syntax functions, categories, and descriptors’

Following lines

half_spat_res_base_pic( )  is specified as follows.

- If all of the following conditions are true, the return value of half_spat_res_base_pic( ) is equal to TRUE.

- The syntax element base_id_plus1 is greater than 0.

- ( pic_width_in_mbs_minus1 + 1 ) is equal to 2 * ( basePicWidthInMbsMinus1 + 1 ), with basePicWidthInMbsMinus1 being the syntax element pic_width_in_mbs_minus1 of the active sequence parameter set for the pictures with DependencyId equal to ( base_id_plus1 – 1 ).

- ( pic_height_in_map_units_minus1 + 1 ) is equal to 2 * ( basePicHeightInMapUnitsMinus1 + 1 ), with basePicHeightInMapUnitsMinus1  being the syntax element pic pic_height_in_map_units_minus1 of the active sequence parameter set for the pictures with DependencyId equal to ( base_ id_plus1 – 1 ).

- Otherwise, the return value of half_spat_res_base_pic( ) is equal to FALSE.

replaced by

half_spat_res_base_pic( )  is specified as follows.

- If all of the following conditions are true, the return value of half_spat_res_base_pic( ) is equal to TRUE.

- The syntax element base_id_plus1 is greater than 0.

- scaled_base_width is equal to 2 *16* ( basePicWidthInMbsMinus1 + 1 ), with basePicWidthInMbsMinus1 being the syntax element pic_width_in_mbs_minus1 of the active sequence parameter set for the pictures with DependencyId equal to ( base_id_plus1 – 1 ).

- scaled_base_height is equal to 2 *16* ( basePicHeightInMbsMinus1 + 1 ), with basePicHeightInMbsMinus1 being the syntax element pic_height_in_mbs_minus1 of the active sequence parameter set for the pictures with DependencyId equal to ( base_id_plus1 – 1 ).

- Otherwise, the return value of half_spat_res_base_pic( ) is equal to FALSE.

Following lines added

spatial_base_layer() specifies whether the current layer is the (spatial) base layer. If the current layer is the (spatial) base layer, spatial_base_layer() is equal to TRUE, else it is equal to FALSE.

in_crop_window( mbAddr )  is specified as follows.

- Let mbIdxX and mbIdxY be defined as follows.

mbIdxX = mbAddr % PicWidthInMbs

mbIdxY = mbAddr / PicWidthInMbs

- If all of the following conditions are true, the return value of in_crop_window ( ) is equal to TRUE.

mbIdxX >= scaled_base_column_in_mbs	and

mbIdxX < scaled_base_column_in_mbs + scaled_base_width / 16	and

mbIdxY >= scaled_base_ line_in_mbs	and

mbIdxY < scaled_base_ line_in_mbs + scaled_base_height / 16

- Otherwise, the return value of in_crop_window ( ) is equal to FALSE. base_mode_flag and base_mode_refinement_flag are enforced to 0.

Changes in Section S.7.3.2.1 ‘Sequence parameter set RBSP syntax’

The main changes (in red characters) are the addition in the slide header syntax of a flag extended_spatial_scalability, and accordingly 4 parameters (scaled_base_column_in_mbs, scaled_base_line_in_mbs, scaled_base_width, scaled_base_height) related to the geometrical transformation to be applied in the base layer upsampling process.

| seq_parameter_set_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| profile_idc | 0 | u(8) |
| constraint_set0_flag | 0 | u(1) |
| constraint_set1_flag | 0 | u(1) |
| constraint_set2_flag | 0 | u(1) |
| constraint_set3_flag | 0 | u(1) |
| reserved_zero_4bits /* equal to 0 */ | 0 | u(4) |
| level_idc | 0 | u(8) |
| seq_parameter_set_id | 0 | ue(v) |
| if( profile_idc = = 100 ¦ ¦ profile_idc = = 110 ¦ ¦ profile_idc = = 122 ¦ ¦ profile_idc = = 144 ¦ ¦ profile_idc = = 83 ) ) { |  |  |
| chroma_format_idc | 0 | ue(v) |
| if( chroma_format_idc = = 3 ) |  |  |
| residual_colour_transform_flag | 0 | u(1) |
| bit_depth_luma_minus8 | 0 | ue(v) |
| bit_depth_chroma_minus8 | 0 | ue(v) |
| qpprime_y_zero_transform_bypass_flag | 0 | u(1) |
| seq_scaling_matrix_present_flag | 0 | u(1) |
| if( seq_scaling_matrix_present_flag ) |  |  |
| for( i = 0; i < 8; i++ ) { |  |  |
| seq_scaling_list_present_flag[ i ] | 0 | u(1) |
| if( seq_scaling_list_present_flag[ i ] ) |  |  |
| if( i < 6 ) |  |  |
| scaling_list( ScalingList4x4[ i ], 16, UseDefaultScalingMatrix4x4Flag[ i ]) | 0 |  |
| else |  |  |
| scaling_list( ScalingList8x8[ i – 6 ], 64, UseDefaultScalingMatrix8x8Flag[ i – 6 ] ) | 0 |  |
| } |  |  |
| } |  |  |
| log2_max_frame_num_minus4 | 0 | ue(v) |
| pic_order_cnt_type | 0 | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |  |
| log2_max_pic_order_cnt_lsb_minus4 | 0 | ue(v) |
| else if( pic_order_cnt_type = = 1 ) { |  |  |
| delta_pic_order_always_zero_flag | 0 | u(1) |
| offset_for_non_ref_pic | 0 | se(v) |
| offset_for_top_to_bottom_field | 0 | se(v) |
| num_ref_frames_in_pic_order_cnt_cycle | 0 | ue(v) |
| for( i = 0; i < num_ref_frames_in_pic_order_cnt_cycle; i++ ) |  |  |
| offset_for_ref_frame[ i ] | 0 | se(v) |
| } |  |  |
| num_ref_frames | 0 | ue(v) |
| gaps_in_frame_num_value_allowed_flag | 0 | u(1) |
| pic_width_in_mbs_minus1 | 0 | ue(v) |
| pic_height_in_map_units_minus1 | 0 | ue(v) |
| frame_mbs_only_flag | 0 | u(1) |
| if( !frame_mbs_only_flag ) |  |  |
| mb_adaptive_frame_field_flag | 0 | u(1) |
| direct_8x8_inference_flag | 0 | u(1) |
| frame_cropping_flag | 0 | u(1) |
| if( frame_cropping_flag ) { |  |  |
| frame_crop_left_offset | 0 | ue(v) |
| frame_crop_right_offset | 0 | ue(v) |
| frame_crop_top_offset | 0 | ue(v) |
| frame_crop_bottom_offset | 0 | ue(v) |
| } |  |  |
| if( !spatial_base_layer() ) { |  |  |
| extended_spatial_scalability | 2 | u(1) |
| if( extended_spatial_scalability ! = 0 ) { |  |  |
| scaled_base_column_in_mbs | 2 | ue(v) |
| scaled_base_line_in_mbs | 2 | ue(v) |
| scaled_base_width | 2 | ue(v) |
| scaled_base_height | 2 | ue(v) |
| } |  |  |
| } |  |  |
| vui_parameters_present_flag | 0 | u(1) |
| if( vui_parameters_present_flag ) |  |  |
| vui_parameters( ) | 0 |  |
| rbsp_trailing_bits( ) | 0 |  |
| } |  |  |

Changes in Section ‘S.7.3.8	Macroblock layer in scalable extension syntax

| macroblock_layer_in_scalable_extension( ) { | C | Descriptor |
| --- | --- | --- |
| if( in_crop_window( CurrMbAddr ) ) { |  |  |
| if( base_id_plus1 != 0 && adaptive_prediction_flag ) { |  |  |
| base_mode_flag | 2 | ae(v) |
| if( ! base_mode_flag && (HalfSpatResBaseFlag ¦¦ extended_spatial_scalability) && ! intra_base_mb( CurrMbAddr ) ) |  |  |
| base_mode_refinement_flag | 2 | ae(v) |
| } |  |  |
| } |  |  |
| if( ! base_mode_flag && ! base_mode_ref inement_flag ) { |  |  |
| mb_type | 2 | ae(v) |
| if( mb_type = = I_NxN && base_id_plus1 != 0 ) |  |  |
| intra_base_flag | 2 | ae(v) |
| } |  |  |
| if( mb_type = = I_PCM ) { |  |  |
| while( !byte_aligned( ) ) |  |  |
| pcm_alignment_zero_bit | 2 | f(1) |
| for( i = 0; i < 256; i++ ) |  |  |
| pcm_sample_luma[ i ] | 2 | u(v) |
| for( i = 0; i < 2 * MbWidthC * MbHeightC; i++ ) |  |  |
| pcm_sample_chroma[ i ] | 2 | u(v) |
| } else { |  |  |
| NoSubMbPartSizeLessThan8x8Flag = 1 |  |  |
| if( mb_type != I_NxN && MbPartPredMode( mb_type, 0 ) != Intra_16x16 && NumMbPart( mb_type ) = = 4 ) { |  |  |
| sub_mb_pred_in_scalable_extension( mb_type ) | 2 |  |
| for( mbPartIdx = 0; mbPartIdx < 4; mbPartIdx++ ) |  |  |
| if( sub_mb_type[ mbPartIdx ] != B_Direct_8x8 ) { |  |  |
| if( NumSubMbPart( sub_mb_type [ mbPartIdx ] ) > 1 ) |  |  |
| NoSubMbPartSizeLessThan8x8Flag = 0 |  |  |
| } else if( !direct_8x8_inference_flag ) |  |  |
| NoSubMbPartSizeLessThan8x8Flag = 0 |  |  |
| } else { |  |  |
| if( transform_8x8_mode_flag && mb_type = = I_NxN ) |  |  |
| transform_size_8x8_flag | 2 | ae(v) |
| mb_pred_in_scalable_extension( mb_type ) | 2 |  |
| } |  |  |
| if( MbPartPredMode( mb_type, 0 ) != Intra_16x16 ) { |  |  |
| coded_block_pattern | 2 | ae(v) |
| if( CodedBlockPatternLuma > 0 && transform_8x8_mode_flag && mb_type != I_NxN && NoSubMbPartSizeLessThan8x8Flag && !( MbPartPredMode( mb_type, 0 ) = = B_Direct_16x16 && !direct_8x8_inference_flag ) ) |  |  |
| transform_size_8x8_flag | 2 | ae(v) |
| } |  |  |
| if( CodedBlockPatternLuma > 0 ¦ ¦ CodedBlockPatternChroma > 0 ¦ ¦ MbPartPredMode( mb_type, 0 ) = = Intra_16x16 ) { |  |  |
| mb_qp_delta | 2 | ae(v) |
| residual_in_scalable_extension( ) | 3 ¦ 4 |  |
| } |  |  |
| } |  |  |
| } |  |  |

## Changes in section ‘S.7.4  Semantics’

Changes in Section S.7.4.2.1	Sequence parameter set RBSP semantics

Following lines added

extended_spatial_scalability specifies the presence of syntax elements related to geometrical parameters for the base layer upsampling in the slice header. When this syntax element is not present, it shall be inferred to be equal to 0.

scaled_base_column_in_mbs corresponds to the horizontal macroblock coordinate of the upper left pixel of the upsampled base layer in the current layer coordinates system (this pixel is aligned on the macroblock structure). When this syntax element is not present, it shall be inferred to be equal to 0.

The horizontal coordinate in the current layer coordinates system of the upper left pixel of the upsampled base layer luma samples is defined as follows:

ScaledBaseColumnL = 16 * scaled_base_column_in_mbs

The horizontal coordinate in the current layer coordinates system of the upper left pixel of the upsampled base layer chroma samples is defined as follows:

ScaledBaseColumnC = ScaledBaseColumnL / SubWidthC

scaled_base_line_in_mbs corresponds to the vertical macroblock coordinate of the upper left pixel of the upsampled base layer in the current layer coordinates system (this pixel is aligned on the macroblock structure). When this syntax element is not present, it shall be inferred to be equal to 0.

The vertical coordinate in the current layer coordinates system of the upper left pixel of the upsampled base layer luma samples is defined as follows:

ScaledBaseLineL  = 16 * scaled_base_ line_in_mbs

The vertical coordinate in the current layer coordinates system of the upper left pixel of the upsampled base layer chroma samples is defined as follows:

ScaledBaseLineC  = ScaledBaseLineL  / SubWidthC

scaled_base_width corresponds to the number of pixels per line of the upsampled base layer in the current layer. When this syntax element is not present, it shall be inferred to be equal to (16 x ( pic_width_in_mbs_minus1 + 1 )).

scaled_base_height corresponds to the number of pixels per column of the upsampled base layer in the current layer. When this syntax element is not present, it shall be inferred to be equal to (16 x ( pic_height_in_mbs_minus1 + 1 )).

The following conditions must be verified:

- scaled_base_column_in_mbs >= 0

- 16*scaled_base_column_in_mbs + scaled_base_width <= 16 * ( pic_width_in_mbs_minus1 + 1 )

- scaled_base_line_in_mbs >= 0

- 16*scaled_base_line_in_mbs + scaled_base_height <= 16 * ( pic_height_in_mbs_minus1 + 1 )

## Changes in section S.8 ‘Decoding Process’

Changes in Section S.8.3.6	Intra_Base prediction process

[Ed. Note(HS): This process is currently only specified for frame_mbs_only_flag equal to 1.]

This process is invoked when the macroblock prediction mode is equal to Intra_Base.

Following lines added

This process is invoked only if the current macroblock coordinates, defined as:

mbIdxX = CurrMbAddr % PicWidthInMbs

mbIdxY = CurrMbAddr / PicWidthInMbs

verify the following conditions:

mbIdxX >= scaled_base_column_in_mbs	and

mbIdxX < scaled_base_column_in_mbs + scaled_base_width / 16	and

mbIdxY >= scaled_base_ line_in_mbs	and

mbIdxY < scaled_base_ line_in_mbs + scaled_base_height / 16

Input to this process are constructed samples of the base picure prior to the deblocking filter process.

Outputs of this process are

- the Intra prediction luma samples for the current macroblock predL[ x, y ] with x = 0..15, y = 0..15

- when chroma_format_idc is not equal to 0, the Intra prediction chroma samples for the current macroblock predCb[ x, y ] and predCr[ x, y ] with x = 0..MbWidthC – 1, y = 0..MbHeightC – 1

The derivation process for base macroblocks in subclause S.6.5.2  is invoked with CurrMbAddr as input and the outputs are assigned to the picture basePic and the macroblock address mbAddrBase.  When the syntax element constrained_intra_pred_flag is equal to 1 for the base picture basePic, the macroblock type of the macroblock mbAddrBase in the base picture basePic shall be an I macroblock type.

NOTE – When constrained_intra_pred_flag is equal to 1, only the samples of I macroblocks of the base picture basePic need to be constructed for generating the prediction samples for macroblocks coded in Intra_Base macroblock prediction mode.

The position ( xP, yP ) of the upper-left luma sample of the current macroblock is derived by invoking the inverse macroblock scanning process in subclause S.6.4.1  with CurrMbAddr as input and the output is assigned to ( xP, yP ).

When chroma_format_idc is not equal to 0, the position ( xC, yC ) of the upper-left chroma samples of the current macroblock are derived by

xC = xP / SubWidthC
yC = yP / SubHeightC

Following lines added

The corresponding position ( xB, yB ) in the base layer of position ( xP, yP ) is computed as:

- If HalfSpatResBaseFlag is equal to 0, the following applies.

xB = (xP – ScaledBaseColumnL )

yB = (yP – ScaledBaseLineL )

- Otherwise (HalfSpatResBaseFlag is equal to 1), the following applies.

xB = (xP – ScaledBaseColumnL ) / 2

yB = (yP – ScaledBaseLineL ) / 2

When chroma_format_idc is not equal to 0, the corresponding position ( xCB, yCB ) in the base layer of position ( xC, yC ) is computed as:

xCB = xB / SubWidthC

yCB = yB / SubWidthC

The Intra prediction luma and chroma samples for the current macroblock are derived as follows.

- If HalfSpatResBaseFlag is equal to 0, the following applies.

- Let S'Base,L[ x, y ] be the array of constructed luma samples of the picture basePic prior to the deblocking filter process.

- When chromat_format_idc is not equal to 0, let S'Base,Cb[ x, y ] and S'Base,Cr[ x, y ] be the arrays of constructed chroma samples of the picture basePic prior to the deblocking filter process.

- Each Intra prediction luma sample predL[ x, y ] with x = 0..15 and y = 0..15 is derived by

Following lines

predL[ x, y ] = S'Base,L[ xP + x, yP+ y ]

replaced by

predL[ x, y ] = S'Base,L[ xB + x, yB + y ]

- When chroma_format_idc is not equal to 0, each Intra prediction chroma sample predCb[ x, y ] and predCr[ x, y ] with x = 0..MbWidthC – 1 and y = 0..MbHeightC – 1 is derived by

Following lines

predCb[ x, y ] = S'Base,Cb[ xC + x, yC + y ]
predCr[ x, y ] = S'Base,Cr[ xC + x, yC + y]

replaced by

predCb[ x, y ] = S'Base,Cb[ xCB  + x, yCB + y ]
predCr[ x, y ] = S'Base,Cr[ xCB  + x, yCB + y]

- Otherwise (HalfSpatResBaseFlag is equal to 1), the following applies.

- The deblocking filter process in subclause S.8.3.6.1  is invoked with basePic as input and the outputs are the luma sample array S'Base,L[ x, y ], and, when chroma_format_idc is not equal to 0, the chroma sample arrays S'Base,Cb[ x, y ] and S'Base,Cr[ x, y ].

- Let baseL[ x, y ] with x = –8..15, y =–8..15 be a luma sample array and, when chroma_format_idc is not equal to 0, let baseCb[ x, y ] and baseCr[ x, y ] with x = –MbWidthC / 2..MbWidthC – 1 and 
y = –MbHeightC / 2..MbHeightC – 1 be two chroma samples arrays.

- Each luma sample baseL[ x, y ] with x = –8..15 and y =–8..15 of the luma sample array is derived as follows.

- If any of the following conditions is true, the sample baseL[ x, y ] is marked as “not existing”.

Following lines

- x is less than ( –xP / 2 ) or x is greater than ( ( PicWidthInSamplesL – xP ) / 2 – 1 )

- y is less than ( –yP / 2 ) or y is greater than ( ( PicHeightInSamplesL – yP ) / 2 – 1 )

replaced by

- x is less than ( –xB ) or x is greater than ( ( BasePicWidthInSamplesL  + xB ) – 1 )

- y is less than ( –yB ) or y is greater than ( ( BasePicWidthInSamplesL  + yB ) – 1 )

Following lines

- Otherwise, if the syntax element constrained_intra_pred_flag is equal to 1 for the base picture basePic and the sample S'Base,L[ ( xP / 2 ) + x, ( yP / 2 ) + y ] does not represent a sample of an I macroblock, the sample baseL[ x, y ] is marked as “not available for Intra_Base prediction”.

replaced by

- Otherwise, if the syntax element constrained_intra_pred_flag is equal to 1 for the base picture basePic and the sample S'Base,L[  xB + x, yB + y ] does not represent a sample of an I macroblock, the sample baseL[ x, y ] is marked as “not available for Intra_Base prediction”.

Following lines

- Otherwise, the sample baseL[ x, y ] is marked as “available for Intra_Base prediction” and derived by

baseL[ x, y ] = S'Base,L[ ( xP / 2 ) + x, ( yP / 2 ) + y ]

replaced by

- Otherwise, the sample baseL[ x, y ] is marked as “available for Intra_Base prediction” and derived by

baseL[ x, y ] = S'Base,L[ xB + x, yB + y ]

- When chroma_format_idc is not equal to 0, each chroma sample baseCb[ x, y ] and baseCr[ x, y ] with 
x = –MbWidthC / 2..MbWidthC – 1 and y = –MbHeightC / 2..MbHeightC – 1 of the chroma sample arrays is derived as follows.

- If any of the following conditions is true, the samples baseCb[ x, y ] and baseCr[ x, y ] are marked as “not existing”.

Following lines

- x is less than ( –xC / 2 ) or x is greater than ( ( PicWidthInSamplesC – xC ) / 2 – 1 )

- y is less than ( –yC / 2 ) or y is greater than ( ( PicHeightInSamplesC – yC ) / 2 – 1 )

replaced by

- x is less than ( –xC / 2 ) or x is greater than ( ( BasePicWidthInSamplesC + xCB ) – 1 )

- y is less than ( –yC / 2 ) or y is greater than ( ( BasePicHeightInSamplesC + yCB ) – 1 )

Following lines

- Otherwise, if the syntax element constrained_intra_pred_flag is equal to 1 for the base picture basePic and the samples baseCb[ x, y ] and baseCr[ x, y ] do not represent samples of an I macroblock, the samples baseCb[ x, y ] and baseCr[ x, y ] are marked as “not available for Intra_Base prediction”:

replaced by

- Otherwise, if the syntax element constrained_intra_pred_flag is equal to 1 for the base picture basePic and the samples S'Base,Cb[ xCB + x, yCB + y ] and S'Base,Cr[ xCB + x, yCB + y ] do not represent samples of an I macroblock, the samples baseCb[ x, y ] and baseCr[ x, y ] are marked as “not available for Intra_Base prediction”:

- Otherwise, the samples baseCb[ x, y ] and baseCr[ x, y ] are marked as “available for Intra_Base prediction” and derived by

Following lines

baseCb[ x, y ] = S'Base,Cb[ ( xC / 2 ) + x, ( yC / 2 ) + y ]
baseCr[ x, y ] = S'Base,Cr[ ( xC / 2 ) + x, ( yC / 2 ) + y ]

replaced by

baseCb[ x, y ] = S'Base,Cb[ xCB + x, yCB + y ]
baseCr[ x, y ] = S'Base,Cr[ xCB + x, yCB + y ]

- The construction process for not available or not existing samples in subclause S.8.3.6.2  is invoked with the sample array baseL[ x, y ] and, when chroma_format_idc is not equal to 0, the sample arrays baseCb[ x, y ] and baseCr[ x, y ] as inputs and the outputs are the sample array baseL[ x, y ] with modified sample values, and when chromat_format_idc is not equal to 0, the sample arrays baseCb[ x, y ] and baseCr[ x, y ] with modified sample values.

- The upsampling process for Intra_Base prediction in subclause S.8.3.6.3 is invoked with the sample array baseL[ x, y ], and when chroma_format_idc is not equal to 0, the sample arrays baseCb[ x, y ] and baseCr[ x, y ] as inputs and the outputs are predL[ x, y ], and when chroma_format_idc is not equal to 0, predCb[ x, y ] and predCr[ x, y ].

# References

[JSVM1]	Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG JVT-N023, “Joint Scalable Video Model JSVM 1”, J.Reichel, M.Wien, H.Schwarz

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | THOMSON |  |  |
| Mailing address | 1 avenue de belle fontaine 35511 CESSON SEVIGNE CEDEX FRANCE |  |  |
| Country | FRANCE |  |  |
| Contact person | Edouard Francois |  |  |
| Telephone | +33 2 99 27 32 62 |  |  |
| Fax |  |  |  |
| Email | edouard.francois@thomson.net |  |  |
| Place and date of submission | Busan, Korea, 16 april 05 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | THOMSON response 1 to Core experiments 9 and 10 : Generic Extended Spatial Scalability |  |
| Contribution number |  | JVT-O041 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  | Provisional filing (Thomson) |  |
| Inventor(s)/Assignee(s) |  | Marquant,Francois,Vieron (Thomson) |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: