| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 15th Meeting: Busan, KR, 16-22 April, 2005 | Document: JVT-O043r1 Filename: input.doc |
| --- | --- |

| Title: | CE5: Cross Verification of France Telecom Proposal |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Patrick Lopez THOMSON R&D France, Video Compression Lab, Corporate Research Rennes, 1, Avenue Belle Fontaine – CS17617, 35 576 Cesson Sevigne Cedex – France P. Onno and F. Le Léannec Canon Research Centre France Rue de la Touche Lambert 35517 Cesson-Sévigné Cedex - France | Tel: Email: Tel: Email: | +33 2 99 27 32 64 patrick.lopez@thomson.net + 33 2 99 87 68 92 patrice.onno@crf.canon.fr |
| Sources: | Thomson |  |  |

_____________________________

# Overview

SVC CE 5 addresses the insertion of optional information in the SVM bitstream, in order to improve the quality of extracted/decoded substreams as well as the access to an extended range of points, according to requirement 13: coding efficiency performance.

Two tools are proposed:

- Tool 1 called “Coding rate coverage extension with dead substreams in the SVM”

- Tool 2 called “Layered quality optimization for SVM”

This paper reports the cross-check result for the tool 1 and tool 2 of CE5 from France Telecom R&D, specified in documents  [1] , [2] and [3]. The cross verification of tool 1 and tool2 were respectively performed by Canon Research Centre France and Thomson R&D France.

# Tool 1 : Coding rate coverage extension with dead substreams in the SVM

## Evaluation Conditions

In this cross-checking procedure, the test conditions as specified in  [1], both for CIF and 4CIF sequences.

## Evaluation results

All PSNR results obtained and the bitrate of extracted/decoded substreams were equal to those claimed in the Core Experiment 5 detailed description. In particular, the experiment shows that all the bitrates listed in the table 3 in  [1] can be reached. In particular, high rate points in low resolutions are reachable thanks to the introduction of the dead substreams into the bitstreams.

Obtained results can be seen in the associated file “CE5_tool1_check.xls”. As for the Bus and City sequences, decoding points where the dead substreams have to be kept in the extracted SVC substreams are highlighted in bold font.  Moreover, decoding points not reachable by the current JSVM are underlined for Bus and City sequences.

Finally, note the slight different decoded bitrate obtained for sequence Bus_CIF7.5. This is explained by the fact that a portion of the dead substream, corresponding to an SNR of the QCIF_7.5 sequence, has to be kept in the extracted bitstream.

These results can be found in the attached Excel file:

Note: that we were not able to report meaningful results for the sequence Soccer in the associated “CE5_tool1_check.xls” file, since we used a different version of the original sequence Soccer than France Telecom.

## Recommendation

The expected gains mentioned in the CE5 are confirmed through this cross-check. Hence, the authors recommend WG11 to consider the dead substream functionality as technically valid.

# Tool 2 : Layered quality optimization for SVM

## Description of tools

The global objective is to improve the quality of extracted/decoded streams, by introducing either optional quality levels, allowing a better extraction of sub-streams, or an information on the localisation of what is called a "dead substream", allowing to extract points with a higher quality from low-level substreams.

## Evaluation conditions

Results have been submitted for the following sequences : Bus, Football, Foreman,  Mobile, City; Crew, Harbour and Soccer. The submission comprises the following items:

- Binary executables (decoder, extractor).

- Bitstreams at the highest spatio-temporal resolution (one bitstream per sequence).

- PSNR values for each target data rate.

To verify the results, we have performed the following steps:

- Extracting the bitstreams for all test points.

- Checking the bitstream sizes with the target bit rates.

- Decoding the bitstreams for every target point.

- Computing the PSNR values for every target point using the original sequences as references, and checking with the results provided by France Telecom.

## Evaluation results

The results can be found in the attached Excel document.

It can be seen that the bitstream sizes match the target sizes with negligible deviations. The bitstreams could be decoded without any problems. The PSNR values match the values provided by France Telecom with negligible deviations.

## Recommendation

We conclude that the results submitted by France Telecom for CE5 can be reproduced without any problem. Consequently we recommend to adopt this tool in the JSVM.

# Conclusion

The authors recommend to adopt the tools (1 and 2) proposed France Telecom for CE5l in the JSVM.

# References

[1] "CE5 Quality Layers description and detailed syntax", France Telecom R&D, April 2005.

[2] "Coding rate coverage extension with dead substreams in the SVM", I. Amonou, N. Cammas, S. Pateux, S. Kervadec, MPEG2005/M11703.

[3] "Layered quality optimization for SVM", I. Amonou, N. Cammas, S. Pateux, S. Kervadec, MPEG2005/M11704.