| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 15th Meeting: Busan, KR, 16-22 April, 2005 | Document: JVT-O044 Filename: JVT-O044.doc NOTE – That first character after the dash up there in the document number is an "O", not a zero. |
| --- | --- |

| Title: | Response to CE 5: Quality Layers |  |  |
| --- | --- | --- | --- |
| Status: | [Input Document to JVT] |  |  |
| Purpose: | [Report] |  |  |
| Author(s) or Contact(s): | Isabelle AMONOU Nathalie CAMMAS Sylvain KERVADEC Stéphane PATEUX France Télécom 4 Rue du Clos Courtel 35510 Cesson-Sévigné France | Tel: +33 2 99 12 44 77 Email: isabelle.amonou@francetelecom.com nathalie.cammas@francetelecom.com sylvain.kervadec@francetelecom.com stephane.pateux@francetelecom.com |  |
| Source: | [France Télécom] |  |  |

_____________________________

# Introduction

This document presents the response to CE 5 on quality layers.

Section 2 presents the tool 1 results for the three extraction paths specified:

- 2.1 inter-layer prediction

- 2.2 intra-layer prediction

- 2.3 additional streams

- 2.4 presents additional tests for the SD sequences

- 2.5 presents the decoding process

Section 3 presents the results for the tool 2 of the core experiment.

And section 5 concludes this paper.

# Tool 1: Coding rate coverage extension with dead substreams in the JSVM

The solution that we propose is to introduce "dead-substreams" in the embedded final bitstream [2]. The dead-substreams are additional streams that allow to reach higher rate points than the one used to predict the next higher layer.

Following is a table showing high rate points that can not be reached by the current J-SVM:

| Sequences | Prediction point | High rate points not reached |
| --- | --- | --- |
| Bus | QCIF15 128kbs | QCIF15 160,192kbs |
| Foreman | QCIF15 80kbs | QCIF15 96kbs |
| Football | QCIF15 256kbs | QCIF15 320,384kbs |
| Mobile | QCIF15 96kbs | QCIF15 112,128kbs |

Table 1: Rate points that can not be reached at QCIF 15Hz, for sequences of scenario 2

As for the SD sequences, all rate points can be reached, as described in the Core Experiment Conditions Tests, the prediction points for those sequences have been decreased. Following is a table showing the modified prediction points and the high rate points that can not be reached:

| Sequences | Prediction point | Rate points not reached |
| --- | --- | --- |
| City | QCIF15 112kbs CIF30 448kbs | QCIF15 128kbs CIF30 512kbs |
| Crew, Harbour, Soccer | QCIF15 160kbs CIF30 640kbs | QCIF15 192kbs CIF30 768kbs |

Table 2: Prediction points and missing rate points, for the sequences of scenario 1

As defined in the description of the Core Experiment Conditions Tests, three extraction paths have been defined:

- inter-layer prediction: each point can generate a lower spatio-temporal point, so dead substreams must be present at every point.  This case is similar to the extraction path proposed at the Munich meeting.

- intra-layer prediction: each high point of each spatio-temporal resolution generate the lower points of this spatio-temporal resolution. And, a high point of a lower spatial resolution is generated by the high point of the higher spatial resolution and same temporal resolution. This case is similar to the extraction path proposed at the Palma meeting. Dead-substreams must be present only at each high point of each spatio-temporal resolution. The use of dead-substreams only impacts on these high points.

- additional streams: dead substreams are considered as additional streams, so they do not impact on the PSNR performances.

These extraction paths are shown on the figure 1.

Figure 1: The different extraction paths

The format and bit rates for each sequence are defined in the table 3. All extraction paths defined must include those points. Those points are the ones defined at the Palma meeting [6521]. Points selected at the Munich meeting appear in red.

For the SD sequences (City, Crew, Harbour, Soccer), in the case of the intra-layer prediction, the Munich points do not respect the extraction path specified. The CIF 30Hz high rate point is not predicted by the 4CIF 30Hz high rate point but by the 4CIF 30Hz middle rate point. So the Munich points have been removed from the test sets for the intra-layer prediction extraction, so that the CIF 30Hz high rate point is predicted by the 4CIF 30Hz high rate point.

The Munich points refers to the extraction path with inter-layer prediction.

| Sequence | Format | Bit rates (kbit/sec) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bus | QCIF 7.5Hz | 64 | 80 | 96 | 112 | 128 |
|  | QCIF 15Hz | 96 | 112 | 128 | 160 | 192 |
|  | CIF 7.5Hz | 128 | 160 | 192 | 224 | 256 |
|  | CIF 15Hz | 192 | 224 | 256 | 320 | 384 |
|  | CIF 30Hz | 256 | 320 | 384 | 448 | 512 |
| Football | QCIF 7.5Hz | 128 | 160 | 192 | 224 | 256 |
|  | QCIF 15Hz | 192 | 224 | 256 | 320 | 384 |
|  | CIF 7.5Hz | 256 | 320 | 384 | 448 | 512 |
|  | CIF 15Hz | 384 | 448 | 512 | 640 | 768 |
|  | CIF 30Hz | 512 | 640 | 768 | 896 | 1024 |
| Foreman | QCIF 7.5Hz | 32 | 40 | 48 | 56 | 64 |
|  | QCIF 15Hz | 48 | 56 | 64 | 80 | 96 |
|  | CIF 7.5Hz | 64 | 80 | 96 | 112 | 128 |
|  | CIF 15Hz | 96 | 112 | 128 | 160 | 192 |
|  | CIF 30Hz | 128 | 160 | 192 | 224 | 256 |
| Mobile | QCIF 7.5Hz | 48 | 56 | 64 | 80 | 96 |
|  | QCIF 15Hz | 64 | 80 | 96 | 112 | 128 |
|  | CIF 7.5Hz | 96 | 112 | 128 | 160 | 192 |
|  | CIF 15Hz | 128 | 160 | 192 | 224 | 256 |
|  | CIF 30Hz | 192 | 224 | 256 | 320 | 384 |
| City | QCIF 15Hz | 64 | 80 | 96 | 112 | 128 |
|  | CIF 7.5Hz | 128 | 160 | 192 | 224 | 256 |
|  | CIF 15Hz | 192 | 224 | 256 | 320 | 384 |
|  | CIF 30Hz | 256 | 320 | 384 | 448 | 512 |
|  | 4CIF 15Hz | 512 | 640 | 768 | 896 | 1024 |
|  | 4CIF 30Hz | 768 | 896 | 1024 | 1280 | 1536 |
|  | 4CIF 60Hz | 1024 | 1280 | 1536 | 1792 | 2048 |
| Crew, Harbour, Soccer | QCIF 15Hz | 96 | 112 | 128 | 160 | 192 |
|  | CIF 7.5Hz | 192 | 224 | 256 | 320 | 384 |
|  | CIF 15Hz | 256 | 320 | 384 | 448 | 512 |
|  | CIF 30Hz | 384 | 448 | 512 | 640 | 768 |
|  | 4CIF 15Hz | 768 | 896 | 1024 | 1280 | 1536 |
|  | 4CIF 30Hz | 1024 | 1280 | 1536 | 1792 | 2048 |
|  | 4CIF 60Hz | 1536 | 1780 | 2048 | 2560 | 3072 |

Table 3: Spatio-SNR-temporal scalability test points (from Palma meeting; Munich test points indicated in red.)

The modified syntax can be found in the annex D.

For the experiments, we use the JSVM1.0 as a basis.

## Inter-layer prediction

In this experiment, every point must keep the dead substream. But, some points are too low to keep the dead substream without truncating it. So, in these cases, we throw away the dead substream and extract the bitstream with the desired rate. These points can be identified on the R/D curves when the curve with dead substream meets the reference curve.

For the other points, the bitrate difference corresponds to the size of the dead-substreams.

Results can be found in the annex A1.

## Intra-layer prediction

In this test, the use of dead substreams impacts only on the high rate points of the spatio-temporal resolutions that have to generate a high rate point of a lower spatial resolution. Except for those points, the PSNRs are the same as the reference ones, and the dead substreams allow to reach higher rate points on QCIF for CIF sequences and on QCIF and CIF for SD sequences.

Results can be found in the annex A2.

## Additional streams

In this test, the use of dead substreams does not impact on the PSNR performances.

The curves are exactly the same as the reference ones, except that the dead substreams allow reaching higher rate points on QCIF for CIF sequences and on QCIF and CIF for SD sequences.

Results can be found in the annex A3.

For all extraction path, the size of the SEI NAL containing the information to identify the dead substream has been taken into account in the final bitstream. This cost does not penalize the PSNR performances as when the dead substream is not present in the bitstream we obtain the same PSNR performances as the reference.

## Additional tests for the SD sequences

For the SD sequences, as no point was missing, in the Core Experiment, we modified the prediction in order to introduce missing points. The results for this test were given above.

Here, we compare the reference software J-SVM with no modification of the prediction points (i.e. it can reach all the rate points) and the software with dead substreams with modified prediction points (i.e. there are missing points)

We compare the Rate/Distortion curves for each extraction path.

On the R/D curves given for the additional stream and intra-layer extraction path, we can see that decreasing the prediction points of QCIF and CIF to code the SD sequences improves the PSNRs results from 0.1 to 0.3dB on the sequences City, Harbour and Soccer. Decreasing the prediction points implies that some high points on QCIF and CIF may not be reached. But, the use of dead substream allows reaching those points.

For the inter-layer extraction path, the gain obtained when the prediction points have been decreased allows to compensate the loss due to the use of the dead substreams in this path scenario.

### Inter-layer prediction

Results can be found in the annex B1.

### Intra-layer prediction

Results can be found in the annex B2.

### Additionnal streams

Results can be found in the annex B3.

## Decoding process with dead susbtreams

Keeping dead substreams in an extracted bitstream that is going to be decoded decreases the PSNR performances because the datas contained in the dead substream is also decoded by the decoder. The decoder can not identify dead substream information from a lower spatial layer from refinement information that must be used for that layer. This implies a drift in the decoding process and impacts on the calculated PSNR. An example of such a drift is given for the sequence Bus in the case of the inter-layer prediction on the figure 2. We compare the PSNRs given with the dead substream in the inter-layer extraction path when dead substream is removed before decoding (DS) and when it is not (DS drift). We can observe a loss until 0.54dB due to the presence of the dead substream at the decoder.

Figure 2: Drift at decoding if no additional extraction is performed to remove the dead substream before the decoding

For the core experiment, we performed an additional extraction before the decoding process, in order to remove the dead substreams from the extracted bitstream.

# Tool 2: Layered quality optimization for JSVM

This section presents the PSNR results for the tool 2 (quality levels).

The aim of this tool is to introduce quality levels information in the final bitstream in order to optimize the reconstruction quality of a target point by selecting the best rate allocation among coded frames [3].  The modified syntax can be found in the annex E.

On the Rate/Distortion curves, we can observe that the quality levels allow to improve PSNR results for some sequences (Mobile, Bus, City), and for other sequences give similar results as the JSVM1.0.

The size of the SEI NAL has been taken into account in the final bitstream containing the quality levels. We can see that this cost does not penalize our results, as in the worst case, quality levels performs as well as the reference.

Results can be found in the annex C.

# Tool 1&2: Combined layered quality optimization for JSVM

Due to lack of time, we have not tested this solution yet.

# Conclusions and further studies

To conclude, we have shown for the tool 1 (dead substream) that the introduction of dead substreams allows to reach higher rate points than the ones used for prediction of the next higher layer with little impact on the PSNR performances, depending on the extraction path used. In the case of simulcast applications, there is no impact on the performances.

We have also shown for the tool 1 the necessity to have a normative syntax in order to avoid the drift at the decoder due to the presence of the dead substream.

For the tool 2 (quality layers), we have shown improved PSNR for some sequences from 0.1 to 0.52 dB.

For other sequences, no gain has been shown and PSNR are the same as those of the reference. Further studies will deal with improving the Rate/Distortion model and looking for an exhaustive search for quality layers.

We need also to test the combination of tool 1 and 2: i.e. considering the dead substream as a high level of quality.

References

[1] "Joint Scalable Video Model JSVM 1", Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6), 14th meeting: Hong-Kong, CN, 17-21 January, 2005, document JVT-N021.

[2] "Coding rate coverage extension with dead substreams in the SVM", ISO/IEC JTC1/SC29/WG11/MPEG2005/M11703, Hong-Kong, January 2005

[3] "Layered quality optimization for SVM", ISO/IEC JTC1/SC29/WG11/MPEG2005/M11704, Hong-Kong, January 2005

[4] "Description of Core Experiments in SVC", ISO/IEC JTC1/SC29/WG11/N6898, Hong-Kong, January 2005

Annex A: Rate/Distortion curves for tool 1

A.1 Inter-layer prediction

A.2 Intra-layer prediction

A.3 Additionnal streams

Annex B: Additional tests results for tool 1: dead-substreams

B.1 Inter-layer prediction

B.2 Intra-layer prediction

B.3 Additional streams

Annex C: Rate/Distortion curves for tool 2: quality layers

Annex D: Modified syntax for the tool 1

For each spatio-temporal frame of a video sequence, a dead substream is optionally indicated.

Since this additional information is desirable but not necessary, we propose to transmit this additional information as SEI (Supplemental enhancement information) messages, by introducing a new SEI message

Note that in this proposal, the dead-substream information (thus the SEI message) is attached to a spatio-temporal image layer (including Base layer + FGS enhancement layers), i.e. what is defined as a CLASS in m11708.

SEI payload syntax

| sei_payload( payloadType, payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| if( payloadType = = 0 ) |  |  |
| buffering_period( payloadSize ) | 5 |  |
| else … |  |  |
| else if( payloadType = = 42 ) |  |  |
| dead_substream_info( payloadSize ) | 5 |  |
| else |  |  |
| reserved_sei_message( payloadSize ) | 5 |  |
| if( !byte_aligned( ) ) { |  |  |
| bit_equal_to_one /* equal to 1 */ | 5 | f(1) |
| while( !byte_aligned( ) ) |  |  |
| bit_equal_to_zero /* equal to 0 */ | 5 | f(1) |
| } |  |  |
| } |  |  |

| dead_substream_info ( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| delta_dead_substream | 5 | ue(v) |
| } |  |  |

SEI payload semantics

dead_substream_info SEI message semantics

dead substream info defines an information useful for the extraction of dead sub-streams from a scalable bit-stream. When used, SEI message must be present before the extraction of a sub-stream of the targeted frame.

delta_dead_substream: specifies the offset in bytes from where the dead substream information starts.

Annex E: Modified syntax for the tool 2

For each spatio-temporal slice of a frame of the video sequence, quality levels are defined. For the sake of simplicity, we define this additional information as SEI message (For the CE process, it thus implies no modifications in the main syntax). When present, one SEI NAL Unit applies to the next spatio-temporal slice (Base layer plus FGS refinement data NAL Units).

Here are the modifications on the current SVM syntax (introducing a new AVC SEI message)

SEI payload syntax

| sei_payload( payloadType, payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| if( payloadType = = 0 ) |  |  |
| buffering_period( payloadSize ) | 5 |  |
| else … |  |  |
| else if( payloadType = = 40 ) |  |  |
| quality_levels_info( payloadSize ) | 5 |  |
| else |  |  |
| reserved_sei_message( payloadSize ) | 5 |  |
| if( !byte_aligned( ) ) { |  |  |
| bit_equal_to_one /* equal to 1 */ | 5 | f(1) |
| while( !byte_aligned( ) ) |  |  |
| bit_equal_to_zero /* equal to 0 */ | 5 | f(1) |
| } |  |  |
| } |  |  |

| quality_levels_info( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| num_quality_levels | 5 | u(4) |
| for( i = 0; i < num_quality_levels; i++ ) { |  |  |
| quality_level[I] | 5 | u(8) |
| delta_quality_level_byte_offset[i] } | 5 | ue(v) |
| } |  |  |

SEI payload semantics

Quality levels SEI message semantics

Quality levels define a set of information useful for an optimal extraction of sub-streams from a scalable bit-stream. When used, SEI message must be present before the extraction of a sub-stream of the targeted slice.

num_quality_levels : specifies the number of quality levels defined for this frame

quality_level[i] : specifies the value of the i-th quality level

delta_quality_level_byte_offset[i] : specifies the number of bytes that should be extracted (starting from the beginning of the first data NAL unit, each quality level offset being coded as an increment to the previous quality level offset) for the i-th quality level.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name |  |  |  |
| Mailing address |  |  |  |
| Country |  |  |  |
| Contact person |  |  |  |
| Telephone |  |  |  |
| Fax |  |  |  |
| Email |  |  |  |
| Place and date of submission |  |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  |  |  |
| Title |  |  |  |
| Contribution number |  |  |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: