| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 15th Meeting: Busan, KR, 16-22 April, 2005 | Document: JVT-O050 Filename: JVT-O050.doc NOTE – That first character after the dash up there in the document number is an "O", not a zero. |
| --- | --- |

| Title: | CE Verification Report of Thomson on CE10 : Extended spatial scalability |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Isabelle Amonou, Nathalie Cammas, Sylvain Kervadec, Stephane Pateux France Telecom Division R&D 4, rue du Clos Courtel. BP 59 35 512 Cesson Sevigne Cedex FRANCE | Tel: Email: | +33 2 99 12 44 77 isabelle.amonou@francetelecom.com nathalie.cammas@francetelecom.com sylvain.kervadec@francetelecom.com stephane.pateux@francetelecom.com |
| Source: | France Telecom Division R&D |  |  |

_____________________________

# Overview

The main purpose of this core experiment is to verify the performance of the CE 10 related to upsampling for Extended Spatial Scalability (horizontal and vertical dimension ratios from one spatial layer to its enhancement spatial layer not equal to 2).

Three ratios between low and high resolution layers are considered : 3/4, 2/3, 3/5.

For each of these ratios, the goal of the CE w to compare three different solutions:

- Anchor solution : ‘classical’ dyadic scalable coding with upsampling of the low dyadic layer to get the right low-layer dimensions

- Proposal 1 : scalable coding with a 2/3 size-ratio between low- and high-layers (3/2 tool), and upsampling of the low 2/3-layer to get the right low-layer dimensions

- Proposal 2 : generic scalable coding (generic tool ) ; any value can be considered as high-low size-ratio, and low-and high-layers are directly encoded with their real dimensions.

# Details of verification task

  - General settings

This core experiment defined new sequences sizes and ratios as well as target bit-rates, as summarized in the tables below.

Table 1  Test sequences and the corresponding sizes and bit-rate points for 3/2 ratio

|  | High | Low | Output (resampled) | Resampling ratio |
| --- | --- | --- | --- | --- |
| Dyadic | 672*576 | 336*288 | 448*384 | 4/3 |
| 3/2 | 672*576 | 448*384 | 448*384 | - |
| Generic | 672*576 | 448*384 | 448*384 | - |
| Rate City (kbps) | 1000 |  | 720 |  |
| Rate Crew, Harbour, Soccer (kbps) | 1470 |  | 1050 |  |

Table 2 Test sequences and the corresponding sizes and bit-rate points for 3/4 ratio

|  | High | Low | Output (Low resampled) | Resampling ratio |
| --- | --- | --- | --- | --- |
| Dyadic | 704*576 | 352*288 | 528*432 | 3/2 |
| 3/2 | 704*576 | 464*384 crop | 522*432 crop | 9/8 |
| Generic | 704*576 | 528*432 | 538*432 | - |
| Rate City (kbps) | 1024 |  | 810 |  |
| Rate Crew, Harbour, Soccer (kbps) | 1500 |  | 1190 |  |

Table 3  Test sequences and the corresponding sizes and bit-rate points for 3/5 ratio

|  | High | Low | Output (Low resampled) | Resampling ratio |
| --- | --- | --- | --- | --- |
| Dyadic | 640*560 | 320*272 crop | 384*326 crop | 6/5 |
| 3/2 | 640*560 | 416*368 crop | 374*330 crop | 9/10 |
| Generic | 640*560 | 384*336 | 384*336 | - |
| Rate City (kbps) | 980 |  | 610 |  |
| Rate Crew, Harbour, Soccer (kbps) | 1430 |  | 890 |  |

The verification task was performed for all three simulation types (dyadic, 3/2 or generic) for the three ratios (3/2, 3/4, 3/5)

In the sequel, the results are provided per ratio.

In three cases, the verification procedure consisted of the following steps:

(1) Coding some of the sequences at the bit-rate points listed above by using the Thomson software, then checking the output streams size and calculating the actual bit-rates ; this was nor done for all sequences (only those showing a figure in the corresponding cell).

(2) Decoding the sequences and computing PSNR values of the reconstructed images

(3) Checking the decoded images (aspect and ratio)

  - Test comparisons (PSNR, rate, visual check)

The results of the test are summarized in the three tables below. Resulats provided by Thomson are in black. Cross-check Results are in blue.

Table 4  Results for CE10 test 1 (ratio 3/2)

| CITY | low layer | 720 | kbps |  |  | high layer | 1000 | kbps |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | real bitrate | psnr Y | psnr U | psnr V | delta bitrate | real bitrate | psnr Y | psnr U | psnr V | delta bitrate | Visual check |
| Thomson results |  |  |  |  |  |  |  |  |  |  |  |
| Dyadic | 705,66 | 29,43 | 42,90 | 43,51 | -1,99% | 981,11 | 32,69 | 45,16 | 46,97 | -1,89% |  |
| Ratio 3/2 | 732,07 | 35,44 | 44,51 | 46,27 | 1,68% | 993,87 | 34,05 | 44,68 | 46,44 | -0,61% |  |
| Generic | 732,07 | 35,44 | 44,51 | 46,27 | 1,68% | 983,97 | 34,02 | 44,68 | 46,45 | -1,60% |  |
| Cross-check results |  |  |  |  |  |  |  |  |  |  |  |
| dyadic | 705,71 | 29,43 | 42,90 | 43,51 | -1,98% | 981,3 | 32,69 | 45,16 | 46,97 | -1,87% | OK |
| ratio 3/2 | - | 35,44 | 44,51 | 46,27 | - | - | 34,05 | 44,68 | 46,44 | - | OK |
| generic | - | 35,44 | 44,51 | 46,27 | - | - | 34,02 | 44,68 | 46,45 | - | OK |
| CREW | low layer | 1050 | kbps |  |  | High layer | 1470 | kbps |  |  |  |
| Thomson results |  |  |  |  |  |  |  |  |  |  |  |
|  | real bitrate | psnr Y | psnr U | psnr V | delta bitrate | real bitrate | psnr Y | psnr U | psnr V | delta bitrate |  |
| Dyadic | 1049,62 | 34,77 | 40,03 | 39,28 | -0,04% | 1465,98 | 35,03 | 41,87 | 41,31 | -0,27% |  |
| Ratio 3/2 | 1069,01 | 36,02 | 41,57 | 40,91 | 1,81% | 1458,41 | 35,29 | 42,02 | 41,45 | -0,79% |  |
| Generic | 1069,01 | 36,02 | 41,57 | 40,91 | 1,81% | 1465,22 | 35,33 | 42,03 | 41,47 | -0,32% |  |
| Cross-check results |  |  |  |  |  |  |  |  |  |  |  |
| Dyadic | - | 34,77 | 40,03 | 39,28 | - | - | 35,03 | 41,87 | 41,31 | - | OK |
| Ratio 3/2 | - | 36,02 | 41,57 | 40,91 | - | - | 35,29 | 42,02 | 41,45 | - | OK |
| Generic | - | 36,02 | 41,57 | 40,91 | - | - | 35,33 | 42,03 | 41,47 | - | OK |
| HARBOUR | low layer | 1050 | kbps |  |  | high layer | 1470 | kbps |  |  |  |
| Thomson results |  |  |  |  |  |  |  |  |  |  |  |
|  | real bitrate | psnr Y | psnr U | psnr V | delta bitrate | real bitrate | psnr Y | psnr U | psnr V | delta bitrate |  |
| Dyadic | 1058,18 | 29,28 | 41,41 | 42,21 | 0,78% | 1479,71 | 31,25 | 43,10 | 45,17 | 0,66% |  |
| ratio 3/2 | 1063,44 | 31,93 | 42,42 | 44,15 | 1,28% | 1481,42 | 31,69 | 42,97 | 44,69 | 0,78% |  |
| generic | 1063,44 | 31,93 | 42,42 | 44,15 | 1,28% | 1474,86 | 31,70 | 42,97 | 44,69 | 0,33% |  |
| Cross-check results |  |  |  |  |  |  |  |  |  |  |  |
| dyadic | - | 29,28 | 41,41 | 42,21 | - | - | 31,25 | 43,10 | 45,17 | - | OK |
| ratio 3/2 | - | 31,93 | 42,42 | 44,15 | - | - | 31,69 | 42,97 | 44,69 | - | OK |
| generic | - | 31,93 | 42,42 | 44,15 | - | - | 31,70 | 42,97 | 44,69 | - | OK |
| SOCCER | low layer | 1050 | kbps |  |  | high layer | 1470 | kbps |  |  |  |
| Thomson results |  |  |  |  |  |  |  |  |  |  |  |
|  | real bitrate | psnr Y | psnr U | psnr V | delta bitrate | real bitrate | psnr Y | psnr U | psnr V | delta bitrate |  |
| dyadic | 1044,03 | 33,74 | 41,50 | 42,37 | -0,57% | 1488,46 | 34,67 | 42,85 | 44,96 | 1,26% |  |
| ratio 3/2 | 1057,82 | 36,24 | 43,31 | 45,22 | 0,74% | 1492,19 | 35,18 | 43,05 | 45,04 | 1,51% |  |
| generic | 1057,81 | 36,24 | 43,31 | 45,22 | 0,74% | 1452,56 | 35,10 | 43,03 | 45,01 | -1,19% |  |
| Cross-check results |  |  |  |  |  |  |  |  |  |  |  |
| dyadic | - | 33,74 | 41,51 | 42,38 | - | - | 34,67 | 42,85 | 44,96 | - | OK |
| ratio 3/2 | - | 36,24 | 43,31 | 45,22 | - | - | 35,18 | 43,05 | 45,04 | - | OK |
| generic | - | 36,24 | 43,31 | 45,22 | - | - | 35,10 | 43,02 | 45,01 | - | OK |

Table 5  Results for CE10 test 2 (ratio 4/3)

| CITY | low layer | 810 | kbps |  |  | high layer | 1024 | kbps |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | real bitrate | psnr Y | psnr U | psnr V | delta bitrate | real bitrate | psnr Y | psnr U | psnr V | delta bitrate | Visual check |
| Thomson results |  |  |  |  |  |  |  |  |  |  |  |
| dyadic | 804,83 | 29,83 | 43,31 | 45,39 | -0,64% | 1010,48 | 32,24 | 42,76 | 45,34 | -1,32% |  |
| ratio 3/2 | 822,71 | 34,89 | 43,24 | 45,16 | 1,57% | 1020,97 | 33,69 | 42,91 | 45,24 | -0,30% |  |
| generic | 827,38 | 35,84 | 43,94 | 45,91 | 2,15% | 1034,01 | 33,96 | 42,92 | 45,21 | 0,98% |  |
| Cross-check results |  |  |  |  |  |  |  |  |  |  |  |
| dyadic | - | 29,83 | 43,31 | 45,39 | - | - | 32,24 | 42,76 | 45,34 | - | OK |
| ratio 3/2 | - | 34,89 | 43,24 | 45,16 | - | - | 33,69 | 42,91 | 45,24 | - | OK |
| generic | - | 35,84 | 43,94 | 45,91 | - | - | 33,96 | 42,92 | 45,21 | - | OK |
| CREW | low layer | 1190 | kbps |  |  | High layer | 1500 | kbps |  |  |  |
| Thomson results |  |  |  |  |  |  |  |  |  |  |  |
|  | real bitrate | psnr Y | psnr U | psnr V | delta bitrate | real bitrate | psnr Y | psnr U | psnr V | delta bitrate |  |
| dyadic | 1211,20 | 35,23 | 40,78 | 40,03 | 1,78% | 1490,08 | 34,97 | 40,41 | 40,43 | -0,66% |  |
| ratio 3/2 | 1176,53 | 36,22 | 40,75 | 40,05 | -1,13% | 1511,90 | 35,35 | 40,62 | 40,75 | 0,79% |  |
| generic | 1201,04 | 36,27 | 41,00 | 40,77 | 0,93% | 1509,83 | 34,99 | 40,42 | 40,45 | 0,66% |  |
| Cross-check results |  |  |  |  |  |  |  |  |  |  |  |
| dyadic | - | 35,23 | 40,78 | 40,03 | - | - | 34,97 | 40,41 | 40,43 | - | OK |
| ratio 3/2 | - | 36,22 | 40,75 | 40,05 | - | - | 35,35 | 40,62 | 40,75 | - | OK |
| generic | - | 36,27 | 41,00 | 40,77 | - | - | 34,99 | 40,42 | 40,45 | - | OK |
| HARBOUR | low layer | 1050 | kbps |  |  | high layer | 1470 | kbps |  |  |  |
| Thomson results |  |  |  |  |  |  |  |  |  |  |  |
|  | real bitrate | psnr Y | psnr U | psnr V | delta bitrate | real bitrate | psnr Y | psnr U | psnr V | delta bitrate |  |
| dyadic | 1212,24 | 29,75 | 41,28 | 43,74 | 1,87% | 1519,73 | 30,93 | 41,88 | 44,09 | 1,32% |  |
| ratio 3/2 | 1174,42 | 31,97 | 40,90 | 43,41 | -1,31% | 1500,57 | 31,41 | 41,78 | 43,76 | 0,04% |  |
| generic | 1183,14 | 31,99 | 41,90 | 43,76 | -0,58% | 1517,93 | 31,22 | 41,76 | 43,75 | 1,20% |  |
| Cross-check results |  |  |  |  |  |  |  |  |  |  |  |
| dyadic | - | 29,75 | 41,28 | 43,74 | - | - | 30,93 | 41,88 | 44,09 | - | OK |
| ratio 3/2 | - | 31,97 | 40,90 | 43,40 | - | - | 31,41 | 41,78 | 43,76 | - | OK |
| generic | - | 31,99 | 41,90 | 43,76 | - | - | 31,22 | 41,76 | 43,75 | - | OK |
| SOCCER | low layer | 1050 | kbps |  |  | high layer | 1470 | kbps |  |  |  |
| Thomson results |  |  |  |  |  |  |  |  |  |  |  |
|  | real bitrate | psnr Y | psnr U | psnr V | delta bitrate | real bitrate | psnr Y | psnr U | psnr V | delta bitrate |  |
| dyadic | 1179,31 | 33,91 | 43,18 | 44,74 | -0,90% | 1499,80 | 34,20 | 42,85 | 44,95 | -0,01% |  |
| ratio 3/2 | 1181,99 | 35,87 | 43,41 | 44,10 | -0,67% | 1504,34 | 34,84 | 42,95 | 44,95 | 0,29% |  |
| generic | 1204,31 | 36,46 | 43,66 | 45,52 | 1,20% | 1505,02 | 34,42 | 42,80 | 44,73 | 0,33% |  |
| Cross-check results |  |  |  |  |  |  |  |  |  |  |  |
| dyadic | - | 33,91 | 43,18 | 44,74 | - | - | 34,20 | 42,85 | 44,95 | - | OK |
| ratio 3/2 | - | 35,87 | 43,41 | 44,10 | - | - | 34,84 | 42,94 | 44,95 | - | OK |
| generic | - | 36,46 | 43,66 | 45,52 | - | - | 34,42 | 42,80 | 44,72 | - | OK |

Table 6  Results for CE10 test 3 (ratio 5/3)

| CITY | low layer | 610 | kbps |  |  | high layer | 980 | kbps |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | real bitrate | psnr Y | psnr U | psnr V | delta bitrate | real bitrate | psnr Y | psnr U | psnr V | delta bitrate | Visual check |
| Thomson results |  |  |  |  |  |  |  |  |  |  |  |
| dyadic | 601,22 | 29,91 | 42,67 | 44,17 | -1,44% | 997,97 | 33,61 | 42,94 | 45,39 | 1,83% |  |
| ratio 3/2 | 601,24 | 34,06 | 42,33 | 44,27 | -1,44% | 967,69 | 34,32 | 42,96 | 45,19 | -1,26% |  |
| generic | 621,78 | 35,30 | 43,21 | 45,11 | 1,93% | 963,08 | 33,99 | 42,89 | 45,18 | -1,73% |  |
| Cross-check results |  |  |  |  |  |  |  |  |  |  |  |
| dyadic | 601,20 | 29,91 | 42,66 | 44,17 | -1,44% | 998 | 33,61 | 42,95 | 45,39 | 1,83% | OK |
| ratio 3/2 | - | 34,06 | 42,33 | 44,27 | - | - | 34,32 | 42,96 | 45,18 | - | OK |
| generic | - | 35,30 | 43,21 | 45,11 | - | - | 33,99 | 42,89 | 45,18 | - | OK |
| CREW | low layer | 890 | kbps |  |  | High layer | 1430 | kbps |  |  |  |
| Thomson results |  |  |  |  |  |  |  |  |  |  |  |
|  | real bitrate | psnr Y | psnr U | psnr V | delta bitrate | real bitrate | psnr Y | psnr U | psnr V | delta bitrate |  |
| dyadic | 887,24 | 33,66 | 39,00 | 37,94 | -0,31% | 1407,53 | 35,07 | 40,32 | 40,40 | -1,57% |  |
| ratio 3/2 | 906,04 | 34,45 | 39,31 | 38,20 | 1,80% | 1426,67 | 35,36 | 40,48 | 40,67 | -0,23% |  |
| generic | 898,96 | 35,09 | 39,68 | 39,08 | 1,01% | 1424,63 | 35,11 | 40,37 | 40,51 | -0,38% |  |
| Cross-check results |  |  |  |  |  |  |  |  |  |  |  |
| dyadic | 887,56 | 33,66 | 39,01 | 37,93 | -0.27% | 1406,23 | 35,07 | 40,32 | 40,40 | -1,66% | OK |
| ratio 3/2 | - | 34,45 | 39,30 | 38,20 | - | - | 35,36 | 40,48 | 40,67 | - | OK |
| generic | - | 35,09 | 39,68 | 39,08 | - | - | 35,10 | 40,37 | 40,51 | - | OK |
| HARBOUR | low layer | 1050 | kbps |  |  | high layer | 1470 | kbps |  |  |  |
| Thomson results |  |  |  |  |  |  |  |  |  |  |  |
|  | real bitrate | psnr Y | psnr U | psnr V | delta bitrate | real bitrate | psnr Y | psnr U | psnr V | delta bitrate |  |
| dyadic | 894,55 | 28,31 | 40,56 | 42,14 | 0,51% | 1451,09 | 31,28 | 41,75 | 43,85 | 1,47% |  |
| ratio 3/2 | 907,37 | 29,83 | 40,31 | 42,60 | 1,95% | 1444,08 | 31,91 | 41,99 | 43,98 | 0,98% |  |
| generic | 875,03 | 30,81 | 40,83 | 42,58 | -1,68% | 1449,78 | 31,46 | 41,95 | 43,89 | 1,38% |  |
| Cross-check results |  |  |  |  |  |  |  |  |  |  |  |
| dyadic | 894,59 | 28,31 | 40,56 | 42,14 | 0,51% | 1451,47 | 31,29 | 41,75 | 43,85 | -1.25% | OK |
| ratio 3/2 | - | 29,83 | 40,31 | 42,60 | - | - | 31,91 | 41,98 | 43,98 | - | OK |
| generic | - | 30,81 | 40,83 | 42,57 | - | - | 31,46 | 41,95 | 43,89 | - | OK |
| SOCCER | low layer | 1050 | kbps |  |  | high layer | 1470 | kbps |  |  |  |
| Thomson results |  |  |  |  |  |  |  |  |  |  |  |
|  | real bitrate | psnr Y | psnr U | psnr V | delta bitrate | real bitrate | psnr Y | psnr U | psnr V | delta bitrate |  |
| dyadic | 893,80 | 33,17 | 41,25 | 42,67 | 0,43% | 1401,96 | 34,72 | 42,77 | 44,88 | -1,96% |  |
| ratio 3/2 | 901,77 | 34,57 | 42,16 | 43,18 | 1,32% | 1421,51 | 35,27 | 43,05 | 45,04 | -0,59% |  |
| generic | 892,08 | 35,58 | 42,43 | 44,42 | 0,23% | 1444,52 | 34,88 | 42,97 | 44,92 | 1,02% |  |
| Cross-check |  |  |  |  |  |  |  |  |  |  |  |
| dyadic | - | 33,18 | 41,25 | 42,67 | - | - | 34,71 | 42,77 | 44,88 | - | OK |
| ratio 3/2 | - | 34,57 | 42,15 | 43,17 | - | - | 35,27 | 43,05 | 45,04 | - | OK |
| generic | - | 35,58 | 42,43 | 44,42 | - | - | 34,88 | 42,97 | 44,91 | - | OK |

# Conclusion

From this cross-check it can be concluded that all the provided bitstreams, coder and decoder executables as well as configuration files for CE10 are all correct.

The results (PSNR, rates, output sizes with or without cropping) given by Thomson were successfully cross-checked, the difference between their results and the cross-check ones being negligible.