| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 13th Meeting: Busan, Korea 18-22 April, 2005 | Document: JVT-O052 Filename: input.docc |
| --- | --- |

| Title: | Technical description of the Thomson proposal for SVC CE6 - non-scalable motion vector coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Peng Yin, Jill Boyce, Purvin Pandit 2 Independence Way Princeton, NJ, 08540 | Tel: Email: | 1-609.987.7355 peng.yin@thomson.net Jill.boyce@thomson.net |
| Source: | Thomson Inc. |  |  |

_____________________________

## Introduction

This document presents the technical description and experimental result in response to CE6 [1], which aims to evaluate the coding efficiency of non-scalable motion vector (MV) coding in the case of complexity scalability for broadcast. The description is based on the Thomson's proposal in VCEG-X06[5], m11241[2]and m11682 [3].

In broadcast environments, all layers of a scalable bitstream are broadcast over the same network and are available to all devices.  In a two-layer system, two layers are broadcast, and low-end devices use only the lower (base) layer, and devices with higher capabilities use both the base and the enhancement layers.  In such an environment, total bitrate efficiency of the combined layers is an important requirement, but the bitrate efficiency of a lower layer individually is not a requirement.  By relaxing the typical scalability requirement of bitrate efficiency of the lower layer, total bitrate efficiency for combined layers can be improved.

In our proposal, the motion vectors transmitted in the base layer bitstream are used in both the low resolution decoder and the full resolution decoder, but with a higher accuracy in the full resolution decoder than in the low resolution decoder.  Table 1 shows the operation of decoder for low resolution and full resolution.

In our scheme, motion vector is non-scalably coded to improve the coding efficiency. So for the base layer decoding, we need to downscale the MV by a factor of 2.  This raises the issue that that P and B slices in the base layer are not compatible with AVC. In this core experiment, we shall evaluate our non-scalable motion vector coding compared to scalable motion vector coding in JSVM1.0 [4] for our proposed complexity scalable video coding scheme.

It is known that for total coding efficiency of MV, non-scalable MV coding has better performance than scalable MV coding. When bitrate scalability is used, non-scalable MV coding suffers for the base layer, because all MVs have to be sent in the base layer. But if the bitrate requirement of base layer is relaxed and total bitrate efficiency is the most important  requirement, for example, in broadcast environment, non-scalable MV coding will have better coding efficiency than scalable MV coding, which is especially true for low to medium bitrate. The fact is confirmed by this core experiment. For very low bitrate, a big total coding efficiency loss is observed when motion vector is scalable coded for both P and B slices.

- Table 1. Operation of decoder

| decoder | MV | residue | MC | ref. store |
| --- | --- | --- | --- | --- |
| low resolution | downscale | - | low resolution | low resolution |
| full resolution | - | upsample | full resolution | full resolution |

## Syntax in tabular form

In this section, only the syntax that is added or extended in comparison to AVC or JSVM 1.0 [4] for our proposal is described.

We use profile_idc = 92 to indicate the bitstream has the feature proposed by Thomson’s complexity scalable video coding techniques where non-scalable MV coding is used[2][3][5].

2.1 slice header syntax

| slice_header( ) { | C | Descriptor |
| --- | --- | --- |
| first_mb_in_slice | 2 | ue(v) |
| slice_type | 2 | ue(v) |
| pic_parameter_set_id | 2 | ue(v) |
| frame_num | 2 | u(v) |
| /* complexity scalability parameters*/ |  |  |
| if (profile_idc == 92 && slice_type != I_SLICE && slice_type!=SI_SLICE) |  |  |
| { |  |  |
| complexity_scalable_flag | 2 | u(1) |
| } |  |  |
| /* end of complexity scalability parameters */ |  |  |
| if( !frame_mbs_only_flag ) { |  |  |
| field_pic_flag | 2 | u(1) |
| if( field_pic_flag ) |  |  |
| bottom_field_flag | 2 | u(1) |
| } |  |  |
| if( nal_unit_type = = 5 ) |  |  |
| idr_pic_id | 2 | ue(v) |
| if( pic_order_cnt_type = = 0 ) { |  |  |
| pic_order_cnt_lsb | 2 | u(v) |
| if( pic_order_present_flag && !field_pic_flag ) |  |  |
| delta_pic_order_cnt_bottom | 2 | se(v) |
| } |  |  |
| if( pic_order_cnt_type = = 1 && !delta_pic_order_always_zero_flag ) { |  |  |
| delta_pic_order_cnt[ 0 ] | 2 | se(v) |
| if( pic_order_present_flag && !field_pic_flag ) |  |  |
| delta_pic_order_cnt[ 1 ] | 2 | se(v) |
| } |  |  |
| if( redundant_pic_cnt_present_flag ) |  |  |
| redundant_pic_cnt | 2 | ue(v) |
| if( slice_type = = B ) |  |  |
| direct_spatial_mv_pred_flag | 2 | u(1) |
| if( slice_type = = P ¦ ¦ slice_type = = SP ¦ ¦ slice_type = = B ) { |  |  |
| num_ref_idx_active_override_flag | 2 | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |  |
| num_ref_idx_l0_active_minus1 | 2 | ue(v) |
| if( slice_type = = B ) |  |  |
| num_ref_idx_l1_active_minus1 | 2 | ue(v) |
| } |  |  |
| } |  |  |
| ref_pic_list_reordering( ) | 2 |  |
| if( ( weighted_pred_flag && ( slice_type = = P ¦ ¦ slice_type = = SP ) ) ¦ ¦ ( weighted_bipred_idc = = 1 && slice_type = = B ) ) |  |  |
| pred_weight_table( ) | 2 |  |
| if( nal_ref_idc != 0 ) |  |  |
| dec_ref_pic_marking( ) | 2 |  |
| if( entropy_coding_mode_flag && slice_type != I && slice_type != SI ) |  |  |
| cabac_init_idc | 2 | ue(v) |
| slice_qp_delta | 2 | se(v) |
| if( slice_type = = SP ¦ ¦ slice_type = = SI ) { |  |  |
| if( slice_type = = SP ) |  |  |
| sp_for_switch_flag | 2 | u(1) |
| slice_qs_delta | 2 | se(v) |
| } |  |  |
| if( deblocking_filter_control_present_flag ) { |  |  |
| disable_deblocking_filter_idc | 2 | ue(v) |
| if( disable_deblocking_filter_idc != 1 ) { |  |  |
| slice_alpha_c0_offset_div2 | 2 | se(v) |
| slice_beta_offset_div2 | 2 | se(v) |
| } |  |  |
| } |  |  |
| if( num_slice_groups_minus1 > 0 && slice_group_map_type >= 3 && slice_group_map_type <= 5) |  |  |
| slice_group_change_cycle | 2 | u(v) |
| } |  |  |

2.2 Slice header semantics

complexity_scalable_flag equal to 1 indicates the presence of the complexity scalability in the bitstream. complexity_scalable_flag equal to 0 indicates that the bitstream does not contain the  complexity scalable bitstream.

## Decoding process

This section describes the decoding process when profile_idc=92.

Comparing to traditional scalable video bitstreams, our complexity scalable video bitstreams only has enhancement layer for I-slices. It does not have enhancement layer for P- and B-slices.

3.1 Low resolution decoding

For low resolution decoding, the decoding process is compliant with AVC except that the motion vectors need to be downscaled before motion compensation. The decoding process is illustrated in Figure 1.

The inter prediction process formulae in MPEG4 AVC specification is modified as

MvL0[ mbPartIdx ][ subMbPartIdx ] = (mvL0+1)>>1    (8-1)

MvL1[ mbPartIdx ][ subMbPartIdx ] = (mvL1+1)>>1    (8-2)

3.2 Full resolution decoding

The decoding process for full resolution is illustrated in Figure 2

For I-slices, the spatial scalability method [4] is invoked for full resolution decoding. For P or B slices, complexity_scalable_flag is set to 1 and RRU based decoding method [2][5] is invoked for full resolution decoding. The syntax in JSVM1.0 for enhancement layer should be inferred as follows:

- base_mode_flag = 1.

- residue_prediction_flag = 1 and coded_block_pattern = 0 (no extra bits are coded for the residue at enhancement layer).

But the motion vectors are not upsampled as JSVM1.0.

The decoding processing is interpreted as following. For inter-pictures, after entropy decoding and inverse quantization and inverse transform, the residual is upsampled.  Motion compensation is applied to the full resolution reference pictures to form a full resolution prediction, and the upsampled residual is added to the prediction.  For the intra pictures, scalable video coding in [4] is conducted.

Figure 1 Low Resolution Decoder

Figure 2  Full Resolution Decoder

## Simulation

The experiment is conducted to compare our proposed non-scalable MV coding scheme with scalable MV coding scheme for the complexity scalable video coding. The same test condition as CE4 [3] in MPEG 71st Hong Kong meeting is used. That is, two layer spatial scalability is supported, CIF/QCIF and separately 4CIF/CIF. Random access is set at 1 sec interval. We set gop_size=2. For CIF/QCIF and 4CIF/CIF, we use the 30 Hz three lower bitrates of the Redmond conditions, shown in Table 2.

Table 2 CE6 test conditions

| CIF/QCIF Sequences | Bitrates |
| --- | --- |
| Bus | 256, 320, 384 kbps |
| Football | 512, 640, 768 kbps |
| Foreman | 128, 160, 192 kbps |
| Mobile | 192, 224, 256 kbps |
| City | 256, 320, 384 kbps |
| Crew/Harbor/Soccer | 384, 448, 512 kbps |

| 4CIF/CIF Sequences | Bitrates |
| --- | --- |
| City | 768, 896, 1024 kbps |
| Crew/Harbor/Soccer | 1024, 1280, 1536 kbps |

Our target is to evaluate non-scalable MV coding versus scalable MV coding for our complexity scalable video coding. So we adopt the same encoding methodology in both methods. In our non-scalable MV coding scheme, since the inter-picure only has one layer, the encoding is only performed and optimized for full-resolution picture (enhancement layer) at the 32x32 MB basis. We performed the same encoding method for scalable MV coding. That is, the motion estimation and mode decision is performed and optimized for full resolution picture at the 32x32 MB basis.  In scalable MV coding scheme, for the base layer, the resulted full-resolution MV needs to be downscaled by 2 and differentiated coded in the base layer bitstream. In enhancement layer, the difference between the full-resolution MV and upsampled base layer MV will be sent using quarter-pel MV refinement. No residue is sent. This can be implemented by using JSVM inter-layer motion prediction and inter-layer residue prediction. If any of the MVs in one MB for full resolution picture has quarter-pel MV, we use quarter pel refinement mode, i.e., set base_mode_flag=0, base_mode_refinement_flag=1. Otherwise, if none of MVs in one MB has quarter-pel MV, we set base_mode_flag=1. Since the enhancement layer uses only upsampled residue from base layer, we set code_block_pattern=0 and residue_prediction_flag=1. During the experiment, a trade-off between PSNR and bitrate is observed. If we perform quarter_pel (qpel) ME for full-resolution picture, we need to send MV quarter pel refinement bits in the enhancement layer. If we perform half_pel (hpel) ME for full-resolution picture, we do not need to send MV quarter pel refinement bits in the enhancement layer. So in total, qpel ME may require higher bitrate than hpel ME. But since qpel ME has higher precision, the PSNR of qpel ME is better than hpel ME. In order to better trade-off between PSNR and bitrate, the encoder will perform both qpel ME and hpel ME, and select the best result from them. The experiment shows for very low bitrate, hpel ME tends to have better performance than qpel ME. This is an encoder's choice, to trade-off between PSNR and bitrate.

Table 3 shows the resulted bitrate and PSNR with scalable MV coding, compared to our non-scalable MV coding, averaging 1.24dB. Table 4 shows the percentage of enhancement layer bits for P frame (enhP) and B frames (enhB) over total bits of P frames (totP) and B frames (totB), separately, assuming qpel ME is used . The plotted RD curves are shown in the attached excel file.

From Table 3 and the corresponding figures, we can see that there is a big quality loss for our proposed complexity scalable coding method using JSVM1 scalable MV coding. The worst case can be up to 4.7 dB. Table 4 shows a comparison of the coding efficiency penalty used for scalable MV coding for P and B frames, which can in some way help to explain why there is such a big loss in Table 3.  Table 4 shows that for low and mid bitrate, if qpel ME is used, a large percentage of bits have to be consumed to code the MV qpel refinement, at least 10% for enhP and 20% for enhB. This explains why at lower bitrate, for many cases, hpel ME can achieve better results. But with less MV precision, it resulted less PSNR comparing to qpel ME. Therefore, it explained why we have a big PSNR drop comparing to non-scalable MV coding method, where qpel ME is performed and no additional bits are sent in enhancement layer to refine the full-resolution MV. We also observed in general, the percentage of enhb is higher than enhP, with an average of 39% for B and 27% for P.  The percentage of enhP and enhB decreases when the bitrate increases

## Conclusion

From this core experiment, we find out that there is big coding efficiency loss for total bitrate if JSVM1 scalable MV coding is used for our proposed RRU based complexity scalable video coding scheme.  The same result can be inferred for other scalable coding methods when base layer bitrate requirement is relax and total bitrate coding efficiency is the major concern. Based on the results, we propose that

- Non-scalable MV coding scheme should be considered by JSVC. Though it is not strictly AVC compliant for the base layer of P and B slices, I slices are AVC compliant. This still achieves coarse scalability with lower frame rate. In addition, the only modification in base layer is MV downscaling, which can be implemented by simple 1 addition and 1 right shift. The additional complexity is quite trivial, comparing to additional MV coding in enhancement layer. .

- Our approach allows picture adaptive decision on non-scalable or scalable MV coding. So the optimal decision can be made based on application. For example, in general enhB% is higher than enhP%. We can use scalable MV coding for P frames but non-scalable MV coding for B frames.  An encoder can make the tradeoff of coding efficiency vs. backwards-compatible base layer frame rate by coding some P frames with scalable MV coding and some without.

Table 3 Comparison between JSVM1 scalable MV coding and our non-scalable MV coding for complexity scalable video coding scheme

| Sequence | JSVM1 (scalable MV coding) |  |  |  | THOMSON (non-scalable MV coding) |  |  |  | diff_psnrY (dB) (Thomon-JSVM1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | bitrate (kbps) | psnrY (dB) | psnrU (dB) | psnrV (dB) | bitrate (kbps) | psnrY (dB) | psnrU (dB) | psnrV |  |
| Bus (CIF) | 251.6608 | 25.7905 | 36.9927 | 38.1918 | 256.4144 | 27.0090 | 37.4943 | 38.7512 | 1.2185 |
|  | 319.0272 | 26.6251 | 37.3904 | 38.6639 | 325.5968 | 27.9637 | 38.0478 | 39.3376 | 1.3386 |
|  | 387.1376 | 27.3376 | 37.4280 | 38.6962 | 377.7616 | 28.5789 | 38.0890 | 39.5219 | 1.2413 |
| Football (CIF) | 516.8469 | 30.0143 | 35.7587 | 38.2903 | 512.8292 | 30.3673 | 36.3180 | 38.5834 | 0.3530 |
|  | 657.3868 | 30.9130 | 37.0242 | 39.3620 | 642.4006 | 31.1505 | 36.8247 | 39.1201 | 0.2375 |
|  | 759.6609 | 31.4374 | 37.1543 | 39.4777 | 773.8726 | 31.7330 | 37.4946 | 39.7004 | 0.2956 |
| Foreman (CIF) | 129.144 | 29.6511 | 37.2499 | 37.9076 | 129.2752 | 31.3023 | 38.1059 | 39.1491 | 1.6512 |
|  | 161.3848 | 30.5515 | 37.7467 | 38.5596 | 162.9664 | 32.2634 | 38.6437 | 39.7579 | 1.7120 |
|  | 189.7584 | 31.3227 | 38.1290 | 39.0431 | 190.2920 | 32.6417 | 38.8171 | 39.8588 | 1.3190 |
| Mobile (CIF) | 192.0808 | 21.1066 | 29.5748 | 28.9637 | 190.3272 | 25.8129 | 30.8980 | 30.1842 | 4.7063 |
|  | 214.2248 | 21.7161 | 29.5885 | 28.9988 | 217.1344 | 26.2952 | 31.4362 | 30.7689 | 4.5791 |
|  | 263.6592 | 22.7287 | 30.2740 | 29.7172 | 261.2104 | 26.9628 | 31.8820 | 31.2204 | 4.2341 |
| City (CIF) | 259.6992 | 31.0780 | 40.8565 | 42.2153 | 263.4712 | 33.5143 | 42.3580 | 43.8553 | 2.4363 |
|  | 323.1728 | 31.8453 | 41.3273 | 42.7103 | 320.5560 | 34.1309 | 42.8768 | 44.3329 | 2.2857 |
|  | 380.3664 | 32.5975 | 41.5565 | 43.0150 | 387.8128 | 34.7015 | 43.4823 | 44.9260 | 2.1040 |
| Crew (CIF) | 380.9008 | 33.2220 | 37.7375 | 36.3656 | 387.6120 | 33.6934 | 38.2439 | 36.8161 | 0.4714 |
|  | 460.4376 | 33.7300 | 38.1949 | 36.8437 | 438.8768 | 34.1732 | 38.3017 | 36.8591 | 0.4432 |
|  | 513.6512 | 34.1577 | 38.2653 | 36.9197 | 500.8008 | 34.6185 | 38.7432 | 37.3482 | 0.4608 |
| Harbor (CIF) | 387.4704 | 27.9108 | 39.0766 | 40.8890 | 377.3432 | 28.8250 | 40.0275 | 41.7781 | 0.9142 |
|  | 444.5112 | 28.4085 | 39.3766 | 41.2493 | 455.1056 | 29.4251 | 40.3030 | 41.9545 | 1.0167 |
|  | 502.2768 | 28.8471 | 39.4750 | 41.3484 | 509.0080 | 29.7711 | 40.6823 | 42.3558 | 0.9240 |
| Soccer (CIF) | 383.2880 | 32.5870 | 40.4749 | 42.1804 | 387.5496 | 33.1139 | 40.6007 | 42.4039 | 0.5269 |
|  | 441.0208 | 33.0504 | 40.5670 | 42.2818 | 447.5208 | 33.6144 | 40.9673 | 42.6321 | 0.5640 |
|  | 505.7992 | 33.4695 | 40.9713 | 42.6685 | 505.4400 | 34.0317 | 41.2739 | 42.8995 | 0.5622 |
| City (4CIF) | 790.9208 | 31.0549 | 41.2102 | 43.2704 | 771.6840 | 33.0188 | 41.7180 | 43.8686 | 1.9639 |
|  | 899.7344 | 31.8258 | 40.6325 | 42.6232 | 904.4968 | 33.3304 | 41.9260 | 44.0710 | 1.5046 |
|  | 1013.7704 | 32.2269 | 40.7294 | 42.7733 | 1016.2432 | 33.4250 | 41.9803 | 44.1232 | 1.1982 |
| Crew (4CIF) | 1050.332 | 34.4209 | 39.0249 | 38.7103 | 1039.1344 | 34.7006 | 39.4790 | 39.2352 | 0.2797 |
|  | 1321.9593 | 35.0943 | 39.4517 | 39.2421 | 1238.0072 | 35.2384 | 39.6120 | 39.4071 | 0.1441 |
|  | 1572.7841 | 35.5726 | 39.8617 | 39.7833 | 1532.7160 | 35.8146 | 40.1562 | 40.1726 | 0.2420 |
| Harbor (4CIF) | 1040.3776 | 29.3334 | 39.7561 | 41.5226 | 1000.1824 | 29.8949 | 40.1550 | 41.9359 | 0.5615 |
|  | 1240.7081 | 29.9397 | 40.1282 | 41.9763 | 1284.0648 | 30.7707 | 40.6181 | 42.5969 | 0.8310 |
|  | 1575.3377 | 30.7677 | 40.6557 | 42.5743 | 1535.4144 | 31.3667 | 40.8708 | 42.8199 | 0.5990 |
| Soccer (4CIF) | 1071.9321 | 33.3310 | 41.2151 | 42.8163 | 1019.4480 | 33.7306 | 41.3830 | 43.2120 | 0.3996 |
|  | 1244.1329 | 33.8019 | 41.6841 | 43.3628 | 1290.6136 | 34.4969 | 41.9204 | 43.6985 | 0.6950 |
|  | 1554.9569 | 34.4776 | 42.0699 | 43.7854 | 1516.3384 | 34.9635 | 42.1748 | 43.9634 | 0.4859 |
| Average |  |  |  |  |  |  |  |  | 1.2361 |

Table 4 Percentage of enhancement layer bits using qpel ME

| - sequence | - bitrate(kbps) | - enhP/totP (%) | - enhB/totB(%) |
| --- | --- | --- | --- |
| Bus (CIF) | 251.6608 | - 28.85 | - 44.00 |
|  | 319.0272 | - 26.03 | - 41.53 |
|  | 387.1376 | - 23.31 | - 39.05 |
| Football (CIF) | 516.8469 | - 12.33 | - 24.35 |
|  | 657.3868 | - 10.81 | - 21.87 |
|  | 759.6609 | - 9.87 | - 19.61 |
| Foreman (CIF) | 129.144 | - 37.39 | - 59.23 |
|  | 161.3848 | - 34.04 | - 56.71 |
|  | 189.7584 | - 32.56 | - 51.87 |
| Mobile (CIF) | 192.0808 | - 57.82 | - 31.46 |
|  | 214.2248 | - 55.15 | - 33.97 |
|  | 263.6592 | - 51.81 | - 34.55 |
| City (CIF) | 259.6992 | - 41.70 | - 57.77 |
|  | 323.1728 | - 38.26 | - 55.38 |
|  | 380.3664 | - 36.11 | - 53.00 |
| Crew (CIF) | 380.9008 | - 15.31 | - 31.58 |
|  | 460.4376 | - 14.43 | - 29.31 |
|  | 513.6512 | - 13.60 | - 27.74 |
| Harbor (CIF) | 387.4704 | - 28.62 | - 46.02 |
|  | 444.5112 | - 26.11 | - 44.12 |
|  | 502.2768 | - 24.02 | - 42.12 |
| Soccer (CIF) | 383.2880 | - 17.69 | - 29.44 |
|  | 441.0208 | - 17.00 | - 28.17 |
|  | 505.7992 | - 16.33 | - 26.19 |
| City (4CIF) | 790.9208 | - 43.87 | - 57.42 |
|  | 899.7344 | - 41.78 | - 56.32 |
|  | 1013.7704 | - 39.43 | - 54.29 |
| Crew (4CIF) | 1050.332 | - 16.93 | - 31.17 |
|  | 1321.9593 | - 14.50 | - 28.42 |
|  | 1572.7841 | - 13.82 | - 25.30 |
| Harbor (4CIF) | 1040.3776 | - 26.24 | - 47.82 |
|  | 1240.7081 | - 24.31 | - 43.59 |
|  | 1575.3377 | - 20.34 | - 39.59 |
| Soccer (4CIF) | 1071.9321 | - 19.85 | - 32.75 |
|  | 1244.1329 | - 18.63 | - 30.16 |
|  | 1554.9569 | - 17.10 | - 27.51 |
| - Average |  | - 26.83 | - 38.98 |

## 6. References

- “Description of Core Experiment in SVC,” ISO/IEC JTC1/SC29/WG11 doc. no. N6898, Hongkong, January 2005.

- P. Yin, J. Boyce, P. Pandit, A.M. Tourapis, “Complexity Scalable Video Codec”, ISO/IEC JTC1/SC29/WG11 doc. no. m11241, Palma, October 2004.

- P. Yin, J. Boyce, P. Pandit, A.M. Tourapis, “Technical Description of the Thomson Proposal for SVC CE4”, ISO/IEC JTC1/SC29/WG11 doc. no. m11682, Hongkong, January 2005.

- J. Reichel, M. Wien and H. Schwarz, “Joint Scalable Video Model JSVM1”, JVT-N023, Hongkong, China, January 2005.

- P. Yin, J. Boyce, P. Purvin, A.M. Tourapis, “Complexity Scalable Video Coding”, ITU-T Q6/SG16 VCEG-X06.doc, Palma, October 2004

## (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Thomson Inc. |  |  |
| Mailing address | 2 Independence Way, Princeton, NJ, 08540 |  |  |
| Country | USA |  |  |
| Contact person | Peng Yin |  |  |
| Telephone | +1 609 987 7355 |  |  |
| Fax | +1 609 987 7299 |  |  |
| Email | peng.yin@thomson.net |  |  |
| Place and date of submission | Busan, Korean April 16, 2005 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT-O052 |  |
| Title |  | Technical description of the Thomson proposal for SVC CE6 - non-scalable motion vector coding |  |
| Contribution number |  | JVT-O052 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
| x | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: