| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 13th Meeting: Busan, Korea 18-22 April, 2005 | Document: JVT-O53 Filename: JVT-O53.doc |
| --- | --- |

| Title: | Technical description of the Thomson proposal for SVC CE7 - spatial intra prediction on enhancement layer residue |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Peng Yin, Jill Boyce, Purvin Pandit 2 Independence Way Princeton, NJ, 08540 | Tel: Email: | 1-609.987.7355 peng.yin@thomson.net Jill.boyce@thomson.net |
| Source: | Thomson Inc. |  |  |

_____________________________

## Introduction

This document presents the technical description and experimental result in response to CE7 [1], which aims to evaluate the intra coding efficiency by enabling spatial intra prediction of the enhancement layer residue for spatial scalability. The description is based on the Thomson's proposal in VCEG-X06 [5], m11241 [2] and m11682 [3].

In JSVM1.0 [4],  intra texture prediction using information from the previous layer is provided in the new I_BL macroblock mode, where the enhancement layer residue (the difference between current MB and upsampled base layer) is transformed and quantized. In this document, we propose to improve I_BL macroblock mode by enabling spatial intra prediction of the enhancement layer residue using the same method specified in subclause 8.3 of AVC.

The rationale behind this method is that for spatial scalability, the enhancement layer residue in general contains a lot of high frequency components, such as edges. Spatial intra prediction should help to keep more details. The drawback of the proposed method is that it requires coding more syntax bits, like mb_type, intra prediction modes or cbp pattern if intra16x16 is selected. The experimental results show that the proposed method can have up to 0.8 dB gain for very high bitrate as compared to JSVM1 I_BL method. For lower bitrate, the proposed method can have up to 0.3 dB loss. A slice header syntax is defined to enable the adaptive selection of the proposed method based on bitrate requirement and application. For example, in digital cinema, higher bitrate tends to be used. By enabling the proposed method, better performance can be achieved.

## Syntax in tabular form

In this section, only the syntax that is added or extended in comparison to AVC or JSVM 1.0 [4] for our proposal is described, which is marked in red.

2.1 syntax

2.1.1 slice header in scalable extension syntax

| slice_header_in_scalable_extension( ) { | C | Descriptor |
| --- | --- | --- |
| first_mb_in_slice | 2 | ue(v) |
| slice_type | 2 | ue(v) |
| pic_parameter_set_id | 2 | ue(v) |
| frame_num | 2 | u(v) |
| if( !frame_mbs_only_flag ) { |  |  |
| field_pic_flag | 2 | u(1) |
| if( field_pic_flag ) |  |  |
| bottom_field_flag | 2 | u(1) |
| } |  |  |
| if( nal_unit_type = = 21 ) |  |  |
| idr_pic_id | 2 | ue(v) |
| if( pic_order_cnt_type = = 0 ) { |  |  |
| pic_order_cnt_lsb | 2 | u(v) |
| if( pic_order_present_flag && !field_pic_flag ) |  |  |
| delta_pic_order_cnt_bottom | 2 | se(v) |
| } |  |  |
| if( pic_order_cnt_type = = 1 && !delta_pic_order_always_zero_flag ) { |  |  |
| delta_pic_order_cnt[ 0 ] | 2 | se(v) |
| if( pic_order_present_flag && !field_pic_flag ) |  |  |
| delta_pic_order_cnt[ 1 ] | 2 | se(v) |
| } |  |  |
| if( slice_type != Progressive_Refinement ) { |  |  |
| if( redundant_pic_cnt_present_flag ) |  |  |
| redundant_pic_cnt | 2 | ue(v) |
| if( slice_type = = BE ) |  |  |
| direct_spatial_mv_pred_flag | 2 | u(1) |
| gop_id | 2 | u(4) |
| gop_size | 2 | ue(v) |
| decomposition_stages | 2 | ue(v) |
| picture_id_inside_gop | 2 | ue(v) |
| base_id_plus1 | 2 | ue(v) |
| if( base_id_plus1 != 0 ) { |  |  |
| adaptive_prediction_flag | 2 | u(1) |
| if (HalfSpatResBaseFlag) |  |  |
| intra_base_residue_prediction_flag | 2 | u(1) |
| } |  |  |
| if( slice_type = = PE ¦ ¦ slice_type = = BE ) { |  |  |
| num_ref_idx_active_override_flag | 2 | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |  |
| num_ref_idx_l0_active_minus1 | 2 | ue(v) |
| if( slice_type = = B ) |  |  |
| num_ref_idx_l1_active_minus1 | 2 | ue(v) |
| } |  |  |
| } |  |  |
| ref_pic_list_reordering( ) | 2 |  |
| for( decLvl = TemporalLevel; decLvl < decomposition_stages; decLvl++ ) { |  |  |
| num_ref_idx_update_l0_active[ decLvl + 1 ] | 2 | ue(v) |
| num_ref_idx_update_l1_active[ decLvl + 1 ] | 2 | ue(v) |
| } |  |  |
| if( ( weighted_pred_flag && slice_type = = PE ) ¦ ¦ ( weighted_bipred_idc = = 1 && slice_type = = BE ) ) |  |  |
| pred_weight_table( ) | 2 |  |
| if( nal_ref_idc != 0 ) |  |  |
| dec_ref_pic_marking( ) | 2 |  |
| if( entropy_coding_mode_flag && slice_type != IE ) |  |  |
| cabac_init_idc | 2 | ue(v) |
| } |  |  |
| slice_qp_delta | 2 | se(v) |
| if( deblocking_filter_control_present_flag ) { |  |  |
| disable_deblocking_filter_idc | 2 | ue(v) |
| if( disable_deblocking_filter_idc != 1 ) { |  |  |
| slice_alpha_c0_offset_div2 | 2 | se(v) |
| slice_beta_offset_div2 | 2 | se(v) |
| } |  |  |
| } |  |  |
| if( slice_type != Progressive_Refinement ) |  |  |
| if( num_slice_groups_minus1 > 0 && slice_group_map_type >= 3 && slice_group_map_type <= 5) |  |  |
| slice_group_change_cycle | 2 | u(v) |
| } |  |  |

2.1.2 Macroblock layer in scalable extension syntax

| macroblock_layer_in_scalable_extension( ) { | C | Descriptor |
| --- | --- | --- |
| if( base_id_plus1 != 0 && adaptive_prediction_flag ) { |  |  |
| base_mode_flag | 2 | ae(v) |
| if( ! base_mode_flag && HalfSpatResBaseFlag && ! intra_base_mb( CurrMbAddr ) ) |  |  |
| base_mode_refinement_flag | 2 | ae(v) |
| } |  |  |
| if( ! base_mode_flag && ! base_mode_refinement_flag ) { |  |  |
| mb_type | 2 | ae(v) |
| if( (intra_base_residue_prediction_flag ¦¦ mb_type = = I_NxN) && base_id_plus1 != 0 ) |  |  |
| intra_base_flag | 2 | ae(v) |
| } |  |  |
| if( mb_type = = I_PCM ) { |  |  |
| while( !byte_aligned( ) ) |  |  |
| pcm_alignment_zero_bit | 2 | f(1) |
| for( i = 0; i < 256; i++ ) |  |  |
| pcm_sample_luma[ i ] | 2 | u(v) |
| for( i = 0; i < 2 * MbWidthC * MbHeightC; i++ ) |  |  |
| pcm_sample_chroma[ i ] | 2 | u(v) |
| } else { |  |  |
| NoSubMbPartSizeLessThan8x8Flag = 1 |  |  |
| if( mb_type != I_NxN && MbPartPredMode( mb_type, 0 ) != Intra_16x16 && NumMbPart( mb_type ) = = 4 ) { |  |  |
| sub_mb_pred_in_scalable_extension( mb_type ) | 2 |  |
| for( mbPartIdx = 0; mbPartIdx < 4; mbPartIdx++ ) |  |  |
| if( sub_mb_type[ mbPartIdx ] != B_Direct_8x8 ) { |  |  |
| if( NumSubMbPart( sub_mb_type [ mbPartIdx ] ) > 1 ) |  |  |
| NoSubMbPartSizeLessThan8x8Flag = 0 |  |  |
| } else if( !direct_8x8_inference_flag ) |  |  |
| NoSubMbPartSizeLessThan8x8Flag = 0 |  |  |
| } else { |  |  |
| if( transform_8x8_mode_flag && mb_type = = I_NxN ) |  |  |
| transform_size_8x8_flag | 2 | ae(v) |
| mb_pred_in_scalable_extension( mb_type ) | 2 |  |
| } |  |  |
| if( MbPartPredMode( mb_type, 0 ) != Intra_16x16 ) { |  |  |
| coded_block_pattern | 2 | ae(v) |
| if( CodedBlockPatternLuma > 0 && transform_8x8_mode_flag && mb_type != I_NxN && NoSubMbPartSizeLessThan8x8Flag && !( MbPartPredMode( mb_type, 0 ) = = B_Direct_16x16 && !direct_8x8_inference_flag ) ) |  |  |
| transform_size_8x8_flag | 2 | ae(v) |
| } |  |  |
| if( CodedBlockPatternLuma > 0 ¦ ¦ CodedBlockPatternChroma > 0 ¦ ¦ MbPartPredMode( mb_type, 0 ) = = Intra_16x16 ) { |  |  |
| mb_qp_delta | 2 | ae(v) |
| residual_in_scalable_extension( ) | 3 ¦ 4 |  |
| } |  |  |
| } |  |  |
| } |  |  |

2.2 Semantics

A flag is added to the enhancement layer slice header to indicate that spatial intra prediction is performed on the residue.

intra_base_residue_prediction_flag equal to 1 specifies that the in I_BL mode macroblocks, spatial intra prediction is used on the enhancement layer residual. By default, intra_base_residue_prediction_flag is set to 0.

## Decoding process

For enhancement layer, if intra_base_residue_prediction_flag is set to 1, then for I_BL macroblock, the current macroblock and neighboring macroblock residuals from the (upsampled) reconstructed base layer are adjusted by adding 128 and clipping to (0, 255), and then used for spatial intra prediction specified in subclause 8.3 of AVC.   A subtraction of 128 and clipping to (-256, 255) is then performed.  The inter-layer intra predicted residue is then added on the (upsampled) reconstructed base layer to form the reconstructed enhancement layer macroblock. If  intra_base_residue_prediction_flag is set to 0, I_BL macroblock is decoded as in [4].

When constrained_intra_prediction_flag is set to 1, it specifies constrained intra prediction, in which case prediction of macroblocks coded using Intra macroblock prediction modes only uses residual data and decoded samples from I or SI macroblock types.

The complexity of the proposed I_BL mode is the same as the spatial intra prediction in subclause 7.4.2.2, plus an additional additition, subtraction and clipping per pixel. The complexity is a little bit higher than JSVM1 I_BL mode. We propose that when the spatial scalability layer is more than 2, the proposed method will be only enabled for the highest layer.

## Simulation

The experiment is conducted to compare our proposed method with JSVM1.0. All the sequences are coded with only I-pictures at fixed QP (8, 12, 16, 20, 24, 28, 32, 36). For 3-layer coding, our proposed method is only enabled for the highest layer. The CIF/QCIF sequences we tested are Bus, Football, Foreman and Mobile, at 15fps. The 4CIF/CIF sequences we tested are City, Crew, Harbour and Soccer, at 30fps. In our experiment, intra_base_residue_prediction_flag is always set to 1.  Since the proposed method fits better for high bitrate application, we also add 2 HD clips to show the improvement at higher bitrate. The 1280x704/640x352 sequences are Harbour and Night.

Table 1 shows the bitrate and PSNR between our proposed method and JSVM1.0. The plotted RD curve is shown in Figure 1.

From Table 1 and Figure 1, we can observe that in general, for fixed QP, our proposed method has higher bitrate and PSNR than JSVM1.  For the same bitrate, according to Figure 1, up to 0.8 dB gain can be achieve at higher bitrate. But there is a little loss at lower bitrate, up to 0.3 dB. Table 2 shows ∆PSNR and ∆bitrate of QP 8,12,26,20 and 24,28,32, 36, separately. For higher bitrate, the average ∆PSNR gain is 0.32dB. For lower bitrate, the average ∆PSNR loss is 0.11dB. This is because spatial intra prediction needs more bits to code syntax, such as mb_type and intra prediction direction. When QP is high, less details on enhancement layer residue can be maintained after quantization, so the gain in PSNR does not validate the increase in the bitrate. For higher bitrate, the QP is lower, we can keep more details, so the gain in PSNR validate the increase in the bitrate. We propose that we add one more syntax element (intra_base_residue_prediction_flag) in the slice header, so spatial intra prediction on the enhancement layer residue can be enabled and disabled on a slice-by-slice basis. In this way, we can get better gain at high bitrates, while keep the same performance as JSVM1 at lower bitrate.

## 5. Conclusion

In this core experiment, we propose to add spatial intra prediction for enhancement layer residue. By adding intra_base_residue_prediction_flag in slice header, a slice adaptive decision can be made to enable or disable this flag. In this way, we can achieve better performance at higher bitrates, while keep the same/better performance at lower bitrates, compared to current JSVM1.

Table 1 Bitrate and PSNR comparison for fixed QP

| Sequence: BUS CIF 15Hz |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QP | JSVM 1 |  |  |  | Thomson-Proposal |  |  |  |
|  | Y-PSNR [dB] | U-PSNR [dB] | V-PSNR [dB] | rate [kbit/s] | Y-PSNR [dB] | U-PSNR [dB] | V-PSNR [dB] | rate [kbit/s] |
| 8 | 53.35471 | 53.28714 | 53.76466 | 10066.6912 | 53.91603 | 53.29041 | 53.70068 | 10038.1456 |
| 12 | 49.68526 | 50.43383 | 51.21938 | 7967.5392 | 50.16982 | 50.44195 | 51.18605 | 7993.0416 |
| 16 | 46.08129 | 47.72876 | 48.76156 | 6238.8112 | 46.45401 | 47.73545 | 48.78717 | 6270.1424 |
| 20 | 42.39906 | 45.26112 | 46.74742 | 4682.9664 | 42.67671 | 45.17272 | 46.6889 | 4712.6368 |
| 24 | 38.93044 | 42.74472 | 44.54806 | 3401.9200 | 39.16726 | 42.71693 | 44.48636 | 3429.3264 |
| 28 | 35.75944 | 40.69191 | 42.55931 | 2409.1136 | 35.84736 | 40.64978 | 42.4717 | 2432.3040 |
| 32 | 32.59281 | 39.3588 | 41.01185 | 1598.6272 | 32.63757 | 39.27065 | 40.89966 | 1620.8064 |
| 36 | 29.71436 | 38.00347 | 39.26132 | 1005.2704 | 29.78092 | 37.88831 | 39.19135 | 1026.1952 |

| Sequence: FOREMAN CIF 15Hz |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QP | JSVM 1 |  |  |  | Thomson-Proposal |  |  |  |
|  | Y-PSNR [dB] | U-PSNR [dB] | V-PSNR [dB] | rate [kbit/s] | Y-PSNR [dB] | U-PSNR [dB] | V-PSNR [dB] | rate [kbit/s] |
| 8 | 53.25519 | 53.11349 | 53.29791 | 8072.4000 | 54.01362 | 53.17517 | 53.33568 | 8119.0512 |
| 12 | 49.74081 | 50.25493 | 50.81068 | 6053.3640 | 50.29172 | 50.30259 | 50.83722 | 6120.2280 |
| 16 | 46.37988 | 47.55743 | 48.55738 | 4448.8600 | 46.72654 | 47.62101 | 48.6438 | 4495.4592 |
| 20 | 43.01161 | 45.17851 | 46.79441 | 3087.8072 | 43.21269 | 45.13583 | 46.76322 | 3126.0680 |
| 24 | 39.96416 | 42.74906 | 44.66224 | 2063.7264 | 40.15129 | 42.73105 | 44.65886 | 2096.6712 |
| 28 | 37.27932 | 40.80869 | 42.62961 | 1349.6472 | 37.33034 | 40.7715 | 42.56499 | 1373.7104 |
| 32 | 34.54451 | 39.61067 | 41.13949 | 840.0272 | 34.61726 | 39.53395 | 41.00981 | 864.6448 |
| 36 | 32.04231 | 38.37949 | 39.46902 | 508.9176 | 32.15209 | 38.19678 | 39.18479 | 528.4088 |

| Sequence: FOOTBALL CIF 15Hz |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QP | JSVM 1 |  |  |  | Thomson-Proposal |  |  |  |
|  | Y-PSNR [dB] | U-PSNR [dB] | V-PSNR [dB] | rate [kbit/s] | Y-PSNR [dB] | U-PSNR [dB] | V-PSNR [dB] | rate [kbit/s] |
| 8 | 53.10842 | 53.30441 | 53.4492 | 8130.2391 | 53.96274 | 53.35002 | 53.46494 | 8166.8178 |
| 12 | 49.71566 | 50.61236 | 50.85746 | 6166.6449 | 50.33238 | 50.65875 | 50.89125 | 6216.2151 |
| 16 | 46.48554 | 48.07503 | 48.39642 | 4602.1108 | 46.90454 | 48.14036 | 48.466 | 4637.4600 |
| 20 | 43.1649 | 45.91945 | 46.5054 | 3289.8665 | 43.41964 | 45.90134 | 46.46602 | 3324.5898 |
| 24 | 40.00694 | 43.48303 | 44.30616 | 2285.7102 | 40.23489 | 43.43175 | 44.27502 | 2323.1326 |
| 28 | 37.13959 | 41.10608 | 42.41041 | 1545.2215 | 37.09175 | 41.03736 | 42.32952 | 1578.9369 |
| 32 | 34.28897 | 39.17635 | 40.94395 | 970.2277 | 34.26434 | 39.04288 | 40.84135 | 1001.894 |
| 36 | 31.75822 | 37.17083 | 39.12606 | 580.3726 | 31.84296 | 37.06421 | 39.0807 | 606.2908 |

| Sequence: MOBILE CIF 15Hz |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QP | JSVM 1 |  |  |  | Thomson-Proposal |  |  |  |
|  | Y-PSNR [dB] | U-PSNR [dB] | V-PSNR [dB] | rate [kbit/s] | Y-PSNR [dB] | U-PSNR [dB] | V-PSNR [dB] | rate [kbit/s] |
| 8 | 53.30091 | 52.94687 | 52.98981 | 13893.6248 | 53.97833 | 53.01082 | 53.04926 | 13941.3256 |
| 12 | 49.68657 | 49.60001 | 49.64548 | 11453.6616 | 50.25458 | 49.64629 | 49.69141 | 11547.0104 |
| 16 | 46.13588 | 46.27663 | 46.32692 | 9371.3920 | 46.62284 | 46.30771 | 46.35735 | 9474.4400 |
| 20 | 42.30808 | 42.90224 | 42.93386 | 7414.4384 | 42.76048 | 42.89696 | 42.93401 | 7519.2128 |
| 24 | 38.59406 | 39.71864 | 39.68749 | 5687.8320 | 39.00163 | 39.7181 | 39.69831 | 5791.2512 |
| 28 | 35.11256 | 36.87402 | 36.77538 | 4225.5856 | 35.43151 | 36.85247 | 36.7613 | 4320.6608 |
| 32 | 31.49986 | 34.818 | 34.62588 | 2974.8728 | 31.77882 | 34.79174 | 34.61991 | 3065.4088 |
| 36 | 28.18949 | 32.71748 | 32.47 | 1961.7672 | 28.42283 | 32.67748 | 32.44431 | 2046.5208 |

| Sequence: CITY 4CIF 30Hz |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QP | JSVM 1 |  |  |  | Thomson-Proposal |  |  |  |
|  | Y-PSNR [dB] | U-PSNR [dB] | V-PSNR [dB] | rate [kbit/s] | Y-PSNR [dB] | U-PSNR [dB] | V-PSNR [dB] | rate [kbit/s] |
| 8 | 53.07649 | 53.90677 | 54.52759 | 62526.1264 | 53.90447 | 53.87744 | 54.42891 | 63067.4416 |
| 12 | 49.60854 | 51.28742 | 52.02018 | 47726.3064 | 50.20368 | 51.31604 | 52.02718 | 48466.6480 |
| 16 | 46.2459 | 48.79091 | 49.6978 | 35704.1720 | 46.63133 | 48.8478 | 49.8022 | 36227.7640 |
| 20 | 42.7534 | 46.77975 | 48.12115 | 25232.3944 | 42.96823 | 46.7059 | 48.07576 | 25637.4264 |
| 24 | 39.52388 | 44.59908 | 46.20701 | 17262.7464 | 39.71167 | 44.60205 | 46.21311 | 17586.7368 |
| 28 | 36.65444 | 42.91618 | 44.80124 | 11519.7536 | 36.63829 | 42.8561 | 44.71931 | 11797.6392 |
| 32 | 33.75792 | 41.7021 | 43.7489 | 9093.0216 | 33.72396 | 41.54038 | 43.53219 | 9377.8384 |
| 36 | 31.11763 | 40.37209 | 42.3689 | 4286.1952 | 31.12345 | 40.15941 | 42.04296 | 4486.5816 |

| Sequence: CREW 4CIF 30Hz |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QP | JSVM 1 |  |  |  | Thomson-Proposal |  |  |  |
|  | Y-PSNR [dB] | U-PSNR [dB] | V-PSNR [dB] | rate [kbit/s] | Y-PSNR [dB] | U-PSNR [dB] | V-PSNR [dB] | rate [kbit/s] |
| 8 | 53.02437 | 53.21718 | 53.5437 | 47942.2840 | 54.08701 | 53.34431 | 53.64965 | 50333.4784 |
| 12 | 49.85232 | 50.43235 | 50.9492 | 33838.7304 | 50.47228 | 50.52624 | 51.0295 | 35627.6288 |
| 16 | 46.72493 | 47.58224 | 48.47013 | 23011.4184 | 46.99979 | 47.63602 | 48.5555 | 24137.4616 |
| 20 | 43.59793 | 45.18825 | 46.534 | 14388.5896 | 43.5417 | 45.12493 | 46.54206 | 15081.6208 |
| 24 | 40.829 | 43.11443 | 44.28188 | 8614.9944 | 40.88856 | 43.14101 | 44.32152 | 8986.4896 |
| 28 | 38.67077 | 41.68555 | 42.24687 | 5224.9192 | 38.55874 | 41.68996 | 42.22386 | 5440.8560 |
| 32 | 36.5261 | 40.6038 | 40.66828 | 3169.0824 | 36.50795 | 40.54395 | 40.55843 | 3333.596 |
| 36 | 34.42862 | 39.28846 | 38.93298 | 1932.3288 | 34.57298 | 39.18862 | 38.73926 | 2052.8128 |

| Sequence: HARBOUR 4CIF 30Hz |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QP | JSVM 1 |  |  |  | Thomson-Proposal |  |  |  |
|  | Y-PSNR [dB] | U-PSNR [dB] | V-PSNR [dB] | rate [kbit/s] | Y-PSNR [dB] | U-PSNR [dB] | V-PSNR [dB] | rate [kbit/s] |
| 8 | 52.86714 | 53.49865 | 54.18245 | 62929.4208 | 53.72038 | 53.57662 | 54.20245 | 62597.0808 |
| 12 | 49.49748 | 50.88148 | 51.74082 | 48193.9376 | 50.0507 | 50.94788 | 51.78572 | 47736.9536 |
| 16 | 46.15161 | 48.35776 | 49.40812 | 35829.9616 | 46.50369 | 48.3998 | 49.51231 | 35223.2720 |
| 20 | 42.70691 | 46.24565 | 47.78789 | 25130.2152 | 42.93922 | 46.14296 | 47.70904 | 24640.4032 |
| 24 | 39.58435 | 44.01585 | 45.87143 | 17158.1544 | 39.78113 | 44.02084 | 45.86984 | 16929.1304 |
| 28 | 36.82467 | 42.38095 | 44.3112 | 11573.8096 | 36.82986 | 42.31869 | 44.2353 | 11564.9880 |
| 32 | 33.93517 | 41.22968 | 43.08085 | 7483.4464 | 33.90987 | 41.05588 | 42.85543 | 7647.5824 |
| 36 | 31.22229 | 39.86657 | 41.42841 | 4808.2912 | 31.20926 | 39.6354 | 41.24245 | 4978.8984 |

| Sequence: SOCCER 4CIF 30Hz |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QP | JSVM 1 |  |  |  | Thomson-Proposal |  |  |  |
|  | Y-PSNR [dB] | U-PSNR [dB] | V-PSNR [dB] | rate [kbit/s] | Y-PSNR [dB] | U-PSNR [dB] | V-PSNR [dB] | rate [kbit/s] |
| 8 | 53.16096 | 54.15545 | 54.95288 | 48924.8744 | 53.99509 | 54.16607 | 54.9536 | 49783.5816 |
| 12 | 49.94324 | 51.45877 | 52.33733 | 35441.1256 | 50.48955 | 51.51181 | 52.38217 | 36292.6592 |
| 16 | 46.82397 | 48.90526 | 49.98187 | 25187.9168 | 47.12204 | 48.98979 | 50.10581 | 25767.6376 |
| 20 | 43.60018 | 47.05795 | 48.60557 | 16954.0256 | 43.66654 | 47.01584 | 48.57641 | 17425.5600 |
| 24 | 40.55308 | 45.17251 | 46.9444 | 11143.0040 | 40.66652 | 45.16186 | 46.94307 | 11516.2392 |
| 28 | 37.86347 | 43.51317 | 45.32567 | 7177.6496 | 37.71756 | 43.42415 | 45.17507 | 7473.2216 |
| 32 | 35.21388 | 42.18728 | 43.87839 | 4179.9192 | 35.10728 | 42.00494 | 43.57256 | 4397.8792 |
| 36 | 32.85631 | 40.63765 | 42.57629 | 2290.9992 | 32.91122 | 40.40001 | 41.95668 | 2425.6408 |

| Sequence: HARBOUR HD 30Hz |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QP | JSVM 1 |  |  |  | Thomson-Proposal |  |  |  |
|  | Y-PSNR [dB] | U-PSNR [dB] | V-PSNR [dB] | rate [kbit/s] | Y-PSNR [dB] | U-PSNR [dB] | V-PSNR [dB] | rate [kbit/s] |
| 8 | 53.3348 | 54.2312 | 55.1589 | 120007.483 | 54.0167 | 54.1969 | 55.0374 | 120750.809 |
| 12 | 50.166 | 51.64 | 52.7205 | 91074.9184 | 50.5976 | 51.6571 | 52.7162 | 91248.9672 |
| 16 | 46.9851 | 48.9988 | 50.174 | 68286.1192 | 47.2355 | 49.118 | 50.3888 | 67988.8728 |
| 20 | 43.5426 | 46.9167 | 48.454 | 48468.9584 | 43.6864 | 46.8505 | 48.4174 | 48263.4376 |
| 24 | 40.3621 | 44.5262 | 46.3351 | 33491.6448 | 40.5055 | 44.5605 | 46.3643 | 33474.5624 |
| 28 | 37.5162 | 42.6777 | 44.5951 | 22793.1264 | 37.4572 | 42.6338 | 44.5474 | 22976.5376 |
| 32 | 34.5625 | 41.3984 | 43.2788 | 14858.6896 | 34.494 | 41.227 | 43.0902 | 15238.6464 |
| 36 | 31.7982 | 39.9706 | 41.6379 | 9598.0432 | 31.7648 | 39.7226 | 41.4077 | 9929.6248 |

| Sequence: NIGHT HD 30Hz |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QP | JSVM 1 |  |  |  | Thomson-Proposal |  |  |  |
|  | Y-PSNR [dB] | U-PSNR [dB] | V-PSNR [dB] | rate [kbit/s] | Y-PSNR [dB] | U-PSNR [dB] | V-PSNR [dB] | rate [kbit/s] |
| 8 | 53.5124 | 53.7059 | 54.3248 | 121802.431 | 54.319 | 53.7704 | 54.3478 | 124482.106 |
| 12 | 50.31 | 50.9486 | 51.6744 | 92029.7635 | 50.8268 | 51.0146 | 51.7201 | 94143.5228 |
| 16 | 47.0867 | 48.1071 | 48.9782 | 68398.7197 | 47.3772 | 48.203 | 49.1266 | 69864.8515 |
| 20 | 43.7686 | 45.6112 | 46.7924 | 48184.7906 | 43.8841 | 45.5741 | 46.7826 | 49255.9398 |
| 24 | 40.7006 | 42.9801 | 44.2743 | 33002.7913 | 40.8525 | 43.0105 | 44.32 | 33709.5621 |
| 28 | 37.9788 | 40.7076 | 42.1186 | 22283.6077 | 37.9599 | 40.6591 | 42.09 | 22761.4070 |
| 32 | 35.1378 | 39.0489 | 40.5488 | 14486.6139 | 35.1682 | 38.9417 | 40.4407 | 14925.4873 |
| 36 | 32.4607 | 37.2608 | 38.7977 | 9169.7760 | 32.5631 | 37.107 | 38.6615 | 9535.8563 |

Table 2 ∆PSNR and ∆bitrate

| sequence | QP:8,12,16,20 |  |  | QP:24,28,32,36 |  |
| --- | --- | --- | --- | --- | --- |
|  | ∆PSNR(dB) | ∆bitrate(%) |  | ∆PSNR(dB) | ∆bitrate(%) |
| BUS_CIF | 0.392 | -2.50 |  | -0.005 | 0.12 |
| FOOTBALL_CIF | 0.450 | -3.71 |  | -0.134 | 2.54 |
| FOREMAN_CIF | 0.340 | -2.99 |  | -0.046 | 0.92 |
| MOBILE_CIF | 0.359 | -2.04 |  | 0.036 | -0.30 |
| CITY_4CIF | 0.310 | -2.66 |  | -0.147 | 2.57 |
| CREW_4CIF | 0.068 | -0.38 |  | -0.205 | 5.35 |
| HARBOUR_4CIF | 0.583 | -5.24 |  | -0.056 | 0.80 |
| SOCCER_4CIF | 0.173 | -2.15 |  | -0.273 | 6.13 |
| HARBOUR_HD | 0.352 | -3.21 |  | -0.143 | 2.17 |
| NIGHT_HD | 0.148 | -1.56 |  | -0.127 | 2.09 |
| average | 0.318 | -2.64 |  | -0.11 | 2.24 |

## 6. References

- “Description of Core Experiment in SVC,” ISO/IEC JTC1/SC29/WG11 doc. no. N6898, Hongkong, January 2005.

- P. Yin, J. Boyce, P. Pandit, A.M. Tourapis, “Complexity Scalable Video Codec”, ISO/IEC JTC1/SC29/WG11 doc. no. m11241, Palma, October 2004.

- P. Yin, J. Boyce, P. Pandit, A.M. Tourapis, “Technical Description of the Thomson Proposal for SVC CE4”, ISO/IEC JTC1/SC29/WG11 doc. no. m11682, Hongkong, January 2005.

- J. Reichel, M. Wien and H. Schwarz, “Joint Scalable Video Model JSVM1”, JVT-N023, Hongkong, China, January 2005.

- P. Yin, J. Boyce, P. Purvin, A.M. Tourapis, “Complexity Scalable Video Coding”, ITU-T Q6/SG16 VCEG-X06.doc, Palma, October 2004

Figure 1 RD plot of PSNR and bitrate

(Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Thomson Inc. |  |  |
| Mailing address | 2 Independence Way Princeton, NJ 08540 |  |  |
| Country | U.S.A |  |  |
| Contact person | Peng Yin |  |  |
| Telephone | +1 609 987 7355 |  |  |
| Fax | +1 609 987 7299 |  |  |
| Email | peng.yin@thomson.net |  |  |
| Place and date of submission | Busan, Korean April 16, 2005 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT-O053 |  |
| Title |  | Technical description of the Thomson proposal for SVC CE7 - spatial intra prediction on enhancement layer residue |  |
| Contribution number |  | JVT-O053 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
| x | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: