| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 15th Meeting: Busan, Korea, 18-22 April, 2005 | Document: JVT-O054 Filename: JVT-O054.doc |
| --- | --- |

| Title: | Improvements to Fine Granularity Scalability for Low-Delay Applications |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yiliang Bao, Marta Karczewicz, Justin Ridge, Xianglin Wang Nokia Research Center 6000 Connection Drive Irving, Texas 75039, USA | Tel: Email: | +1 972 374 1369 yiliang.bao@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Introduction

For real-time video communication applications, such as video conferencing, minimizing end-to-end delay is very critical to ensure good interaction among the participants. The low-delay requirement is usually met by encoding each video frame as either I-frames or P-frame.

JSVM could be configured to generate a video stream meeting the low-delay requirement using one of the following 2 prediction structures. The first prediction structure shown in Figure 1 uses only the normal P frames encoded in closeloop. In the second prediction structure shown in Figure 2, the sequence is encoded in groups of pictures. The anchor frames are encoded as close-loop P frames to prevent long-term drift, and the frames between 2 anchor frames are encoded as P frames in open-loop.

Figure 1 Normal close-loop P-frame encoding

Figure 2 Open-loop P-frame encoding in JSVM1.0

Fine granularity scalability is an important feature that can be used in video communications since it could make channel adaptation much easier.  Currently JSVM1.0 encodes the FGS layer of an anchor frame using the prediction only from the base layer reconstructed frame because JSVM1.0 is not capable of exploiting the temporal redundancy in the FGS layer with effective control on the drift.  The FGS layer coding performance for such a P frame is very poor.  For an open-loop P frame, FGS coding performance is less an issue since the temporal correlation is removed by using the original reference frame and the drift of open-loop P frames is confined within a GOP.  However, to maximize the overall coding efficiency for such a prediction structure, the majority of bits are spent on encoding the anchor frames. The ratio between the size of the anchor frame and the size of an open-loop P frame can be larger than 10 even when the GOP length is as small as 4. The excessive buffer delay on transmitting the anchor frame could easily result in the skipping of the following frames. So this structure is not very suitable for low-delay application. It is to be tested whether the coding performance of this prediction structure is satisfactory if some frame-level rate control is performed to reduce the frame-to-frame variation.

We propose the improvements to the FGS coder in JSVM1.0 so that the FGS layer of anchor frames can be efficiently encoded. It will especially benefit the low-delay prediction structure in Figure 1.

# Improvements to JSVM1.0 FGS coder

## FGS coding with adaptive reference block formation

In the JSVM1.0 coder, FGS layer of an anchor P-frame is predicted only from the base layer of the current frame. In order to increase coding efficiency, we propose to predict FGS layers also from the enhancement layers of the reference frames.

Since the FGS stream can be arbitrarily truncated, very often the decoder will use a different reference frame from the one used by encoder. The mismatch in the reference frames could cause the accumulation of error, and result in drift.

The drift can be put under control if the accumulated error is bounded. Leaky prediction is an effective method to achieve that. In this proposal, the coefficients to be encoded in the enhancement layer are classified, and different leaky factors are used for coefficients in different categories. If a coefficient is more likely to become nonzero in the enhancement layer, the coefficient should be predicted with more weight from the base layer. The error introduced by the removal of this coefficient when bitstream is truncated will be added to an error in the reference frame that is already reduced. And statistically the errors are balanced across the frame and overall performance is improved.

Figure 3 Formation of reference block for prediction in the new FGS coder

An adaptive reference block formation algorithm is developed based on the reasoning presented above. In this algorithm, the coefficients in the enhancement layer are classified into 3 categories with increasing probability of being a nonzero in the FGS layer.

- The coefficient in a block that does not have any nonzero coefficients in the base layer,

- The coefficient in a block that has some nonzero coefficients in the base layer, but the coefficient itself is coded in the base layer as zero,

- The coefficient in a block that has some nonzero coefficients in the base layer, in addition, the coefficient has already been coded as a non-zero value in the base layer.

Using one of 2 methods described below forms the reference block for a block coded in the FGS layer.

- If all the quantized coefficients in the base layer block are zero, the block-based reference block formation method is used. The actual reference block used in FGS layer coding is calculated as the weighted average of base layer reconstructed block and the enhancement layer reference block. Ra(i,j)=α⋅Xb(i,j)+(1−α)⋅Re(i,j), 0≤i<N, 0≤j<N

- Otherwise, the coefficient-based reference block formation method is used. Firstly transform is performed on both the base layer reconstructed block Xb and enhancement layer reference block Re to obtain coefficient blocks FXb and FRe. A coefficient in the actual reference block, FRa(u,v), is set to be equal to the coefficient of the same frequency in the base layer reconstructed block if the quantized coefficient Qb(u,v) in the base layer is nonzero, otherwise it is set to be equal to the weighted average of the coefficient of the same frequency in the base layer reconstructed block and the coefficient of the same frequency in the enhancement layer reference block. After all the coefficients are calculated, inverse transform is performed on the coefficient block FRa to get the reference block Ra in the spatial domain. Here are the equations that describe the process of forming the coefficient block.

FRa(u,v)=FXb(u,v),                                            	if Qb(u,v)≠0

FRa(u,v)=β⋅FXb(u,v)+(1−β)⋅FRe(u,v), 		if Qb(u,v)=0

0≤u<N, 0≤v<N

Once the new reference block is formed, it replaces the base layer reconstructed block as the prediction in the FGS coding. No modifications to the FGS entropy coder are needed.

It is also proposed to signal the weighting factors base_ref_weight_for_zero_base_block and base_ref_weight_for_zero_base_coeff in the slice header of FGS slice. base_ref_weight_for_zero_base_block corresponds to α in block-based reference block formation method, and base_ref_weight_for_zero_base_coeff corresponds to β in coefficient-based reference block formation method. It is found that the optimal weighting factors are sequence dependent. These weighting factors can also be used for controlling the refreshing of the FGS layer. A larger weighting factor means more prediction from the base layer. Such a stream can recover faster from loss of FGS packets.

## Reduce the drift in discrete layer due to the partial FGS decoding

If a discrete enhancement layer such as spatial enhancement layer is coded on top of the FGS layer as illustrated in Figure 4, the enhancement layer can still be decoded even the FGS layer in the middle is only partially decoded, but the reconstructed enhancement layer will have drift. This drift can be greatly reduced if the reconstructed residual from the FGS layer is not used in the residual prediction of the enhancement layer. In this case, the residual of the enhancement layer is predicted directly from the residual reconstructed of the base layer.

Figure 4 Formation of reference block for prediction in the new FGS coder

# Proposed syntax modifications

## Change to the slice header in scalable extension

| slice_header_in_scalable_extension( ) { | C | Descriptor |
| --- | --- | --- |
| …… |  |  |
| if( slice_type != Progressive_Refinement ) |  |  |
| if( num_slice_groups_minus1 > 0 && slice_group_map_type >= 3 && slice_group_map_type <= 5) |  |  |
| slice_group_change_cycle | 2 | u(v) |
| if( slice_type == Progressive_Refinement ) { |  |  |
| base_ref_weight_for_zero_base_block |  | ue(v) |
| base_ref_weight_for_zero_base_coeff |  | ue(v) |
| fgs_used_in_residual_prediction |  | u(1) |
| } |  |  |
| } |  |  |

## Semantics

base_ref_weight_for_zero_base_block is the weighting factor to be applied to base layer reconstructed block in the block-based reference block formation method. base_ref_weight_for_zero_base_block is in the range [0, 32] with 32 equivalent to 1.0.

base_ref_weight_for_zero_base_coeff is the weighting factor to be applied to the coefficient from the base layer reconstructed block in the coefficient-based reference block formation method. base_ref_weight_for_zero_base_coeff is in the range [0, 16] with 16 equivalent to 1.0.

fgs_used_in_residual_prediction is a flag that indicates whether the residual reconstructed from this FGS layer will be used in predicting the upper discrete enhancement layer. If the flag is 0, the reconstructed residual from this FGS layer is not added to base layer reconstructed residual to use in the residual prediction of the upper discrete enhancement layer.

# Results

Attached excel file JVT-O054.xls has 3 sets of simulation results to demonstrate the effectiveness of the adaptive reference block formation algorithm. The first set of results is generated using modified JSVM1.0 with the adaptive reference block formation and prediction structure in Figure 1 is used. The second set of results is generated using the original JSVM1.0 with the prediction structure in Figure 1. The third set of results is generated using the original JSVM1.0 with the prediction structure in Figure 2. The length of GOP is set to 8. Some adjustment on the slice qp values is performed to reduce the variation of the compressed frame size to be suitable for low-delay applications. Frame size variation of bitstreams generated by using the proposed method and JSVM1.0 with the prediction structure in Figure 2 is given.

The coding performance with the new algorithm is significantly better than JSVM1.0 with either configuration. The proposed method has much smaller variations of bits per frame.

# Conclusions

Compared to JSVM1.0 with either low-delay prediction structure, the new FGS coder with the normal close-loop P frames has significantly better performance. We propose that the method is included in the next release of JSVM.

# References

[1]. Julien Reichel, Heiko Schwarz, and Mathias Wien, “Working Draft 1.0 of 14496-10:200x/AMD1 Scalable Video Coding”, ISO/IEC JTC 1/SC 29/WG 11 N6901, January, 2005.

(Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Nokia Corporation |  |  |
| Mailing address | 6000 Connection Drive |  |  |
| Country | USA |  |  |
| Contact person | Yiliang Bao |  |  |
| Telephone | 972-374-1369 |  |  |
| Fax |  |  |  |
| Email | Yiliang.bao@nokia.com |  |  |
| Place and date of submission | Busan, Korea, 18-22 April, 2005 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | ITU-T Rec. H.264 ¦ ISO/IEC 14496-10 |  |
| Title |  | Improvements to FGS for low-delay applications |  |
| Contribution number |  | JVT-O054 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
| X | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: