| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 72th Meeting: Busan, Korea, 18-22 Apr, 2005 | Document: JVT-O055 Filename: JVT-O055.doc |
| --- | --- |

| Title: | FGS coding efficiency enhancements |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Justin Ridge, Marta Karczewicz, Yiliang Bao, Xianglin Wang Nokia Research Center 6000 Connection Drive Irving, TX 75039, USA | Tel: Email: | +1 972 374 0628 justin.ridge@nokia.com |
| Source: | Nokia Inc. |  |  |

_____________________________

# Introduction

The fine granularity scalability (FGS) algorithm in the current JSVM codes coefficients within an FGS slice in an order such that “more significant” coefficients are coded first.  By arranging the bit stream in this way, the extraction process is biased so that a simple truncation is likely to retain those “more significant” coefficients, and therefore improve the reconstructed quality.

Building upon this idea, in this proposal we introduce two modifications that can further improve the FGS coding efficiency.

# Block ordering within a coding cycle

When FGS data is truncated, the decoder assumes the missing values to be zero.  Consequently, coding zero values into the bit stream contributes nothing to the reconstruction, and coefficients with the greatest probability of being zero should be deferred until the end of the slice.  Conversely, coefficients with the greatest probability of being non-zero should be coded first.

In cyclical block coding, unlike subband coding, the current scan position in a given coding pass will differ from one block to another.  Furthermore, a correlation was observed between scan position and the probability of the next coefficient being non-zero.  The function describing this correlation varies according to QP and sequence content.

Our proposal exploits this correlation by grouping blocks according to their scan position (i.e. 16 groups for a 4x4 block) then coding the groups in decreasing order of the probability that the next coefficient is nonzero (nnz).  We base the nnz probability on the previous FGS slice, but other means of calculation could be investigated.

## Complexity, memory considerations

This method requires a very small amount of additional memory for measuring the number of nonzero coefficients at each of 16 scan positions in the previous slice, and for storing the order of scan positions to be processed in the current slice.  This modification requires some additional memory access operations.

Ordering effectively involves 16 sub-passes for each cycle.  For each scan position, blocks are processed in raster order, as with the current FGS algorithm.  In terms of complexity, ordering the blocks involves sorting a vector of fixed size 16; the vector size is not based on sequence resolution.

# Temporal level-based truncation

Varying the QP setting according to temporal level is the key to achieving good coding efficiency with MCTF – causing both high and low pass pictures to have the same QP value is known to dramatically reduce performance.

The current JSVM bit stream extractor truncates all FGS slices proportionally, reducing the difference in effective quality between temporal levels, and ultimately reduces coding efficiency.

Rather than using a fixed truncation ratio (or “bit plane fraction”) when extracting, this proposal uses a different truncation ratio for each temporal level.

The simplest modification is to supply the truncation ratio independently for each temporal level at the time of extraction, just as the overall truncation ratio is now specified.  However, this complicates the extraction/decoding process.

A better approach is to supply the overall truncation ratio to the extractor (as in the current JSVM) and use it to scale a vector of temporal truncation ratios.  For example, if the supplied ratio is y then the scaling vector might be {0.4y, 0.5y, 0.6y, 0.75y, 1y 1.2y}, where 0.4y is the truncation ratio used for the first temporal level and so on.  This would retain the simplicity of the current extractor/decoder, and is non-normative.

While varying the truncation ratio according to temporal level can improve performance, selection of the scaling vector, {0.4, 0.5, 0.6, 0.75, 1, 1.2} in the above example, is not straightforward.  The optimal vector is both sequence-dependant and QP-dependant.  We propose creating a set of allowable vectors and signaling the one to be used for a given layer in the SPS.  A small set of between 4-8 vectors (requiring 3 bits in the SPS) is sufficient.

## Complexity, memory considerations

This proposal requires no additional memory in either encoder or decoder.

Any effect on complexity is negligible.  Selection of the optimal scaling vector by the encoder is considered analogous to rate control, and would be non-normative.

## Proposed syntax changes

We propose adding two bits in the NAL unit header indicating the truncation scaling vector to be used:

| nal_unit( NumBytesInNALunit ) { | C | Descriptor |
| --- | --- | --- |
| …forbidden_zero_bit | All | f(1) |
| nal_ref_idc | All | u(2) |
| nal_unit_type | All | u(5) |
| nalUnitHeaderBytes = 1 |  |  |
| if( nal_unit_type = = 14 ¦ ¦ nal_unit_type = = 15 ¦ ¦ nal_unit_type = = 20 ¦ ¦ nal_unit_type = = 21 ) { |  |  |
| decodability_dependency_information | All | u(6) |
| temporal_scaling_vec_idx | All | u(2) |
| nalUnitHeaderBytes++ |  |  |
| } |  |  |
| … |  |  |
| } |  |  |

temporal_scaling_vec_idx selects between proposed truncation scaling vectors:

| Value | Vector |
| --- | --- |
| 0 | {0.2, 0.4, 0.75, 1.1, 1.5} |
| 1 | {0.3, 0.5, 1.2, 1.2, 1.2} |
| 2 | {0.7, 0.8, 0.9, 1.1, 1.5} |
| 3 | {1, 1, 1, 1, 1} |

# Results

Software used to generate our results is attached.  Note that currently, only luminance blocks are re-ordered and only luminance results are presented.  We would expect a similar gain for chroma.

The attached spreadsheet JVT-O055.xls demonstrates the coding efficiency gain achievable with this proposal.  As with all ordering-based proposals, the “end points” of the FGS layer are unchanged – the goal is to improve performance in between.  In this case, the optimal weighting vector was determined by hand and not implemented in the encoder.  As noted previously, this is merely a syntax issue and does not affect the FGS decoding algorithm.

Bit rate savings for the same PSNR were up to 6% (e.g. Bus QCIF 15 Hz from 89 to 83 kbps), which roughly translates into a gain of up to 0.3 dB.  To reiterate, this gain should not be expected to occur at all points in the FGS layer, and was not as profound on all sequences.  However, the performance was always improved compared to the current JSVM.  Given the minor nature of the modifications, we believe that they are justified given the observed gain.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Nokia Inc. |  |  |
| Mailing address | 6000 Connection Drive, Irving, TX 75039 USA |  |  |
| Country | USA |  |  |
| Contact person | Justin Ridge |  |  |
| Telephone | +1 972 374 0628 |  |  |
| Fax |  |  |  |
| Email | justin.ridge@nokia.com |  |  |
| Place and date of submission | Busan, Korea, 18/4/2005 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | FGS coding efficiency enhancements |  |
| Contribution number |  | JVT-O055 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: