| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 15th Meeting: Busan, KR, 16-22 April, 2005 | Document: JVT-O064 Filename: input.doc NOTE – That first character after the dash up there in the document number is an "O", not a zero. |
| --- | --- |

| Title: | Adaptive quantization techniques for SVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sangchang Cha Woo-Jin Han | Tel: Email: | 82-31-200-9339 micro21@samsung.com wjhan.han@samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

- Motivation

- Context-based Adaptive Binary Arithmetic Coding (CABAC) method is part of the main profile of H.264/AVC and JSVM also uses it as a quantization method. CABAC is based on the key elements of binarization, context modeling, and binary arithmetic coding. The basic self-contained unit in H.264/AVC and SVC is a slice. All models have to be re-initialized at the slice boundaries using some pre-defined probability states. This re-initialization process can be the reason of the performance degradation in MCTF-based SVC. We suggest an adaptive re-initialization method to improve the overall performance of the CABAC.

- Adaptive quantization techniques for SVC

- Our proposal is based on following two facts.

- 1. At the very first stage of a slice, the performance of CABAC is lower than the latter stages due to the insufficient accumulation of the symbol.

- 2. A MCTF-based video codec is different from a multiple reference coder such as the H.264/AVC. It uses the specific frames as the reference owing to keep the MCTF structure. Therefore, we can predict the propagation of the bit errors and restrict them.

- We can obtain performance gain by providing better probability model at the first stage of a slice and restricting the error propagation in temporally localized frames. In the specific condition such as an error-free network or storage applications, we are able to re-initialize CABAC model states with previous model states.

  - Multi-layer CABAC re-initialization.

- We can use the base-layer CABAC states as the re-initialization information. Re-initialization CABAC with base-layer model states is shown in Figure 1. With this method, we can obtain a more reasonable probability model at the very first part of a slice.

Figure 1 Re-initialization CABAC with base-layer model states.

  - Inter-temporal level CABAC re-initialization.

- Figure 2 shows the re-initialization of the inter-temporal level slices. In the error-free network or local storage application, the reuse of the low temporal level CABAC probability model states can show the better prediction performances. In the Figure 2, each slice can be re-initialized with the nearest lower temporal level CABAC states, previous states of same temporal level frame, or corresponding base-layer’s adaptively. Each slice has the specific prediction mode bits for CABAC re-initialization.

Figure 2  Inter-temporal level re-initialization

- Syntax, Semantics and Decoding Process

  - Syntax in tabular form

| slice_header_in_scalable_extension( ) { | C | Descriptor |
| --- | --- | --- |
| first_mb_in_slice | 2 | ue(v) |
| slice_type | 2 | ue(v) |
| pic_parameter_set_id | 2 | ue(v) |
| frame_num | 2 | u(v) |
| if( !frame_mbs_only_flag ) { |  |  |
| field_pic_flag | 2 | u(1) |
| if( field_pic_flag ) |  |  |
| bottom_field_flag | 2 | u(1) |
| } |  |  |
| if( nal_unit_type = = 21 ) |  |  |
| idr_pic_id | 2 | ue(v) |
| if( pic_order_cnt_type = = 0 ) { |  |  |
| pic_order_cnt_lsb | 2 | u(v) |
| if( pic_order_present_flag && !field_pic_flag ) |  |  |
| delta_pic_order_cnt_bottom | 2 | se(v) |
| } |  |  |
| if( pic_order_cnt_type = = 1 && !delta_pic_order_always_zero_flag ) { |  |  |
| delta_pic_order_cnt[ 0 ] | 2 | se(v) |
| if( pic_order_present_flag && !field_pic_flag ) |  |  |
| delta_pic_order_cnt[ 1 ] | 2 | se(v) |
| } |  |  |
| if( slice_type != Progressive_Refinement ) { |  |  |
| if( redundant_pic_cnt_present_flag ) |  |  |
| redundant_pic_cnt | 2 | ue(v) |
| if( slice_type = = BE ) |  |  |
| direct_spatial_mv_pred_flag | 2 | u(1) |
| gop_id | 2 | u(4) |
| gop_size | 2 | ue(v) |
| decomposition_stages | 2 | ue(v) |
| picture_id_inside_gop | 2 | ue(v) |
| base_id_plus1 | 2 | ue(v) |
| if( base_id_plus1 != 0 ) |  |  |
| adaptive_prediction_flag | 2 | u(1) |
| if( slice_type = = PE ¦ ¦ slice_type = = BE ) { |  |  |
| num_ref_idx_active_override_flag | 2 | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |  |
| num_ref_idx_l0_active_minus1 | 2 | ue(v) |
| if( slice_type = = B ) |  |  |
| num_ref_idx_l1_active_minus1 | 2 | ue(v) |
| } |  |  |
| } |  |  |
| ref_pic_list_reordering( ) | 2 |  |
| for( decLvl = TemporalLevel; decLvl < decomposition_stages; decLvl++ ) { |  |  |
| num_ref_idx_update_l0_active[ decLvl + 1 ] | 2 | ue(v) |
| num_ref_idx_update_l1_active[ decLvl + 1 ] | 2 | ue(v) |
| } |  |  |
| if( ( weighted_pred_flag && slice_type = = PE ) ¦ ¦ ( weighted_bipred_idc = = 1 && slice_type = = BE ) ) |  |  |
| pred_weight_table( ) | 2 |  |
| if( nal_ref_idc != 0 ) |  |  |
| dec_ref_pic_marking( ) | 2 |  |
| if( entropy_coding_mode_flag && slice_type != IE ) { |  |  |
| cabac_init_idc | 2 | ue(v) |
| if( cabac_init_idc == 3) { |  |  |
| cabac_prediction_mode | 2 | ue(v) |
| } |  |  |
| } |  |  |
| } |  |  |
| slice_qp_delta | 2 | se(v) |
| if( deblocking_filter_control_present_flag ) { |  |  |
| disable_deblocking_filter_idc | 2 | ue(v) |
| if( disable_deblocking_filter_idc != 1 ) { |  |  |
| slice_alpha_c0_offset_div2 | 2 | se(v) |
| slice_beta_offset_div2 | 2 | se(v) |
| } |  |  |
| } |  |  |
| if( slice_type != Progressive_Refinement ) |  |  |
| if( num_slice_groups_minus1 > 0 && slice_group_map_type >= 3 && slice_group_map_type <= 5) |  |  |
| slice_group_change_cycle | 2 | u(v) |
| } |  |  |

  - Semantics

- cabac_init_idc specifies the index for determining the initialisation table used in the initialisation process for context variables. The value of cabac_init_idc shall be in the range of 0 to 3, inclusive. cabac_init_idc equal to 3 specifies that CABAC model states are updated with previous generated model states decided by cabac_prediction_mode.

- cabac_prediction_mode specifies a parameter that is used for identifying which CABAC context model states is used for re-initialization. cabac_prediction_mode equal to 1 specifies corresponding base-layer model states, 2 specifies previous states of same temporal level frame, 3 specifies model states of nearest frame in the lower temporal level frames.

- Experimental results

- One of proposed algorithm which uses only previous CABAC model states with decoding order for re-initializing B frame is illustrated in Figure 3. The proposed algorithm is integrated into JSVM 1.0 software [1] and the experimental results are compared to that of JSVM 1.0. As you can see in the experimental results, almost CIF test points showed stable performance improvements. The detailed experimental results are illustrated as follows. Munich test points are used for experiment.

Figure 3 The simplest re-initialization method for SVC

Figure 4 Bus

Figure 5 Foreman

Figure 6 Football

Figure 7 Mobile

- Conclusion

- We tested the basic idea of the CABAC re-initialization with previous model states not pre-defined model states. Though simplified method which uses CABAC model states of just previous B frame is adopted with decoding order, the meaningful results are obtained. There can be various update method of CABAC model with acceptable robustness and provide a better probability model.

- References

- [1] JVT-N024, “JSVM 1.0 software,” Hong Kong, Jan., 2005.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name |  |  |  |
| Mailing address |  |  |  |
| Country |  |  |  |
| Contact person |  |  |  |
| Telephone |  |  |  |
| Fax |  |  |  |
| Email |  |  |  |
| Place and date of submission |  |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  |  |  |
| Title |  |  |  |
| Contribution number |  |  |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: