| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 15th Meeting: Busan, KR, 16-22 April, 2005 | Document: JVT-O065 Filename: JVT-O065.doc NOTE – That first character after the dash up there in the document number is an "O", not a zero. |
| --- | --- |

| Title: | Improved IntraBL coding for SVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Woo-Jin Han Yeongtong-Gu Suwon-City Gyeonggi- Do South Korea Digital Media R&D Center Samsung Electronics Co., Ltd. | Email: Tel: | wjhan.han@samsung.com +82 31 200 9319 |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# 1. Problem with IntraBL coding in JSVM 1.0.

Currently, the residual signal of IntraBL macroblock in JSVM 1.0 is coded by 4x4 integer transform, which is also used for inter-macroblocks. It provides better performance compared to the quantization method used for Intra16x16 mode because each subblock may have very different statistical properties. However, in many IntraBL macroblocks, residual signal of each subblock in IntraBL mode has notable statistical similarities, such as DC values. It is natural phenomenon since the residual signal of IntraBL macroblock is generated by subtracting the base-layer image from the original image. It is very similar to the case that the reference frame is the quantized original and the all motion vectors are zeros. Surely, it generates a statistically 'flat' signal. That's why we think that Hadamard transform used in Intra16x16 mode may be useful for quantizing residuals in IntraBL mode.

# 2. Proposal for New IntraBL coding

Our proposal is to change the quantization process of IntraBL mode from one used in inter-modes to one used in Intra16x16 mode to utilize the Hadamard transform to reduce the similarities existing in each subblock. Let’s name the current quantization process for IntraBL mode in JSVM 1.0 as 4x4T_J and the modified quantization process for IntraBL mode as 16x16T_A. It should be noted that the 16x16T_A is very similar to the Intra16x16 quantization process except the prediction is generated from the base-layer, not the neighbouring pixels. Actually, we just replaces the 4x4 integer transform part by the Hadamard transform of 16 DC components followed by the 4x4 integer transform as Intra16x16 quantization process.

Figure 1 shows a structure of 4x4 Hadamard transform of 16x16T_A for encoding DC elements of luma component. DC elements of chroma components are also coded separately from AC elements through 2x2 Hadamard transform.

Figure 1 4x4 Hadamard transform of Intra16x16 mode

## 2.2. Coding of IntraBL with adaptive selection between 4x4T_J and 16x16T_A

From our experimental results, when the resolution is not changed between layers, 16x16T_A shows a better performance compared to the 4x4T_J, however, in the other case, 4x4T_J provides slightly better performance. Therefore, an adaptive approach can be taken. Our proposal covers both slice-header based adaptive selection between 16x16T_A and 4x4T_J, and macroblock-based adaptive selection although 16x16T_A was always used when the resolution is not changed, whereas 4x4T_J was always used for other cases to generate the experimental results. For the future works, we will try to the macroblock-based adaptive selection.

# 3. Experimental results

We used 8 test sequences using Munich test condition, however, some test-points of Harbour and Soccer sequence cannot be included due to the time limit. We will provide full results in the presentation material. As you can see, some sequences including significant number of IntraBL modes, such as Football and Crew, clear performance improvements can be observed. It is interesting to see the performance improvement in Mobile sequence, which has a very steady motion fields, except the red ball areas.

Figure 2 PSNR curves of BUS sequence.

Figure 3 PSNR curves of Foreman sequence.

Figure 4 PSNR curves of Football sequence.

Figure 5 PSNR curves of Mobile sequence

Figure 6 PSNR curves of CITY sequence.

Figure 7 PSNR curves of CREW sequence

Figure 8 PSNR curves of Harbour sequence.

Figure 9 PSNR curves of Soccer sequence.

# 4. Conclusion

The experimental results show that the proposed quantization process for IntraBL mode is efficient to improve the coding efficiency, especially for the sequences having many IntraBL macroblocks. Even for other sequences, such as Mobile, some performance gain can be observed.

(Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name |  |  |  |
| Mailing address |  |  |  |
| Country |  |  |  |
| Contact person |  |  |  |
| Telephone |  |  |  |
| Fax |  |  |  |
| Email |  |  |  |
| Place and date of submission |  |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  |  |  |
| Title |  |  |  |
| Contribution number |  |  |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: