| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 15th Meeting: Busan, KR, 16-22 April, 2005 | Document: JVT-O069 Filename: input.doc NOTE – That first character after the dash up there in the document number is an "O", not a zero. |
| --- | --- |

| Title: | Adaptive Quantization Offset |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Peter Amon, Michael Kapralov Siemens AG CT IC 2 Otto-Hahn-Ring 6 81730 Munich | Tel: Email: | +49 (0)89 636 54642 p.amon@siemens.com |
| Source: | Siemens AG |  |  |

_____________________________

# Introduction

This document proposes to introduce an adaptive quantization offset in scalable video coding (SVC), which is dependent on the quantization step size and on the coefficient value. By defining different classes for the coefficients, the specific characteristics of these different coefficient classes are exploited.

# Quantization in JSVM 1.0

In JSVM 1.0 [1,2] the quantization of the magnitude of a transform coefficient Xk≥0 is done as expressed in the following equation:

Xk=Xk+Δ⋅QQ, where

Xkis the unquantized coefficient,

Xkis the level representing the quantized value,

Qis the step size of the quantizer,

k is the index des unquantized/quantized coefficient and

Δ is the quantization offset

The current JSVM (version 1.0) uses the quantization offsets

ΔINTRA=13 and ΔINTER=ΔRESUDIAL=16.

Inverse quantization (‘dequantization’) is done by

Yk=Xk⋅Q, where

Yk is the reconstructed coefficient.

# Proposal for Adaptive Quantization Offset

The proposal consist of

- the definition of coefficient classes,

- the adaptivity for the quantization step size and

- the additional adaptivity for the coefficient value.

## Context Adaptive Context (Coefficient Classes)

The following 14 classes were used for the adaptive quantization as defined in the subsequent sections.

- INTRA DC 4x4,

- INTRA AC 4x4,

- INTRA DC 8x8,

- INTRA AC 8x8,

- INTRA AC 16x16,

- INTER DC 4x4,

- INTER AC 4x4,

- INTER DC 8x8,

- INTER AC 8x8,

- INTER AC 16x16,

- RESIDUAL DC 4x4,

- RESIDUAL AC 4x4.

- RESIDUAL DC 8x8,

- RESIDUAL AC 8x8.

For other combinations, the quantization was not modified.

## Dependency on Quantization Step Size

At encoder and decoder side, a standard distribution is assumed (Laplacian distribution). The encoder then measures the real distribution for a specific amount of coefficients (e.g. for a group of pictures (GOP)). (In low delay mode, the adaptation of the quantization is done e.g. for the coefficients of one frame.) The difference between assumed and measured distribution is transmitted in terms of additional quantization offset. This quantization offset is then dependent on the step size Q and therefore also on the decoded bit-rate.

The assumed Laplacian distribution is given by its probability density function

p(x)=λ2e−λx.

Figure 1 shows a Laplacian distribution as it can be regarded as an approximation for the distribution for DCT coefficients, especially AC coefficients [3]. This figure shows that the probability for smaller coefficients is higher.

Figure 1: Laplacian distribution

The discrete version of the Laplacian distribution is the geometric distribution given by the probability distribution

pk=1−q1+qqk.

In the algorithm we use the one-sided version for the magnitude of the coefficients:

pk=(1−q)qk.

From this distribution, an optimal quantization offset independent on the value being dequantized can be analytically derived depending on the quantization step size, see Figure 2. This figure also shows the optimal offset measured for a real sequence, revealing a systematic gap between the two curves.

Figure 2: Optimal offset for Laplacian distribution and a real sequence

Figure 3 shows a characteristic function for the normalized difference between optimal offset of Laplacian distribution and real distribution depending on the quantization step size. This characteristic gap is transmitted to the encoder.

Figure 3: Normalized difference between optimal offset of Laplacian distribution and real distribution; linear interpolation

Figure 3 also shows a linear approximation of this systematic difference for the quantization offset. By transmitting the parameters for this characteristic curve, a improvement for the coding efficiency can be achieved. The parameters can be abbreviated by

v=v1,v2,...vl.

This estimation is done for the 14 classes of coefficients described above.

The algorithm is described in more detail in the annex.

## Additional Dependency on Quantized Value

It has been noted above that the experimental distribution cannot be perfectly approximated by the geometric distribution. The optimal shift parameter is in fact dependent on the value being dequantized. Thus, the above approach cannot capture the optimal value-dependent dequantization offset fully. Figure 4 shows the dependency of the optimized quantization offset on the coefficient valueXk (and Xkrespectively) for four different quantization step sizes Q.

Figure 4: Dependency of optimized quantization offset on coefficient value and quantization step size

To compensate for this, the dependency of the coefficient value is explicitly respected. This can be done by modeling the measurements in Figure 4.

Therefore a fixed amount of reference points is defined for different quantization step sizes Q and coefficient values X. This can be expressed by

Δ(Q,X) for Q∈Q0,Q1,...,Qmand X∈X0,X1,...,Xm.

By using a linear transformation

Δ(Q,X)=AQv+dQfor Q∈Q0,Q1,...,Qm and X=(X0,X1,...,Xm)T

a parameter set

v=(v,vl+1,vl+2,...vl+r)

is generated that optimizes the quantization offset. (r=1 in this proposal. So only one additional parameter is used in the algorithm.) This parameter set is transmitted to the encoder for the 14 classes defined above. The parameters for the linear transform (matrix AQand vectordQ) are generic and are fixed for encoder and decoder. These parameters depend on the quantization step size Q and are defined for all reference points Q∈Q0,Q1,...,Qm.

We used m=7 in our implementation.

By using a linear interpolation the optimal offset Δ(Q,X) is generated for all values of Q and X(respectivelyX).

Also this algorithm is described in more detail in the annex.

Remark: Also the addition of a constant improves the performance. This is given by

Δ(Q,X)=Δ(Q)+c.

# Results

The current results for QCIF are shown in the attached Excel file.

# Conclusions

The introduction of a adaptive quantization offset depending on the quantization step size and the coefficient values reveals a gain in compression efficiency. The dependence on the quantization step size allows an adaptation to different bit-rates in the case of SNR scalability. By defining several classes for the coefficients, also the characteristics of these coefficients are respected.

Therefore, we propose to evaluate the proposal in the context of a core experiment.

# References

[1] ITU-T and ISO/IEC JTC1, JSVM 1 Text, ISO/IEC MPEG und ITU-T VCEG, document JVT-N023, January 2005.

[2] ITU-T and ISO/IEC JTC1, JSVM 1 Software, ISO/IEC MPEG und ITU-T VCEG, document JVT-N024, January 2005.

[3] S. R. Smoot, L. A. Rowe, Study of DCT coefficient distributions, Proc. SPIE, pp. 403-311, January 1996.

[4] Yucel Altunbasak, Nejat Kamaci, An analysis of the DCT distribution with H.264 video coder, IEEE Int. Conf. On Acoustics, Speech and Signal Processing, Montreal, Kanada, May 2004.

# Annex

## Procedure for Dependency on Quantization Step Size

The following procedure is repeated for all types of coefficients:

Input is the experimental distribution pk.

The procedure outputs a vector of approximation parameters (v1,v2,...,v8).

1. Approximate the experimental distribution with the geometric distribution:

pk=(1−q)qk,

producing the optimal parameter qopt by using squared error minimization.

We use geometric distribution instead of Laplacian, because the samples are integer values.

2. Calculate the function Δ(Q):

Δ(Q)=1Ql=0Q−1l(k=0∞pl+kQ) and

Δqopt(Q)=q((Q−1)qQ−QqQ−1+1)Q(1−q)(1−qQ)

3. Approximate the difference with four linear segments as depicted Figure 3. The approximation is assumed to be zero at point 0. The x-coordinate of the joining point between the first and the second line segments is fixed to the values of iBasePoint. This step produces the following output  parameters:

- fNorm - the maximum absolute value of the difference,

- iP1, iP2 - the x-coordinates of the joining points between line segments 2-3 and 3-4,

- β0 - the y-coordinate at point iBasePoint,

- α0,α1,α2 - the slopes of the three line segments.

Thus, process produces a vector of parameters:

(qopt, fNorm, iP1, iP2, β0, α0, α1,α2).

These parameters are transmitted to the decoder and used to produce the appropriate optimal offset for the given quantization step size independently of the value being dequantized.

## Procedure for Additional Dependency on Quantized Value

The procedure described above produces a vector of parameters that characterize the distribution:

(qopt, fNorm, iP1, iP2,β0, α0, α1,α2).

Given a quantization step size Q, the decoder produces the optimal value independent quantization offset Δval.indep.

This value can be used for dequantization if the distribution is close to geometric, but if the deviation from geometric is considerable, it is necessary to correct this value.  The decoder needs to reproduce the curves in Figure 4 from the vector of parameters. This can is done in the following way. A fixed set of quantization step sizes is chosen (5, 20, 50, 100, 200, 300, 399) and for each fixed quantization step size the points with x-coordinates (5, 20, 50, 100, 200, 250) on the corresponding curve is reconstructed. Thus, to obtain the optimal value-dependent quantization offset for an arbitrary quantization step size and value, the decoder needs to do linear interpolation with respect to both variables.

The y-coordinates of a curve for quantization parameter Q are reconstructed as follows. The decoder calculates the optimal value independent offset Δval.indep and forms a vector of parameters

v =(qopt, fNorm, iP1, iP2, β0, α0, α1,α2, Δval.indep).

The y-coordinates of the chosen points on the curve for quantization step size Q are obtained by calculating

y=AQv+dQ

for some fixed matrix AQ of size 6x9 and vector dQ of length 6. Thus, the y-coordinates of the optimal offset curve are obtained by linear prediction from the parameters of the distribution.  The parameters of the prediction (AQ,dQ) are determined for several values of Q by least squares optimization on a set of test sequences.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Siemens AG |  |  |
| Mailing address | Otto-Hahn-Ring 6 71730 Munich |  |  |
| Country | Germany |  |  |
| Contact person | Peter Amon |  |  |
| Telephone | +49 89 636 54642 |  |  |
| Fax | +49 89 636 51115 |  |  |
| Email | p.amon@siemens.com |  |  |
| Place and date of submission | Busan, Korea, April 2005 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | ITU T Rec. H.264 ¦ ISO/IEC 14496 10 |  |
| Title |  | Adaptive Quantization Context |  |
| Contribution number |  | JVT-O069 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
| X | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: