| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 15th Meeting: Busan, KR, 16-22 April, 2005 | Document: JVT-O071 Filename: input.doc NOTE – That first character after the dash up there in the document number is an "O", not a zero. |
| --- | --- |

| Title: | Verification of SVC CE6 – Non-Scalable Motion Vector Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Heiko Schwarz Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49-30-31002-226 heiko.schwarz@hhi.fhg.de |
| Source: | Fraunhofer Institute for Telecommunications – Heinrich Hertz Institute (HHI) |  |  |

_____________________________

### Summary

The Fraunhofer HHI has performed the verification of Thomson’s results for SVC Core Experiment 6 on Non-Scalable Motion Vector Coding.

### Verification process

The supplied bit-streams have been decoded. and the PSNR’s and bit-rates have been calculated using the scripts and binaries provided by Thomson.

### Verification results

The following tables summarize the measured bit-rates and PSNR values. All values are identical to the results that have been reported by Thomson.

Diagrams with rate-distortion plots are contained in the embedded Excel document.

| Sequence: BUS CIF 30Hz (Base layer QCIF 15Hz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Target rate | Y-PSNR [dB] | U-PSNR [dB] | V-PSNR [dB] | rate [bit/s] |
| 256 kbit/s | 25.79049 | 36.99268 | 38.19178 | 251660.8 |
| 320 kbit/s | 26.6251 | 37.39039 | 38.66385 | 319027.2 |
| 384 kbit/s | 27.33762 | 37.42798 | 38.6962 | 387137.6 |

| Sequence: FOOTBALL CIF 30Hz (Base layer QCIF 15Hz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Target rate | Y-PSNR [dB] | U-PSNR [dB] | V-PSNR [dB] | rate [bit/s] |
| 512 kbit/s | 30.01426 | 35.75872 | 38.29027 | 516864.9 |
| 640 kbit/s | 30.91304 | 37.02422 | 39.36203 | 657386.8 |
| 768 kbit/s | 31.43742 | 37.15432 | 39.47769 | 759660.9 |

| Sequence: FOREMAN CIF 30Hz (Base layer QCIF 15Hz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Target rate | Y-PSNR [dB] | U-PSNR [dB] | V-PSNR [dB] | rate [bit/s] |
| 128 kbit/s | 29.65109 | 37.24988 | 37.90762 | 129144 |
| 160 kbit/s | 30.55145 | 37.74666 | 38.55962 | 161384.8 |
| 192 kbit/s | 31.3227 | 38.12905 | 39.04308 | 189758.4 |

| Sequence: MOBILE CIF 30Hz (Base layer QCIF 15Hz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Target rate | Y-PSNR [dB] | U-PSNR [dB] | V-PSNR [dB] | rate [bit/s] |
| 192 kbit/s | 21.10662 | 29.57476 | 28.9637 | 192080.8 |
| 224 kbit/s | 21.71606 | 29.58845 | 28.99879 | 214224.8 |
| 256 kbit/s | 22.72868 | 30.27396 | 29.71722 | 263659.2 |

| Sequence: CITY CIF 30Hz (Base layer QCIF 15Hz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Target rate | Y-PSNR [dB] | U-PSNR [dB] | V-PSNR [dB] | rate [bit/s] |
| 256 kbit/s | 31.07803 | 40.85649 | 42.21528 | 259699.2 |
| 320 kbit/s | 31.84525 | 41.32726 | 42.71033 | 323172.8 |
| 384 kbit/s | 32.59748 | 41.55645 | 43.015 | 380366.4 |

| Sequence: CREW CIF 30Hz (Base layer QCIF 15Hz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Target rate | Y-PSNR [dB] | U-PSNR [dB] | V-PSNR [dB] | rate [bit/s] |
| 384 kbit/s | 33.22196 | 37.73747 | 36.36565 | 380900.8 |
| 448 kbit/s | 33.72995 | 38.19494 | 36.84366 | 460437.6 |
| 512 kbit/s | 34.15769 | 38.26534 | 36.9197 | 513651.2 |

| Sequence: HARBOUR CIF 30Hz (Base layer QCIF 15Hz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Target rate | Y-PSNR [dB] | U-PSNR [dB] | V-PSNR [dB] | rate [bit/s] |
| 384 kbit/s | 27.91077 | 39.07659 | 40.88895 | 387470.4 |
| 448 kbit/s | 28.40845 | 39.37656 | 41.24929 | 444511.2 |
| 512 kbit/s | 28.84713 | 39.47498 | 41.34838 | 502276.8 |

| Sequence: SOCCER CIF 30Hz (Base layer QCIF 15Hz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Target rate | Y-PSNR [dB] | U-PSNR [dB] | V-PSNR [dB] | rate [bit/s] |
| 384 kbit/s | 32.58696 | 40.47493 | 42.18035 | 383288 |
| 448 kbit/s | 33.05036 | 40.56696 | 42.28183 | 441020.8 |
| 512 kbit/s | 33.4695 | 40.97129 | 42.66849 | 505799.2 |

| Sequence: CITY 4CIF 30Hz (Base layer CIF 15Hz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Target rate | Y-PSNR [dB] | U-PSNR [dB] | V-PSNR [dB] | rate [bit/s] |
| 768 kbit/s | 31.05494 | 40.63247 | 42.62321 | 790920.8 |
| 896 kbit/s | 31.82581 | 40.72937 | 42.77333 | 1005594 |
| 1024 kbit/s | 32.22685 | 41.21025 | 43.27036 | 1122190 |

| Sequence: CREW 4CIF 30Hz (Base layer CIF 15Hz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Target rate | Y-PSNR [dB] | U-PSNR [dB] | V-PSNR [dB] | rate [bit/s] |
| 1024 kbit/s | 34.42089 | 39.0249 | 38.71031 | 1050332 |
| 1280 kbit/s | 35.09432 | 39.45165 | 39.24208 | 1321959 |
| 1536 kbit/s | 35.57261 | 39.8617 | 39.78328 | 1572784 |

| Sequence: HARBOUR 4CIF 30Hz (Base layer CIF 15Hz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Target rate | Y-PSNR [dB] | U-PSNR [dB] | V-PSNR [dB] | rate [bit/s] |
| 1024 kbit/s | 29.33344 | 39.75607 | 41.52261 | 1040378 |
| 1280 kbit/s | 29.93974 | 40.12821 | 41.97627 | 1240708 |
| 1536 kbit/s | 30.76774 | 40.65571 | 42.57433 | 1575338 |

| Sequence: SOCCER 4CIF 30Hz (Base layer CIF 15Hz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Target rate | Y-PSNR [dB] | U-PSNR [dB] | V-PSNR [dB] | rate [bit/s] |
| 1024 kbit/s | 33.33098 | 41.21511 | 42.81634 | 1071932 |
| 1280 kbit/s | 33.80188 | 41.68414 | 43.36277 | 1244133 |
| 1536 kbit/s | 34.47762 | 42.06986 | 43.78544 | 1554957 |